3GPP TSG-RAN4 Meeting #112bis	                                                                R4-2417013

Hefei, China, 14th – 18th Oct 2024

Agenda item:			6.10.5

Source:	CMCC

Title:	WF on UE RF for ATG enhancement

Document for:	Approval

Introduction

This document captures the agreements on Rel-19 ATG UE RF requirements in RAN4#112bis.

Topic #1: R19 UE RF for intra-band contiguous CA

Issue 1-1: Whether to specify a separate configuration table for ATG UE

Agreement:

- Since ATG reuse the existing configuration table for ATG UE supporting DL CA_n79C, no separate configuration table is needed for ATG UE CA_n79C.

Issue 1-2: ACS testing parameters for case 2

Agreement:

- Table 3: Test parameters for intra-band contiguous CA with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz, case 2

| Rx Parameter | Units | NR CA bandwidth class |
| --- | --- | --- |
|  |  | C |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | -73.54 -61.55 |
| PInterferer | dBm | -424 -305 |
| BWInterferer | MHz | BWchannel CA |
| FInterferer (offset) | MHz | BWchannel CA / -BWchannel CA |
| NOTE 1: The transmitter shall be set to 24 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the carrier closest to the interferer in MHz. The interferer is an NR signal with an SCS equal to that of the closest carrier. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. NOTE 4: Pinterferer shall be set to -42dBm for omni-directional antenna. NOTE 5: Pinterferer shall be set to -30dBm for antenna array. |  |  |

Topic #2: R19 UE RF for inter-band CA

Issue 2-1: whether to allow n39 UL for CA_n3A-n39A

Agreement:

- The conclusions about allowing n39 UL for CA_n3A-n39A if any should only apply to ATG and will not have any impact on existing NR specification for legacy NR UE.

- Further check if the performance of these parameters in the following table can be achieved in ATG UE with 5MHz frequency gap in n39 filter.

| - Parameters | Values |
| --- | --- |
| Antenna Isolation | 20dB |
| The rejection for band n3 Rx filter at 1885~1915MHz | 50dB |
| n39 Tx filter rejection at 1805~1880MHz | 50dB |

- Other RF architectures are not precluded.

Issue 2-2: BCS for DL CA_n3_n39

WF:

Further consider the following table.

Table 1.  Supported bandwidths per CA band combination of band n3+n39

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n39A | n3, [n39] | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n39 | 5, 10, 15, 20, 25, 30, 35, 40 |  |

Issue 2-3: MSD for CA_n3A-n39A with n39 UL

WF:

Further check the MSD value with 5MHz frequency gap in 1880~1885MHz.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n39 | n3 | 1900 | 30 | 15 | 160 (RBstart=0) | 1827.5 | 5 | [TBD] | ACLR2 |

Topic #3: R19 UE RF for UL-MIMO

Issue 3-1-1: ULFPTx mode for UL-MIMO

WF:

Consider whether and how to introduce this requirement.

- Option 1: ULFPTx mode 1/2/Full in the following table can be supported depending on UE PA implementation

- Table 2: Example of ATG ULFPTx (R-MOP = x dBm)

| ATG ULFPTx mode | PA1(dBm) | PA2(dBm) |
| --- | --- | --- |
| Mode2 | x | (x-3)~(x-1) |
| Mode-Full | x | x |
| Mode1 | (x-3)~(x-1) | (x-3)~(x-1) |

- Option 2: Current ULFPTx mode requirements for legacy UE in TS38.101-1 can be reused for ATG UE, and there is no other specification affect.

- Option 3: No need to extend ULFPTx mode to ATG UE.

Issue 3-1-2: whether preclude the single antenna port related requirement

WF:

- Option 1: Yes.

- Option 2: No.

Issue 3-2-1: how to modify the NR UL MIMO requirement with ATG capability antennaArrayType-r18

Agreement in RAN4#102 meeting: - If the legacy requirement for UL MIMO is defined at each antenna connector, - For ATG UE with antenna array, the requirement should be updated [at each TAB connector]. - If the legacy requirement for UL MIMO is defined per layer, FFS - Option 1: for ATG UE with capability antennaArrayType-r18, the requirement should be updated as sum of all TAB connectors per layer. - Option 2: for ATG UE with capability antennaArrayType-r18, the requirement should be updated per layer without explicitly emphasize the TAB connector.

Agreement:

- If the legacy requirement for UL MIMO is defined at each antenna connector,

  - For ATG UE with antenna array, the requirement should be updated at each TAB connector

- If the legacy requirement for UL MIMO is defined per layer

  - for ATG UE with capability antennaArrayType-r18, the requirement should be updated as sum of all TAB connectors per layer

Issue 3-2-2: whether to apply coherent UL MIMO requirement for ATG UE

Agreement:

Since coherent UL MIMO is the optional requirement, it could be supported without specification impact.

Issue 3-3-1: How to test Rx requirements

Agreement:

No need to test the Rx requirements under the UL MIMO scenario for ATG UE, since general Rx requirements have been tested.

Issue 3-3-2: How to test Tx requirements

Agreement:

Further discuss the method of definition for general ATG Tx requirements referring to agreed CR R4-2414321 can be used.

Issue 3-3-3: Transmit modulation quality for ATG

Agreement:

- 6.4J.2 Transmit modulation quality for ATG The requirements for transmit modulation quality defined in clause 6.4.2 shall apply for ATG UE except for the phase continuity requirements for DMRS bundling in 6.4.2.5. And the requirements for 256QAM modulation are only applicable to ATG UE indicating support of 256QAM. For ATG UE, the requirements for Error Vector Magnitude, Carrier leakage and EVM equalizer spectrum flatness are defined per layer. The requirements for In-band emission are defined at each transmit antenna connector or each TAB connector.