3GPP TSG-RAN WG4 Meeting #112bis	R4-2417098

Hefei, China, 14-18 Oct, 2024

Title: 	WF on HPUE RF requirements

Agenda Item:	6.1.1.1

Source: 	Samsung, Skyworks, Nokia

Document for:	Approval

1. PC1.5 for intra-band contiguous and non-contiguous UL CA

1.1 MPR assumption

Agreement:

- Keep the following last meeting’s agreement for intra-band non-contiguous ULCA

- Define MPR/A-MPR requirements based on equal PSD assumption for intra-band contiguous ULCA

  - It does not preclude companies to bring evaluation for unequal PSD

- MPRc = MPR, which means no change of spec

1.2 PCMAX,f,C and PCMAX

Agreement:

- For a contiguous intra-band combination with PC1.5 capability supporting by txDiversity-r16

  - The PCMAX,f,c and the total configured output power PCMAX for the band combination are given by clause 6.2A.4.1.1 for intra-band contiguous CA in the current version of 38.101-1, no change for PC1.5

- For a non-contiguous intra-band combination with PC1.5 capability supporting dualPA-Architecture

  - For PCMAX,f,c, keep the following last meeting’s agreement, and the detailed wording can be discussed based on the CR in future

  - For PCMAX, FFS

2. 2-band Inter-band UL NR-CA/EN-DC with 2Tx and/or 3Tx

  - Invalid Configuration

Agreement:

- Do not consider 3ULCC with PC2/PC1.5 2Tx CA_nXA-nY(2A)/ DC_XA_nY(2A) scenario.

- Do not consider 4UL CC with PC2/PC1.5 2Tx/3Tx inter-band UL NR CA/ENDC scenario.

3. Increasing UE transmission power

Note: No open issue, focus on the CR drafting

4. General aspect

4.1 MSD rules

Option 1:

  - RAN4 agrees on the following directions in this meeting and continues on details in subsequent meetings.

  - RAN 4 defines the default power class inter-band CA/DC and intra-band CA/DC MSD requirements, and RAN4 specifies equations/simplified guidelines/lookup tables “LUT” that define the relationship between the default power class MSD and the HPUE MSD requirements to replace the existing HPUE MSD tables:

  - Equations/LUT/Simplified guidelines identifying the HPUE MSD requirements are expected to be captured in RAN4 spec as normative requirements, in Rel-19 if possible, the details are FFS

  - FFS the expected spec changes and procedure changes (i.e., continue/stop providing TP’s for each combination, remove/keep all existing HPUE notes/ HPUE MSD tables etc) are recommended to be applicable from Rel-20, or the late stage of Rel-19

- Decision can be deferred until the Equations/LUT/Simplified guidelines are getting done

  - Inform RAN5 after all the discussion are finalized, and it is up to RAN5 to decide how to simplify HPUE CA/DC MSD conformance testing, [and RAN4 can make recommendation if any consensus]

- [Agree to capture just single worst case MSD in TS for HPUE PC which is agnostic to number of TX, for each kind of MSD]

NOTE 1: RAN4 continues normal procedure specifying MSD for HPUE band combination supports at least until down-selection is done.

NOTE 2: RAN4 discuss/consider all HPUE scenarios/configurations as a package for sake of consistence. For the scenarios out of this WI scope, the agreements (if any) are recommended for band combination HPUE basket WIs to follow.

Option 2:

  - Current working procedure of HPUE CA/DC introduction are kept unchanged

  - HPUE MSD tables are kept

  - Unified Excel tool of calculating HPUE MSD are being developed with following details to be addressed

- Companies to evaluate if the interferer level for PC2/PC1.5 can be scaled by less than current 100% to achieve more meaningful PC2/PC1.5 IMD4 and IMD5.

- [Agree to capture just single worst case MSD value in TS for each PC which is agnostic to number of TX].

- Agree to use the following rules in deriving PC2/PC1.5 MSD for cases where PC3 MSD is 0dB:

- If one of the rules above is met, PC2 1TX MSD=0.4dB, PC2 2TX MSD=0.5dB, PC1.5 2TX MSD=0.8dB.

- If both rules above are met, PC2 1TX MSD=0.8dB, PC2 2TX MSD=0.9dB, PC1.5 2TX MSD=1.5dB.

- In case it is discovered that the PC3 MSD is omitted or is illogical compared to respective similar band combinations, then naturally the PC3 MSD can be further evaluated before PC2/PC1.5 assessment.

- Agree that the assumed PRX/DRX difference is 6dB, and additional noise into PRX is 1dB due to 2 TX case for PC2/PC1.5 NR CA and EN-DC.

- The tool is published in 3GPP in Excel format if all proposals are agreeable.

NOTE: Freedom for any company to provide PC3 MSD evaluations using companies’ calculation methods and other assumptions fully remains because PC3 is the baseline to derive PC2/PC1.5 MSD. It is not precluded to provide evaluations for PC2/PC1.5.

Option 3:

  - PC2/PC1.5 MSD requirements are not specified. UE only need to be verified under the Rx requirements of default power class, with default power class transmission

  - Add the Equations/LUT/Simplified guidelines into the Annex of TS to derive the HPUE MSD for information, in Rel-19 if possible, the details are FFS

  - FFS the expected spec changes and procedure changes (i.e., continue/stop providing TP’s for each combination, remove/keep all existing HPUE notes/ HPUE MSD tables etc) are recommended to be applicable from Rel-20, or the late stage of Rel-19

-Decision can be deferred until the Equations/LUT/Simplified guidelines are getting done

  - Inform RAN5 after all the discussion are finalized, and it is up to RAN5 to decide how to simplify HPUE CA/DC MSD conformance testing, [and RAN4 can make recommendation if any consensus]

NOTE 1: RAN4 continues normal procedure specifying MSD for HPUE band combination supports at least until down-selection is done.

NOTE 2: RAN4 discuss/consider all HPUE scenarios/configurations as a package for sake of consistence. For the scenarios out of this WI scope, the agreements (if any) are recommended for band combination HPUE basket WIs to follow.

Option 4:

  - PC2/PC1.5 MSD requirements are not specified, MSD for PC2 and PC1.5 are only specified in TR or Annes of TS which is not mandate, UE shall meet the Rx requirements of default power class, with default power class transmission

Agreement:

- To clarify the Equations/LUT/Simplified guidelines mentioned in Option 1 and Option 3

  - Some examples are provided in the Annex of this WF

- Other options such as Option2 are not precluded

4.2 Duty cycle solution for SAR compliance

Agreement:

- For intra-band CA and inter-band CA PC1.5, duty cycle solution for SAR will be discussed.

- For EN-DC PC1.5 and PC2, duty cycle solution for SAR will discussed for the scenario without new IE introduction.

Annex:

## A.1	Table approach from R4-2415392, Nokia, RAN4#112.

Tables below were not specifically drafted for HPUE purposes as they have been presented in three consecutive RAN4 meeting under specification improvement session in RAN4. But with the modifications made for this WF they can give indication of the intent of the table approach method.

Table 1 defines band groups

Table 1: Band group definition

| FR1 band group range |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Name | FR1-1 | FR1-2 | FR1-3 | FR1-4 | FR1-5 |
| Range (MHz) | 600-1000 | 1400-2200 | 2300-2700 | 3300-5000 | 5150-7125 |

Table 2 is not needed in specification but when the default power class MSD is defined then the IMD order is calculated.

Table 2: 2 UL CA configurations with MSD requirements.

| NR CA band combination (CA_nX-nY) | NR Rx band (nZ) | Uplink Harmonic | Harmonic Mixing | Cross Band Interf. | Order | NC ULCA | IMD test point equation UL Band(x,y) to DL Band(z) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1-n3 | n1 |  |  |  | IMD3 same |  | I2*nX-nYI=nZ |
| CA_n1-n3 | n3 |  |  | 1,>2 |  |  |  |
| CA_n1-n8 | n1 |  |  |  | IMD4 |  | I2*nX-2*nYI=nZ |
| CA_n1-n28 | n1 | UL3 |  |  |  |  |  |
| CA_n1-n38 | n1 |  |  | >2 |  |  |  |
| CA_n1-n38 | n38 |  |  | >2 |  |  |  |
| CA_n1-n40 | n1 |  |  | >2 |  |  |  |
| CA_n1-n40 | n40 |  |  | >2 |  |  |  |
| CA_n1-n41 | n41 |  |  | >2 |  |  |  |
| CA_n1-n46 | n1 |  |  |  | IMD5 |  | InY-4*nXI=nZ |
| CA_n1-n77 | n1 |  |  |  | IMD24 | IMD5 diff | InY-nXI=nZ; nY-2*2CCBW=nZ |
| CA_n1-n77 | n77 | UL2 |  |  |  |  |  |
| CA_n1-n78 | n1 |  |  |  | IMD4 | IMD7 | I3*nX-nYI=nZ; nY-3*2CCBW=nZ |
| CA_n1-n102 | n1 |  | UL1/DL3 |  | IMD3 diff |  |  |
| CA_n1-n105 | n105 |  | UL1/DL3 |  | IMD3 diff |  |  |
| CA_n2-n38 | n2 |  |  | >2 |  |  |  |
| CA_n2-n41 | n2 |  |  | >2 |  |  |  |
| CA_n2-n48 | n2 |  |  |  | IMD4 |  |  |
| CA_n2-n48 | n48 | UL2 |  |  |  |  |  |
| CA_n2-n66 | n2 |  |  |  | IMD3 same |  |  |
| CA_n2-n66 | n66 |  |  |  | IMD5 |  |  |
| CA_n2-n71 | n2 | UL3 |  |  |  |  |  |
| CA_n2-n71 | n71 |  | UL1/DL3 |  |  |  |  |
| CA_n2-n772,3 | n2 |  | UL1/DL2 |  | IMD24, IMD4 | IMD7 |  |
| CA_n2-n772,3 | n77 | UL2 |  |  |  |  |  |
| CA_n2-n78 | n2 |  |  |  | IMD24 |  |  |
| CA_n2-n78 | n78 | UL2 |  |  |  |  |  |
| CA_n3-n5 | n3 |  |  |  | IMD4 |  |  |
| CA_n3-n5 | n5 |  |  |  | IMD2 |  |  |
| CA_n3-n7 | n3 |  |  | >2 |  |  |  |
| CA_n3-n7 | n7 |  |  |  | IMD4 |  |  |
| CA_n3-n8 | n3 |  |  |  | IMD5 |  |  |
| CA_n3-n8 | n8 |  |  |  | IMD4 |  |  |
| CA_n3-n18 | n3 |  |  |  | IMD4 |  |  |
| CA_n3-n20 | n3 |  |  |  | IMD4 |  |  |
| CA_n3-n20 | n20 |  |  |  | IMD4 |  |  |
| CA_n3-n26 | n3 |  |  |  | IMD4 |  |  |
| CA_n3-n26 | n26 |  | UL1/DL2 |  | IMD24 |  |  |
| CA_n3-n34 | n3 |  |  | >2 |  |  |  |
| CA_n3-n38 | n3 |  |  |  | IMD4 |  |  |
| CA_n3-n41 | n3 |  |  | >2 | IMD3 diff, IMD4 | IMD9 |  |
| CA_n3-n41 | n41 |  |  | >2 |  |  |  |
| CA_n3-n74 | n74 |  |  | >2 |  |  |  |
| CA_n3-n75 | n75 |  |  | >2 |  |  |  |
| CA_n3-n772,3 | n3 |  |  |  | IMD24, IMD4 | IMD7 |  |
| CA_n3-n772,3 | n77 | UL2 |  |  |  |  |  |
| CA_n3-n782,3 | n3 |  |  |  | IMD24, IMD4 | IMD7 |  |
| CA_n3-n782,3 | n78 | UL2 |  |  |  |  |  |
| CA_n5-n7 | n5 |  |  |  | IMD3 diff |  |  |
| CA_n5-n14 | n5 |  |  |  | IMD3 same |  |  |
| CA_n5-n14 | n5 |  |  |  | IMD3 same |  |  |
| CA_n5-n41 | n28 |  |  | 2 |  |  |  |
| CA_n5-n41 | n5 |  | UL1/DL3 |  | IMD3 diff |  |  |
| CA_n5-n41 | n41 | UL3 |  |  |  |  |  |
| CA_n5-n66 | n5 |  |  |  | IMD24 |  |  |
| CA_n5-n71 | n5 |  |  | >2 |  |  |  |
| CA_n5-n71 | n71 |  |  | >2 |  |  |  |
| CA_n5-n772,3 | n5 |  |  |  | IMD24, IMD4 | IMD4 |  |
| CA_n5-n772,3 | n77 | UL4, UL5 |  |  |  |  |  |
| CA_n5-n78 | n5 |  |  |  | IMD4 | IMD4 |  |
| CA_n5-n78 | n78 | UL4 |  |  |  |  |  |
| CA_n7-n8 | n7 | UL3 |  |  |  |  |  |
| CA_n7-n26 | n7 |  |  |  | IMD5 |  |  |
| CA_n7-n26 | n26 |  |  |  | IMD3 diff |  |  |
| CA_n7-n40 | n7 |  |  | >2 | IMD3 same |  |  |
| CA_n7-n40 | n40 |  |  | >2 |  |  |  |
| CA_n7-n464 | n7 |  |  |  | IMD24 |  |  |
| CA_n7-n464 | n46 |  | UL1/DL2 |  |  |  |  |
| CA_n7-n66 | n7 |  |  |  | IMD4 |  |  |
| CA_n7-n71 | n7 | UL4 |  |  |  |  |  |
| CA_n7-n77 | n7 |  |  | >2 | IMD4 | IMD5 diff |  |
| CA_n7-n78 | n7 |  |  | >2 |  |  |  |
| CA_n7-n105 | n7 | UL3 |  |  |  |  |  |
| CA_n8-n20 | n8 |  |  |  | IMD3 same |  |  |
| CA_n8-n20 | n20 |  |  |  | IMD3 same |  |  |
| CA_n8-n41 | n8 |  |  |  | IMD3 diff |  |  |
| CA_n8-n41 | n41 | UL3 |  |  |  |  |  |
| CA_n8-n77 | n77 | UL4 |  |  |  |  |  |
| CA_n8-n782 | n8 |  |  |  | IMD4 |  |  |
| CA_n8-n782 | n78 | UL4 |  |  |  |  |  |
| CA_n8-n792,3 | n8 |  | UL1/DL5 |  | IMD5 |  |  |
| CA_n8-n792,3 | n79 | UL5 |  |  |  |  |  |
| CA_n12-n48 | N48 | UL5 |  |  |  |  |  |
| CA_n12-n66 | n66 | UL3 |  |  |  |  |  |
| CA_n12-n71 | n12 |  |  | 2,>2 |  |  |  |
| CA_n12-n772,3 | n12 |  | UL1/DL5 |  |  |  |  |
| CA_n12-n772,3 | n77 | UL5 |  |  |  |  |  |
| CA_n12-n78 | n12 |  | UL1/DL5 |  |  |  |  |
| CA_n12-n78 | n78 | UL5 |  |  |  |  |  |
| CA_n13-n772,3 | n13 |  | UL1/DL5 |  |  |  |  |
| CA_n13-n772,3 | n77 | UL5 |  |  |  |  |  |
| CA_n14-n772,3 | n14 |  | UL1/DL5 |  |  |  |  |
| CA_n14-n772,3 | n77 | UL5 |  |  |  |  |  |
| CA_n18-n28 | n28 |  |  | 1,2 |  |  |  |
| CA_n18-n41 | n18 |  | UL1/DL3 |  |  |  |  |
| CA_n18-n77 | n18 |  |  |  | IMD4, IMD5 |  |  |
| CA_n18-n77 | n77 | UL5 |  |  |  |  |  |
| CA_n18-n78 | n18 |  |  |  | IMD4 |  |  |
| CA_n20-n40 | n20 |  | UL1/DL3 |  |  |  |  |
| CA_n20-n78 | n20 |  |  |  | IMD4 |  |  |
| CA_n20-n78 | n78 | UL4 |  |  |  |  |  |
| CA_n24-n77 | n24 |  |  |  | IMD4 |  |  |
| CA_n25-n38 | n25 |  |  | >2 |  |  |  |
| CA_n25-n41 | n25 |  |  | >2 |  | IMD7 |  |
| CA_n25-n48 | n25 |  |  |  | IMD4 |  |  |
| CA_n25-n66 | n25 |  |  |  | IMD3 same |  |  |
| CA_n25-n66 | n66 |  |  |  | IMD3 same, IMD5 |  |  |
| CA_n25-n71 | n25 | UL3 |  |  |  |  |  |
| CA_n25-n71 | n71 |  | UL1/DL3 |  |  |  |  |
| CA_n25-n772,3 | n25 |  | UL1/DL2 |  | IMD24, IMD4 | IMD7 |  |
| CA_n25-n772,3 | n77 | UL2 |  |  |  |  |  |
| CA_n25-n78 | n25 |  |  |  | IMD24 | IMD7 |  |
| CA_n25-n78 | n78 | UL2 |  |  |  |  |  |
| CA_n26-n29 | n29 |  |  | >2 |  |  |  |
| CA_n26-n48 | n26 |  | UL1/DL4 |  |  |  |  |
| CA_n26-n66 | n26 |  |  |  | IMD24 |  |  |
| CA_n26-n70 | n26 |  |  |  | IMD24 |  |  |
| CA_n26-n77 | n26 |  | UL1/DL4 |  |  |  |  |
| CA_n26-n78 | n26 |  | UL1/DL4 |  | IMD4 |  |  |
| CA_n26-n78 | n78 | UL4 |  |  |  |  |  |
| CA_n28-n40 | n28 |  | UL1/DL3 |  |  |  |  |
| CA_n28-n50 | n28 |  |  |  | IMD2, IMD4 |  |  |
| CA_n28-n50 | n50 | UL2 |  |  |  |  |  |
| CA_n28-n71 | n28 |  |  | >2 |  |  |  |
| CA_n28-n71 | n71 |  |  | 2 |  |  |  |
| CA_n28-n74 | n28 |  |  |  | IMD2, IMD4 |  |  |
| CA_n28-n74 | n74 | UL2 |  |  | IMD4, IMD5 |  |  |
| CA_n28-n75 | n75 | UL2 |  |  |  |  |  |
| CA_n28-n772,3 | n28 |  |  |  | IMD2, IMD5 | IMD4 |  |
| CA_n28-n772,3 | n77 | UL5 |  |  |  |  |  |
| CA_n28-n782 | n28 |  |  |  | IMD5 |  |  |
| CA_n28-n782 | n78 | UL5 |  |  |  |  |  |
| CA_n28-n94 | n94 | UL2 |  |  |  |  |  |
| CA_n29-n48 | n29 |  | UL1/DL5 |  |  |  |  |
| CA_n29-n71 | n29 |  |  | 2 |  |  |  |
| CA_n29-n772,3 | n29 |  | UL1/DL5 |  |  |  |  |
| CA_n30-n66 | n66 |  |  | >2 |  |  |  |
| CA_n30-n772,3 | n30 |  | UL2/DL3 |  | IMD4 | IMD7 |  |
| CA_n34-n41 | n34 |  |  | >2 |  |  |  |
| CA_n34-n41 | n41 |  |  | >2 |  |  |  |
| CA_n38-n66 | n66 |  |  | >2 |  |  |  |
| CA_n38-n78 | n38 |  |  | >2 |  |  |  |
| CA_n38-n78 | n78 |  |  | >2 |  |  |  |
| CA_n39-n41 | n39 |  | UL3/DL4 | >2 |  |  |  |
| CA_n39-n41 | n41 |  |  | >2 |  |  |  |
| CA_n40-n41 | n40 |  |  | 2 |  |  |  |
| CA_n40-n41 | n41 |  |  | 2 |  |  |  |
| CA_n40-n772 | n40 |  | UL2/DL3 |  |  |  |  |
| CA_n40-n772 | n77 |  |  | >2 |  |  |  |
| CA_n40-n78 | n40 |  | UL2/DL3 | >2 |  |  |  |
| CA_n41-n48 | n41 |  |  | >2 |  |  |  |
| CA_n41-n48 | n48 |  | UL4/DL3 | >2 |  |  |  |
| CA_n41-n66 | n66 |  |  | >2 |  | IMD5 same |  |
| CA_n41-n70 | n70 |  |  | >2 |  |  |  |
| CA_n41-n71 | n41 | UL4 |  |  |  |  |  |
| CA_n41-n71 | n71 |  |  |  | IMD4 |  |  |
| CA_n41-n772 | n41 |  | UL2/DL3 | >2 |  |  |  |
| CA_n41-n772 | n77 |  |  | >2 |  | IMD9 |  |
| CA_n41-n78 | n41 |  | UL2/DL3 | >2 |  | IMD9 |  |
| CA_n41-n78 | n78 |  |  | >2 |  |  |  |
| CA_n46-n484 | n46 |  |  | >2 |  |  |  |
| CA_n46-n484 | n48 |  | UL2/DL3 | >2 |  |  |  |
| CA_n46-n77 | n46 |  | UL2/DL3 |  |  | IMD5 diff, IMD6, IMD7 |  |
| CA_n46-n77 | n77 |  |  | >2 |  |  |  |
| CA_n46-n784 | n46 |  | UL2/DL3 | >2 |  | IMD6, IMD7 |  |
| CA_n46-n784 | n78 |  |  | >2 |  |  |  |
| CA_n48-n66 | n48 | UL2 |  |  |  |  |  |
| CA_n48-n66 | n66 |  |  |  | IMD5 |  |  |
| CA_n48-n70 | n70 |  |  |  | IMD24 |  |  |
| CA_n48-n964 | n48 |  | UL2/DL3 | >2 |  |  |  |
| CA_n48-n964 | n96 |  |  | >2 |  |  |  |
| CA_n66-n71 | n66 |  |  |  | IMD4 |  |  |
| CA_n66-n77 | n66 |  |  |  | IMD2, IMD5 | IMD5 diff, IMD7 |  |
| CA_n66-n77 | n77 | UL2 |  |  |  |  |  |
| CA_n66-n78 | n66 |  |  |  | IMD5 | IMD7 |  |
| CA_n66-n78 | n78 | UL2 |  |  |  |  |  |
| CA_n66-n85 | n66 | UL3 |  |  | IMD4 |  |  |
| CA_n67-n78 | n67 |  | UL1/DL5 |  |  | IMD4 |  |
| CA_n70-n71 | n70 |  |  |  | IMD4 |  |  |
| CA_n70-n71 | n71 | UL3 |  |  |  |  |  |
| CA_n70-n77 | n70 | UL2 |  |  | IMD2, IMD5 |  |  |
| CA_n70-n78 | n70 | UL2 |  |  | IMD2, IMD5 |  |  |
| CA_n71-n77 | n71 |  |  |  | IMD5 | IMD4 |  |
| CA_n71-n78 | n71 |  |  |  | IMD5 |  |  |
| CA_n71-n85 | n71 |  |  | >2 |  |  |  |
| CA_n71-n85 | n85 |  |  | 1,2 |  |  |  |
| CA_n77-n85 | n77 | UL5 |  |  |  |  |  |
| CA_n77-n85 | n85 |  | UL1/DL5 |  | IMD5 |  |  |
| CA_n78-n79 | n78 |  |  | >2 |  |  |  |
| CA_n78-n79 | n79 |  |  | >2 |  |  |  |
| CA_n78-n92 | n78 | UL4 |  |  |  |  |  |
| CA_n78-n94 | n78 | UL4 |  |  |  |  |  |
| CA_n78-n102 | n102 |  |  |  |  | IMD4 |  |
| CA_n78-n105 | n78 | UL5 |  |  |  |  |  |
| CA_n78-n105 | n105 |  |  |  | IMD5 |  |  |
| Note 2: Power class 2 supported. Note 3: Power class 1.5 supported. Note 4: Power class 5 supported. |  |  |  |  |  |  |  |

Then finally delta MSD on top of default power class MSD is specified in Tables 3-5.

Table 3: delta MSD for Harmonic mixing requirements PC2/PC1.5.

| PC2/PC1.5 |  |  |  |
| --- | --- | --- | --- |
| Order | UHB/LB | UHB/MB | UHB/HB |
| UL1/DL2 | N/A | ?dB/?dB | N/A |
| UL1/DL4 | ?dB/?dB | N/A | N/A |
| UL1/DL5 | ?dB/?dB | N/A | N/A |
| UL2/DL3 | N/A | N/A | ?dB/?dB |

Table 4: delta MSD for IMD requirements for PC2.

| 2 CA IMD |  |  |  | 3 CA IMD |  |
| --- | --- | --- | --- | --- | --- |
| Order | MSD | 2CA (NC_ULCA) | MSD | Order | MSD |
| IMD2 | ?dB | IMD4 | ?dB | IMD2 | ?dB |
| IMD3 same b.gr | ?dB | IMD5 same b.gr | ?dB | IMD3 same b.gr | ?dB |
| IMD3 diff. b.gr | ?dB | IMD5 diff. b.gr | ?dB | IMD3 diff b.gr. | ?dB |
| IMD4 | ?dB | IMD7 same b.gr | ?dB | IMD4 | ?dB |
| IMD5 | ?dB | IMD7 diff. b.gr | ?dB | IMD5 | ?dB |
|  |  | IMD9 | ?dB | IMD7 |  |

Table 5: delta MSD for IMD requirements for PC1.5.

| 2 CA IMD |  |  |  | 3 CA IMD |  |
| --- | --- | --- | --- | --- | --- |
| Order | MSD | 2CA (NC_ULCA) | MSD | Order | MSD |
| IMD2 | ?dB | IMD4 | N/A | IMD2 | N/A |
| IMD3 same b.gr | N/A | IMD5 same b.gr | N/A | IMD3 same b.gr | N/A |
| IMD3 diff. b.gr | N/A | IMD5 diff. b.gr | N/A | IMD3 diff b.gr. | N/A |
| IMD4 | ?dB | IMD7 same b.gr | N/A | IMD4 | N/A |
| IMD5 | ?dB | IMD7 diff. b.gr | N/A | IMD5 | N/A |
|  |  | IMD9 | N/A | IMD7 | N/A |

## A.2	Tabular or equation-based approach from Skyworks, RAN4#112bis

Some examples of how a tabular or equation-based derivation of HPUE MSD based on PC3 MSD value are provided for 1UL MSDs and 2UL MSD cases.

Example of one UL HPUE scenarios compared to default power class case.

The scenarios described in Table 1 apply to all one UL one CC MSD types:

- UL harmonic, harmonic mixing and cross band isolation

- The first column provides the per band power class supported by the UE in DLCA and the associated number of Tx paths. With PC3 (1Tx) being the reference

- The second column provides the output power condition for the MSD for a given power class support.

- The third column provides the expected interferer increase versus the PC3 reference and where (-MRC) denotes when the MRC gain is lost due to 2Tx support.

Table 1: HPUE one UL scenarios compared to default power class PC3 case.

| 1UL power class | 1UL power condition | 1UL interference |
| --- | --- | --- |
| PC3 | 23 | reference |  |
| PC2 1Tx | 26 | reference+3dB |  |
| PC2 2Tx | 23+23 | reference-MRC |  |
| PC1.5 2Tx | 26+26 | ref+3dB-MRC |  |

Based on the above table and assuming (for example) 7dB interference level difference between diversity receiver and main receiver, the 1UL MSD for the different HPUE cases can be derived from the curves in Figure 1 where the delta MSD is calculated for the interference level increase and MRC gain based on Table 1. A 5th polynomial order fitting equation is also provided as example.

Figure 1: HPUE delta MSD versus PC3 MSD for 1Tx PC2, 2TX PC2 and 2Tx PC1.5 in increasing values.

Example of two UL HPUE scenarios compared to default power class case.

The scenarios described in Table 2 apply to two UL IMDs with one CC per band

- The first column provides the CA power class supported by the UE and the associated number of Tx paths. With PC3 being the reference

- The second and third columns provide the per band power class support

- The fourth column provides the output power condition for the MSD for a given power class support.

- The third column provides the expected interferer increase versus the PC3 reference and where (-MRC) denotes when the MRC gain is lost due to 2Tx support:

  - In some cases the interference increase is provided

  - In other cases the interference increased has an upper bound (<)

  - The more complex cases where the 2Tx UL power is limited by the CA power class, some further work is needed.

Table 2: HPUE two UL scenarios compared to default power class PC3 case.

| ULCA power class | per band power class |  | 2UL power condition | 2UL interference |
| --- | --- | --- | --- | --- |
|  | Bd1 | Bd2 |  |  |
| PC3 | PC3 | PC3 | 20+20 | Reference |
| PC2 2Tx | PC3 | PC3 | 23+23 | reference+3xIMDorder |
| PC2 2Tx | PC3 | PC2 1Tx | 23+23 | <reference+3xIMDorder |
| PC2 3Tx | PC3 | PC2 2Tx | 23+(20+20) | <reference+3xIMDorder-MRC |
| PC2 2Tx | PC2 1TX | PC2 1Tx | 23+23 | reference+3dB |
| PC2 3Tx | PC2 1TX | PC2 2Tx | 23+(20+20) | <reference+3dB-MRC |
| PC1.5 2Tx | PC2 1TX | PC2 1Tx | 26+26 | reference+3dB+3xIMDorder |
| PC1.5 3Tx | PC2 1TX | PC2 2Tx | 26+(20+20) | <reference+3dB+3xIMDorder-MRC |
| PC1.5 3Tx | PC3 | PC1.5 | 23+(24.7+24.7) | FFS |
| PC1.5 3Tx | PC2 1TX | PC1.5 | 23+(23+23) | FFS |

Based on the above table scenarios where the interference increase is known (symmetric PA cases) and assuming (for example) 7dB interference level difference between diversity receiver and main receiver, the 2UL IMD MSD can be derived from the curves:

- Figure 3 provides the delta MSD for the PC2 2Tx ULCA implemented with two PC3 PAs per IMD order.

- Figure 4 provides the delta MSD for the PC2 2Tx ULCA implemented with two PC2 PAs for all IMD orders.

- Figure 5 provides the delta MSD for the PC1.5 2Tx ULCA implemented with two PC2 PAs per IMD order.

- A 5th polynomial order fitting equation is also provided as example. Note that the fitting can be improved by limiting the fitting to the lower PC3 MSD part and use a single value above some PC3 MSD threshold.

Figure 2: PC2 2Tx with two PC3 PAs delta MSD per IMD order

Figure 3: PC2 2Tx with two PC2 PAs delta MSD for all IMD order

Figure 4: PC1.5 2Tx with two PC2 PAs delta MSD per IMD order