3GPP TSG-RAN WG4 Meeting#112bis                              R4-2417107

Hefei , China, 14th – 18th Oct, 2024

Agenda item:			6.2.6

Source:	ZTE Corporation

Title:	WF on 15GHz BS parameters

Document for:	Approval

- Agreement for RAN4#112bis

Issue 3-2: Scope of scenarios

Agreement:

- Limit simulations to urban macro

Issue 3-9: ISD for simulation

Agreement:

- 450m

- Agreement:

Issue 4-17: BS antenna parameters

For Urban Macro:

Agreement:

- Make assumption that reply can say that array size is  [3072 (16*24, SA size 4)], and the TR will document that other arrays are possible.

- Double check the coverage and [3072] before making final decision on reply

  - If coverage issues is found for [3072], minimum array size [4096] to achieve coverage could be discussed

For Dense Urban:

Agreement:

- Dense urban antenna parameters Same as urban macro, mechanical tilt can differ in reply

For Indoor scenario:

Agreement:

- For reply LS : Antenna configuration without sub-array assumption: 4x4

- Simulation: Not prioritized

For output power for different BS class:

Agreement:

- For wide area BS, 46dBm for TRP for dual polarization;

- For dense urban BS, [36dBm]for TRP for dual polarization;

- For Indoor:  23dBm per polarization

Issue 4-10: Emissions mask

Agreement:

For the reply,  refer to the following table as basis

- [category B] and category A

- Table 6.1.2-1: Wide Area BS operating band unwanted emission limits for 6.425 - 7.125 GHz and 10.0 - 10.5 GHz for Category A

| Frequency offset of measurement filter 3dB point, Df | Frequency offset of measurement filter centre frequency, f_offset | Basic limits | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz £ Df < 50 MHz | 0.05 MHz £ f_offset < 50.05 MHz | −7dBm−750foffsetMHz−0.05 | 100 kHz |
| 50 MHz f < min(100 MHz, Dfmax) | 50.05 MHz f_offset < min(100.05 MHz, f_offsetmax) | -14 dBm | 100 kHz |
| 100 MHz £ Df £ Dfmax | 100.5 MHz £ f_offset < f_offsetmax | -13 dBm | 1MHz |

- [Table 6.1.2-2: Wide Area BS operating band unwanted emission limits for 6.425 - 7.125 GHz and 10.0 - 10.5 GHz for Category B

| Frequency offset of measurement filter 3dB point, Df | Frequency offset of measurement filter centre frequency, f_offset | Basic limits | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz £ Df < 50 MHz | 0.05 MHz £ f_offset < 50.05 MHz | −7dBm−750foffsetMHz−0.05 | 100 kHz |
| 50 MHz f < min(100 MHz, Dfmax) | 50.05 MHz f_offset < min(100.05 MHz, f_offsetmax) | -14 dBm | 100 kHz |
| 100 MHz £ Df £ Dfmax | 100.5 MHz £ f_offset < f_offsetmax | -15 dBm | 1MHz |

- ]

Issue 4-14: Spurious emission

Agreement:

For the reply,  refer to the following table as basis

- [category B] and category A

- Table 2.2.2.3-1: BS spurious emission limits for 14800 to 15350 MHz for Category A

| Frequency range | Limit | Measurement Bandwidth | Note |
| --- | --- | --- | --- |
| 30 MHz – 1 GHz | -13 dBm | 100 kHz | Note 1 |
| 1 GHz – 2nd harmonic of the upper frequency edge of the DL operating band |  | 1 MHz | Note 1, Note 2 |
| NOTE 1: Bandwidth as in ITU-R SM.329, s4.1 NOTE 2: Upper frequency as in ITU-R SM.329, s2.5 table 1. |  |  |  |

- Table 2.2.2.3-2: BS spurious emission limits for 14800 to 15350 MHz for Category B

| Spurious frequency range | Limit | Measurement bandwidth |
| --- | --- | --- |
| 30 MHz – 1 GHz | -36 dBm | 100 kHz |
| 1 GHz – 18 GHz | -30 dBm | 1 MHz |
| 18 GHz – 2nd harmonic of the upper frequency edge of the DL operating band | -20 dBm | 10 MHz |

Issue 4-12: Sensivitity

Agreement:

For LS:  “To be specified”

For TR: Define based on NF and antenna gain

Issue 4-8: BS output power

Agreement:

- Do not mention average output power

- Open issues

Issu 4-13: ACLR

- Potential agreement:

- Postpone to next meeting according to the simulation results.

Issue 4-16: ACS

- agreement:

- Postpone to next meeting according to the simulation results.

Issue 4-15: Blocking response

Potential Agreement:

  - Further evaluate the power level for in-band requirement based on the simulation

  - Further evaluate the f_OOBB requirement based on the agreed IBB and OOBB power level;

- Agreement for previous RAN4 meeting

This sub-topic considers BS parameters

Issue 4-9: Power dynamic range

Previous agreement: 0dB

Issue 4-11: Noise figure

Previous agreement:

- 8 dB for Wide-Area BS

- 13 dB for Medium Range BS

- 16 dB for Local Area BS