3GPP TSG-RAN WG4 Meeting #112bis												R4-2417112

Hefei China, 14th – 28th Oct, 2024

Agenda item:			6.23.5

Source:	vivo

Title:	WF on UE RF requirements for LP-WUS

Document for:	Approval

Introduction

This is WF for Rel-19 LP-WUS UE RF.

Topic #1: General and system parameters

Sub-topic 1-1 General

Issue 1-1-1: draft skeleton for LP-WUS RF TR 38.774

Agreements:

  - The draft skeleton for TR 38.774 in R4-2415778 is endorsed.

  - Below is the TR drafting work splitting among interested companies:

| Section in TR | Title | Owner(s) |
| --- | --- | --- |
| General part | (scope, reference, definition, background) | vivo |
| 5 | System parameters | Ericsson, CATT |
| 6 | Simulation assumptions and evaluation | CATT, CMCC, vivo |
| 7.1 | UE RF | Huawei, Nokia, CMCC |
| 7.2 | BS RF | ZTE, Ericsson |
| 8 | Testability | Nokia, Samsung |
| Annex | Other aspects (if needed) | vivo |

Issue 1-1-2: Consideration on RAN1 LS R4-2414909

Agreements:

  - no specific RF work related to RAN1 LS.

Issue 1-1-3: MDR value for RF requirements

Agreements:

  - 1% MDR. Same for requirements and conformance testing.

Issue 1-1-4: FAR assumption for MDR evaluation

Agreements:

  - Use </=1% FAR as evaluation assumption for RF

Sub-topic 1-2 System parameters

Issue 1-2-1: Channel raster for LP-WUR

Agreements:

  - Companies directly bring the specification impact if any and those will be discussed instead of whether a channel raster is required or not.

Issue 1-2-2: considerations on LP-WUS allocation

Agreements:

  - RAN4 to assume PRB grid alignment between LP-WUS and NR.

  - RAN4 to assume the LP-WUS RBs can be flexibly allocated within the wider NR carrier.

Issue 1-2-3: number of RBs for LP-WUS with 15kHz SCS

Agreements:

  - Follow RAN1 agreement for number of RBs for LP-WUS with 15KHz SCS, i.e., 11RBs.

Topic #2: REFSENS, ASCS and ACS requirements

Sub-topic 2-1 SNR simulation and values

Issue 2-1-1: Alignment of FR1 SNR calculation/definition in simulation

Agreements:

  - SNR is defined as the ratio of average energy of the LP-WUS signal per sub carrier over the average energy of Noise per sub carrier, over all the LP-WUS RBs.

    - Consider the sub-carriers allocated to LP-WUS excluding Guard RBs.

Issue 2-1-2: FR1 SNR reference point

Agreements:

  - same SNR reference point as MR

Issue 2-1-3: Target FR1 SNR value for LP-WUS/WUR

Agreements:

  - RAN4 can further discuss target SNR for OOK-based and OFDM-based receivers.

Issue 2-1-5: Link level simulation assumption for FR2 LP-WUR

Agreements:

  - RAN4 can further discuss simulation assumption for FR2 LP-WUS next meeting.

Sub-topic 2-2 NF and REFSENS requirements

Issue 2-2-1: Baseline architecture for FR1 OFDM-based LP-WUS

Agreements:

  - use zero-IF receiver as a baseline RF architecture for OFDM based LP-WUR.

Issue 2-2-3: REFSENS, NF, IM, SNR aspects for FR1 OOK-based and OFDM-based LR

Agreements:

  - Collect input on REFSENS with values for each element（NF, IM, SNR） in the table next meeting. The value for each element can be discussed.

  - Encourage companies to also input the justification for each element.

Issue 2-2-10: Baseline architecture for FR2 OOK-based and OFDM-based LP-WUR

Agreements:

  - Companies to study baseline architecture for FR2 LP-WUS.

Sub-topic 2-3 ASCS simulation and requirements

Issue 2-3-1: ASCS requirements value

Agreements:

  - Confirm ASCS RF requirements is needed for LP-WUS/WUR.

Issue 2-3-2: Required number of guard RB for ASCS

Agreements:

  - RAN4 further discuss required guard RBs for ASCS requirements.

Issue 2-3-3: Whether Test case for ASCS should be defined in RAN4

Agreements:

  - Detailed test case for ASCS should be defined.

Sub-topic 2-4 ACS simulation and requirements

Issue 2-4-1: Phase noise for simulation assumption

Agreements:

  - Phase noise can be considered in LLS simulation analysis.

    - Phase noise used in R4-2415201, R4-2415780, and R4-2309204 could be considered as a reference. Other profiles are not precluded.

  - Number of guard RBs will be further discussed until Feb meeting. Encourage companies to provide simulation results with phase noise modelling.

Issue 2-4-4: ACS requirements value

Agreements:

  - FFS.

Issue 2-4-5: Required guard RB for ACS requirements

Agreements:

  - RAN4 further discuss the required number of guard RBs for ACS requirements

Issue 2-4-7: Test parameters for LP-WUR ACS case

Agreements:

  - Further discuss test parameters for LP-WUS ACS

Issue 2-4-8: whether limit ADC bit for ACS requirements

Agreements:

  - No limiting to a specific ADC bit for ACS requirements.

Topic #3: Other RF requirements

Issue 3-2-2: Maximum input level requirements

Agreements:

  - The same maximum input power of [-25 dBm] could be starting point for LP-WUR.

  - FFS whether this value should be scaled-down based on the number of LP-WUS RBs.

Issue 3-2-3: Reference channel for LP-WUR requirements

Agreements:

  - RAN4 should define reference measurement channel for LP-WUR RF requirements.

  - FFS details.

Topic #4: Testability issues

Sub-topic 4-1 Testability for UE RF requirements

Issue 4-1-1: General framework on LP-WUS testing

Way forward: FFS

Issue 4-1-2: Separate RF test case for idle and connection mode

Way forward: FFS

Issue 4-1-3: How to get feedback from LP-WUR during the test

Way forward: FFS

Issue 4-1-4: detailed Test procedure

Way forward: FFS

Issue 4-1-5: UE test mode for LP-WUR

Agreements:

  - FFS A UE test mode for LP-WUR is needed.

Issue 4-1-6: whether a LS to RAN5 on test issue

Way forward: FFS