3GPP TSG-RAN WG4 Meeting #112-bis	R4-2417116

Hefei, China, 14 ‒ 18 October, 2024

Title: 	WF on UE RF requirements for MIMO enhancement

Agenda Item:	6.19.4

Source: 	Samsung

Document for:	Approval

<Topic 1> 3Tx

<Sub-topic 1-1> Power class

<Agreement>: PC for 3Tx requirement

- PC for 3Tx requirement

  - PC3

  - PC2

  - FFS on PC1.5

    - FFS on UE type, i.e., FWA and/or handheld UE

    - FFS on support of full power mode 0

- RAN4 understanding is that TxD for 3Tx is not in the scope of this WI

- Full power mode 0 apply to PC2 and PC3 only.

  - FFS on for which UE types it should apply.

<Way forward>: FFS on New capability for power class per band per declared UL power mode for 3Tx

<Sub-topic 1-2> PA configuration

<Agreement>: Assumption for power splitting between antenna ports

- Equal power split among non-zero ports according to RAN1 spec

<Agreement>: PA configurations

- For MPR evaluation, the assumptions of PA architecture are:

  - For PC3, 3x23dBm

  - For PC2

    - 3x23dBm

    - 3x26dBm is not precluded and interested company can provide the evaluation

<Sub-topic 1-3> MPR

<Agreement>: Different MPR for different UE type

- Define separate MPR requirements for handheld and FWA UE

<Way forward>: MPR derivation method for 3Tx

- Companies are encouraged to evaluate MPR based on existing MPR requirements, e.g., PC, 2/4Tx, UE type, considering 3PA configuration agreed above

  - Option 1: Reuse legacy MPR requirements for each power class (OPPO, Huawei, Nokia)

  - Option 2: Further evaluate with 3PA configuration (Apple)

<Sub-topic 1-4> Supported layer for mode 0

<Way forward >: FFS on Supported MIMO layer for ULFPTx mode 0

<Topic 2> Asymmetric TRPs

<Sub-topic 2-1> UE RF impact

<Agreement>: Whether to assume different TCI state between DL and UL

- Assume different TCI state between DL and UL

<Way forward>: Whether to study FR2 enhancements for asymmetric TRPs scenario in Rel-19

- RAN4 will not study FR2 enhancements until RAN1 considers the scenario in Option 1

  - Option 1: Yes, UL-specific TRPs are assumed to have limited DL capability

  - Option 2: No, need more check

  - Option 3: Check with RAN1 sending an LS