3GPP TSG-RAN WG4 Meeting # 112-bis					R4-2417130

Hefei Meeting, Oct. 10th – Oct 14th, 2024

Title:	TP to TR 38.719-02-01 CA_n71-n78

Source:	Nokia, Etisalat

Agenda item:	5.3.3

Document for:	Approval

# 1	Introduction

This is a TP to TR 38.719-02-01 to add CA_n71-n78.

************************************* Start of TP*****************************************

## 5.x	CA_n71-n78

### 5.x.1	Common for 1 band UL and 2 bands UL CA

#### 5.x.1.1	Operating bands for CA

Table 5.x.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n71 | 663-698 | 617-652 | FDD |
| n78 | 3300-3800 | 3300-3800 | TDD |

#### 5.x.1.2	Channel bandwidths per operating band for CA

Table 5.x.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n71A-n78C | CA_n78C CA_n71A-n78A CA_n71A-n78C | n71 | 5, 10, 15, 20 | 4 and 5 |
|  |  | n78 | CA_n78C_BCS4 & 5 |  |

#### 5.x.1.3	UE co-existence studies for 1 band UL

UL studies have been completed for CA_n71A-n78A, but the intra-band non-contiguous ULCA must be analyzed for impact into the other FDD band.

Table 5.X.1.3-3 summarizes frequency ranges where IMD products caused by one UL band with 2CC intra-band UL CA may occur for headline

Table 5.x.1.3-3: Intra-band ULCA IMD overlap with the other DL band analysis.

| All in MHz | flow | fhigh | BB IMD range3 |  |  |
| --- | --- | --- | --- | --- | --- |
| n78 fUL | 3300 | 3800 | Order | flow | fhigh |
| n71 fDL | 663 | 698 |  |  |  |
| 2CCBW1 | Minimum | Maximum | IMD2 | Min2CCBW | Max2CCBW |
|  | 70 | 200 | (1-1) | 70 | 200 |
| Close to UL IMD range2 |  |  | IMD4 | 2*Min2CCBW | 2*Max2CCBW |
| Order | flow | fhigh | (2-2) | 140 | 400 |
| IMD3 | fULlow-Max2CCBW | fULhigh+Max2CCBW | IMD6 | 3*Min2CCBW | 3*Max2CCBW |
| (2-1) | 3100 | 4000 | (3-3) | 210 | 600 |
| IMD5 | fULlow-2*Max2CCLBW | fULhigh+2*Max2CCBW | Close to H2 IMD range4 |  |  |
| (3-2) | 2900 | 4200 | Order | flow | fhigh |
| IMD7 | fULlow-3*Max2CCBW | fULhigh+3*Max2CCBW | IMD4 | 2*fULlow-Max2CCBW | 2*fULhigh+Max2CCBW |
| (4-3) | 2700 | 4400 | (3-1) | 6400 | 7800 |
| IMD9 | fULlow-4*Max2CCBW | fULhigh+4*Max2CCBW | IMD6 | 2*fULlow-2*Max2CCBW | 2*fULhigh+2*Max2CCBW |
| (5-4) | 2500 | 4600 | (4-2) | 6200 | 8000 |
| IMD11 | fULlow-5*Max2CCBW | fULhigh+5*Max2CCBW | Close to H3 IMD range4 |  |  |
| (6-5) | 2300 | 4800 | Order | flow | fhigh |
| IMD13 | fULlow-6*Max2CCBW | fULhigh+6*Max2CCBW | IMD5 | 3*fULlow-Max2CCBW | 3*fULhigh+Max2CCBW |
| (7-6) | 2100 | 5000 | (4-1) | 9700 | 11600 |
| Analysis | No issues found |  |  |  |  |
| NOTE 1: 2CCBW is the instantaneous transmit bandwidth of the two intra-band UL CCs: - The minimum 2CCBW for contiguous / non-contiguous intra-band ULCA is 0 / minimum UL channel bandwidth - The maximum 2CCBW for contiguous / non-contiguous ULCA is Min(maximum aggregated bandwidth / maximum separation bandwidth(600MHz),fULhigh-fULlow) NOTE 2: The close to UL IMD range is the most critical when the victim DL band in proximity to the UL band: - For contiguous/non-contiguous intra-band ULCA within a TDD band, IMD order up to 9/7 should be considered and MPR assumed - For intra-band ULCA within a FDD band, IMD order up to 13 should be considered for bands in the same band group and MPR is not assumed. If justified by poor filtering performance, higher order IMD may need to be specified. NOTE 3: The BB IMD range should only be considered if the DL band is below the UL band and for non-contiguous ULCA within a TDD band >3GHz (assuming CA with 450MHz bands is not considered) - IMD2 is not considered assuming CA with 450MHz bands is not considered - IMD4 is considered for FDD or SimRx/Tx TDD bands <1GHz - IMD6 is considered for FDD or SimRx/Tx TDD bands <1.68GHz NOTE 4: The harmonic 2 and 3 IMD ranges should only be considered if the DL band is above the UL band |  |  |  |  |  |

#### 5.x.1.4	∆TIB,c and ∆RIB,c values

Not relevant.

#### 5.x.1.5	REFSENS requirements

Not relevant.

#### 5.x.1.6	OOB blocking exception requirements

Not relevant.

### 5.x.2	Specific for 2 bands UL CA

#### 5.x.2.1	Maximum output power for inter-band CA

Not relevant.

#### 5.x.2.2	UE co-existence studies for 2 bands UL

Table 5.x.2.2-2 lists Band n71 + Band n78 2 bands UL CA(3CC) triple beat IMD analysis for Band n71- Band n78

Table 5.x.2.2-2: Band n71 and Band n78 for 3CC UL IMD products

| Band / CA1 | n71 |  | CA_n78B/C |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | fx_low | fx_high | fy_low | fy_high |
| fUL | 663-698 |  | 3300-3800 |  |
| fDL | 617-652 |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 -200 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | 463-663 |  | 698-898 |  |
| Analysis | There is an issue of IMD3, but no requirements, since the bands are not in same or adjacent bandgroups |  |  |  |
| NOTE 1: If the two bands are not part of the same or adjacent band groups as defined in table A.1, the analysis can be ignored. NOTE 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

#### 5.x.2.3	REFSENS requirements

Not relevant.

************************************* End of TP******************************************