3GPP TSG-RAN WG4 Meeting # 112-bis				        draftR4-2417156

Hefei, China, 14th – 18th October 2024

Source:	Skyworks, Murata, (..)

Title:	WF on DC_(n)8AA test point and MSD test point clean-up

Agenda item:	5.2

Document for:	Approval

Introduction

At RAN4 meeting #112, it was agreed in WF [1] that for DC_(n)8AA only single switched uplink (UL) operation is supported and companies were invited to evaluate the following two MSD test points:

| EN-DC configuration / channel allocations /MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC configuration | E-UTRA/NR band | FC (UL) (MHz) | Channel bandwidth (MHz) | UL allocation (LCRB) | FC (DL) (MHz) | MSD (dB) | Duplex mode |
| DC_(n)8AA | 8 | N/A | 10 | N/A | 935 | FFS | FDD |
|  | n8 | 905 | 20 | 20 (RBstart = 86) | 950 | FFS |  |
|  | 8 | 910 | 10 | 25 (RBstart = 25) | 955 | FFS |  |
|  | n8 | N/A | 20 | N/A | 940 | FFS |  |
| NOTE 2: The transmitters powers shall be set to PUMAX, as defined in TS 38.101-1 [2], TS 38.101-2 [3], and TS 36.101 [4], with additional limits on configured maximum output power for the uplink according to clause 6.2B.4. |  |  |  |  |  |  |  |

At this meeting, one company observed [2] that the test point with single-uplink transmission in E-UTRA Band 8 does not comply to the clause 7.3B.2.1 core requirements on the placement of downlink (DL) MCG/SCG relative to the UL band. It was also observed in [2] that this issue is not specific to DC_(n)8AA as several previously agreed test points also fail to meet these requirements. During the meeting, the proponents discussed the options proposed to resolve this issue and it was agreed to pursue proposal 1- option 3 [2]. This WF captures these agreements.

WF

<Way forward on correction of DC_(n)XAA configurations in 38.101-3 for REFSENS>

WF1: For EN-DC intra-band contiguous Reference Sensitivity where single-uplink only is supported, change the MCG/SCG placement rule so that the MSD test point verifies only the LTE anchor SCG MSD by:

1.1 adopting the following changes to the core requirements of clause 7.3B.2.1:

“For EN-DC configurations where uplink is not available in either the MCG or the SCG or for EN-DC configurations where the UE only supports single uplink operation, reference sensitivity requirements are verified with the downlink carrier(s) from the cell group without uplink configured closer to the uplink operating band than any of the downlink carriers from the cell group with uplink.”

1.2: replacing the 0dB MSD of the NR DL MCG with “N/A”.

1.3: wording for 1.1 may be refined for the CR

WF2: The single-uplink MSD test points which specify the NR band SCG MSD may be removed from Table 7.3B.2.1-1.

<Way forward on MSD test point for DC_(n)8AA>

WF3: Interested companies are invited to evaluate the E-UTRA B8 single-uplink MSD for the following DC_(n)8AA MSD test point. The MSD values proposed by [2,3] are captured in brackets as an initial input to the next RAN4 meeting.

| EN-DC configuration / channel allocations /MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC configuration | E-UTRA/NR band | FC (UL) (MHz) | Channel bandwidth (MHz) | UL allocation (LCRB) | FC (DL) (MHz) | MSD (dB) | Duplex mode |
| DC_(n)8AA | 8 | N/A | 10 | N/A | 935 | [19] or [21.5] | FDD |
|  | n8 | 905 | 20 | 20 (RBstart = 86) | 950 | N/A |  |
| NOTE 2: The transmitters powers shall be set to PUMAX, as defined in TS 38.101-1 [2], TS 38.101-2 [3], and TS 36.101 [4], with additional limits on configured maximum output power for the uplink according to clause 6.2B.4. |  |  |  |  |  |  |  |

WF1 – WF 2 MSD test point simplification example

The changes of WF1 and WF2 simplifications are captured in the table below (based on Table 7.3B.2.1-1).

| EN-DC configuration / channel allocations /MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC configuration | E-UTRA/NR band | FC (UL) (MHz) | Channel bandwidth (MHz) | UL allocation (LCRB) | FC (DL) (MHz) | MSD (dB) | Duplex mode |
| DC_(n)3AA | 3 | N/A | 15 | N/A | 1842.5 | 6.2 | FDD |
|  | n3 | 1770.0 | 30 | [10 (RBstart = 150)] | 1865.0 | N/A |  |
| DC_(n)5AA | 5 | 826.5 | 5 | N/A | 871.5 | 5.2 | FDD |
|  | n5 | 839 | 20 | 20 (RBend = 105) | 884 | N/A |  |
| DC_(n)5AA | 5 | 829 | 10 | N/A | 874 | 5.2 |  |
|  | n5 | 841.5 | 15 | 20 (RBend = 78) | 886.5 | N/A |  |
| DC_(n)5AA | 5 | 831.5 | 5 | N/A | 876.5 | 5.2 |  |
|  | n5 | 841.5 | 15 | 20 (RBend = 78) | 886.5 | N/A |  |
| DC_(n)5AA | 5 | 834 | 10 | N/A | 879 | 1.5 |  |
|  | n5 | 844 | 10 | 25 (RBend = 51) | 889 | N/A |  |
| DC_(n)12AA | 12 | 703.5 | 5 | N/A | 733.5 | 4.5 | FDD |
|  | n12 | 711 | 10 | 20 (RBend = 51) | 741 | N/A |  |
| DC_(n)71AA | 71 | 665.5 | 5 | 5 (RBend =24) | 619.5 | 0 | FDD |
|  | n71 | 675.5 | 15 | 15 (RBstart = 0) | 629.5 | 1.8 |  |
| DC_(n)71AA | 71 | 670.5 | 15 | 15 (RBend = 74) | 624.5 | 0 |  |
|  | n71 | 680.5 | 5 | 5 (RBstart = 0) | 634.5 | 1.6 |  |
| DC_(n)71AA | 71 | 668 | 10 | 10 (RBend = 49) | 622 | 0 |  |
|  | n71 | 678 | 10 | 10 (RBstart = 0) | 632 | 1.7 |  |
| DC_(n)71AA | 71 | 668 | 10 | 10 (RBstart = 0) | 622 | 17.2 |  |
|  | n71 | 678 | 10 | 10 (RBend = 51) | 632 | 29.4 |  |
| DC_(n)71AA | 71 | 665.5 | 5 | 5 (RBend =24) | 619.5 | 0 | FDD |
|  | n71 | 675.5 | 151 | 15 (RBstart = 0) | 6321 | 2.5 |  |
| DC_(n)71AA | 71 | 670.5 | 15 | 15 (RBend = 74) | 624.5 | 0 |  |
|  | n71 | 680.5 | 51 | 5 (RBstart = 0) | 6371 | 2.2 |  |
| DC_(n)71AA | 71 | 668 | 10 | 10 (RBend = 49) | 622 | 0 |  |
|  | n71 | 678 | 101 | 10 (RBstart = 0) | 634.51 | 2.5 |  |
| DC_(n)71AA | 71 | 668 | 10 | 10 (RBstart = 0) | 622 | 17.2 |  |
|  | n71 | 678 | 101 | 10 (RBend = 51) | 634.51 | 29.1 |  |
| DC_(n)71AA3 | 71 | N/A | 10 | N/A | 642.0 | [21.3] | FDD |
|  | n71 | 673.0 | 20 | [5 (RBstart = 0)] | 627.0 | N/A |  |
| DC_(n)71AA3 | 71 | N/A | 15 | N/A | 639.5 | [5.4] |  |
|  | n71 | 670.5 | 15 | [5 (RBstart = 0)] | 624.5 | N/A |  |
| DC_(n)71AA4 | 71 | 680.5 | 5 | N/A | 639.5 | [6.8] |  |
|  | n71 | 670.5 | 151 | [5 (RBstart = 2)] | 627.0 | N/A |  |
| DC_(n)71AA5 | 71 | 680.5 | 5 | N/A | 634.5 | [6.4] |  |
|  | n71 | 670.5 | 15 | [5 (RBstart = 0) | 624.5 | N/A |  |
| NOTE 1: In accordance to BCS1, the NR uplink bandwidth is specified as in this table, but the corresponding NR downlink bandwidth is 5 MHz larger. NOTE 2: The transmitters powers shall be set to PUMAX, as defined in TS 38.101-1 [2], TS 38.101-2 [3], and TS 36.101 [4], with additional limits on configured maximum output power for the uplink according to clause 6.2B.4. NOTE 3: Applicable only to BCS 2. NOTE 4: Applicable only to BCS 1. NOTE 5: Applicable only to BCS 0. |  |  |  |  |  |  |  |

- References

- R4-2414274 WF on introduction of PC2 and 40MHz CBW in NR band n28, 3GPP TSG-RAN WG4 Meeting # 112, Maastricht, The Netherlands, Skyworks Solutions, Inc., Murata Manufacturing Co. Ltd.

- R4-2416228 DC_(n)8AA MSD and MSD test point clean-up, 3GPP TSG-RAN WG4 Meeting # 112-bis, Hefei, China, Skyworks Solutions, Inc.

- R4-2415728 MSD for DC_(n)8AA, 3GPP TSG-RAN WG4 Meeting # 112-bis, Hefei, China, Murata Manufacturing Co. Ltd.