3GPP TSG-RAN WG4 Meeting #112-bis	R4-2417172

Hefei, China, 14 – 18 October 2024

Source:	Ericsson

Title:	TP for TR 36.719-01-01 to add CA_7A-68A

Agenda item:			5.4.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 36.719-01-01 to add CA_7A-68A.

# 2. Text Proposal

# ---Start of changes---

## 5.3	LTE-A inter-band CA for 2 bands DL with 2 bands UL

### 5.3.x	CA_7-68

#### 5.3.x.1	Channel bandwidths per operating band for CA

Table 5.3.x.1-1: Inter-band CA operating bands

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
| 68 | 698 MHz | – | 728 MHz | 753 MHz | – | 783 MHz | FDD |

Table 5.3.6-1: E-UTRA CA configurations and bandwidth combination sets

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_7A-68A | CA_7A-68A | 7 |  |  | Yes | Yes | Yes | Yes | 35 | 0 |
|  |  | 68 |  |  | Yes | Yes | Yes |  |  |  |

#### 5.3.x.2	Co-existence studies

For 2UL / 2DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.3.x.2-1.

Table 5.3.x.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 698 | 728 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1396 | 1456 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2094 | 2184 | 7500 | 7710 |
| Two tone 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1872 | 1772 | 3198 | 3298 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1174 | 1044 | 4272 | 4442 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3896 | 4026 | 5698 | 5868 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3744 | 3544 | 6396 | 6596 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 476 | 316 | 6772 | 7012 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4594 | 4754 | 8198 | 8438 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9582 | 9272 | 412 | 222 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10698 | 11008 | 5292 | 5482 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 6314 | 6044 | 2816 | 3046 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8896 | 9166 | 7094 | 7324 |

Based on Table 5.3.x.2-1, there are no IMD issues.

Table 5.3.x.2-2: Requirements for uplink inter-band carrier aggregation (two bands)

| E-UTRA CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_7-68 | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 15, 21, 26 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 15, 21, 26 |
|  |  | Frequency range |  | 2595 |  | - |  | 2620 |  | -40 |  | 1 |  | 15, 21 |  |

#### 5.3.x.3	∆TIB and ∆RIB values

∆TIB and ∆RIB values for CA_7-28 are reused.

Table 5.4.x.3-1: IB,c

| CA_7-68 | 7 | 0.3 |
| --- | --- | --- |
|  | 68 | 0.3 |

Table 5.4.x.3-2: R IB,c

| CA_7-68 | 7 | 0 |
| --- | --- | --- |
|  | 68 | 0 |

#### 5.3.x.4	REFSENS requirements

#### Based on analysis of 5.3.x.2, there are no additional MSD requirements for this combination.

---End of changes---

# Reference

[1] 		RP-242086, Rel-19 LTE-Advanced Carrier Aggregation for x bands (2<=x<= 6) DL with y bands (y=1, 2) UL; Huawei