3GPP TSG-RAN WG4 Meeting #112bis	R4-2417176

Hefei, China, 14th – 18th October 2024

Title: 	WF on NTN and IoT-NTN bands

Agenda Item:	5.17, 5.18, 5.19

Source: 	Moderator (Inmarsat)

Document for:	Approval

# Topic #1: NR-NTN S-band

### Sub-topic 1-1: General Requirements and Coexistence Aspects

#### Issue 1-1-1: Regional Applicability

- Agreement

  - Consider adding a note in clause 5.2 operating band to clarify regional applicability of band n252

#### Issue 1-1-2: General Methodology for UE to UE Coexistence between n252 UL and n2/b2/n25/b25 DL

- Agreement:

  - Define a new more relaxed spurious emission requirement for TN-NTN UE to UE coexistence compared to -50dBm/MHz, that allows the NTN service to work effectively, across the full n252 band, whilst still protecting the TN UE at least in the majority of cases

    - Further discuss the actual value and methodology for the requirement (e.g. corresponding separation assumption)

    - Companies are requested to propose what emission level in dBm/MHz could be achieved for frequencies below 1995 MHz, considering the UE RF implementation, different uplink channel bandwidths and RB allocations

    - Further discuss how to capture the solutions in the specification

#### Issue 1-1-3: TN-NTN Isolation Distance

- Agreement

  - For band n252 UE-to-UE co-existence study, consider a new TN-NTN separation distance lower than 1.5 km and further study the appropriate separation based on coexistence analysis

    - partial geographic overlap should also be considered.

      - [There could be impact on NTN DL from TN DL]

    - Check if this approach can be applied to other NTN bands in the future

### Sub-topic 1-2: UE RF

Sub-topic description: General UE RF Requirements

#### Issue 1-2-1: SS Raster

- Agreement

  - Specify both Case A and [Case B] as follows

| NTN satellite operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n252 | 15 kHz | Case A | 5456 – <1> – 5494 |
|  | [30 kHz] | [Case B] | [5460 – <1> – 5488] |
| NOTE : SS Block pattern is defined in clause 4.1 in 3GPP TS 38.213 [7]. |  |  |  |

#### Issue 1-2-3: UE Maximum Output Power

- Agreement

  - Specify PC3 maximum output power for band n252 as follows:

| NR satellite band | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| n252 | 23 | ±2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 3 is default power class unless otherwise stated |  |  |

#### Issue 1-2-3: Two antenna port PREFSENS

- Agreement

  - FFS

#### Issue 1-2-4: UL Configuration for REFSENS

- Agreement

  - Agree Option 1

| Operating band / SCS (kHz) / Channel bandwidth (MHz) / Duplex mode |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS | 5 | 10 | 15 | 20 | Duplex Mode |
|  | 15 | 25 | 50 | 75 | 100 |  |
| n252 | 30 |  | 24 | 36 | 50 | FDD |
|  | 60 |  | 10 | 18 | 24 |  |
| NOTE: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth in Table 5.3.2-1. |  |  |  |  |  |  |

#### Issue 1-2-5: In-band blocking

- Agreement

  - Agree Option 1

Table 7.6.2-2: In-band blocking for NR satellite bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| Operating Band | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -56 | -44 |
| n252, n254, n255, n256 | Finterferer (offset) | MHz | -BWChannel/2 – FIoffset, case 1 and BWChannel/2 + FIoffset, case 1 | ≤ -BWChannel/2 – FIoffset, case 2 and ≥ BWChannel/2 + FIoffset, case 2 |
|  | Finterferer | MHz | NOTE 2 | FDL_low – 15 to FDL_high + 15 |
| NOTE 1: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 2: For each carrier frequency, the requirement applies for two interferer carrier frequencies: a: -BWChannel/2 – FIoffset, case 1; b: BWChannel/2 + FIoffset, case 1 |  |  |  |  |

#### Issue 1-2-6: Out-of-band Blocking

- Agreement

  - FFS

#### Issue 1-2-7: Narrow Band Blocking

- Agreement

  - Agree to Option 1

- Table 7.6.4-1: Narrow Band Blocking

| Operating Band | Parameter | Unit | Channel Bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 |
| n252, n254, n255, n256 | Pw | dBm | PREFSENS + channel-bandwidth specific value below |  |  |  |
|  |  |  | 16 | 13 | 14 | 16 |
|  | Puw (CW) | dBm | -55 |  |  |  |
|  | Fuw (offset SCS= 15 kHz) 3 | MHz | BWChannel2+0.2SCS+0.5+0.5SCS |  |  |  |
|  | Fuw (offset SCS= 30 kHz)3 | MHz | NA |  |  |  |
| NOTE 1: The transmitter shall be set a 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in clause 7.3.2 with PCMAX_L,f,c defined in clause 6.2.4 NOTE 2: The PREFSENS power level is specified in clause 7.3.2. NOTE 3: Fuw shall be rounded to half of SCS. |  |  |  |  |  |  |

### Sub-topic 1-3: SAN RF

Sub-topic description: SAN RF requirements for band n252

#### Issue 1-3-1: SAN Operating Band

- Agreement

  - Specify the following SAN operating band for band n252:

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n252 | 2000 MHz - 2020 MHz | 2180 MHz - 2200MHz | FDD |
| NOTE: Satellite bands are numbered in descending order from n256. |  |  |  |

#### Issue 1-3-2: SAN Channel Bandwidths and SCS

- Agreement

  - Specify the following SAN channel bandwidths and SCS for band n252

| SAN Operating Band | SCS (kHz) | SAN channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 |
|  | 15 | 5 | 10 | 15 | 20 |
| n252 | 30 |  | 10 | 15 | 20 |
|  | 60 |  | 10 | 15 | 20 |

#### Issue 1-3-3: SAN NR-ARFCN with 100 kHz channel raster

- Agreement

  - Specify the following SAN NR-ARFCN for band n252

| SAN operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n252 | 100 | 400000 – <20> – 404000 | 436000 – <20> – 440000 |

#### Issue 1-3-4: SAN NR-ARFCN with enhanced channel raster

- Agreement

  - Specify the following SAN NR-ARFCN for enhanced channel raster for band n252:

| SAN operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n252 | 10 | 400000 – <2> – 404000 | 436000 – <2> – 440000 |

#### Issue 1-3-5: SAN SS Raster

- Agreement:

  - Agree Option 2 with both Case A and [Case B]

    - Need check if the 30KHz SCS SS raster is needed and whether case B is correct

Table 5.4.3.3-1: Applicable SS raster entries per operating band (FR1-NTN)

| SAN operating band | SS Block SCS | SS Block pattern (NOTE) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n252 | 15 kHz | Case A | 5456 – <1> – 5494 |
|  | 30 kHz | Case B | 5460 – <1> – 5488 |
| NOTE: SS Block pattern is defined in clause 4.1 in TS 38.213 [7]. |  |  |  |

#### Issue 1-3-5: Other SAN RF Requirements

- Agreement

  - Reuse existing SAN 1-H and 1-O RF requirements for band n252

### Sub-topic 1-4: RRM Requirements

Sub-topic description: RRM requirements for band n252

#### Issue 1-4-1: Band Grouping

- Agreement

  - Agree Option 1 as follows

Table 3.5.2A-1: NR frequency band groups for satellite access in FR1

| Group | NR FDD |  |
| --- | --- | --- |
|  | Band group notation | Operating bands |
| A | NR_FDD_SAB_FR1_A | n254, n255, n256, n252 |
| B | NR_FDD_SAB_FR1_B |  |
| C | NR_FDD_SAB_FR1_C |  |
| D | NR_FDD_SAB_FR1_D |  |
| E | NR_FDD_SAB_FR1_E |  |
| F | NR_FDD_SAB_FR1_F |  |
| G | NR_FDD_SAB_FR1_G |  |
| H | NR_FDD_SAB_FR1_H |  |
| I | NR_FDD_SAB_FR1_I |  |
| J | NR_FDD_SAB_FR1_J |  |

## Other Sub-topics/Issues

Agreement:

- Other topics and issues are postponed to future meetings

# Topic #2 IoT-NTN S-band

### Sub-topic 2-1: UE RF

Sub-topic description: UE RF requirements

#### Issue 2-1-1: UE operating bands

- Agreement

  - Specify UE operating band as follows:

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 252 | 2000 MHz | - | 2020 MHz | 2180 MHz | - | 2200 MHz | FDD |
| NOTE 1: Satellite bands are numbered in descending order from 256 NOTE 2: UE assigned to channels and allocated frequency resources in the lower portion of Band 253 may experience blocking or harmful interference from terrestrial networks in adjacent or nearby frequencies when operating in the proximity with terrestrial base stations. |  |  |  |  |  |  |  |

#### Issue 2-1-2: E-UTRA Channel Numbers

- Agreement

  - Specify E-UTRA channel numbers as follows:

| E-UTRA Operating Band | ΔFRaster (kHz) | Downlink |  |  | Uplink |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | FDL_low (MHz) | NOffs-DL | Range of NDL (First – <Step size> – Last) | FUL_low (MHz) | NOffs-UL | Range of NUL (First – <Step size> – Last) |
| 252 | 100 | 2180 | 228301 | 228301 –<1>- 228500 | 2000 | 261069 | 261069 –<1>- 261268 |
| NOTE 1: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. This implies that the first 7 channel numbers at the lower operating band edge and the last 6 channel numbers at the upper operating band edge shall not be used for channel bandwidth of 1.4 MHz. |  |  |  |  |  |  |  |

#### Issue 2-1-2: UE TX-RX Separation

- Agreement

  - Consider 180MHz default Tx-Rx separation as starting point.

    - Support of flexible Tx-Rx separation is not precluded based

    - Applicability for other bands should also be considered

#### Issue 2-1-3: UE TX Regulatory

- Agreement

  - Further discuss whether and how NTN antenna gain could be considered when evaluating other IoT-NTN UE TX requirements related to regulatory requirement.

#### Issue 2-1-4: UE Maximum Output Power for Cat M1 and NB1/NB2

- Agreement

  - FFS on the solutions

    - FFS on whether the general solution is needed.

#### Issue 2-1-6: UE Cat-M1 Reference Sensitivity

- Agreement:

  - Agree to either Option 1/Option 2 (they are equal)

- Table: Reference sensitivity for FDD UE category M1 QPSK PREFSENS

| NTN Band | REFSENS (dBm) | Duplex Mode |
| --- | --- | --- |
| 252 | -102.2 | FDD |

- Table: Reference sensitivity for HD-FDD UE category M1 QPSK PREFSENS

| NTN Band | REFSENS (dBm) | Duplex Mode |
| --- | --- | --- |
| 252 | -103 | HD-FDD |

#### Issue 2-1-7: UE Cat-NB1/NB2 REFSENS

- Agreement:

  - Agree to Option 1

| Operating band | REFSENS [dBm] |
| --- | --- |
| According to subclause 5.2B | - 108.2 |

#### Issue 2-1-8: UL Configuration for REFSENS for Cat-M1

- Agreement:

  - Agree to Option 1 as follows

Table 7.3A-3: FDD UE category M1 Uplink configuration for reference sensitivity

| E-UTRA Band | NRB | Duplex Mode |
| --- | --- | --- |
| 252 | 61 | FDD and HD-FDD |
| 253 | 61 | FDD and HD-FDD |
| 254 | 61 | FDD and HD-FDD |
| 255 | 61 | FDD and HD-FDD |
| 256 | 61 | FDD and HD-FDD |
| NOTE 1: 1 refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3A-1). |  |  |

#### Issue 2-1-9: In-band blocking for Cat-M1

- Agreement:

  - Agree to Option 1

- Table 7.6A.2-2: In-band blocking

| E-UTRA band | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | PInterferer | dBm | -56 | -44 |
|  | FInterferer (offset) | MHz | =-BW/2 – FIoffset,case 1 & =+BW/2 + FIoffset,case 1 | ≤-BW/2 – FIoffset,case 2 & ≥+BW/2 + FIoffset,case 2 |
| 256, 255, 254, 253, 252 | FInterferer | MHz | (NOTE 2) | FDL_low – 15 to FDL_high + 15 |
| NOTE 1: For certain bands, the unwanted modulated interfering signal may not fall inside the UE receive band, but within the first 15 MHz below or above the UE receive band NOTE 2: For each carrier frequency the requirement is valid for two frequencies: a. the carrier frequency -BW/2 - FIoffset, case 1 and b. the carrier frequency +BW/2 + FIoffset, case 1 NOTE 3: FInterferer range values for unwanted modulated interfering signal are interferer center frequencies |  |  |  |  |

#### Issue 2-1-10: Out-of-band blocking for Cat-M1

- Agreement

  - FFS

#### Issue 2-1-11: Out-of-band blocking for Cat-NB1/NB2

- Agreement

  - Further study whether same issue as B256 applies

### Sub-topic 2-2: SAN RF

Sub-topic description: SAN RF Requirements

#### Issue 2-2-1: SAN Operating Bands

- Agreement:

  - Specify SAN operating band for band 252 as follows:

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 252 | 2000 MHz | - | 2020 MHz | 2180 MHz | - | 2200 MHz | FDD |
| NOTE 1: Satellite bands are numbered in descending order from 256 NOTE 2: UE assigned to channels and allocated frequency resources in the lower portion of Band 253 may experience blocking or harmful interference from terrestrial networks in adjacent or nearby frequencies when operating in the proximity with terrestrial base stations. |  |  |  |  |  |  |  |

#### Issue 2-2-2: E-UTRA Channel Numbers

- Agreement:

  - Specify SAN E-UTRA Channel Numbers for band 252 as follows:

| E-UTRA Operating Band | ΔFRaster (kHz) | Downlink |  |  | Uplink |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | FDL_low (MHz) | NOffs-DL | Range of NDL (First – <Step size> – Last) | FUL_low (MHz) | NOffs-UL | Range of NUL (First – <Step size> – Last) |
| 252 | 100 | 2180 | 228301 | 228301 –<1>- 228500 | 2000 | 261069 | 261069 –<1>- 261268 |
| NOTE: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. This implies that the first 7 channel numbers at the lower operating band edge and the last 6 channel numbers at the upper operating band edge shall not be used for channel bandwidth of 1.4 MHz. |  |  |  |  |  |  |  |

### Sub-topic 2-3: RRM Requirements

Sub-topic description: RRM requirements

#### Issue 2-3-1: Band grouping for Cat NB1/NB2

- Agreement:

  - Specify the following band grouping for Cat NB1/NB2

Table 3.5.1A-1: Band groups for NB-IoT for satellite access

| Group | E-UTRA FDD |  |
| --- | --- | --- |
|  | Band group notation | Operating bands |
| A | NFDD_SAB_A |  |
| B | NFDD_SAB_B |  |
| C | NFDD_SAB_C |  |
| D | NFDD_SAB_D |  |
| E | NFDD_SAB_E |  |
| F | NFDD_SAB_F |  |
| G | NFDD_SAB_G | 253, 254, 255, 256, 252 |
| H | NFDD_SAB_H |  |
| I | NFDD_SAB_I |  |
| J | NFDD_SAB_J |  |
| K | NFDD_SAB_K |  |
| L | NFDD_SAB_L |  |
| M | NFDD_SAB_M |  |
| N | NFDD_SAB_N |  |

#### Issue 2-3-2: Band grouping for Cat M1

- Agreement:

  - Agree to Option 1 as follows:

- Table 3.5.1A-2: Band groups for category M1 for satellite access

| Group | E-UTRA FDD |  |
| --- | --- | --- |
|  | Band group notation | Operating bands |
| A | FDD-M1_SAB_A | 253, 255 |
| B | FDD-M1_SAB_B | 254, 256, 252 |
| C | FDD-M1_SAB_C |  |
| D | FDD-M1_SAB_D |  |
| E | FDD-M1_SAB_E |  |
| F | FDD-M1_SAB_F |  |
| G | FDD-M1_SAB_G |  |
| H | FDD-M1_SAB_H |  |
| I | FDD-M1_SAB_I |  |
| J | FDD-M1_SAB_J |  |
| K | FDD-M1_SAB_K |  |
| L | FDD-M1_SAB_L |  |
| M | FDD-M1_SAB_M |  |
| N | FDD-M1_SAB_N |  |

## Other Sub-topics/Issues

Agreement:

- Other topics and issues are postponed to future meetings

# Topic #3: NR-NTN L-bands

### Sub-topic 3-1: General Aspects and Regulatory Input

Sub-topic description: Regulatory aspects

#### Issue 3-1-1: ETSI additional blocking Requirements

- Agreement

  - Further discuss whether and how the additional ETSI requirements for in-band/out-of-band blocking should be captured.

### Sub-topic 3-2: UE RF

Sub-topic description: System parameters and UE RF aspects

#### Issue 3-2-1: UE Duplexer

- Agreement:

  - Further discuss whether implementation with one duplexer is feasible, considering the requirements to protect Radio Astronomy in the 1660-1668MHz range between the two UL.

### Sub-topic 3-3: SAN RF

Sub-topic description: SAN RF requirements

#### Issue 3-3-1: SAN Operating Bands

- Agreement

  - Specify the following SAN operating bands for bands n253, n251 and n250:

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n253 | 1668 MHz - 1675 MHz | 1518 MHz - 1525 MHz | FDD |
| n251 | 1626.5 MHz - 1660.5 MHz | 1518 MHz - 1559 MHz | FDD |
| n250 | 1668 MHz - 1675 MHz | 1518 MHz - 1559 MHz | FDD |
| NOTE: Satellite bands are numbered in descending order from n256. |  |  |  |

#### Issue 3-3-2: SAN Channel Bandwidths

- Agreement:

  - Specify the following initial SAN DL channel bandwidths for bands n253, n251 and n250:

    - FFS which channel bandwidth applies for UL

| SAN Operating Band | SCS (kHz) | SAN channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 30 (NOTE) |
|  | 15 | 5 |  |  |  |  |
| n253 | 30 |  |  |  |  |  |
|  | 60 |  |  |  |  |  |
|  | 15 | 5 | 10 | 15 | 20 | 30 |
| n251 | 30 |  | 10 | 15 | 20 | 30 |
|  | 60 |  | 10 | 15 | 20 | 30 |
|  | 15 | 5 | 10 | 15 | 20 | 30 |
| n250 | 30 |  | 10 | 15 | 20 | 30 |
|  | 60 |  | 10 | 15 | 20 | 30 |
| NOTE: Deployment of 30 MHz channel bandwidth for NTN SAN needs to be preceded by introduction of all applicable Tx RF, Rx RF, and demodulation requirements. |  |  |  |  |  |  |

#### Issue 3-3-3: SAN NR-ARFCN

- Agreement

  - Specify the SAN NR-ARFCN for 100kHz channel raster for bands n253, n251 and n250 as follows:

| SAN operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n253 | 100 | 333600– <20> – 335000 | 303600 – <20> – 305000 |
| n251 | 100 | 325300– <20> – 332100 | 303600 – <20> – 311800 |
| n250 | 100 | 333600– <20> – 335000 | 303600 – <20> – 311800 |

#### Issue 3-3-4: SAN NR-ARFCN for enhanced channel raster

- Agreement

  - Specify the SAN NR-ARFCN for 10kHz enhanced channel raster for bands n253, n251 and n250 as follows:

| SAN operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n253 | 10 | 333600– <2> – 335000 | 303600 – <2> – 305000 |
| n251 | 10 | 325300– <2> – 332100 | 303600 – <2> – 311800 |
| n250 | 10 | 333600– <2> – 335000 | 303600 – <2> – 311800 |
| NOTE 1: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. These channel numbers shall also be such that the minimum guard band for each channel bandwidth and SCS specified in Table 5.3.3-1 are met for carriers located at the upper or lower edge of an operating band. |  |  |  |

#### Issue 3-3-5: SAN SS Raster

- Agreement

  - FFS

### Sub-topic 3-4: RRM Requirements

Sub-topic description: RRM aspects

#### Issue 3-4-1: Band grouping

- Agreement:

  - Agree Option 1 as follows

- Table 3.5.2A-1: NR frequency band groups for satellite access in FR1

| Group | NR FDD |  |
| --- | --- | --- |
|  | Band group notation | Operating bands |
| A | NR_FDD_SAB_FR1_A | n254, n255, n256, n253, n251, n250, |
| B | NR_FDD_SAB_FR1_B |  |
| C | NR_FDD_SAB_FR1_C |  |
| D | NR_FDD_SAB_FR1_D |  |
| E | NR_FDD_SAB_FR1_E |  |
| F | NR_FDD_SAB_FR1_F |  |
| G | NR_FDD_SAB_FR1_G |  |
| H | NR_FDD_SAB_FR1_H |  |
| I | NR_FDD_SAB_FR1_I |  |
| J | NR_FDD_SAB_FR1_J |  |

## Other Sub-topics/Issues

Agreement:

- Other topics and issues are postponed to future meetings