3GPP TSG-RAN WG4 Meeting #112bis	R4-2417192

Hefei, China, October 14th – 18th, 2024

Agenda item:			6.4.4

Source:	Intel Corporation

Title:	WF on UE RF requirements for Rel-19 NR channel BW less than 5MHz for FR1 Phase 2

Document for:	Approval

# Topic #1: UE RF requirements for inter-band NR CA/DC with less than 5MHz CBW

## Sub-topic 1-1 Scell SSB position restrictions (Sync raster applicability)

### Issue 1-1-1: SCell SSB position restrictions

Agreement

- Update RAN4 #112 agreement as follows:

  - SCell with 15 PRB transmission bandwidth configuration within 3 MHz channel can be configured only with SS Block frequency position same as defined in TS 38.101-1 Table 5.4.3.1-2

  - For band n100

    - SCell with 12 PRB transmission bandwidth configuration within 3 MHz channel can be configured only with SS Block frequency positions same as defined in TS 38.101-1 Table 5.4.3.1-3

    - SCell with 20 PRB transmission bandwidth configuration within 5 MHz channel can be configured only with SS Block frequency positions same as defined in TS 38.101-1 Table 5.4.3.1-3

  - Capture corresponding restrictions as notes in TS 38.101-1 TS 38.101-1 Table 5.4.3.1-2 and Table 5.4.3.1-3

## Sub-topic 1-2 UE Capability signalling

### Issue 1-2-1: UE capability signalling

Agreement

- UE needs to be able to indicate for each carrier in the CA/DC band combination whether it supports less than 5 MHz channel bandwidth operation for DL/UL

  - To declare the support of less than 5 MHz bandwidth operation in the CA/DC band combination UE needs to indicate UE capabilities defined for NR < 5MHz channel bandwidth in Rel-18 and the existing UE capability applicability restriction to single carrier needs to be removed.

  - UE also indicates BCS including support of less than 5MHz channel bandwidth

- For BCS signalling the following framework is used:

  - Traditional BCSs (i.e., not BCS4 or BCS5): UE is expected to support 3MHz channel bandwidth in the band in the band combination if it indicates a corresponding BCS, which includes 3MHz channel bandwidth in RAN4 specifications. No signalling changes are expected by RAN4.

  - BCS4: if UE indicates support for BCS4 including a band that it also supports 3 MHz channel bandwidth for, then it shall support 3 MHz for CA for that band in the band combination. No signalling changes are expected by RAN4.

  - BCS5: if UE indicates support for BCS5 and minimum channel bandwidth of 3MHz as a part of BCS signalling, then it shall support 3 MHz for CA for that band in the combination.  The existing BCS5 signalling framework needs to be modified to allow indication of 3 MHz as minimum channel bandwidth.

- It should be possible for UE to indicate support of CA/DC with less than 5MHz channel bandwidth starting from Rel-18 (i.e., allow early implementation from Rel-18)

- RAN4 has not identified a need for new signaling element. The signalling details are up to RAN2.