3GPP TSG-RAN WG4 Meeting #112bis											R4-2415221

Hefei, China, 14th ‒ 18th October, 2024

Agenda item:			6.2.6

Source:	Moderator (Ericsson)

Title:	Topic summary for [112bis][117] FS_NR_IMT

Document for:	Information

# Introduction

This summary covers the IMT parameters area. The areas covered are:

- Some general aspects for the TR

- Corrections for the 4400-4800 and 7125-8400 MHz frequency ranges

- Co-existence simulations for the 14800-15350 MHz range

- Parameters for the LS response for the 14800 – 15350 MHz range

- Other aspects

# Topic #1: General aspects

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2415044 | Korea Testing Laboratory | Proposal 1: add a section in TR 38.922 that summarizes the use cases of the global frequency allocation in the three IMT-2030 bands based on the contribution from companies. |
| R4-2415048 | Korea Testing Laboratory | Text proposal capturing frequency allocations in Korea |
| R4-2415343 | Cablelabs | Proposal 1: add a section in TR 38.922 that summarizes the use cases of the global frequency allocation in the three IMT-2030 bands based on the contribution from companies. |
| R4-2415344 | Cablelabs | Text proposal capturing frequency allocations in the USA |
| R4-2416145 | Huawei, HiSilicon | Text proposal to add two notes to the TR: NOTE 1: Findings captured in this technical report represent status of Rel-18 NR specifications at the time of writing, unless otherwise stated. NOTE 2: While ITU-R W5PD asked for BS parameters, findings on BS parameters captured in this TR may be r re-used in future also for other network nodes, such as IAB, repeaters, etc. |

## Open issues summary

The contributions in this topic relate to adding information on spectrum allocations in the TR, and also on some clarifying notes.

### Sub-topic 1-1 Spectrum allocation information

This sub-topic is on adding information on spectrum allocations to the TR

Issue 1-1: Adding spectrum allocation information to the TR

- Proposals

  - Option 1: add a section in TR 38.922 that summarizes the use cases of the global frequency allocation in the three IMT-2030 bands based on the contribution from companies. (Korea Testing Laboratory, Cablelabs)

- Recommended WF

  - TBA

Issue 1-2: Korea spectrum allocations

- Proposals

  - Option 1: Approve the TP in R4-2415048 on Korea spectrum allocations (Korea Testing Laboratory)

  - Option 2: Revise the TP

  - Option 3: Do not approve the TP

- Recommended WF

  - TBA

Issue 1-3: US spectrum allocations

- Proposals

  - Option 1: Approve the TP in R4-2415344 on US spectrum allocations (Cablelabs)

  - Option 2: Revise the TP

  - Option 3: Do not approve the TP

- Recommended WF

  - TBA

### Sub-topic 1-2 Additional notes for the TR

This sub-topic is on addition of some general notes to the TR

Issue 2-2: Additional notes

- Proposals

  - Option 1: Add the following notes to the TR:

- NOTE 1: 	Findings captured in this technical report represent status of Rel-18 NR specifications at the time of writing, unless otherwise stated.

- NOTE 2:	While ITU-R W5PD asked for BS parameters, findings on BS parameters captured in this TR may be r				re-used in future also for other network nodes, such as IAB, repeaters, etc.

- Recommended WF

  - TBA

# Topic #2: Corrections for 4400-4800 and 7125-8400MHz frequency ranges

This topic handles proposed corrections for the frequency ranges for which the parameters, LS response and TR text were already agreed.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2415117 | CATT | Text proposal with several re-wordings |
| R4-2415478 | Nokia | Text proposal with several re-wordings |
| R4-2416173 | Ericsson | Text proposal correcting references and some re-wordings. |
| R4-2416413 | Huawei, HiSilicon | TR clean up |
| R4-2415118 | CATT | Text proposal with several re-wordings |
| R4-2415473 | Qualcomm | Text proposal to include simulation results demonstrating the lack of impact of higher channel bandwidths and higher UE power on ACIR |
| R4-2415479 | Nokia | Text proposal with some re-wordings |
| R4-2416172 | Ericsson | Text proposal to add missing UE spectrum mask |
| R4-2416280 | Huawei, HiSilicon | Correction to OBUE limits and remove spurious limits table (reference instead) |

## Open issues summary

### Sub-topic 2-1 Correction TPs

Sub-topic description: Update TPs to the TR

Issue 2-1: Correction TPs

The correction TPs in R4-2415117. R4-2415478, R4-2416173, R4-2416413, R4-2415118, R4-2415479, R4-2416172 and R4-2416280 are overlapping. It is proposed that the Editors of the original TPs according to the worksplit provide merged versions of the corrections CRs (taking into account comments):

- 4400-4800 range + sections 1-3: Ericsson

- 7125-8400 range BS: Huawei

- 7125-8400 range UE: Apple

Issue 2-1: Simulation results on impact of higher bandwidth or UE power.

R4-2415473 proposes to add some simulation results that establish that co-existence properties are the same for higher bandwidths or UE TX power.

- Proposals

  - Option 1: Approve the TP (Qualcomm)

  - Option 2: Revise the TP

  - Option 3: Do not approve the TP

- Recommended WF

  - TBA

# Topic #3: 15GHz co-existence simulations

This topic deals with the co-existence study for 15GHz. Some proposals for parameters are also for parameters that should be replied in the LS (e.g. bandwidth). For these parameters, the parameter value will be discussed jointly for both the simulation assumption and the LS reply, including whether the LS reply and simulation assumption can be the same.

The proposals for simulation parameters and for the LS are formally captured in topics 3 and 4.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2416000 | Samsung | Skeleton TP for co-existence results |
| R4-2415119 | CATT | Simulation results with different BS array sizes |
| R4-2415378 | ISSDU | - Subcarrier spacing of 30 kHz or larger should be considered for bandwidth higher than 100 MHz. - Beamforming should not be use on resource elements of CSI-RS - Both SU-MIMO and MU-MIMO cases should be considered. - The evaluation of ACLR for SU-MIMO and MU-MIMO cases should consider different number of data streams and TX configurations. - Both uplink MIMO and downlink MIMO cases should be considered. - We need to consider FR-1 like and FR-2 like UEs for uplink MIMO cases. In the previous sections we made the following observations: - There is a need to understand how MIMO technology and PA nonlinearities affect out-of-band emissions, particularly in both SU-MIMO and MU-MIMO cases. - Frequency dependent nonlinearity model for high-power amplifier should be further considered. As the PAs are distributed, a model for PA nonlinearity variation across the PAs need to be agreed. - Under ideal conditions, an ACLR value of approximately 45 dB can be achieved, meeting current requirements. However, PA nonlinearity causes significant degradation in ACLR. - With filtering applied, an ACLR exceeding 45 dB can be achieved in the absence of PA nonlinearity. - SU-MIMO implementation can enhance ACLR performance. - Filtering is effective in improving ACLR only if the current ACLR value is not severely degraded. - Without PA nonlinearity, ZF beamforming does not affect the ACLR performance. - MU-MIMO implementation can enhance ACLR performance. - Filtering improves ACLR only when the ACLR value is not severely degraded. - The ACLR values and behaviors for SU-MIMO and MU-MIMO cases are similar when their total number of data streams, antenna pattern and TX configuration are identical. |
| R4-2415435 | Mediatek | - Observation 1: The outcome of coexistence study is determined by the worst-case scenario. - Observation 2: For outdoor scenario, uncoordinated network operation is the bottleneck scenario for determining the ACIR irrespective of UE types. - Observation 3: The assumption of using an omnidirectional antenna without BF underestimates the interference suppression capabilities and array gains that would be realized with multiple antennas, especially since UE is likely to be equipped with multiple antennas in the 14 GHz frequency range - Observation 4: A combination of option2 and option3 can be an alternative assumption for FR1-like UE to provide both the spatial selectivity and the digital BF gains at the UE. - Observation 5: Based on the evaluation in all scenarios, the required ACIR value for DL is no higher than 26dB, and the required ACIR value for UL is no higher than 13dB - Proposal 1: As an alternative for FR1-like UE assumption, on top of option2, a constant gain G dB is applied to desired signals, and no additional gain is applied to interfering signals, assuming four antennas at the UE. |
| R4-2415476 | Qualcomm | Proposal 1: RAN4 to consider 200 MHz as a baseline in the adjacent channel coexistence framework. Proposal 2: RAN4 to consider one of the options as follows: 3072, 4096, or 2048 total antenna elements, with the subarray configuration and element/ subarray spacing to be further discussed in RAN4. Observation 1: The isotropic 0 dBi model and the analog BF (i.e., FR2 like) model represents the two extremes for the UE beamforming models with other FR1 variants provide performance falling between those two extremes. Observation 2: From system level performance, FR1-like UE beamforming models can provide comparable performance to the FR2-like UE beamforming model. Observation 3: There is an observed performance enhancement when considering directionality at the UE via single elements compared to the isotropic 0 dBi assumption. Observation 4: With isotropic model, DL and UL ACIR are observed to be around 27 and 18 dB while for analog BF model (i.e., FR2 like), DL and UL ACIR are observed to be around 23 and 14 dB. Observation 5: Different FR1-like UE beamforming model (two elements + antenna selection + digital BF model) can provide close performance to FR2-like UE beamforming model. Observation 6: Large antenna arrays at the BS enables feasible adjacent channel coexistence via directing the wanted signal to the target destination and reducing the adjacent channel interference. Observation 7: Due to the regulatory occupied bandwidth requirement, required UE ACLR is technically equal to 24 dBc. |
| R4-2415480 | Nokia | 1) The average and 5%-tile downlink throughput losses of the victim UE in all simulated cases are below 5% with downlink ACIR offset of -9 dB. 2) The throughput losses for FR1 like UE are higher than those for FR2 like UE with the use of beamforming at the FR2 like UE, except the 5%-tile throughput losses at larger ACIR offsets. 3) The average and 5%-tile uplink throughput losses in all simulated cases are below 5% with an uplink ACLR offset of -9 dB. 4) The throughput losses for FR1 like UE are higher than those for FR2 like UE with the use of beamforming at the FR2 like UE. 5) The 99.99%-tile received blocking signal power levels are less than -36 dBm and -40 dBm, respectively, in un-coordinated operation with FR1 like UE and coordinated operation with FR2 like UE. |
| R4-2415481 | Nokia | Text proposal updating simulation parameters for BS array assumption |
| R4-2415761 | Vivo | Proposal 1: For BS antenna assumption, taking 64*24 for Uma and 4*4 for Indoor as the worst case. Proposal 2: For UE Tx power, using 23 dBm as the worst assumption. Proposal 3: For ISD assumption in Uma, 350m is used as worst case. Proposal 4: For FR1-like UE, coordinated layout is assumed while for FR2-like UE, un-coordinated is assumed. |
| R4-2416002 | Samsung | Observation 1: Overall system impact to legacy services on the frequency range should be considered for coexistence study. Observation 2: Trade-off between performance benefits and coverage/energy costs are taken into account in terms of the feasibility of antenna related parameter options. Observation 3: It is observed that the throughput loss between two antenna element sizes is comparable enough for both average and edge cases, e.g., 3072 vs. 1536 AEs, as expected. Proposal 1: Potential ACIR requirements should be determined with up to 3072 AEs assumption for urban macro scenarios. |
| R4-2416170 | Ericsson | Proposal 1 RAN 4 to narrow down the co-existence simulation evaluation scope to Urban Macro and Indoor scenarios. Proposal 2 RAN4 to clarify the association of typical channel bandwidth assumption with the maximum transmit power in both UL and DL. Proposal 3 RAN4 to narrow down to ISD as 450m, to maintain the coverage of lower frequency bands and reuse the available macro grid Proposal 4 To maximize the operation of this band, RAN4 to prioritize the scenario with 20% indoor user. Keep 0% Indoor user as second priority until other companies can verify UL coverage feasibility based on 20% probability. Proposal 5 Given the timeline to finalize the parameters, we propose to narrow down to only Uncoordinated scenarios for both FR1 and FR2 UE type. Proposal 6 RAN4 to consider as 23 dBm UE maximum output power when BW is 100 MHz. Proposal 7 With the objective to narrow down the possible combinations of BS-UE antenna configurations, RAN4 to limit the combination that are feasible from UL coverage perspective and evaluate them to determine the more restrictive combination in terms of requirements. Proposal 8 For UE maximum transmission power narrow down to 23 dBm/100 MHz. Consider higher power of 26 dBm, only if the channel bandwidth is considered important to be higher than 100 MHz. Proposal 9 Most of the system parameters are dependent on other, so to make progress RAN4 should start by tentatively agreeing on parameters, and then confirm them only after verifying the overall system feasibility. |
| R4-2416171 | Ericsson | Observation 1 Uncoordinated scenario represents the worst case and drives the ACIR requirements. Observation 2 UL coverage is challenged by the presence of indoor users. UL 5%tile throughput statistics are not stable if UL coverage is poor. UL coverage can be improved by increasing UE maximum transmission power or the structure of the UE antenna array. Observation 3 To achieve stable 5%tile throughput, and consequently requirements, 26 dBm can be considered as maximum UE transmission power, if the channel bandwidth is 200 MHz. Observation 4 A lower channel bandwidth, for the same maximum transmission power, increases the PSD and consequently the coverage, so the channel bandwidth and the maximum transmission power to be proposed to ITU-R should be tied together in order to guarantee coverage feasibility. Observation 5 It would be important to share with ITU-R a typical combination of transmission power and bandwidth, and not leave the two parameters to be independent from each other. Observation 6 It is possible to improve UL performance in the scenario, by increasing the UE maximum transmission power or by reducing the channel badiwdth, to increase the PSD. Observation 7 In an Uncoordinated scenario, with ISD 450 mt and 20% indoor probability, ACIR requirements for the 4 different studied combinations are in the range of 15–17 dBs for the DL case, and 5–7 dBs for UL, for 26 dBm/200MHz or 23 dBm/100 MHz UE maximum transmission power/channel bandwidth. Observation 8 The Coordinated scenario represents a best case scenario, with respect to the uncoordinated case. The requirements in UL and DL are less restrictive than the uncoordinated case, they fall in the range 7-10 dB for DL, and below 3 dB for UL. Observation 9 With FR1 omni-directional UE in Indoor DL and UL, ACIR requirements are quite low in the proposed deployment. Observation 10 The ACIR requirements in the indoor scenario are in the order of 8 and 10 dB for UL and DL, respectively. |
| R4-2416242 | ZTE, Sanechips | Proposal 1: use the antenna array assumption in Table 2.1.1 and Table 2.1.2 with the transmission power per sub-array Proposal 2: follow the legacy approach and do not consider the Rx number from UE perspective in RAN4 SLS coexistence evaluation. Observation 1: based on the FR1 like UE assumption, then DL ACIR requirement could be relaxed around 8dB in Urban macro scenario. Observation 2: based on the FR1 like UE assumption, for non-co-located deployment, even with the existing FR1 ACIR requirements under the 20% indoor deployment, it is still challenging to meet the 5% throughput loss criteria in Urban macro scenario. If the indoor deployment is assumed to be 0%, then ACIR requirement could be relaxed around 10dB. Observation 3: based on the FR2 like UE assumption, then DL ACIR requirement could be relaxed around 10dB in Urban macro scenario. Observation 4: based on the FR2 like UE assumption, UL ACIR requirement could be relaxed around 10dB in Urban macro scenario. Observation 5: based on the FR1 like UE assumption, then DL ACIR requirement could be relaxed more than 10dB in the indoor scenario. Observation 6: based on the FR1 like UE assumption, then UL ACIR requirement could be relaxed more than 10dB in the indoor scenario. Observation 7: based on the FR2 like UE assumption, then DL ACIR requirement could be relaxed more than 10dB in the indoor scenario. Observation 8: based on the FR2 like UE assumption, then UL ACIR requirement could be relaxed more than 10dB in the indoor scenario. |

## Open issues summary

### Sub-topic 3-1 Further refinement of simulation assumptions

Issue 3-1: criteria for deciding parameters

- Proposals

  - Option 1: Only consider combinations of BS-UE configurations where UL coverage works (Ericsson)

  - Option 2: Tentatively agree parameters, then confirm after verifying the overall system feasibility (Ericsson)

- Recommended WF

  - Moderator proposal: Parameters (like BS array size, UE type and antenna gain, power, bandwidth etc.) are discussed and agreed one by one. The agreement shall firstly be captured in []. Once all parameters are agreed, a sanity check can be made that e.g. the link budget works out.

Issue 3-2: Scope of scenarios

- Proposals

  - Option 1: Limit simulations to urban macro and dense urban (Ericsson)

- Recommended WF

Issue 3-3: BS total array size assumed for the simulation for urban macro

Previous agreement:

- Recommended for further discussions

- 3072 AEs dual polarization :

- 16x24 array and 4x1 sub-array

- 4096 AEs dual polarization:

- 8x32 array and 8x1 sub-array

- 16x32 array and 4x1 sub-array

- 2048 AEs dual polarization:

- 8x32 array and 4x1 sub-array

- 1536 AEs dual polarization:

- 8x16 and 6x1 sub-array

- Other options are not precluded.

Previous agreement:

- Recommended for further discussions

- 3072 AEs dual polarization :

- 16x24 array and 4x1 sub-array

- 4096 AEs dual polarization:

- 8x32 array and 8x1 sub-array

- 16x32 array and 4x1 sub-array

- 2048 AEs dual polarization:

- 8x32 array and 4x1 sub-array

- 1536 AEs dual polarization:

- 8x16 and 6x1 sub-array

- Other options are not precluded.

- Proposals

  - Option 1: 4096 elements over both polarizations (Qualcomm, Ericsson)

    - 64 rows * 32 columns * 2 polarizations

  - Option 2: 3072 elements over both polarizations (Qualcomm, Nokia, Vivo, Samsung, ZTE)

    - 64 rows * 24 columns * 2 polarizations

  - Option 3: 2048 elements over both polarizations (Qualcomm, ZTE)

    - 32 rows * 32 columns * 2 polarizations

- Recommended WF

  - Moderator proposal: Discuss together with the same issue in topic 4, and check if the same array dimensions can be assumed for the simulations and for the LS response.

Issue 3-4: BS total array size assumed for the simulation for dense urban

- Proposals

  - Option 1: Same as urban macro (Ericsson)

- Recommended WF

  - Moderator proposal: Discuss together with the same issue in topic 4, and check if the same array dimensions can be assumed for the simulations and for the LS response, or alternatively dense urban can be deprioritized for simulations.

Issue 3-5: BS sub-array size

- Proposals

  - Option 1: 4 (Nokia)

- Recommended WF

  - Discuss in topic 4, check if same assumption can be used for simulations.

Issue 3-6: UE array type used for simulations

- Proposals

  - Option 1: Both FR1 like and FR2 like (ISSDU)

  - Option 2: Single, omnidirectional antenna (Ericsson)

  - Option 3: FR1 like, with some assumed antenna gain

    - Gain G for the wanted signal, no gain for interference (Mediatek, Qualcomm with G=2)

- Recommended WF

  - Discuss in topic 4, check if same assumption can be used for simulations.

Issue 3-7: UE TX power assumption

- Proposals

  - Option 1: 23dBm (Vivo, Ericsson if bandwidth 100MHz, or else higher)

  - Option 2: other

- Recommended WF

  - Discuss in topic 4, check if same assumption can be used for simulations.

Issue 3-8: Deployment type

- Proposals

  - Option 1: Coordinated

  - Option 2: Un-coordinated (Mediatek, Ericsson)

  - Option 3: Co-ordinated for FR2 like UE, un-coordinated for FR1 like UE (Vivo, Nokia)

- Recommended WF

  - TBA

Issue 3-9: ISD

- Proposals

  - Option 1: 450m (Ericsson, Nokia)

  - Option 2: 350m (Vivo)

- Recommended WF

  - TBA

Issue 3-10: Indoor users

- Proposals

  - Option 1: Prioritize case with 20% indoor (Ericsson, Nokia)

  - Option 2: 20% indoor coverage challenging, taking 0% indoor relaxed ACIR by 10dB (ZTE)

- Recommended WF

  - TBA

Issue 3-11: Bandwidth assumption

- Proposals

  - Option 1: 100MHz (Ericsson when UE power is 23dBm)

  - Option 2: 200MHz (Qualcomm)

- Recommended WF

  - TBA

Issue 3-12: SCS

- Proposals

  - Option 1: 30 kHz or larger (ISSDU)

  - Option 2: Other

- Recommended WF

  - TBA

Issue 3-13: Beamforming and MIMO details

- Proposals

  - Option 1: Consider both SU-MIMO and MU-MIMO with different numbers of data streams for co-existence simulations. No beamforming for CSI-RS. (ISSDU)

  - Option 2: Other

- Recommended WF

  - TBA

### Sub-topic 3-2 First view of ACIR results for information

In this sub-topic, observed ACIR values are collected for information. Agreement of ACIR will not happen this meeting, and there is a need to further align on assumptions on e.g. BS array size and UE size. Further analysis should take place on the difference between results and assumptions. Not much online or ad-hoc time is expected for this sub-topic.

The numbers are not directly comparable as underlying assumptions differ (Array size, UE type, bandwidth, power etc.). Once the assumptions are fully aligned, results should be collected in spreadsheet format.

Issue 3-14: Observed DL ACIR (Outdoor)

- Proposals

  - 26 dB (Mediatek)

  - 27dB for FR1 like, 23dB for FR2 like UE (Qualcomm)

  - 9dB better than FR1 ACIR (i.e., around 24dB) (Nokia)

  - 15-17dB (Ericsson)

  - 8-10dB lower than FR1 (i.e., around 25-27dB) (ZTE)

  - 20-25dB depending on bandwidth and number of elements (Samsung)

  - 15-20dB for FR2 like, 20-25dB for FR1 like, depending on average or 5th percentile (Vivo)

Issue 3-15: Observed UL ACIR (Outdoor)

- Proposals

  - 13 dB (Mediatek)

  - 18dB for FR1 like, 14dB for FR2 like UE (Qualcomm)

  - 9dB better than FR1 ACIR (i.e., around 21dB) (Nokia)

  - 5-7dB (Ericsson)

  - 10dB lower than FR1 (i.e., around 20dB) (ZTE)

  - 10dB or lower for FR2 like, 10-15dB for FR1 like (Vivo)

Issue 3-16: Observed DL ACIR (Indoor)

- Proposals

  - 10dB (Ericsson)

  - 10dB lower than FR1 (i.e., around 23dB?) (ZTE)

  - 5-10dB (Samsung)

Issue 3-17: Observed UL ACIR (Indoor)

- Proposals

  - 8dB (Ericsson)

  - 10dB lower than FR1 (i.e., around 20dB?) (ZTE)

Issue 3-18: Observed Blocking level (Outdoor)

- Proposals

  - -36dBm (FR1 like UE), -40dBm (FR2 like UE) 99.99th percentile (Nokia)

# Topic #4: 15GHz parameters for ITU-R response

This topic covers issues and parameters that need to be considered to formulate the LS response.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2415982 | Nokia | Proposal 1: 200-400 MHz is considered as typical maximum channel bandwidth for 14800 – 15350 MHz. Proposal 2: Typical signal bandwidth is given by a formula of RBs * SCS without number of RBs with the reference to the TR 38.922. Proposal 3: For the maximum UE output power of FR1 option, 23 dBm is indicated in the LS as a typical value, mentioning the TR for other power classes for FR1 option. Proposal 4: For the maximum UE output power of FR2 option, the agreed coexistence simulation assumptions are informed in the LS, mentioning the TR for other power classes if they are present in the TR. Observation 1: FR1-like architecture may be more feasible for handheld devices for 14800-15350 MHz. FR2 may not be excluded for other UE types. It may be premature to single out the UE architecture for 14800-15350 MHz. Proposal 5: Unless there is no clear consensus among UE vendors, both FR1 and FR2 architecture assumptions are informed to WP5D in the LS reply and captured in the TR 38.922. |
| R4-2415999 | Samsung | Text proposal on UE parameters |
| R4-2415120 | CATT | Proposal 1: Set the BS antenna array configuration to 8 x 32 as an optimal trade-off between cost and performance. Proposal 2: Set the BS sub-array size as 4 as the optimal trade-off between cost and performance. |
| R4-2415281 | Fujitsu | Observation 1: For achieving the same coverage in each frequency range, the base station output power/sector (e.i.r.p) for 14.8 – 15.35 GHz needs 10.1 dB and 5.2 dB higher than that for 4.4 – 4.8 GHz and 7.125 – 8.4 GHz, respectively. Proposal 1: The target value for maximum base station output power/sector (e.i.r.p.) for 14.8 – 15.35 GHz should be within 82.3 - 83.5 dBm. Observation 2: For achieving the same coverage in each frequency range, BS antenna array size of 2048 is suitable. Proposal 2: RAN4 sets BS antenna array size for 15GHz to 2048. If the additional margin needs to be considered, the BS antenna array size should be discussed based on the value of margin. |
| R4-2415315 | Apple, Samsung | Text proposal justifying a choice of FR1 like rather than FR2 like antenna for the UE. |
| R4-2415316 | Apple | Proposal 1: The FR1-like antenna model can be abstracted by an individual patch antenna model, number of patch antennas and distance between them. Proposal 2: Individual patch antenna model can follow the same model as FR2 antenna array element with the gain factor of [2] dB |
| R4-2415475 | Qualcomm | Proposal 1: RAN4 to agree on TDD as a baseline duplexing for 14800 – 15350 MHz frequency range and capture the following text in TR 38.922. “Even though FDD is not precluded, most likely TDD will be used in this frequency range. In addition, SBFD is considered as a candidate duplexing method. The core requirements for Rel-19 SBFD work item can be tracked through the list of impacted specs captured in [6].” Proposal 2: RAN4 to adopt 200MHz is a typical channel bandwidth. Additionally, it should be stated clearly in TR 38.922 that higher channel bandwidths should not be precluded for this frequency range. Proposal 3: As signal bandwidth depends on CHBW and SCS, RAN4 to not specify a fixed signal bandwidth but rather mention its dependency on SCS and number of RBs, similar to what was done to 7125-8400 MHz frequency range. Observation 1: The impact of modelling near-by users using spherical waves on system level evaluations is not clear in RAN4 and currently being studied in RAN1. Proposal 4: RAN4 to not consider near-field impacts in this frequency range in its correspondence to WP5D. Proposal 5: For BS ACLR and ACS will be based on the outcome of the adjacent channel coexistence. In addition, RAN4 should make sure that the defined ACLR/ ACS are consistent with the defined ACLR/ACS for FR1 and FR2-1. Proposal 6: RAN4 to discuss if ΔfOOB = 100 MHz should be revisited or not for the definition of in-band and out-of-band blocking. Proposal 7: RAN4 to agree on 23 dBm as UE typical maximum output power in the LS reply and to include in TR that higher power classes are not precluded in the normative phase of this frequency range. Proposal 8: RAN4 to agree on 56 dBm as UE power dynamic range. Observation 2: Per WP5D request in LS R4-2400333, RAN4 is expected to provide a clear response on the AAS model at the UE, in case a model different than isotropic 0 dBi model is agreed. Observation 3: IF RAN4 responds with more than one model for UE beamforming, WP5D will chose the model with worst case implications from sharing studies point of view. Proposal 9: If RAN4 cannot converge on a UE beamforming model, RAN4 could reply to WP5D with two variants; UE without beamforming and with beamforming, without putting a label of FR1 or FR2 on the response. Proposal 10: RAN4 to consider 11 dB as UE noise figure for both simulation and LS reply, to ensure consistency within the TR between simulation assumptions and RF parameters. Proposal 11: RAN4 to consider UE SEM given in TR 38.921, which is relaxed compared to NR general NR FR1 SEM at the FOOB edge ± 0 – 5 MHz by 3 dB. Proposal 12: For UE ACLR and ACS, RAN4 to decide on the UE ACLR and ACS based on the outcome of the adjacent channel coexistence study. RAN4 should make sure that the BS and UE ACLR/ACS values communicated to WP5D are reasonable considering other RF requirements. Proposal 13: RAN4 to adopt the UE general spurious emissions defined in TS 38.101-1 clause 6.5.3.1. Proposal 14: For UE blocking response, RAN4 to consider the blocking characteristic specified in clause 7.6 of TS 38.101-1 for frequency larger than 3300 MHz could be applied. |
| R4-2415482 | Nokia | Proposal 1: Adopt 16x24 array and 4x1 sub-array as the BS antenna array size in urban macro scenario. Proposal 2: No need to prioritize 4x4 array for indoor scenario in the coexistence studies. |
| R4-2415762 | Vivo | Proposal 1: 200MHz is used as typical channel bandwidth for 14800 to 15350 MHz in the LS. Proposal 2: For the signal bandwidth in the 15GHz, it is suggested to quote formula of RBs * SCS without number of RBs Observation 1: It is inefficient to try to down-selection from different UE form factor since there no commercial UE is designed in this new frequency range at this stage. Proposal 3: The UE parameters for both FR1-like and FR2-like are provided in the reply LS for 14800-15300MHz. Proposal 4: For FR1-like UE, the maximum output power is 23 dBm in the reply LS and other higher power can be captured in TR just like 8GHz; For FR2-like UE, the maximum output power is same as n258 PC3. Proposal 5: For FR1-like UE, the noise figure is 13 dB; For FR2-like UE, the noise figure is 11 dB. |
| R4-2415962 | Mediatek | Observation 1: At least 1024x2 (dual polarizations) BS antennas are required to achieve a comparable DL/UL coverage to that of FR1 BS in the 15GHz frequency range. Proposal 1: To ensure non-degraded network performance, it is preferrable to deploy at least 1024x2 (dual polarizations) BS antennas for 14800MHz to 15350MHz. Observation 2: FR1-like UE outperformed FR2-like UE in most scenarios in terms of SE, assuming that four antenna elements (AEs) are active simultaneously for both UE types. Proposal 2: RAN4 to consider FR1-like UE antenna architecture as the baseline for 15GHz. Proposal 3: RAN4 to consider 4Rx UE for DL as the baseline for 15GHz. Proposal 4: RAN4 to consider not concluding on the UE ACLR and ACS values until the outcomes of the adjacent channel coexistence studies are available. Proposal 5: RAN4 to consider 26 dBm as the baseline for the UE's maximum output power, and higher power classes, such as 29 dBm, can also be considered. Proposal 6: RAN4 to consider 15dB as the UE NF value. Proposal 7: RAN4 to consider the sensitivity for the UE as “to be specified”. |
| R4-2416001 | Samsung | <BS and system parameters> Observation 1: Given the various implementation factors and plans, legacy parameters in the previous two LSs could also be good references to determine antenna related parameters for 15 GHz. Observation 2: From our prior studies, A larger antenna array size does not necessarily guarantee the better performance in terms of the efficiency and coverage at 15 GHz range, which is higher than the previous frequency ranges. Proposal 1: RAN4 to take 1536 or 3072 AEs (dual polarization) with 6x1 sub-array and dual polarization, which are still double and quadruple AEs of 8 GHz, respectively. Observation 3: Potential contiguous spectrum would be limited given other legacy services around the range, e.g., fixed and passive satellite services, military uses, etc. Proposal 2: 200 MHz could be considered as a nominal channel bandwidth for the frequency range around 15 GHz. Proposal 3: TDD can be sent in the future LS while TR 38.922 captures SBFD as a promising duplex scheme with other frequency ranges. <UE antenna parameters> Observation 4: For FR2 like UE type, taking the complexity of beam management into account, the benefit from using a beam steering antenna system with such a small number of elements need to be reconsidered. Observation 5: It would not make sense for handheld UE to expect the similar uplink and downlink performances when compared with FR2, considering the estimated antenna array size and limited number of modules/panels. Proposal 4: In terms of both performance and feasibility, the option for FR2-like UE type with 2x2 or 2x1 can be excluded from the simulation work and LS. Observation 6: For FR1-like UE type, it should be noted that increasing the number of antennas does not necessarily guarantee the better performance given the higher frequency ranges and the limited form factor. Proposal 5: Although we could see some directivity gain due to the higher frequency, 5dBi with more than 4-port transmission cannot be considered for UE antenna parameters at this stage. Proposal 6: RAN4 to take FR1-like UE type with omnidirectional antenna, e.g., 0dBi, as a baseline for the both simulation work and LS. The targeted total antenna gain can be further discussed or refined together with the default Tx number for this frequency range, if necessary. + tables for radio and antenna parameters |
| R4-2416169 | Ericsson | Proposal 1 In Urban Macro scenario, RAN4 to consider sub-array size of 4x1 and drop 8x1 as a larger sub-array would limit the coverage angular range due to narrow sub-array radiation pattern in vertical axis. Proposal 2 RAN4 to consider following BS array antenna architectures: 1. Urban Macro: 4096 AEs with 16x32 array and 4x1 sub-array 2. Indoor Hotspot: 4x4 3. Dense Urban: Same HW as Urban Macro Proposal 3 RAN4 to converge on the combination of the BS-UE antenna configuration that seems the most viable based on implementation feasibility aspects, to achieve a reasonable coverage. Proposal 4 Considering the given timeline to agree on the parameters, RAN4 to consider the combination 4096 Antenna Elements (AE) (in total) (16x32 and 4x1 sub-array) and FR1-like UE - 1TX assumption with omnidirectional UE antenna as baseline. Proposal 5 Most of the system parameters are dependent on other, so to make progress RAN4 should start by tentatively agreeing on parameters, and then confirm them only after verifying the overall system feasibility. Proposal 6 The additional information should be updated with the following text in the TR – “There is no defined 3GPP band for the 14800 - 15350 MHz frequency range. Similar to the 4400 – 4800 MHz and 7125 – 8400 MHz frequency ranges, RAN4 assumed TDD as a baseline duplexing for the 14800 – 15350 MHz frequency range even though FDD is not precluded. SBFD can be a candidate duplexing method for this frequency range. The core requirements for Rel-19 SBFD work item can be tracked through the list of impacted specs captured in [6].” Proposal 7 RAN4 to consider 100 MHz as a typical channel bandwidth, with higher channel bandwidth not precluded for this range, and add a note on association of the typical channel bandwidth with maximum transmission power. Proposal 8 RAN4 to not specify a fixed signal bandwidth but rather mention the dependency of signal bandwidth on the SCS and number of RBs. Proposal 9 RAN4 to consider the Table 6.1.2-1 and Table 6.1.2-2 from TR 38.921 as baseline for starting discussions on BS OBUE. Proposal 10 RAN4 to consider BS spurious emission limits in Table 2.2.3-1 and 2.2.3-2 as baseline for starting discussions. Proposal 11 RAN4 to mention “to be specified” for sensitivity in the LS response to ITU-R for 14800 to 15350 MHz range. Proposal 12 RAN4 to consider 100 MHz for both ΔfOBUE and ΔfOOB Proposal 13 RAN4 to consider power dynamic range of 56 dB for 100 MHz CBW as baseline for discussions. Proposal 14 RAN4 to consider UE spurious emission limits from TS 38.101-1, - 30 dBm / 1 MHz as baseline for discussions. Proposal 15 RAN4 to consider as 23 dBm UE maximum output power. Proposal 16 RAN4 to consider UE NF as 10 dB based on information in TR 38.820. Proposal 17 RAN4 to mention “to be specified” for sensitivity in the LS response to ITU-R for 14800 to 15350 MHz range. |
| R4-2416243 | ZTE, Sanechips | Proposal 1: choose the 100MHz channel bandwidth in reply LS and indicate other channel bandwidth are not precluded. Proposal 3: define ACLR requirement as 35dBc; Proposal 4: propose to reuse the spectral mask for 7 and 10GHz for 15GHz; Proposal 5: propose the typical Noise Figure for a Wide Area BS operating at 15 GHz is 8 dB (13 dB for Medium Range BS and 16 dB for Local Area BS). Proposal 6:propose to define BS OTA sensitivity requirement based on the NF and the antenna gain similar 7 and 10GHz; Proposal 7:propose to reuse FR1 licensed band IBB requirement for it. Proposal 8:propose to define BS ACS requirement as 38dBc. Proposal 9: postpone the ACLR requirement until the UE implementation assumptions are confirmed. Proposal 10: define UE ACS requirement around 30dBc for 15GHz. |
| R4-2416412 | Huawei, HiSilicon | Proposal 1: Decision on typical channel bandwidth for 14800 to 15350 MHz frequency range shall not influence future decision on 14800 to 15350 MHz assignment to FR1 or FR2, i.e. it shall not exclude FR1 from future discussion. Proposal 2: Exclude 400 MHz channel bandwidth from consideration. Proposal 3: Select 100 MHz as typical channel bandwidth for 14800 to 15350 MHz frequency range (200 MHz channel bandwidths not precluded). Proposal 4: Capture the following disclaimer in the TR 38.922: Agreements on parameters and requirements for 14800 to 15350 MHz frequency range shall not be considered decisive for FR1 vs FR2 assignment of this range, nor on related RF requirements limits. Those aspects are expected to be decided in future 6G work item, pending WRC-27 decision. Proposal 5: Clarify in the LS on 14800 to 15350 MHz, that “Parameters for the frequency range 14800 to 15350 MHz were derived based on [FR1/FR2/mix of FR1 and FR2] RF requirements.”. Text in [] to be updated once all BS and UE requirements are settled. Proposal 6: For BS antenna array and sub-array size, select 4096 AEs dual polarization case with 16x32 array and 4x1 sub-array. Proposal 7: For 14800 to 15350 MHz frequency range, do not consider non-AAS BS architecture in the LS reply. Proposal 8: For Category A OBUE limits, consider table 2.2.3-1 and 2.2.3-2 as baseline for spectral mask adjustment for 14800 to 15350 MHz frequency range. Table 2.2.3-1: WA BS OBUE limits for 10.0 - 10.5 GHz, Category A Frequency offset of measurement filter -3dB point, Df; Frequency offset of measurement filter centre frequency, f_offset; Basic limits; Measurement bandwidth. 0 MHz £ Df < 50 MHz; 0.05 MHz £ f_offset < 50.05 MHz; -7dBm-750foffsetMHz-0.05; 100 kHz. 50 MHz f < min(100 MHz, Dfmax); 50.05 MHz f_offset < min(100.05 MHz, f_offsetmax); -14 dBm; 100 kHz. 100 MHz £ Df £ Dfmax; 100.5 MHz £ f_offset < f_offsetmax; -13 dBm; 1MHz. Table 2.2.3-2: OBUE limits, 24.25 – 33.4 GHz, Category A Frequency offset of measurement filter -3B point, f; Frequency offset of measurement filter centre frequency, f_offset; Limit; Measurement bandwidth. 0 MHz f < 0.1*BWcontiguous; 0.5 MHz f_offset < 0.1* BWcontiguous +0.5 MHz; Min(-5 dBm, Max(Prated,t,TRP – 35 dB, -12 dBm)); 1 MHz. 0.1*BWcontiguous f < fmax; 0.1* BWcontiguous +0.5 MHz f_offset < f_ offsetmax; Min(-13 dBm, Max(Prated,t,TRP – 43 dB, -20 dBm)); 1 MHz. Proposal 9: BS ACLS and BS ACS requirement levels for 14800 to 15350 MHz frequency range are to be derived from adjacent channel coexistence analyses. Proposal 10: For spurious emissions limits in the LS reply, follow SM.329 limit of -13 dBm, for the range of 30 MHz to 2nd harmonic. Proposal 11: For ΔfOBUE value, re-evaluate estimated filtering capabilities before proceeding to FR1, or FR2 value reuse. Proposal 12: Agree that the average output power won’t be mentioned in the reply LS for 14800 to 15350 MHz frequency range. Proposal 13: for the sensitivity inputs to the LS table, reuse the approach from previous LS, i.e. To be specified, as the sensitivity is not a critical parameter for sharing and compatibility studies. Proposal 14: for the SINR operating range inputs to the LS table, reuse the approach from previous LS, i.e. refer to “SINR operating range and mapping function”. Proposal 15: For Issue 3-1 on UE type assumptions, select option 2 for the FR1-like alternative (i.e. two discrete elements following Table 1-1 with maximum-gain across elements selection criterion). Proposal 16: Postpone the final decision on the UE type assumption at least until November 2024, to collect more inputs on the analysis of the FR2-like option before taking the decision. Proposal 17: UE noise figure for the LS reply on 14800 to 15350 MHz frequency range to be 11 dB. Based on the above proposals, TP to TR 38.922 in [6] was drafted for the 14800 to 15350 MHz frequency range parameters. Proposal 18: It is proposed to be use a Draft running LS for 14800 to 15350 MHz frequency range parameters in [7] as starting point to collect all the agreements so far. One revised, it is proposed for Endorsement. |
| R4-2416414 | Huawei | Text proposal for the 15GHz BS parameters |
| R4-2416416 | Huawei | Draft LS response |

## Open issues summary

### Sub-topic 4-1 General issues

This topic considers general issues that apply to both BS and UE.

Issue 4-1: Duplex Mode

Previous agreement:

- TDD as a baseline.

- Suggestion for TR text found in R4-2411521 which can be further discussed

Previous agreement:

- TDD as a baseline.

- Suggestion for TR text found in R4-2411521 which can be further discussed

- Proposals

  - Option 1: RAN4 to agree on TDD as a baseline duplexing for 14800 – 15350 MHz frequency range and capture the following text in TR 38.922. (Qualcomm)

“Even though FDD is not precluded, most likely TDD will be used in this frequency range. In addition, SBFD is considered as a candidate duplexing method. The core requirements for Rel-19 SBFD work item can be tracked through the list of impacted specs captured in [6].”

Option 2: The additional information should be updated with the following text in the TR – “There is no defined 3GPP band for the 14800 - 15350 MHz frequency range. Similar to the 4400 – 4800 MHz and 7125 – 8400 MHz frequency ranges, RAN4 assumed TDD as a baseline duplexing for the 14800 – 15350 MHz frequency range even though FDD is not precluded. SBFD can be a candidate duplexing method for this frequency range. The core requirements for Rel-19 SBFD work item can be tracked through the list of impacted specs captured in [6].” (Ericsson)

- Option 3: TDD can be sent in the future LS while TR 38.922 captures SBFD as a promising duplex scheme with other frequency ranges. (Samsung)

- Recommended WF

  - TBA

Issue 4-2: FR1/FR2 decision

- Proposals

Option 1: Capture the following disclaimer in the TR 38.922:  (Huawei)

  - Agreements on parameters and requirements for 14800 to 15350 MHz frequency range shall not be considered decisive for FR1 vs FR2 assignment of this range, nor on related RF requirements limits. Those aspects are expected to be decided in future 6G work item, pending WRC-27 decision.

Option 2: Clarify in the LS on 14800 to 15350 MHz, that “Parameters for the frequency range 14800 to 15350 MHz were derived based on [FR1/FR2/mix of FR1 and FR2] RF requirements.”. Text in [] to be updated once all BS and UE requirements are settled. (Huawei)

- Recommended WF

  - TBA

Issue 4-3: AAS BS

- Proposals

Option 1: For 14800 to 15350 MHz frequency range, do not consider non-AAS BS architecture in the LS reply. (Huawei)

Recommended WF

  - TBA

Issue 4-4: Approach to parameter agreement

- Proposals

  - Option 1: Most of the system parameters are dependent on other, so to make progress RAN4 should start by tentatively agreeing on parameters, and then confirm them only after verifying the overall system feasibility.(Ericsson)

- Recommended WF

  - Moderator proposal: Parameters (like BS array size, UE type and antenna gain, power, bandwidth etc.) are discussed and agreed one by one. The agreement shall firstly be captured in []. Once all parameters are agreed, a sanity check can be made that e.g. the link budget works out.

Issue 4-5: Typical channel bandwidth

Previous  agreement:

For LS about 15GHz, choose the single value out of 100MHz, 200MHz or 400MHz as baseline with understanding that the other channel bandwidths are not precluded.

Previous  agreement:

For LS about 15GHz, choose the single value out of 100MHz, 200MHz or 400MHz as baseline with understanding that the other channel bandwidths are not precluded.

- Proposals

  - Option 1: 200 – 400 MHz for typical maximum channel bandwidth (Nokia)

  - Option 2: 200MHz typical bandwidth, document in TR other bandwidths not precluded (Qualcomm, Vivo, Samsung)

  - Option 3: 100MHz typical bandwidth, document in TR other bandwidths not precluded (ZTE, Huawei, Ericsson)

    - Option 3a: The note should capture the association of channel bandwidth with maximum transmission power /PSD (Ericsson)

- Recommended WF

  - TBA

Issue 4-6: Typical signal bandwidth

- Proposals

  - Option 1: Typical signal bandwidth is given by a formula of RBs * SCS without number of RBs with the reference to the TR 38.922. (Nokia, Qualcomm, Vivo, Ericsson, Samsung)

- Recommended WF

  - TBA

Issue 4-7: SINR operating range

- Proposals

  - Option 1: Reuse the approach from previous LS, i.e. refer to “SINR operating range and mapping function” (Huawei, Samsung)

### Sub-topic 4-2: BS related parameters

This sub-topic considers BS parameters

Issue 4-8: BS output power

- Proposals

  - Option 1: Do not mention average output power (Huawei)

Issue 4-9: Power dynamic range

Previous agreement: 0dB

Issue 4-10: Emissions mask

- Proposals

  - Option 1: RAN4 to consider the Table 6.1.2-1 and Table 6.1.2-2 from TR 38.921 as baseline for starting discussions on BS OBUE. RAN4 to consider BS spurious emission limits in Table 2.2.3-1 and 2.2.3-2 as baseline for starting discussions. (Ericsson, ZTE)

- Table 6.1.2-1: Wide Area BS operating band unwanted emission limits for 6.425 - 7.125 GHz and 10.0 - 10.5 GHz for Category A

| Frequency offset of measurement filter 3dB point, Df | Frequency offset of measurement filter centre frequency, f_offset | Basic limits | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz £ Df < 50 MHz | 0.05 MHz £ f_offset < 50.05 MHz | -7dBm-750foffsetMHz-0.05 | 100 kHz |
| 50 MHz f < min(100 MHz, Dfmax) | 50.05 MHz f_offset < min(100.05 MHz, f_offsetmax) | -14 dBm | 100 kHz |
| 100 MHz £ Df £ Dfmax | 100.5 MHz £ f_offset < f_offsetmax | -13 dBm | 1MHz |

- Table 6.1.2-2: Wide Area BS operating band unwanted emission limits for 6.425 - 7.125 GHz and 10.0 - 10.5 GHz for Category B

| Frequency offset of measurement filter 3dB point, Df | Frequency offset of measurement filter centre frequency, f_offset | Basic limits | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz £ Df < 50 MHz | 0.05 MHz £ f_offset < 50.05 MHz | -7dBm-750foffsetMHz-0.05 | 100 kHz |
| 50 MHz f < min(100 MHz, Dfmax) | 50.05 MHz f_offset < min(100.05 MHz, f_offsetmax) | -14 dBm | 100 kHz |
| 100 MHz £ Df £ Dfmax | 100.5 MHz £ f_offset < f_offsetmax | -15 dBm | 1MHz |

  - Option 2: Consider the following as baseline for spectrum mask adjustment (Huawei):

- Table 2.2.3-1: WA BS OBUE limits for 10.0 - 10.5 GHz, Category A

| Frequency offset of measurement filter -3dB point, Df | Frequency offset of measurement filter centre frequency, f_offset | Basic limits | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz £ Df < 50 MHz | 0.05 MHz £ f_offset < 50.05 MHz | -7dBm-750foffsetMHz-0.05 | 100 kHz |
| 50 MHz f < min(100 MHz, Dfmax) | 50.05 MHz f_offset < min(100.05 MHz, f_offsetmax) | -14 dBm | 100 kHz |
| 100 MHz £ Df £ Dfmax | 100.5 MHz £ f_offset < f_offsetmax | -13 dBm | 1MHz |

- Table 2.2.3-2: OBUE limits, 24.25 – 33.4 GHz, Category A

| Frequency offset of measurement filter -3B point, f | Frequency offset of measurement filter centre frequency, f_offset | Limit | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.1*BWcontiguous | 0.5 MHz f_offset < 0.1* BWcontiguous +0.5 MHz | Min(-5 dBm, Max(Prated,t,TRP – 35 dB, -12 dBm)) | 1 MHz |
| 0.1*BWcontiguous f < fmax | 0.1* BWcontiguous +0.5 MHz f_offset < f_ offsetmax | Min(-13 dBm, Max(Prated,t,TRP – 43 dB, -20 dBm)) | 1 MHz |

- Recommended WF

  - The main difference between Ericsson/ZTE and Huawei proposals is whether to consider the FR1 like OBUE or consider also the FR2 like as a candidate.

  - A second issue for FR1 like is whether to consider Category B.

Issue 4-11: Noise figure

Previous agreement:

- 8 dB for Wide-Area BS

- 13 dB for Medium Range BS

- 16 dB for Local Area BS

Previous agreement:

- 8 dB for Wide-Area BS

- 13 dB for Medium Range BS

- 16 dB for Local Area BS

- Proposals

  - Option 1: 8dB for WA (13dB for MR, 16dB for LA) (ZTE, Samsung)

- Recommended WF:

  - Option 1 aligns with previous agreement.

Issue 4-12: Sensivitity

- Proposals

  - Option 1: Define based on NF and antenna gain (ZTE)

  - Option 2: Write “To be specified” (Huawei, Samsung, Ericsson)

- Recommended WF

Issue 4-13: ACLR

- Proposals

  - Option 1: Decide based on co-existence study (Qualcomm, Huawei, Nokia)

  - Option 2: Define ACLR as 35dB (ZTE)

- Recommended WF

Issue 4-14: Spurious emission

- Proposals

  - Option 1: For spurious emissions use limits in TR 38.921 for 10 to 10.5 GHz as baseline. According to ERC 74-01 report, the upper frequency limit could be defined as 2nd harmonic. Take the following tables as baseline (Ericsson):

- Table 2.2.2.3-1: BS spurious emission limits for 14800 to 15350 MHz for Category A

| Frequency range | Limit | Measurement Bandwidth | Note |
| --- | --- | --- | --- |
| 30 MHz – 1 GHz | -13 dBm | 100 kHz | Note 1 |
| 1 GHz – 2nd harmonic of the upper frequency edge of the DL operating band |  | 1 MHz | Note 1, Note 2 |
| NOTE 1: Bandwidth as in ITU-R SM.329, s4.1 NOTE 2: Upper frequency as in ITU-R SM.329, s2.5 table 1. |  |  |  |

- Table 2.2.2.3-2: BS spurious emission limits for 14800 to 15350 MHz for Category B

| Spurious frequency range | Limit | Measurement bandwidth |
| --- | --- | --- |
| 30 MHz – 1 GHz | -36 dBm | 100 kHz |
| 1 GHz – 18 GHz | -30 dBm | 1 MHz |
| 18 GHz – 2nd harmonic of the upper frequency edge of the DL operating band | -20 dBm | 10 MHz |

- For spurious emissions limits in the LS reply, follow SM.329 limit of -13 dBm, for the range of 30 MHz to 2nd harmonic. (Same as table 2.2.2.3-1 above) (Huawei)

- Recommended WF

  - Ericsson and Huawei proposals align for Category A. Discuss whether there is a need to consider category B.

Issue 4-15: Blocking response

- Proposals

  - Option 1: RAN4 to discuss if ΔfOOB = 100 MHz should be revisited or not for the definition of in-band and out-of-band blocking.(Qualcomm)

  - Option 2: RAN4 to consider 100 MHz for both ΔfOBUE and ΔfOOB (Ericsson)

  - Option 3: Use FR1 licensed band IBB (ZTE)

  - Option 4: From co-existence simulations, Nokia observe -36dBm (FR1 like UE), -40dBm (FR2 like UE) 99.99th percentile

- Recommended WF

Issue 4-16: ACS

- Proposals

  - Option 1: Wait until co-existence study outcome (Qualcomm, Huawei, Nokia)

  - Option 2: 38dB (ZTE)

- Recommended WF

Issue 4-17: BS antenna parameters

Previous agreement:

- Recommended for further discussions

- 3072 AEs dual polarization :

- 16x24 array and 4x1 sub-array

- 4096 AEs dual polarization:

- 8x32 array and 8x1 sub-array

- 16x32 array and 4x1 sub-array

- 2048 AEs dual polarization:

- 8x32 array and 4x1 sub-array

- 1536 AEs dual polarization:

- 8x16 and 6x1 sub-array

- Other options are not precluded.

Previous agreement:

- Recommended for further discussions

- 3072 AEs dual polarization :

- 16x24 array and 4x1 sub-array

- 4096 AEs dual polarization:

- 8x32 array and 8x1 sub-array

- 16x32 array and 4x1 sub-array

- 2048 AEs dual polarization:

- 8x32 array and 4x1 sub-array

- 1536 AEs dual polarization:

- 8x16 and 6x1 sub-array

- Other options are not precluded.

- Proposals on sub-array size Urban Macro

  - Option 1: Set sub-array size 4 (CATT, Nokia, Ericsson, Huawei)

  - Option 2: Set sub-array size 6 (Samsung)

- Recommended WF

  - TBA

- Proposals on array size Urban Macro

  - Option 1: Set array size 8*32 (SA size 4) (CATT, Fujitsu, Mediatek

  - Option 2: Set array size 16*24 (SA size 4) (Nokia)

  - Option 3: Set array size 16*32 (SA size 4) (Ericsson, Huawei)

  - Option 4: Set array size 8*32 (SA size 6) (Samsung)

  - Option 5: Set array size 16*32 (SA size 6) (Samsung)

- Recommended WF

- Proposals on array parameters Dense Urban

  - Option 1: Same as urban macro (Ericsson)

- Recommended WF

- Proposal on indoor (for LS response, whether to do co-existence considered separately)

  - Option 1: 4x4 (Nokia, but not prioritized for co-existence?, Ericsson, vivo, ZTE)

- Recommended WF

  - TBA

### Sub-topic 3-2: UE related parameters

This sub-topic considers UE parameters

Issue 4-18: UE output power

- Proposals

  - Option 1: 23dBm, mention other power classes possible in TR (Nokia, Qualcomm, Vivo, Samsung, Ericsson)

  - Option 2: 26dBm as the baseline, higher PC considered (Mediatek)

- Recommended WF

Issue 4-19: Power dynamic range

- Proposals

  - Option 1: 56dB for 100MHz assuming 23dBm (Qualcomm, Samsung, Ericsson)

- Recommended WF

Issue 4-20: Emissions mask

- Proposals

  - Option 1: RAN4 to consider UE SEM given in TR 38.921, which is relaxed compared to NR general NR FR1 SEM at the FOOB edge ± 0 – 5 MHz by 3 dB. (Qualcomm)

- Recommended WF

Issue 4-21: ACLR

- Proposals

  - Option 1:  Decide based on co-existence study and UE implementation assumptions (Qualcomm, ZTE, Mediatek, Nokia)

- Recommended WF

Issue 4-22: Spurious emission

- Proposals

  - Option 1:  RAN4 to adopt the UE general spurious emissions defined in TS 38.101-1 clause 6.5.3.1. (Qualcomm, Ericsson)

- Recommended WF

Issue 4-23: Noise figure

- Proposals

  - Option 1: 10dB (Ericsson)

  - Option 2: 11 dB (Qualcomm, vivo for FR2 like, Samsung, Nokia)

  - Option 3: 13dB (vivo for FR1 like)

  - Option 4: 15dB (Mediatek)

Issue 4-24: Sensitivity

- Proposals:

  - Option 1: “To be specified” (Samsung, Qualcomm, Huawei, Mediatek, Ericsson)

- Recommended WF

Issue 4-25: Blocking and spurious response

- Proposals

  - Option 1: For UE blocking response, RAN4 to consider the blocking characteristic specified in clause 7.6 of TS 38.101-1 for frequency larger than 3300 MHz could be applied. (Qualcomm)

Issue 4-26: ACS

- Proposals

  - Option 1: Decide based on co-existence study (Qualcomm, Mediatek, Nokia)

  - Option 2: 30 dB (ZTE)

Issue 4-27: FR1 like or FR2 like UE

- Proposals

  - Option 1: Reply with both architectures unless clear consensus (Nokia, Qualcomm, vivo)

  - Option 2: FR1 like (Apple, Samsung, Mediatek, Samsung, Ericsson)

  - Option 4: FR1 patch with 2dB gain (Apple)

  - Option 5: FR1 like with omni-directional antenna (Samsung, Apple)

  - Option 5: FR1 like with 4 antennas and some directionality (Mediatek)

  - Option 6: Postpone the decision (Huawei)

  - Option 7: For FR1 like UE, assume 2 elements with maximum gain selection (Huawei)

- Recommended WF

  - Discuss whether FR2 like can be ruled out or not

  - Clarify FR1 like; omnidirectional, directional or G

Issue 3-23: TP R4-2415315 on UE implementation

- Proposals

  - Option 1: Agree the TP (Apple, Samsung)

  - Option 2: Revise the TP

  - Option 3: Do not agree the TP

Issue 3-23: Considering near field impacts

- Proposals

  - Option 1: Do not consider near field impacts in the response to ITU-R (Qualcomm)

# Topic #5: Other aspects

This topic covers the additional questions on MIMO modelling and on ACLR modelling

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2415969 | Ericsson | Observation 1: Since array correlation is difficult to measure for an AAS base station with many transceivers it is more practical to measure the directivity response and convert it to array correlation via a directivity/correlation evaluation. Observation 2: Based on measured characteristics parameter values for a piece-wise linear model for the array correlation can be established. Observation 3: For adjacent channel sharing studies where AAS BS EIRP is considered additional parameters such as ACRL and RF filter suppression should be considered. + Text proposal to capture the array correlation model |
| R4-2414921 | Spark | Proposal 1: The text proposals in this document be included in TR 38 922, section 7 Proposal 2:ZF requires the computation of a pseudo inverse of the concatenation of channel matrices. It is important that the pseudo inverse is based on the correct formulation of the channel matrices as otherwise the results will be misleading The array response vmn ( in the 3GPP AAS model) can be made to provide maxima at given angular directions by an appropriate choice of the angles in wmn which can be viewed as additional phase terms in the array response. It should be noted that pseudo inverse based on concatenated channels in eqn (1) that are in turn based on wmn alone is flawed and will not provide an exact impact of ZF on the complete array response. : We could advise WP 5D on how to calculate the BF weights with ZF with correct formulation of the pseudo inverse and that the differences between the current approach ie using ITU R M 2101 and ZF cannot be ignored. This text could be sent in an LS to WP 5D. Alternatively the RAN4 experts could agree on common simulation assumptions and resubmit the simulations to the RAN4 113 meeting so that a unified answer may be provided to WP Proposal 3: There is a need to define array correlation factor so that it may be computed. Equation (1) may be used. Proposal 4: Define the constituent components of P oob (θ,φ) and their dependence on f Proposal 5: There is a need to investigate array ACLR and see that its computation method in the standards is still the correct way. Proposal 6: An array will not drop down to a single element in adjacent bands. Nor its performance will drop down to a sub array. Proposal 7: Power Amplifier non linearities should be carefully modelled when considering adjacent channel impacts when MIMO arrays are used. The choice of a PA model should be agreed (say via polynomial models), then it may be validated by measurements and also agree on a model for the differences in the parameters for the distributed PAs. Proposal 8: We may assume for the sake of simplicity there are no differences in the PA model parameters in a distributed PA AAS. Proposal 9: A model for the BPF needs to be agreed Proposal 10: MU-MIMO precoder type and numbers of simultaneous users need to be agreed and could impact out of band distortions to fall in un-intended directions. + Text proposal covering beamforming and ACLR |
| R4-2415121 | CATT | Observation 1: Evaluation may be interesting for a channel bandwidth 100 MHz ~ 400 MHz at 15GHz for the cases where spatial sensitivity of ACLR is observed for 5MHz channel bandwidth at 2GHz. Observation 2: The current coexistence simulation assumes the worst-case scenario in terms of spatial ACLR by applying a uniform ACIR value, which is the basis for defining the minimum ACIR requirement. Observation 3: In M2101 downlink simulation methodology, no multi-user spatial beam-forming is considered. Observation 4: With sufficient number of snapshots for one simulated scenario and ideally paired UEs for co-scheduling, multi-user spatial beam-forming does not improve the assessment of interference and accuracy of studies. Proposal 1: RAN4 to reply WP5D that the current coexistence methodology is sufficient without a need of taking into account spatial ACLR. Proposal 2: RAN4 to reply WP5D that no multi-user spatial beam-forming is needed for assessing AAS implementation with a sufficient number of snapshots for one simulated scenario and ideally paired UEs for co-scheduling. |
| R4-2415122 | CATT | Skeleton for adjacent channel modelling TP |
| R4-2415474 | Qualcomm | Observation 1: WP5D is currently investigating whether it is needed or not to consider multi-user beamforming techniques impact on top of the AAS model (i.e., ITU-R M.2101). Observation 2: RAN4 is expected to provide its technical response on the impact of multi-UE beamforming techniques impact on system level studies to WP5D and capture its findings in TR 38.922. Observation 3: From Monte-Carlo studies, similar IMT BS gain above the horizon is observed when zero forcing is considered at the BS IMT when compared to the case without zero forcing beamforming. Observation 4: The IMT BS radiation pattern is nearly identical when considering the methodology given in Annex A and the modified methodology based w_(n,m) and ν_(n,m) from ITU-UR M.2101 when constructing the channel matrix H_k. Observation 4: The IMT BS radiation pattern is nearly identical when considering the methodology given in Annex A and the modified methodology based on the vectorization of U_(k_(n,m) )and ν_(n,m) when constructing the channel matrix H_k. Proposal 1: RAN4 to reply to WP5D that it is sufficient to adopt the legacy AAS model based on ITU-R Recommendation M.2101 since the performance when ZF beamforming is considered is similar to the performance without ZF beamforming technique. Thus, the difference between M.2101 model and the ZF precoder scheme is minor, and therefore M.2101 should be used for WRC-27 study cycle. |
| R4-2415483 | Nokia | 1) Power amplifier non-linearities have been included in the current AAS model for UE specific beamforming with multiple UE. 2) Impact of correlation level to the system coexistence has been evaluated in the AAS SI, and simulation results have shown that different correlation levels have little impact on the throughput loss. |
| R4-2415523 | Nokia | 1) With 400 MHz channel bandwidth at the 15 GHz carrier frequency, the composite array radiation patterns formed by the AAS in the adjacent bands are similar to that formed at the designed carrier frequency, while the ACLR patterns experience some changes in the adjacent bands, using the current AAS model with correlation of 2 transceiver paths (ρ) equals to 1 when 3 UE specific beams are formed by the AAS. 2) The average and 5%-tile downlink and uplink throughput losses are similar for the three simulated cases. |
| R4-2415970 | Ericsson | Proposal 1: Based on outcome of beamforming modelling evaluation, the current modelling approach remains effective for its intended purposes. Modifying the existing beamforming model is not necessary, as the variations observed are insignificant and do not justify the complexity of updating the model. Proposal 2: Capture simulation assumption, simulation results and corresponding conclusion in TR 38.922. + Text proposal capturing simulaitons and information. |
| R4-2416244 | ZTE, Sanechips | the correlation level at the adjacent channel is highly dependent on the implementation and exact level should be also up to the filed measurement results. The correlation level on the adjacent channel will have limited or marginal impacts on the coexistence study. In addition, if considering the varying correlation level in the adjacent channel, then simulation case or simulation efforts in RNA4 will be increased a lot. |
| R4-2416281 | Huawei, HiSilicon | Based on the simulation and theory analysis, there is no strong motivation to change the beamforming assumptions in Recommendation ITU-R M.2101 considering extended AAS modelling for sub-array. |

## Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 2-1 ACLR model and MIMO model

Issue 5-1: ACLR model

- Proposals

  - Option 1: Agree to use a piece-wise antenna correlation model (Ericsson)

  - Option 2: Endorse the model in the TP in R4-2415969 as a basis for the piecewise model (Ericsson)

    - Moderator: This would be endorsed as a basis. The TP editor identified in the work split should merge to the final TP

  - Option 3: Agree to define a correlation factor and capture it’s dependence on frequency based on analysis of the constituent components (Spark)

  - Option 4: Array ACLR should be investigated. An array will not drop down to a single element / sub-array. (Spark)

  - Option 5: A PA model should be adopted and compared to measurements (Spark)

  - Option 6: A model for the band pass filter is needed (Spark)

  - Option 7: Agree multi-user MIMO precoder type and number of users in order to model the distortions falling in un-intended directions. (Spark)

  - Option 8: Endorse the model in the TP in R4-2414921 on modelling (Spark)

    - Moderator: This would be endorsed as a basis. The TP editor identified in the works split should merge to the final TP

  - Option 9: There is no need to take into account spatial ACLR. Monte-carlo simulations show no dependency on the spatial pattern compared with the existing ITU model (CATT, ZTE)

  - Option 10: Existing simulations based on the current AAS model show that the correlation factor for adjacent channel emissions does not impact the co-existence performance much (Nokia)

- Recommended WF

  - Based on the above “options”, the moderator recommends firstly to discuss between these 4 high level options:

    - Option A: Define a correlation factor, but do not give any indication of what it could be; just that dependent on implementation it could be between 0 and 1.

    - Option B: Define a correlation factor with a piece wise model for the correlation value

    - Option C: Investigate the correlation factor in detail with models, such as PA model, BPF  model etc. and considering MU-MIMO precoding and number of users

    - Option D. No correlation factor; the adjacent channel correlation factor does impact co-existence results

Issue 5-2: MIMO model

- Proposals

  - Option 1: Provide a detailed description to ITU-R on how to calculate the ZF beamforming weights (Spark, proposal 1)

  - Option 2: Endorse the model in the TP in R4-2414921 on modelling (Spark)

    - Moderator: This would be endorsed as a basis. The TP editor identified in the work split should merge to the final TP

  - Option 3: Modelling of multi-user spatial beamforming is not needed for co-existence/sharing studies; the M.2101 methodology is sufficient since simulations show similar spatial distribution of interference over monte-carlo runs (CATT, Qualcomm, Ericsson, Huawei)

  - Option 4: Endorse the model in the TP in R4-2415970 on the simulation comparison between ZF and ITU model (Ericsson)

    - Moderator: This would be endorsed as a basis. The TP editor identified in the work split should merge to the final TP

- Recommended WF

# Worksplit for TP and WF

Already completed TPs and WFs are highlighted in green. The TPs highlighted in yellow should be allocated this meeting for merging correction TPs. The remaining TPs in white should only be allocated at RAN4#112bis if all parameters are agreed (otherwise, incremental agreements are captured in the WF).

The WF for 112bis should all be allocated.

| Item | Company |
| --- | --- |
| TP for 4GHz parameters | Ericsson |
| TP for 8GHz BS parameters | Huawei |
| TP for 8GHz UE parameters | Apple |
| TP for 15GHz BS RF parameters | CATT |
| TP for 15GHz BS Antenna parameters | ZTE |
| TP for 15GHz UE RF parameters | Samsung |
| TP for 15GHz UE Antenna parameters | MTK |
| TP for 15GHz simulation parameters | Nokia (including further revisions at RAN4#112 etc.) |
| TP for 15GHz simulation results | Samsung |
| TP for other issues (MIMO modelling) | Nokia/Spark |
| TP for other issues (Adjacent channel modelling) | CATT |
| TP on BS antenna model | Ericsson |
| RAN4#110bis WF on 4GHz | Ericsson |
| RAN4#110bis WF on 8 and 15GHz | CATT |
| RAN4#111 WF | Ericsson |
| RAN4#112 WF on 8GHz BS parameters | Huawei |
| RAN4#112 WF on 8GHz UE parameters | Vivo |
| RAN4#112 WF on 15GHz BS parameters | ZTE |
| RAN4#112 WF on 15GHz UE parameters | Qualcomm |
| RAN4#112bis WF on 15GHz BS parameters | ZTE |
| RAN4#112bis WF on 15GHz UE parameters | Qualcomm |
| RAN4#113 WF on 15GHz BS parameters | ZTE |
| RAN4#113 WF on 15GHz UE parameters | Qualcomm |
| RAN4#112 WF on other issues (MIMO) | Nokia/Spark |
| RAN4#112bis WF on other issues (MIMO) | Nokia/Spark |
| RAN4#113 WF on other issues (MIMO) | Nokia/Spark |