3GPP TSG-RAN WG4 Meeting #112bis												    		R4-2415652

Hefei, China, 14th – 18th October 2024

Agenda item:			6.4.4

Source:	Moderator (Intel Corporation)

Title:	Topic summary for [112bis][205] NR_FR1_lessthan_5MHz_BW_Ph2

Document for:	Information

# Introduction and recommendations

This document is the TDocs summary for [112][214] NR_FR1_lessthan_5MHz_BW_Ph2 with the following topics covered.

- Topic 1:	 RRM scope discussions for NR_FR1_lessthan_5MHz_BW_Ph2 work item (AI 6.4.3)

The moderator recommends the below topics to be discussed during the online session under the Vice Chair guidance.

The recommendation to discuss the below issues online is in order of priority identified by the moderator. Details subject to further arrangements from the Vice Chair.

Network indications on SSB bandwidth

Issue 1-3-1a: Allowing 12PRB SSB bandwidth off sync raster:

Issue 1-3-1b: NW indication on PBCH BW in MO configuration:

Issue 1-3-1c: NW indication on PBCH BW in HO command/SCell addition command/SIB4 message:

Issue 1-3-2: LS to RAN1 and RAN2:

CA

Issue 1-1-1a: Known SCell activation delay requirements:

Issue 1-1-1b: Unknown SCell activation delay requirements:

DC

Issue 1-2-1a: PSCell addition delay requirements:

Issue 1-2-1b: Conditional PSCell addition delay requirements:

Issue 1-2-1c: Subsequent Conditional PSCell addition delay requirements:

Issue 1-2-3a: Handover with PSCell:

Issue 1-2-3b: NR Conditional Handover including target MCG and target SCG:

Issue 1-2-4: SCG activation delay requirements:

Others

Issue 1-4-1: EMR requirements:

# Topic #1: RRM scope discussion

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2415540 | Apple | On RRM core for less than 5MHz Phase 2 Proposal 1: In CA/DC with less than 5MHz, if PCell is using 3MHz, the legacy R18 less than 5MHz RRM requirement in RRC connected mode can still be applied on this PCell. Proposal 2: Even though UE capability indicates to support fast SCell activation or inter-band SSB-less SCell activation, no requirement shall be applied for fast SCell activation or inter-band SSB-less SCell activation when the target SCell BW is less than 5MHz, i.e., UE capability of fast SCell activation and inter-band SSB-less SCell activation only applies to SCell on 5MHz and larger BW. Proposal 3: SCell activation requirement shall not be applied when CSI-RS is configured for L1-RSRP measurement/report for this being-activated SCell. Proposal 4: RAN4 to decide the corresponding RRM requirement for intra-band CA after RF session has concrete conclusions. Proposal 5: If SCell can use 12PRB SSB bandwidth in R19, it’s necessary to provide assistance information to UE regarding whether the PBCH is 12 or 20 PRBs in MO and/or HO command. Proposal 6: agree the following summary table. Section ID; CA/DC requirements; Impacts due to less than 5MHz band. 6.1.5; HO with PSCell; Yes. SSB index acquisition of less than 5MHz cell can be extended in legacy requirement.. 6.1.6; NR Conditional Handover including target MCG and target SCG; Yes. SSB index acquisition of less than 5MHz cell can be extended in legacy requirement.. 7.1; UE transmit timing; No, the reference cell for timing requirement can reuse the legacy assumption.. 7.5, 7.6; MRTD/MTTD requirement; No, reuse the legacy CA/DC requirement.. 8.2.2, 8.2.4; Interruption requirement related with SCell/PSCell operation; No, reuse the legacy interruption requirement.. 8.3; SCell Activation and Deactivation Delay; Need FFS, if target to-be-activated SCell is using 3MHz, the SSB index acquisition of less than 5MHz cell may extend the SCell activation requirement.. 8.4; UL carrier RRC reconfiguration delay; No, reuse the legacy requirement.. 8.5.9; Requirements for Beam Failure Recovery in SCell; No, reuse the legacy BFR requirement.. 8.9, 8.9A, 8.9C; NR-DC: PSCell Addition and Release Delay; Conditional PSCell Addition Delay; Subsequent Conditional PSCell Addition Delay; Need FFS, if target PSCell is using 3MHz, the SSB index acquisition of less than 5MHz cell may extend the PSCell addition requirement.. 8.11, 8.11B, 8.11E; PSCell Change; Conditional PSCell Change; Subsequent Conditional PSCell Change; Need FFS, if target PSCell is using 3MHz, the SSB index acquisition of less than 5MHz cell may extend the PSCell change requirement.. 8.17; SCG Activation and Deactivation Delay; Need FFS, if target PSCell is using 3MHz, the SSB index acquisition of less than 5MHz cell may extend the PSCell activation requirement.. 9.1.2; Measurement gap; No, reuse the legacy MG requirement.. 9.1.3; UE Measurement capability; No, reuse the legacy requirement.. 9.1.4; Capabilities for Support of Event Triggering and Reporting Criteria; No, reuse the legacy requirement.. 9.1.5; CSSF; No, reuse the legacy CSSF requirement.. 9.1.7~13; MG enh and MUSIM MG; NA for this WI (MG enh is not considered). 9.2.3; Number of cells and number of SSB (FR1); No, reuse the legacy requirement.. 9.2.5; Intra-frequency measurements without measurement gaps; Yes, time period for time index detection for deactivated SCell or deactivated PSCell can be extended of this SCell or PSCell is using 3MHz.. 9.2.6; Intra-frequency measurements with measurement gaps; No, reuse the legacy requirement. SCell and PSCell measurement is not in this section.. 9.2.7; Intra-frequency measurements with NCSG; NA for this WI (MG enh is not considered). 9.3; NR inter-frequency measurements; No, reuse the legacy requirement. SCell and PSCell measurement is not in this section.. |
| R4-2415862 | Huawei, HiSilicon | Discussion on RRM requirements for less than 5MHz Ph2 Proposal 1: For SCell with less than 5MHz BW, - SCell activation delay for unknown SCell is extended by [2] SMTC period, - FFS whether SCell activation delay for known SCell needs to be extended or not. Proposal 2: Wait for RF session conclusion before deciding whether to consider intra-band CA in RRM requirements for less than 5MHz in Rel-19. Proposal 3: Specify new PSCell change delay requirements for PSCell configured with less than 5MHz channel bandwidth. Proposal 4: RAN4 to define new SCG activation requirements for PSCell with 12RB PBCH, and same delay extension as for PSCell addition or PSCell change is re-used. Proposal 5: RAN4 to discuss whether NW indication on PBCH BW in MO configuration is needed considering a cell with less than 5MHz can be SCell. Proposal 6: RAN4 to define EMR requirements for target cell with 12RB PBCH. Delay requirements for SSB index reading in EMR is extended by [3] samples. |
| R4-2416085 | Intel Corporation | Discussion on the RRM requirements for Rel-19 less than 5MHz UE Proposal 1: Specify time period for time index detection for a UE operating on a target cell with 12PRB SSB, deactivated SCell in FR1 as in the following table. Proposal 2: Specify fine time tracking time equals 3 SSB periods instead of 1 when the target cell is with 12PRB SSB bandwidth, as is included in the SCell activation delay requirements in FR1 in Rel-19. Proposal 3: Specify time period for time index detection for a UE operating on a target cell with 12PRB SSB, deactivated PSCell in FR1 as in the following table. Table 9.2.5.1-y: Time period for time index detection for a UE operating on a target cell with 12PRB SSB, deactivated PSCell (FR1) DRX cycle; TSSB_time_index_intra. No DRX; Ceil(7 x Kp) x measCyclePSCell x CSSFintra. DRX cycle≤ 320ms; Ceil(7 x Kp) x max(measCyclePSCell, 1.5xDRX cycle) x CSSFintra. DRX cycle> 320ms; Ceil(7 x Kp) x max(measCyclePSCell, DRX cycle) x CSSFintra. Proposal 4: Specify fine time tracking time equals 3 SSB periods instead of 1 when the target cell is with 12PRB SSB bandwidth, as is included in the PSCell addition delay requirements. Proposal 5: Specify fine time tracking time equals 3 SSB periods instead of 1 when the target cell is with 12PRB SSB bandwidth, as is included in the PSCell change delay requirements. Proposal 6: RAN4 allows that 12 PRB SSB can be transmitted off sync raster. Proposal 7: Introduce network signalling in the MO to facilitate UE knowledge about the unknown target SSB bandwidth. Proposal 8: No need to introduce new indication of SSB bandwidth in HO command or in SCell addition command. Proposal 9: No need to introduce indications in SIB4 message for IDLE mode measurements. Proposal 10: Send an LS to RAN2 about introducing SSB bandwidth indications in the MO configurations. |
| R4-2416090 | Ericsson | Further Discussion on RRM requirements for NR CA/DC in less than 5 MHz Observation 1 The PBCH Puncturing impacts only case 3b and case 4. Observation 2 RAN4 RF discussed and conclude to not introduce new network signalling on PRB transmission. Based on the discussion in the previous sections, we propose the following: Proposal 1 RAN4 to support option 1, provided that no additional effort is needed and WID should be revised and approved. Proposal 2 RAN4 should set contiguous and non-contiguous intra-band CA as a second priority, after completing the inter-band work, if time permits, we can specify contiguous and non-contiguous intra-band CA. Proposal 3 For case 3b, SSB index reading is needed. It can be performed during the fine timing acquisition phase as agreed in Rel-18. Proposal 4 For case 4a and 4b, SSB index reading is needed during the cell search phase as UE need to perform L1-RSRP. Proposal 5 RAN4 to decide and limit the scope of the WI to Single SCell considering the time constraint. Proposal 6 RAN4 to capture the following text for PSCell activation delay for NR-DC. T∆ = Trs for both known and unknown target cells operating with 20 PRB SSB BW. T∆ = 3*Trs for both known and unknown target cells operating with 12 PRB SSB BW. Proposal 7 RAN4 to capture the following text for Conditional PSCell activation delay for NR-DC. T∆ = Trs for both known and unknown target cells operating with 20 PRB SSB BW. T∆ = 3*Trs for both known and unknown target cells operating with 12 PRB SSB BW. Proposal 8 The HO requirements with PSCell-PCell and PSCell-PSCell can be reused for NR CA/DC in less than 5MHz. Proposal 9 RAN4 should define conditional handover requirements to support NR CA/DC for less than 5 MHz. T∆ is time for fine time tracking and acquiring full timing information of the target cell. Where, T∆ = Trs for target cells operating with 20 PRB SSB BW. T∆ = 3*Trs for target cells operating with 12 PRB SSB BW. Proposal 10 RAN4 to capture the following text for NR Conditional Handover. T∆ is time for fine time tracking and acquiring full timing information of the target cell. Where, T∆ = Trs for target cells operating with 20 PRB SSB BW. T∆ = 3*Trs for target cells operating with 12 PRB SSB BW. Proposal 11 SSB time index detection for inter-frequency CA is extended by 4 samples. Proposal 12 RAN4 to capture the following text for SCG activation delay. T∆ = Trs for target cells operating with 20 PRB SSB BW. T∆ = 3*Trs for target cells operating with 12 PRB SSB BW. Proposal 13 RAN4 RRM should not introduce new network signalling on PRB transmission in alignment with RF. Proposal 14 RAN4 to discuss and study the impact of reduced PBCH BW of 12PRBs for 3MHz channel bandwidth on EMR measurement with SSB index reading. Proposal 15 RAN4 to send LS to inform potential impacts to other WGs in the November meeting after the RRM requirements are finalized. |
| R4-2416288 | Nokia | Discussion on RRM requirements for NR less than 5MHz phase 2 - Proposal 1: In rel-19, follow rel-18 baseline and add 2 additional SSB samples for SBI and MIB reading due to PBCH puncturing as part of T∆, where applicable. - Proposal 2: In rel-19, follow rel-18 baseline and add 2 additional SSB samples where SSB index reading is required. - Proposal 3: Consider only inter-band CA for contiguous and non-contiguous case. - Proposal 4: No changes on top of rel-18 EMR measurement reporting are needed. - Proposal 5: Legacy PSCell requirements are used as a baseline, T∆ may be updated, if needed. - Proposal 6: No changes needed, follow the agreement in 18. - Proposal 7: Legacy PSCell requirements are used as a baseline, T∆ may be updated, if needed. - Proposal 8: Discuss what information is needed and when CD-SSB of SCell can be off sync raster |
| R4-2416282 | ZTE Corporation, Sanechips | Discussion on RRM impacts for less than 5MHz BW Proposal 1: Focus on inter-band CA and wait until RF room has concrete conclusion on intra-band CA combinations. Observation 1: In R18, more time is specified to read the SSB due to the puncturing. With the same principle, more time maybe needed for SCell activation delay to support less than 5MHz bandwidth in the target SCell in FR1. Proposal 2: Need to study whether the SCell activation delay should be updated, for example, more time maybe needed for SCell activation delay to support less than 5MHz bandwidth in the target SCell in FR1. Observation 2: In Rel-18, SSB index identification delay - Unknown intra-frequency cell (Es/Iot≥-6 dB): 3+4 samples - Unknown inter-frequency cell (Es/Iot≥-4 dB): 3+3 samples Proposal 3: For the SSB index identification delay requirements for deactivated SCC, time index detection for a UE operating on a target cell with 12 PRB SSB defined in R18 can be used as baseline. Observation 3: In the latest version of TS 38.133, T∆ = 3*Trs for both known and unknown target cells operating with 12 PRB SSB BW. Proposal 4: Samilar as HO requirements, T∆ should be updated and additional Trs should be added for PSCell addition delay. Proposal 5: The extension for conditional PSCell Addition delay and subsequent conditoinal PSCell Delay delay should be consisered, additional Trs should be added for T∆. Observation 4: PScell change requirement is based on PScell addition and no need to update PScell change requirement. Proposal 6: The extension for conditional PSCell change delay and subsequent conditoinal PSCell change delay requirements should be consisered, additional Trs should be added for T∆. Observation 5: Partial components of handover with PSCell requirement is based on PScell addition and handover, maybe no need to update handover with PSCell requirement. Proposal 7: RAN4 to study impact of reduced PBCH BW of 12PRBs for 3MHz channel bandwidth on EMR measurement with SSB index reading. |
| R4-2416455 | MediaTek inc. | Discussion on core part for FR1 less than 5MHz Observation 1: The frequency spacing between two consecutive points of sync raster is equal to 100 kHz. Observation 2: the granularity of the global frequency raster is ΔFGlobal is equal to 5 kHz, which is much smaller than the granularity of the sync raster, which is equal to 100 kHz. Observation 3: the bands allowing <5MHz channel BW also supports other channel BW, UE cannot determine the SSB BW by simply checking the band index. Observation 4: for handover unknown cell the UE cannot predict whether this SSB is punctured or not because the cell was not measured and the SSB in that cell was not indicated by the MO. Proposal 1: RAN4 shall ask RAN2 to add the information of whether the SSB is 12 or 20 PRBs in the measurement object (MO). Proposal 2: There is no need to add the information of whether the SSB is 12 or 20 PRBs in the handover command (HO). Proposal 3: RAN4 shall send the following LS as shown in this document: - Overall Description: Based on discussions in RAN4#112-bis on the WI of NR_FR1_lessthan_5MHz_BW_Ph2, RAN4 has reached consensus that for the UE to differentiate whether the SSB is 12 PRBs or 20 PRBs during the measurements of SCell. Therefore, RAN4 would like to ask RAN2 to update the measurement object (MO) to include whether the SSB is 12 PRBs or 20 PRBs. 3. Actions: To RAN2: RAN4 respectfully asks RAN2 to take the above information into account and add the indication on whether the SSB is 12 PRBs or 20 PRBs in the measurement object (MO).. |
| R4-2416086 | Intel Corporation | Draft Big CR for less than 5MHz RRM requirements in Rel-19 |
| R4-2416389 | ZTE Corporation, Sanechips | draft CR to TS 38.133: NR channel BW less than 5MHz for FR1 |

## Open issues summary for topic #1

### Sub-topic 1-1 RRM scope for SCell configured with 12PRB SSB

Issue 1-1-1a: Known SCell activation delay requirements:

- Proposals

  - Option 1a: When the measurement period of the SCell being activated is larger than 2400ms, specify fine time tracking time equals 3 SSB periods instead of 1 when the target cell is with 12PRB SSB bandwidth.

  - Option 1b: When UE is not configured with beam level reporting for the SCell, extend the SCell activation delay by 2 Trs.

  - Option 2: No new requirement is specified on any case for known SCell activation.

- Recommended WF

  - Discussion is needed.

Issue 1-1-1b: Unknown SCell activation delay requirements:

- Proposals

  - Option 1: Specify fine time tracking time equals 3 SSB periods instead of 1 when the target cell is with 12PRB SSB bandwidth, as is included in the SCell activation delay requirements in FR1 in Rel-19.

If the SCell is unknown and belongs to FR1, and if one of the following conditions is met - ‘ssb-PositionInBurst’ indicates only one SSB is being actually transmitted, or - ‘ssb-PositionInBurst’ indicates multiple SSBs and TCI indication is provided in same MAC PDU with SCell activation, - TFirstSSB_MAX + TSMTC_MAX + 4*Trs + 5ms, if the UE supporting [12PRB SSB bandwidth UE feature] is configured with a target SCell that is with 12PRB SSB bandwidth.

- Otherwise - 6ms + TFirstSSB_MAX + TSMTC_MAX + 3 * Trs + TL1-RSRP, measure + TL1-RSRP,report + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP), if semi-persistent CSI-RS is used for CSI reporting, and the UE supporting [12PRB SSB bandwidth UE feature] is configured with a target SCell that is with 12PRB SSB bandwidth, - 3ms + TFirstSSB_MAX + TSMTC_MAX + 3 * Trs + TL1-RSRP, measure + TL1-RSRP,report + max(THARQ + Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay), if periodic CSI-RS is used for CSI reporting, and the UE supporting [12PRB SSB bandwidth UE feature] is configured with a target SCell that is with 12PRB SSB bandwidth.

- Recommended WF

  - Agree on option 1.

Issue 1-1-2: Measurement requirements for deactivated SCC:

- Proposals

  - Option 1: Specify time period for time index detection for a UE operating on a target cell with 12PRB SSB, deactivated SCell in FR1 as in the following table.

- Table 9.2.5.1-x: Time period for time index detection for a UE operating on a target cell with 12PRB SSB, deactivated SCell (FR1)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | Ceil(7 x Kp) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320ms | Ceil(7 x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320ms | Ceil(7 x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: The requirements also apply to deactivated SCG SCell. |  |

- Recommended WF

  - Agree on option 1.

Issue 1-1-3: Whether intra-band CA is in the scope:

- Proposals

  - Option 1: Consider intra-band CA (both contiguous and non-contiguous) within the scope of the RRM requirements in Rel-19.

  - Option 2: Wait until RF room has concrete conclusion on intra-band CA combinations.

  - Option 3: No need to discuss since there is no impact to RRM requirements in Rel-19.

- Recommended WF

  - Discussion is needed.

### Sub-topic 1-2 RRM scope for PSCell configured with 12PRB SSB

Issue 1-2-1a: PSCell addition delay requirements:

- Proposals

  - Option 1: Specify fine time tracking time equals 3 SSB periods instead of 1 when the target cell is with 12PRB SSB bandwidth, as is included in the PSCell addition delay requirements.

### 8.9.2 PSCell Addition Delay Requirement Upon receiving PSCell addition in subframe n, the UE shall be capable to transmit PRACH preamble towards PSCell in FR1 or FR2 no later than in slot n+Tconfig_PSCellNR slot length: where: T∆ is time for fine time tracking and acquiring full timing information of the target cell. T∆ = 1*Trs ms for a known or unknown PSCell. T∆ = 3*Trs ms for either known or unknown target PSCell operating with 12 PRB SSB BW.

- Recommended WF

  - Agree on option 1.

Issue 1-2-1b: Conditional PSCell addition delay requirements:

- Proposals

  - Option 1: RAN4 to capture the following text for Conditional PSCell activation delay for NR-DC: T∆ = 3*Trs for both known and unknown target cells operating with 12 PRB SSB BW.

- Recommended WF

  - Discussion is needed.

Issue 1-2-1c: Subsequent Conditional PSCell addition delay requirements:

- Proposals

  - Option 1: In subclause 8.9C.2 add: T∆ = 3*Trs ms for PSCell operating with 12 PRB SSB BW.

- Recommended WF

  - Discussion is needed.

Issue 1-2-2a: PSCell change delay requirements:

- Proposals

  - Option 1: Specify fine time tracking time equals 3 SSB periods instead of 1 when the target cell is with 12PRB SSB bandwidth, as is included in the PSCell change delay requirements.

## 8.11 PSCell Change This clause defines requirements for the delay within which the UE shall be able to change PSCell to other cell in EN-DC or NR-DC. The requirements in this clause are applicable to EN-DC and NR-DC. In the case of NR-DC PSCell change in FR1, when the UE supporting [12PRB SSB bandwidth UE feature] is configured with a target PSCell that is with 12PRB SSB bandwidth, the UE shall transmit PRACH preamble towards the target PSCell no later than the delay requirements specified in clause 8.9.2.

  - Option 2: No need to update the text.

- Recommended WF

  - Agree on option 1.

Issue 1-2-2b: Conditional PSCell change delay requirements:

- Proposals

  - Option 1: In subclause 8.11B.2, add T∆ = 3*Trs ms for PSCell operating with 12 PRB SSB BW.

- Recommended WF

  - Discussion is needed.

Issue 1-2-3a: Handover with PSCell:

- Proposals

  - Option 1: SSB index acquisition of less than 5MHz cell can be extended in legacy requirement.

  - Option 2: The HO requirements with PSCell-PCell and PSCell-PSCell can be reused for NR CA/DC in less than 5MHz.

- Recommended WF

  - Discussion is needed.

Issue 1-2-3b: NR Conditional Handover including target MCG and target SCG:

- Proposals

  - Option 1: SSB index acquisition of less than 5MHz cell can be extended in legacy requirement.

  - Option 2: T∆ = 3*Trs for target cells operating with 12 PRB SSB BW.

- Recommended WF

  - Discussion is needed.

Issue 1-2-4: SCG activation delay requirements:

- Proposals

  - Option 1: RAN4 to define new SCG activation requirements for PSCell with 12RB PBCH, and same delay extension as for PSCell addition or PSCell change is re-used.

    - RAN4 to capture the following text for SCG activation delay: T∆ = 3*Trs for target cells operating with 12 PRB SSB BW.

- Recommended WF

  - Agree on option 1.

Issue 1-2-5: Measurement requirements for deactivated PSCell:

- Proposals

  - Option 1: Specify time period for time index detection for a UE operating on a target cell with 12PRB SSB, deactivated PSCell in FR1 as in the following table.

- Table 9.2.5.1-y: Time period for time index detection for a UE operating on a target cell with 12PRB SSB, deactivated PSCell (FR1)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | Ceil(7 x Kp) x measCyclePSCell x CSSFintra |
| DRX cycle≤ 320ms | Ceil(7 x Kp) x max(measCyclePSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320ms | Ceil(7 x Kp) x max(measCyclePSCell, DRX cycle) x CSSFintra |

- Recommended WF

  - Agree on option 1.

### Sub-topic 1-3 Network indication on SSB bandwidth

Issue 1-3-1a: Allowing 12PRB SSB bandwidth off sync raster:

- Proposals

  - Option 1: RAN4 allows that 12 PRB SSB can be transmitted off sync raster except for FRMCS services deployed in band n100.

- Recommended WF

  - Agree on option 1.

Issue 1-3-1b: NW indication on PBCH BW in MO configuration:

- Background

  - RF room Agreement (online session)

    - No new signalling from network to inform UE the number of PRB transmission is required in the RF scope of this item.

  - Rapporteur comment

    - The intention of this agreement is to discuss any possible signalling design not in the RF room but in RRM room.

    - This agreement does not prevent any RRM discussions.

- Proposals

  - Option 1: Introduce network signalling in the MO to facilitate UE knowledge about the unknown target SSB bandwidth.

  - Option 2: RAN4 RRM should not introduce new network signalling on PRB transmission in alignment with RF.

- Recommended WF

  - Discussion is needed.

Issue 1-3-1c: NW indication on PBCH BW in HO command/SCell addition command/SIB4 message:

- Proposals

  - Option 1: No need to introduce new indication.

  - Option 2: If SCell can use 12PRB SSB bandwidth in R19, it’s necessary to provide assistance information to UE regarding whether the PBCH is 12 or 20 PRBs in MO and/or HO command.

- Recommended WF

  - Discussion is needed.

Issue 1-3-2: LS to RAN1 and RAN2:

- Proposals

  - Option 1: RAN4 sends RAN2 LS on the agreement if any about the network indications.

  - Option 2: RAN4 to send LS to inform potential impacts to other WGs in the November meeting after the RRM requirements are finalized.

- Recommended WF

  - Depend on discussions in Issue 1-3-1a/b/c.

### Sub-topic 1-4 Other RRM impacts

Issue 1-4-1: EMR requirements:

- Proposals

  - Option 1: RAN4 to define EMR requirements for target cell with 12RB PBCH. Delay requirements for SSB index reading in EMR is extended by [3] samples.

  - Option 2: No changes on top of rel-18 EMR measurement reporting are needed.

- Recommended WF

  - Discussion is needed.

Issue 1-4-2: Requirements applicability:

- Proposals

  - Proposal 1: Even though UE capability indicates to support fast SCell activation or inter-band SSB-less SCell activation, no requirement shall be applied for fast SCell activation or inter-band SSB-less SCell activation when the target SCell BW is less than 5MHz, i.e., UE capability of fast SCell activation and inter-band SSB-less SCell activation only applies to SCell on 5MHz and larger BW.

  - Proposal 2: SCell activation requirement shall not be applied when CSI-RS is configured for L1-RSRP measurement/report for this being-activated SCell

- Recommended WF

  - Discussion is needed.

Issue 1-4-3: Conditional handover delay requirements to PCell with 12PRB SSB:

- Proposals

  - Option 1: T∆ = 3*Trs for target PCell operating with 12 PRB SSB BW.

  - Option 2: No changes to Rel-18 requirement.

- Recommended WF

  - Discussion is needed.