3GPP TSG-RAN WG4 Meeting #112bis										          R4-2415653

Hefei, Anhui, China, 14th – 18th Oct 2024

Agenda item:			6.5.4

Source:	Moderator (Huawei, HiSilicon)

Title:	Topic summary for [112bis][206] NonCol_intraB_ENDC_NR_CA_Ph2

Document for:	Information

Introduction

This email thread discusses the RRM core requirements of WI on R19 Support of intra-band non-collocated EN-DC/NR-CA deployment Phase2: new receiver type(s).

Online handling:

(Online) Issue 1-1: Whether to introduce MRTD/MTTD requirements for non-collocated FR1 intra-band asynchronous NR-CA for Type 4 UE in Rel-19

(Online) Issue 1-2: Whether to introduce MRTD/MTTD requirements for FR1 inter-band asynchronous EN-DC with overlapping DL bands for TDD-TDD band combinations in Rel-19

Note: Issue 1-1 and Issue 1-2 are similar.

(Online) Issue 1-4: Interruption requirements for Type 4 capable UE when UE operates with shared RF chains in collocated condition

(Online) Issue 1-5: SCell activation delay requirements for Type 4 capable UE when UE operates with shared RF chains in collocated condition

(Online) Issue 1-6: Scheduling availability for Type 4 capable UE when UE operates with shared RF chain in collocated condition

Note: Issue 1-4, 1-5 and 1-6 are similar.

(Online) Issue 1-7: SCell BFD/CBD requirements for Type 4 capable UE

(Online) Issue 1-3: Other RRM requirements (apart from MRTD/MTTD requirements) for Type 4 UE

Topic #1:	RRM requirements

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2415519 | Apple | Proposal 1: Not to introduce MRTD/MTTD requirements for non-collocated FR1 intra-band asynchronous NR-CA for Type 4 UE in Rel-19. Proposal 2: The following RRM requirements (apart from MRTD/MTTD requirements) are also impacted due to Type 4 UE (what’s open is when collocated condition is met) ­ Interruption ­ SCell activation delay ­ Scheduling availability ­ SSB based and CSI-RS based BFD (Beam failure detection) and CBD (candidate beam detection) Proposal 3: The legacy FR1 inter-band CA interruption requirements apply for type 4 UE in the aspects of interruption at SCell addition/release/activation/deactivation and interruption during measurements on deactivated SCC. Need to wait for the RF session conclusion on signalling. Proposal 4: The existing SCell activation delay requirements in clause 8.3.2 apply for type 4 UE if the values of TSMTC_MAX and TFirstSSB_MAX for FR1 inter-band CA SCell activation delay requirements is reused for FR1 intra-band non-contiguous CA. Need to wait for the RF session conclusion on signalling. Proposal 5: Scheduling availability requirement should be introduced for type 4 UE performing beam failure detection with a different subcarrier spacing as PDSCH/PDCCH on FR1, L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1, measurement in TDD band, or measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1. Need to wait for the RF session conclusion on signalling. Proposal 6: It is suggested that that Type 4 UE can be configured to perform beam failure detection on no more than 2 serving cells per band. |
| R4-2415520 | Apple | Draft CR |
| R4-2415571 | Nokia, Nokia Shanghai Bell | Observation 1: Existing MRTD/MTTD requirements are not differentiated between synchronous and asynchronous for NR-CA. Proposal #1: Do not discuss MRTD/MTTD requirements for non-collocated FR1 intra-band asynchronous NR-CA. Proposal #2: If the UE type transitions between Type 4 and other types are agreed in RF session, the RRM requirements need to be adapted based on the eventual UE type in operation. Proposal #3: To wait for RF conclusion on the UE type transition before specifying the RRM requirements. |
| R4-2415840 | Huawei, HiSilicon | Proposal 1: Not to introduce MRTD/MTTD requirements for non-collocated FR1 intra-band asynchronous NR-CA for Type 4 UE in Rel-19. Proposal 2: Type 4 UE when operating in non-collocated condition can be configured with BFD/CBD measurements on up to two serving cells in the same band. Proposal 3: When type 4 capable UE operates in collocated condition, interruption requirements (below listed) defined for type 2 UE in collocated scenario in R18 can be applied - Interruption at SCell addition/release - Interruptions at SCell activation/deactivation - Interruptions during measurements on deactivated SCC Note: Some clarifications are needed with regard to the UE capability signalling and network indication(s). Proposal 4: When type 4 capable UE operates in collocated condition, SCell activation delay defined for type 2 UE in collocated condition in R18 can be applied. Note: Some clarifications are needed with regard to the UE capability signalling and network indication(s). Proposal 5: When type 4 capable UE operates in collocated condition, scheduling availability (below listed) defined for type 2 UE in collocated condition in R18 can be applied - scheduling availability in RLM - scheduling availability in BFD - scheduling availability in CBD - scheduling availability in measurement requirement in clause 9 Note: Some clarifications are needed with regard to the UE capability signalling and network indication(s). |
| R4-2416071 | Ericsson | Observation 1: Asynchronous operation means that we have no predefined relation between slot timing of different carriers to be aggregated and the handling at least a relative receive timing difference given by ±1/2 slot duration. Proposal 1: We are open to not to state MRTD/MTTD requirements for non-collocated FR1 intra-band asynchronous NR-CA and have this implicit. Max MRTD will be ±1/2 slot duration, regardless. Observation 2: We still have the same 2 CC non-colocated case in release 19, as in release 18. The only difference is that we had RANK2 MIMO per CC in release-18 and we have up to RANK 4 MIMO per CC in release-19. Proposal 2: The development for release-19 non-colocated intra band ENDC and NRCA requirements can use release-18 Scheduling restriction/availability, SCell operation related delay and interruptions as starting points for analysis. Observation 3: We are dealing with a shared receiver, also in type 4 (with different MIMO rank capability). Proposal 3: It is a fair assumption that when Type 4 capable UE operates with shared RF chains in collocated scenario, interruption requirements (below listed) defined for type 2 UE in collocated scenario in R18 can be applied for Interruption at SCell addition/release, Interruptions at SCell activation/deactivation and Interruptions during measurements on deactivated SCC. Observation 4: We are dealing with a shared receiver, also in type 4 (with different MIMO rank capability). Proposal 4: When Type 4 capable UE operates with shared RF chain in collocated condition, SCell activation delay defined for type 2 UE in collocated condition in R18 can be applied. Observation 5: We are dealing with a shared receiver, also in type 4 (with different MIMO rank capability). Proposal 5: Scheduling availability defined for type 2 UE in collocated condition in R18 can be applied for type 4. Observation 6: BFD/CBD measurements on up to two serving cells per band is in existing specification for rel-18 Type 2 UE. Proposal 6: Type 4 capable UE when operating in non-collocated condition can be configured with BFD/CBD measurements on up to two serving cells per band. |
| R4-2416130 | Samsung | Proposal 1: Not to introduce MRTD/MTTD requirements for non-collocated FR1 intra-band asynchronous NR-CA for Type 4 UE in Rel-19 Proposal 2: The impact on the following RRM requirements for type 4 UE can be the same as for type 2 UE in non-collocated condition - Scheduling restriction - Interruption - SCell activation delay - SSB/CSI-RS based BFD - SSB/CSI-RS based CBD Proposal 3: The following requirements for Type 2 capable UE when it operates with shared RF chain in collocated condition in Rel 18 can be applied to Type 4 capable UE when it operates with the same condition: - Interruptions at SCell addition/release/activation/deactivation/ during measurements on deactivated SCC - SCell activation delay requirement for Deactivated SCell - Scheduling availability Proposal 4: Type 4 capable UE when operating in non-collocated condition can be configured with BFD/CBD measurements on up to two serving cells per band Proposal 5: How to specify the impacted RRM requirements for Rel-19 type 4 UE need to wait for RF progress on the design of new UE capabilities/NW signallings |
| R4-2416386 | ZTE Corporation, Sanechips | Proposal 1: Similar as Type 2 UE, for Type 4 UE - Do not consider MRTD/MTTD requirements for non-collocated FR1 inter-band asynchronous EN-DC with overlapping DL bands for TDD-TDD band combinations in the scope of this work item - Do not introduce MRTD/MTTD requirements for non-collocated FR1 intra-band asynchronous NR-CA for Type 4 UE Observation 1: For type 4 capable UE, MRTD/MTTD and the other RRM requirements need to be adapted based on the eventual UE type in operation which is indicated by the BS signaling. Proposal 2: Requirements for Type 4 capable UE when UE operates with shared RF chains should be discussed based on RF progress on UE type transition and design of new UE capabilities/NW signalling. Proposal 3: For Type 4 UE with FR1 intra-band CA, there are no scheduling restrictions on carriers in the same band. |

Open issues summary

Sub-topic 1: RRM requirements impact

(Online) Issue 1-1: Whether to introduce MRTD/MTTD requirements for non-collocated FR1 intra-band asynchronous NR-CA for Type 4 UE in Rel-19

- Proposals

  - Option 1(Apple, Nokia, Huawei, Samsung, KDDI, ZTE): Not to introduce MRTD/MTTD requirements for non-collocated FR1 intra-band asynchronous NR-CA for Type 4 UE in Rel-19.

- Recommended WF

Agree on option 1.

(Online) Issue 1-2: Whether to introduce MRTD/MTTD requirements for FR1 inter-band asynchronous EN-DC with overlapping DL bands for TDD-TDD band combinations in Rel-19

- Proposals

  - Option 1(ZTE): Not to introduce MRTD/MTTD requirements for FR1 inter-band asynchronous EN-DC with overlapping DL bands for TDD-TDD band combinations in Rel-19.

- Recommended WF

Further discussion.

(Online) Issue 1-3: Other RRM requirements (apart from MRTD/MTTD requirements) for Type 4 UE

- Proposals

  - Option 1(Apple, Huawei, Ericsson, Samsung, KDDI): The following RRM requirements (apart from MRTD/MTTD requirements) are also impacted due to Type 4 UE

- Interruption

- SCell activation delay

- Scheduling availability

- SSB based and CSI-RS based BFD (Beam failure detection) and CBD (candidate beam detection)

- Recommended WF

Agree on option 1.

(Online) Issue 1-4: Interruption requirements for Type 4 capable UE when UE operates with shared RF chains in collocated condition

- Proposals

  - Option 1(Huawei, Ericsson, Samsung, KDDI): when Type 4 capable UE operates in collocated condition, interruption requirements (below listed) defined for type 2 UE in collocated scenario in R18 can be applied:

- Interruption at SCell addition/release

- Interruptions at SCell activation/deactivation

- Interruptions during measurements on deactivated SCC

Note: Some clarifications are needed with regard to the UE capability and network signalling. The specific UE capability and network signalling can wait for the RF session conclusion.

  - Option 2 (Nokia, ZTE): If the UE type transitions between Type 4 and other types are agreed in RF session, the RRM requirements need to be adapted based on the eventual UE type in operation. To wait for RF conclusion on the UE type transition before specifying the RRM requirements.

- Recommended WF

Further discussion on whether option 1 is agreeable.

(Online) Issue 1-5: SCell activation delay requirements for Type 4 capable UE when UE operates with shared RF chains in collocated condition

- Proposals

  - Option 1(Apple, Huawei, Ericsson, Samsung, KDDI): When Type 4 capable UE operates in collocated condition, SCell activation delay defined for type 2 UE in collocated scenario in R18 can be applied.

Note: Some clarifications are needed with regard to the UE capability and network signalling. The specific UE capability and network signalling can wait for the RF session conclusion.

  - Option 2 (Nokia, ZTE): If the UE type transitions between Type 4 and other types are agreed in RF session, the RRM requirements need to be adapted based on the eventual UE type in operation. To wait for RF conclusion on the UE type transition before specifying the RRM requirements.

- Recommended WF

Further discussion on whether option 1 is agreeable.

(Online) Issue 1-6: Scheduling availability for Type 4 capable UE when UE operates with shared RF chain in collocated condition

- Proposals

  - Option 1(Apple, Huawei, Ericsson, Samsung, KDDI): When type 4 capable UE operates in collocated scenario, scheduling availability defined for type 2 UE in collocated scenario in R18 can be applied.

Note: Some clarifications are needed with regard to the UE capability and network signalling. The specific UE capability and network signalling can wait for the RF session conclusion.

  - Option 2 (Nokia, ZTE): If the UE type transitions between Type 4 and other types are agreed in RF session, the RRM requirements need to be adapted based on the eventual UE type in operation. To wait for RF conclusion on the UE type transition before specifying the RRM requirements.

- Recommended WF

Further discussion on whether option 1 is agreeable.

(Online) Issue 1-7: SCell BFD/CBD requirements for Type 4 capable UE

- Proposals

  - Option 1(Apple, Huawei, Ericsson, Samsung, KDDI, ZTE): Type 4 capable UE when operating in non-collocated condition can be configured with BFD/CBD measurements on up to two serving cells per band.

- Recommended WF

Agree on option 1.