3GPP TSG-RAN WG4 Meeting # 112bis											R4-2415659

Hefei, Anhui, China, 14th ‒ 18th, October, 2024

Agenda item:			6.19.4

Source:	Moderator (Samsung)

Title:	Topic summary for [112bis][212] NR_MIMO_Ph5_Part1

Document for:	Information

Introduction

This topic summary covers the contributions submitted under the following AI for first part of Rel-19 RRM for NR MIMO Phase 5:

- 6.19	NR MIMO Phase 5

- 6.19.3 RRM core requirements

  - 6.19.3.1 Enhancement for UE-initiated/event-driven beam management

Recommended issues to online discussion:

Issue 1-1

Issue 1-2

Issue 1-3

Issue 1-5

Issue 1-8

Issue 1-11

Issue 1-13

Topic #1: RRM requirements for UE-initiated/event-driven beam management

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2415075 | Xiaomi | Observation 1: For legacy event triggered L3 reporting, the starting point and ending point is defined as: - the starting point of reporting is the time when channel condition of cell is changed and UE needs to evaluate for L3 report. - The ending point of reporting is the time when UE starts to send L3 report Observation 2: For legacy event triggered L3 reporting, the maximum delay is defined as one measurement period for neighbour cell to guarantee that UE can obtain at least one L3 result to compare with criteria. Observation 3: For legacy event triggered L3 reporting, No L3 related processing and no UL resource waiting time are included in reporting delay. Proposal 1: For L1 event-driven/UE-initiated reporting, the starting point is an event that beam quality of current beam or other beams has changed. From that point on, the UE needs to evaluate for L1 reporting through measurement. Proposal 2: For L1 event-driven/UE-initiated reporting, the ending point is the time when UE starts to send L1 report. Proposal 3: If multiple beams are measured with different measurement period, RAN4 to discuss how to consider the total measurement time. Proposal 4: For measurement time, RAN4 needs to consider the impact of triggering window defined in RAN1. Proposal 5: RAN4 needs to consider the impact of UL resource waiting time for L1 event triggered reporting. Proposal 6: RAN4 to discuss whether and how to define requirement when measurement period is shorter than the UL resource waiting time. Proposal 7: No new accuracy requirements for existing L1 measurements are required. |
| R4-2415238 | CMCC | Proposal 1: for UE-initiated/event-driven beam reporting, it is proposed that the reporting delay is defined as the time between an event that will trigger a beam report and the point when the UE starts to transmit the beam report over the air interface. Proposal 2: for UE-initiated/event-driven beam reporting, it is proposed to support both mode A and mode B. Proposal 3: it is proposed to follow the approach for L3 measurement report delay to add clarification in the spec that the beam reporting delay excludes a delay which caused by no UL resources being available for UE to send the measurement report on. In this case, no need to differentiate mode A and mode B. Proposal 4: for reporting delay for SSB based UE-initiated/event-driven beam reporting, it is proposed to take measurement period defind for Rel-18 LTM as baseline - in detail, the reporting delay for SSB bsed UE-initiated/event-driven beam reporting is less than TL1-RSRP_Measurement_Period or TL1-RSRP_Measurement_Period + TSSB_time_index , which are specified for L1-RSRP measurement for Rel-18 LTM . Proposal 5: for UE-initiated/event-driven beam reporting, except reporting delay requirements, it is proposed to define reporting accuracy requirements. - The reporting accuracy requirement is that the reported L1 measurement contained in event triggered beam reports shall meet the accuracy requirements - For SSB based beam report, Rel-18 LTM accuracy requirements can be used as baseline. - For CSI-RS based beam reprt, FFS whether to use legacy accuracy requirements which only cover serving cell, or to define new accuracy requirements. Proposal 6: for UE-initiated/event-driven beam reporting, it is proposed to define requirements for both the case without filtering and the case with filtering (i.e. timer and counter M). Proposal 7: it is proposed that same reporting requirements are applied to the case without filtering and the case with filtering (i.e. timer and counter M). |
| R4-2415301 | Qualcomm Technologies Ireland | Observation 1: The time between the triggering event and the transmission of the 1st PUCCH is common in both mode A and mode B. Proposal 1: RAN4 should use the time when the 1st PUCCH can be sent as the reference point for the definition of the delay requirements (Issue 2-1-3). Observation 2: Because of the periodicity of resources used for measurements, a certain delay Tgap can occur till the UE can start the L1 measurements for the triggered event. Observation 3: Depending how long the L1 measurements take to identify the event, there is also a certain gap called TRS from the end of the measurements till the next PUCCH opportunity to inform the network. Observation 4: Both delays Tgap and TRS contribute to the overall measurement reporting delay. Their magnitude depend on the periodicity of the CSI-RS / SSB resources and the PUCCH resources. Proposal 2: Both delays Tgap (time from occurrence of the event till first available resource for measurement) and TRS (time from detection of event in the UE till first available PUCCH resource for event reporting) need to be taken into account for defining the measurement reporting delay (Issue 2-1-3). Observation 5: Requirements for L1-RSRP reporting based on SSB and CSI-RS measurement are already defined in [3], section 9.5 fulfilling the absolute and relative measurement accuracy requirements defined [3], section 10.1. Proposal 3: RAN4 should investigate whether those measurement periods can be reused for defining measurement reporting delay requirements for UE-initiated/event-driven beam management (Issue 2-1-3). Proposal 4: No additional accuracy requirements are needed for L1-RSRP measurements (Issue 2-1-11). Proposal 5: No additional time-window-based filtering or other filtering should be applied for the L1-RSRP measurements. (Issue 2-1-8). Observation 6: RAN4 should wait for RAN1 to make further progress whether TRS can be used for L1-RSRP measurement. There is no need for RAN4 to spend time on this point as of now. Observation 7: In Rel-17, measurement requirements for inter-cell beam management have only been introduced for SSB based L1-RSRP but not for CSI-RS based L1-RSRP. Proposal 6: RAN4 should not introduce in Rel-19 CSI-RS based L1-RSRP measurements for inter-cell beam management as long as RAN1 has not identified a clear use case (Issue 2-1-6). Observation 8: The existing conditions for known and unknown TCI state can be reused for UE-initiated/event-triggered beam management. Proposal 7: RAN4 should investigate whether any new required TCI state switch delays need to be defined or existing requirements are sufficient (Issue 2-1-9). Proposal 8: Discussions on beam sweeping enhancements should be postponed till the baseline requirement for delay and measurement period have taken place (Issue 2-1-10). |
| R4-2415508 | Apple | Observation #1: RAN1 would possibly define additional events other than Event 2 to trigger beam reporting. Observation #2: The same design as Event 2 will be reused for other events introduced. Observation #3: RAN4 requirements should be agnostic to the event that triggers the UE measurement report. Proposal #1: RAN4 defined requirements for UE initiated beam management are applicable to all events covered in RAN1. Observation #4: UE sends PUCCH for both Mode-A and Mode-B and the time to transmit PUCCH is the same for both modes. Observation #5: Time to PUCCH transmission need not be distinguished for the two modes. Observation #6: The time for beam report on 2nd UL channel is different for Mode-A and Mode-B. Observation #7: It would be incomplete to define reporting delay requirements up to PUCCH transmission only. Proposal #2: RAN4 should cover event triggered measurement reporting delay for both Mode-A and Mode-B – Starting point is when event is triggered at the UE; Ending point is when the UE transmits the measurement report on the 2nd UL channel. Proposal #3: Report delay is defined as Tfirst-PUCCH or TPUCCH + TDCI + T1. Where TDCI = 0 for Mode B, Time to receive DCI with UL grant in Mode A; T1 = K2 for Mode A; Tfirst-PUSCH or TPUSCH for Mode-B Observation #8: It would be incomplete to define only reporting delay requirements without covering measurement period/delay for the new beams for event triggered beam report. Proposal #4: Define measurement period for new beam measurement for event triggered reporting with L1-RSRP measurement period as baseline for intra-cell SSB, inter-cell SSB, CSI-RS. Observation #9: It would be incomplete to define only reporting delay requirements without covering measurement period/delay for the new beams for event triggered beam report. Proposal #5: From legacy measurement period, for new beam report remove – (1) Treport (2) considerations for HST enhancements/ scenarios Proposal #6: From legacy measurement period, for new beam report further discuss if M is needed or can be removed/assumed 1. Proposal #7: From legacy measurement period, for new beam report further discuss if P can be the same or modified for simplification. Proposal #8: Do not introduce any filtering for L1-RSRP – time window based or any other filtering for new beam measurement for event triggered reporting. Observation #10: No impact of UE triggered beam report to known/unknown conditions for TCI state. Observation #11: The known/unknown condition would be determined by the same criteria as legacy. Observation #12: Faster RX, TX beam refinement is not in scope of this WI. Proposal #9: Use legacy assumptions for beam sweeping for requirements for UE/ event triggered beam reporting. Proposal #10: Legacy accuracy requirements for L1-RSRP shall be applicable for UEIBM. |
| R4-2415534 | vivo | Observation 1 For Mode-A, if the one-bit/multi-bit PUCCH is SR, then following legacy UE behaviour, UE needs not to send such SR, if gNB has already provides the uplink grant that is big enough to carry the UCI when UE generates the UCI. In this case requirements in 9.2.4 of TS 38.133 would be a starting point for event-triggered L1 reporting delay. Proposal 1 RAN4 specifies RRM requirements at least for Mode-A UEIBR. Proposal 2 RAN4 does not define RRM requirements for Mode-B if it can only be carried by CG-PUSCH. If RAN1 comes to further conclusion that it can be carried by PUCCH or DG-PUSCH, RAN4 may also define RRM requirements for Mode-B. Proposal 3 To determine how to count for the uncertainty in acquiring UL resource for UEIBR, RAN4 waits further conclusion from RAN1 on whether the first PUCCH is SR or not. Proposal 4 RAN4 specify RRM requirements for event-triggered L1 reporting without time-window-based filtering or other filtering configured. Proposal 5 Regarding the impact to requirement applicability conditions of TCI state activation from event-triggered L1 reporting, RAN4 waits further RAN1/2 conclusions about the detailed UE reporting behaviour, e.g. whether single-shot or multi-shot reporting after event-entering condition is met, and whether UE reports when event-leaving condition is met, etc. |
| R4-2415551 | Samsung | Proposal 1: RAN4 to specify the Event triggered measurement reporting delay as time between an event that will trigger a measurement report and the point when the UE transmits a first PUCCH. Proposal 2: If P1 is agreed, this requirement can be applied for both mode A and mode B. There is no difference of the delay for mode A and mode B. Proposal 3: The event triggered measurement reporting delay shall be within T_L1_measurement_period. T_L1_measurement_period can be based on TL1-RSRP_Measurement_Period_SSB or TL1-RSRP_Measurement_Period_CSI-RS. If it is based on SSB for inter-cell, T_L1_measurement_period can be based on TL1-RSRP_Measurement_Period_SSB_CDP. Proposal 4: For FR2, the T_L1_measurement_period is scaled by N. Proposal 5: No need to define new TCI state switching delay requirements. This can be revisited if the RS in the RS resource set can be updated by MAC-CE in further RAN1 conclusion. Proposal 6: The L1-RSRP measurement period in event triggered measurement reporting delay is no need to be filtered. Proposal 7: No enhancement for beam sweeping in event triggered L1 beam reporting. Proposal 8: The existing L1-RSRP accuracy requitements can be reused for event-driven beam management. |
| R4-2415701 | ZTE Corporation, Sanechips | Observation 1: When considering the measurement reporting delay, no need to distinguish between Mode A and B as they only develop different UL resource request/allocation solutions, which is RAN1 scope, RAN4 does not need to penetrate into this. Proposal 1: A universal requirement of measurement reporting delay could be used for both Mode A and B. Observation 2: In legacy L1-RSRP measurement, - only measurement period requirement is defined with the assumption of periodic/semi-persistent reporting, not touch aperiodic reporting. - Not any measurement reporting delay requirement is defined, only some reporting condition are clarified. Observation 3: In legacy event triggered of RRM measurment reporting, the measurement reporting delay is specified. Observation 4: RAN 1 has defined two types of trigger-event detection for UEIBM, i.e. the basic trigger-event detection and the optional trigger-event detection. Proposal 2: The requirement of measurement reporting delay for UEIBM can be defined as the time between an trigger-event detection and the point when UE starts to transmit the reporting on the allocated UL resource over the air interface. Proposal 3: For CSI-RS based inter-cell L1-RSRP measurement, since no specific discussion before, RAN4 needs to identify whether additional clarifications on the following points are necessary: - Known condition for the NSC, especially for the time offset assumption between serving cell and NSC; - The prerequisite on L3 measurement on NSC. Proposal 4: Discuss the necessity of filtering based on the two trigger-event detection solutions approved in RAN1. - For the basic solution, one-shot measurement is defined by RAN1. No room to consider filtering. - For the optional solution, multi-shot measurement with at least M instance is defined by RAN1. Filtering is feasible. Proposal 5: Discuss the accuracy for the two trigger-event solutions respectively. - For the basic trigger-event detection, tight the existing L1-RSRP accuracy requirement; - For the optional trigger-event detection, resue the existing L1-RSRP accuracy requirement. |
| R4-2415830 | Huawei, HiSilicon | Observation 1: Following events are supported based on RAN1 progress: - Event-1: Quality of the current beam is worse than a certain threshold. - Event-2: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the current beam. - Event-7: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the M-th best quality. Observation 2: From measurement behaviour and requirements perspective, no significant difference among these Events. Observation 3: For the current beam and new beam determination, at least followings are agreed in RAN1 so far: - Current beam: Implicitly derived from a QCL RS of indicated TCI state (QCL RS or SSB QCL-ed with QCL RS) - FFS one TRS case - New beam: configured in one RS resource set associated with an CSI reporting configuration Observation 4: For Event-2, following two triggering determinations are supported: - Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs - If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs. Proposal 1: Mode A and Mode B has minor impact on RRM requirements (e.g. reporting uncertainty). Both Mode A and Mode B are supported, and no differentiation between Mode A and Mode B unless necessary. Proposal 2: RAN4 to discuss how to define one Event-2 instance for following Event-2 case: - If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs. - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam |
| R4-2416016 | Nokia | Proposal 1: Define event-triggered L1 reporting delay requirements for both mode A and mode B. Observation 1: In both mode A and mode B the delay to send the UEI beam report in a second UL channel after the UL indication in a first PUCCH channel has already been defined by RAN1 work. Proposal 2: RAN4 to define event-triggered L1 reporting delay requirements with a maximum delay for the UE transmission of the UL indication in the first PUCCH channel after the event that triggers the beam report is met. Proposal 3: The maximum delay requirement for the first PUCCH channel is determined in relation to the reception time instant of the RS that triggered the UEIBM event. Proposal 4: Reuse known/unknown conditions for Rel-17 unified TCI states when defining requirements for TCI-switching based on UEIBM. Proposal 5: Wait for further RAN1 progress before discussing TCI switching delay requirements. Proposal 6: Reuse legacy measurement accuracy requirements for measurement reports with UEIBM. Proposal 7: Specify RAN4 requirements for a UE supporting Rel-18 multi-Rx fast beam sweeping and configured with UEIBM reporting. Proposal 8: Wait for further RAN1 progress before discussing specific RRM requirements with configured time window and configured number of event instances within the time window for event determination. |
| R4-2416334 | Ericsson | Proposal 1: Event triggered measurement report requirements are applicable for intra and inter-cell scenario for SSB and CSI-RS. Proposal 2: RAN4 to define CSI-RS based L1-RSRP measurements requirements for inter-cell mTRP scenario in FR2. Proposal 3: RAN4 to use principles of sharing agreed for SSB based L1-RSRP for inter-cell CSI-RS based L1-RSRP measurements. Proposal 4: SSB based L1-RSRP measurement delay for event triggered measurement report is TL1-RSRP_Measurement_Period_SSB (ms) with TReport as 0. Proposal 5: CSI-RS based L1-RSRP measurement delay for event triggered measurement report is TL1-RSRP_Measurement_Period_CSI-RS with TReport as 0. Proposal 6: RAN4 to discuss the event evaluation time (i.e., the minimum time required to complete the event evaluation and be ready to send on first UL channel from the measurement occasion). Proposal 7: Existing unified TCI state switching delay requirements are applicable for event triggered measurement reporting based TCI state switching. Proposal 8: RAN4 not to specify new Rx beam sweeping factor or new accuracy requirements as part of the event triggered measurement requirements. |
| R4-2416513 | MediaTek inc. | Proposal 1: RAN4 to discuss two phases of UE-initiated/event-driven beam reporting: - Phase 1: Before UE initiate event beam reporting.  The measurement delay for L1-RSRP is the same for both Mode A and Mode B. - Phase 2: After UE initiate event beam reporting.  The reporting delay could be differentiated based on RAN1 conclusion. Proposal 2: RAN4 to reuse L1-RSRP measurement requirement as a baseline for UE-initiated/event-driven beam management on the following aspect: - Requirement applicability. - Measurement requirement for aperiodic reporting. Proposal 3: RAN4 to consider only periodic CSI-RS based L1-RSRP measurement for UE initiated reporting. Proposal 4: RAN4 to reuse the legacy L1-RSRP accuracy requirement for UE-initiated/event-driven beam management. Proposal 5: Define a capability for supported number of UE-initiated/event-driven beam reporting on intra-frequency. Proposal 6: Events configured for mobility and MIMO should be considered as different events even when they are configured for the same beams and the same cell with the same threshold. Proposal 7: Define the single capability for total number of L1 events shared by mobility L1 events and MIMO L1 events on intra-frequency. |

Open issues summary

Issue 1-1: How to define Event triggered measurement reporting delay?

- Proposals

  - Option 1: (Xiaomi)

    - Starting point is an event that beam quality of current beam or other beams has changed. From that point on, the UE needs to evaluate for L1 reporting through measurement.

    - Ending point is the time when UE starts to send L1 report.

  - Option 2: (CMCC, ZTE)

    - Reporting delay is defined as the time between an event that will trigger a beam report and the point when the UE starts to transmit the beam report over the air interface.

      - Option 2a: (CMCC)

      - SSB based:

        - Less than TL1-RSRP_Measurement_Period or TL1-RSRP_Measurement_Period + TSSB_time_index , which are specified for L1-RSRP measurement for Rel-18 LTM .

  - Option 3: (Qualcomm)

    - RAN4 should use the time when the 1st PUCCH can be sent as the reference point for the definition of the delay requirements

    - Both delays Tgap (time from occurrence of the event till first available resource for measurement) and TRS (time from detection of event in the UE till first available PUCCH resource for event reporting) need to be taken into account for defining the measurement reporting delay

  - Option 4: (Apple)

    - Starting point is when event is triggered at the UE

    - Ending point is when the UE transmits the measurement report on the 2nd UL channel.

    - Report delay is defined as Tfirst-PUCCH or TPUCCH + TDCI + T1. Where TDCI = 0 for Mode B, Time to receive DCI with UL grant in Mode A; T1 = K2 for Mode A; Tfirst-PUSCH or TPUSCH for Mode-B

  - Option 5: (vivo)

    - RAN4 does not define RRM requirements for Mode-B if it can only be carried by CG-PUSCH. If RAN1 comes to further conclusion that it can be carried by PUCCH or DG-PUSCH, RAN4 may also define RRM requirements for Mode-B.

  - Option 6: (Samsung)

    - RAN4 to specify the Event triggered measurement reporting delay as time between an event that will trigger a measurement report and the point when the UE transmits a first PUCCH. No difference for mode A and mode B.

  - Option 6: (Huawei)

    - Mode A and Mode B has minor impact on RRM requirements (e.g. reporting uncertainty). Both Mode A and Mode B are supported, and no differentiation between Mode A and Mode B unless necessary.

  - Option 7: (Nokia)

    - RAN4 to define event-triggered L1 reporting delay requirements with a maximum delay for the UE transmission of the UL indication in the first PUCCH channel after the event that triggers the beam report is met.

    - The maximum delay requirement for the first PUCCH channel is determined in relation to the reception time instant of the RS that triggered the UEIBM event.

  - Option 8: (Ericsson)

    - RAN4 to discuss the event evaluation time (i.e., the minimum time required to complete the event evaluation and be ready to send on first UL channel from the measurement occasion).

  - Option 9: (MediaTek)

    - RAN4 to discuss two phases of UE-initiated/event-driven beam reporting:

      - Phase 1: Before UE initiate event beam reporting.

        - The measurement delay for L1-RSRP is the same for both Mode A and Mode B.

      - Phase 2: After UE initiate event beam reporting.

        - The reporting delay could be differentiated based on RAN1 conclusion.

- Recommended WF

  - RAN4 to discuss and specify what is Event triggered measurement reporting delay?

  - Start point of Event triggered measurement reporting delay

  - End point of Event triggered measurement reporting delay

Moderator: From the contributions, it can be observed that companies have different views of how to counted the event triggered measurement reporting delay. And for mode A and B, except the measurement time typically in RAN4, how to reflect the different time for UL resource in the total measurement reporting delay is related to Issue 1-5

Issue 1-2: How to define measurement period in Event triggered measurement reporting delay?

- Proposals

  - Option 1: (Xiaomi)

    - If multiple beams are measured with different measurement period, RAN4 to discuss how to consider the total measurement time.

  - Option 2: (CMCC)

    - It is proposed to take measurement period defend for Rel-18 LTM as baseline

  - Option 3: (Qualcomm)

    - RAN4 should investigate whether those measurement periods can be reused for defining measurement reporting delay requirements for UE-initiated/event-driven beam management

  - Option 4: (Apple, Samsung, Ericsson, MediaTek)

    - Use legacy measurement period for TL1-RSRP_Measurement_Period_SSB for SSB based intra-cell , inter-cell, TL1-RSRP_Measurement_Period_CSI-RS for CSI RS based.

    - Option 4a (Apple, Ericsson)

      - Remove Treport or set to 0 in legacy requirements.

    - Option 4b (Apple)

      - Remove considerations for HST enhancements/ scenarios

      - From legacy measurement period, for new beam report further discuss if M is needed or can be removed/assumed 1.

      - From legacy measurement period, for new beam report further discuss if P can be the same or modified for simplification.

    - Option 4c: (Samsung)

      - For FR2, the T_L1_measurement_period is scaled by N.

- Recommended WF

  - To be discussed

Moderator: For the measurement period: Whether to reuse legacy L1-RSRP measurement period or Rel-18 LTM. Besides that, from the contributions, it can be observed that companies have different views of the relation of measurement period and the time point of “Event-triggered” or “Event-trigger condition is met”. Need further discuss and clarification. In addition, how to handle it multiple beams should be measured?

Issue 1-3: Applicability of Event triggered measurement reporting delay requirements?

- Proposals

  - Option 1: (Apple, Ericsson)

    - Applicable for all events

  - Option 2: (Xiaomi, CMCC, Qualcomm, Apple, Samsung, Huawei, Nokia, Ericsson, ZTE)

    - Cover both Mode A and B

  - Option 3: (vivo)

    - At least for Mode A.

- Recommended WF

  - Applicable for Event-1; Event-2; Event-7?

  - Both Mode A and B?

Issue 1-4: How to consider the time window of triggering event determination in Event triggered measurement reporting?

- Proposals

  - Option 1: (Xiaomi)

    - RAN4 needs to consider the impact of triggering window defined in RAN1.

  - Option 2: (CMCC)

    - It is proposed to define requirements for both the case without filtering and the case with filtering (i.e. timer and counter M).

    - It is proposed that same reporting requirements are applied to the case without filtering and the case with filtering (i.e. timer and counter M)

  - Option 3: (Qualcomm, Apple, Samsung, vivo)

    - No additional time-window-based filtering or other filtering should be applied for the L1-RSRP measurements

  - Option 5 (ZTE)

    - Discuss the necessity of filtering based on the two trigger-event detection solutions approved in RAN1.

- For the basic solution, one-shot measurement is defined by RAN1. No room to consider filtering.

- For the optional solution, multi-shot measurement with at least M instance is defined by RAN1. Filtering is feasible.

  - Option 6 (Huawei)

    - RAN4 to discuss how to define one Event-2 instance for following Event-2 case:

      - If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs.

        - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam

  - Option 7 (Nokia)

    - Wait further RAN1 progress.

- Recommended WF

  - To be discussed

Issue 1-5: How to consider UL resource in Event triggered measurement reporting?

- Proposals

  - Option 1: (Xiaomi)

    - RAN4 needs to consider the impact of UL resource waiting time for L1 event triggered reporting.

  - Option 2: (CMCC, ZTE)

    - The beam reporting delay excludes a delay which caused by no UL resources being available for UE to send the measurement report on.

    - No need to differentiate mode A and mode B

  - Option 3: (Apple)

    - Report delay is defined as Tfirst-PUCCH or TPUCCH + TDCI + T1. Where TDCI = 0 for Mode B, Time to receive DCI with UL grant in Mode A; T1 = K2 for Mode A; Tfirst-PUSCH or TPUSCH for Mode-B

  - Option 4: (vivo)

    - To determine how to count for the uncertainty in acquiring UL resource for UEIBR, RAN4 waits further conclusion from RAN1 on whether the first PUCCH is SR or not (for Mode-A).

- Recommended WF

  - To be discussed

Issue 1-6: Whether to define requirements if end of measurement period is before the UL resource?

- Proposals

  - Option 1: (Xiaomi)

    - RAN4 to discuss whether and how to define requirement when measurement period is shorter than the UL resource waiting time

- Recommended WF

  - To be discussed

Issue 1-7: Whether to support CSI-RS based L1-RSRP for aperiodic?

- Proposals

  - Option 1: (MediaTek)

    - RAN4 to consider only periodic CSI-RS based L1-RSRP measurement for UE initiated beam reporting.

- Recommended WF

  - Since RAN1 only agree periodic CSI-RS until now, RAN4 can only define periodic CSI-RS measurement

Issue 1-8: CSI-RS based L1-RSRP for inter-cell BM?

- Proposals

  - Option 1: (Qualcomm)

    - RAN4 should not introduce in Rel-19 CSI-RS based L1-RSRP measurements for inter-cell beam management as long as RAN1 has not identified a clear use case

  - Option 2: (ZTE)

    - For CSI-RS based inter-cell L1-RSRP measurement, since no specific discussion before, RAN4 needs to identify whether additional clarifications on the following points are necessary:

-  Known condition for the NSC, especially for the time offset assumption between serving cell and NSC;

-  The prerequisite on L3 measurement on NSC.

  - Option 3: (Ericsson)

    - RAN4 to define CSI-RS based L1-RSRP measurements requirements for inter-cell mTRP scenario in FR2.

    - RAN4 to use principles of sharing agreed for SSB based L1-RSRP for inter-cell CSI-RS based L1-RSRP measurements.

- Recommended WF

  - To be discussed

Issue 1-9: Whether to specify new unified TCI switching delay requirements?

- Proposals

  - Option 1: (Qualcomm)

    - RAN4 should investigate whether any new required TCI state switch delays need to be defined or existing requirements are sufficient

  - Option 2: (vivo, Nokia)

    - Wait further RAN1/RAN2 progress.

  - Option 3: (Samsung)

    - No need to define new TCI state switching delay requirements. This can be revisited if the RS in the RS resource set can be updated by MAC-CE in further RAN1 conclusion.

  - Option 3a: (Ericsson)

    - Existing unified TCI state switching delay requirements are applicable for event triggered measurement reporting based TCI state switching.

- Recommended WF

  - To be discussed

Issue 1-10: Impact of beam on known/unknown conditions for TCI states for TCI states switching based on UEIBM?

- Proposals

  - Option 1: (Nokia)

    - Reuse known/unknown conditions for Rel-17 unified TCI states when defining requirements for TCI-switching based on UEIBM.

  - Option 2: (Apple)

    - No impact of UE triggered beam report to known/unknown conditions for TCI state.

    - The known/unknown condition would be determined by the same criteria as legacy.

- Recommended WF

  - To be discussed

Issue 1-11: Whether to enhance beam sweeping in event triggered L1 beam reporting?

- Proposals

  - Option 1: (Qualcomm)

    - Discussions on beam sweeping enhancements should be postponed till the baseline requirement for delay and measurement period have taken place

  - Option 2: (Apple, Samsung, Ericsson)

    - Reuse the current assumption of beam sweeping for requirements for UE/ event triggered beam reporting. No enhancement.

  - Option 3: (Nokia)

    - Specify RAN4 requirements for a UE supporting Rel-18 multi-Rx fast beam sweeping and configured with UEIBM reporting.

- Recommended WF

  - To be discussed

Issue 1-12: Whether to define new accuracy requirements by Enhancement for UE-initiated/event-driven beam management?

- Proposals

  - Option 1: (Xiaomi, Qualcomm, Apple, Samsung, Nokia, Ericsson, MediaTek)

    - The existing accuracy requirements for L1 measurements can be applied. No new accuracy requirements.

  - Option 2: (CMCC)

    - For SSB based beam report, Rel-18 LTM accuracy requirements can be used as baseline.

    - For CSI-RS based beam reprt, FFS whether to use legacy accuracy requirements which only cover serving cell, or to define new accuracy requirements.

  - Option 3: (ZTE)

    - Discuss the accuracy for the two trigger-event solutions respectively.

- For the basic trigger-event detection, tight the existing L1-RSRP accuracy requirement;

- For the optional trigger-event detection, reusethe existing L1-RSRP accuracy requirement.

- Recommended WF

  - TBA

Issue 1-13: Capabilities for Support of Event Triggering and Reporting Criteria?

- Proposals

  - Option 1: (MediaTek)

    - Define a capability for supported number of UE-initiated/event-driven beam reporting for intra-frequency.

- Recommended WF

  - To be discussed

Issue 1-14: Compatibility of capabilities with LTM

- Proposals

  - Option 1: (MediaTek)

    - Events configured for mobility and MIMO should be considered as different events even when they are configured for the same beams and the same cell with the same threshold.

    - Define the single capability for total number of L1 events shared by mobility L1 events and MIMO L1 events on intra-frequency.

- Recommended WF

  - To be discussed