3GPP TSG-RAN WG4 Meeting # 112bis						       			       			R4-2415663

Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:			6.22.3

Source:	Moderator (Huawei)

Title:	Topic summary for [112bis][216] Netw_Energy_NR_enh_Part2

Document for:	Information

# Introduction

This topic summary includes RRM core requirements for Rel-19 network energy saving (6.22.2.2).

Recommended Topics to be treated online (in order of decreasing priority):

Issue 1-1-1: Scenario clarification based on RAN1 progress

Issue 1-2-1: Requirements need updating

Issue 1-2-2: Whether and how to define requirements for “transition period”

Issue 1-2-3: L1 Requirements after “transition period”

Issue 1-2-5: L3 requirements for “transition period”

Issue 1-2-4: SCell activation requirement for SSB adaptation

Issue 1-2-6: L3 requirements after “transition period”

# Topic #1: Adaptation of common signal/channel transmission

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2416122 | OPPO | Observation 1: For the impact of SSB adaptation on L1 measurement on serving cells, SSB periodicity can be reused. Observation 2: For the impact of SSB adaptation on L3 measurement on neighbor cells, SMTC periodicity can be used as baseline. Observation 3: For the impact of SSB adaptation on L1 measurement on neighbor cells, current time windows W can be reused, which already considers the periodicity of the target SSB. Proposal 1: Don’t pursue SMTC-based L3 measurement for SCell activation, and don’t support L3 measurement after the SCell has been activated without relying on SMTC. Proposal 2: RAN4 not to introduce new SMTC due to SSB adaptation. Proposal 3: RAN4 to specify requirements for network-controlled adaptation mechanism. |
| R4-2415006 | CMCC | Proposal 1: RAN4 also starts the SSB adaptation discussion for SCell NCD-SSB, including related L1 and L3 measurement. Proposal 2: For the measurement requirement during SSB periodicity transitions period, the legacy transition requirement can be applied. |
| R4-2415061 | Xiaomi | Observation 1: For the SSB adaptation, the more than one SSB burst periodicity will be used. But how to indicate the one NW used for UE measurement are still FFS. Observation 2: In current spec, the total L3 measurement delay requirements depend on the SMTC periodicity which is configured by NW RRC message. Observation 3: UE’s measurement requirements can be little impacted because SMTC period in TPSS/SSS_sync and T SSB_measurement_period can be explicitly indicated by NW. Proposal 1: RAN4 to wait RAN1’s progress on SSB adaptation to define the necessary requirements or update the current spec. Proposal 2: The current SSB based measurement requirements in 9.2, 9.3 and 9.4 [2,TS38.133] shall update the applicability for the adaptation SSB for PCell. Observation 4: The adaptation SSB configuration procedure happened within a successful measurement report period may result in the measurement requirements updates. Proposal 3: When defining the requirement with adaptation SSB, RAN4 needs only consider the case in which there is no any SSB adaptation period changed within one reporting interval. |
| R4-2415184 | CATT | Proposal 1: RAN4 to discuss SSB periodicity adaptation in time-domain as starting point. Proposal 2: For SSB periodicity adaptation in time-domain, the existing L1/L3 measurement requirements can still apply. Proposal 3: For SSB periodicity adaptation in time-domain, the existing minimum requirement at transitions for L1 measurement can still apply. Proposal 4: Deprioritize the minimum requirement at transitions for L3 measurement until RAN1 has further progress on SMTC configuration. Proposal 5: The defined requirements due to SSB periodicity adaptation apply to SCell. And RAN4 wait for RAN1 progress on whether the requirements apply to PCell. |
| R4-2415304 | Qualcomm Technologies Ireland | Observation 1: RAN1 has not yet agreed whether SSB adaption in time domain is applicable for a PCell. Proposal 1: RAN4 should postpone all work on SSB adaptation related to the PCell till RAN1 has decided that SSB adaption is applicable to the PCell. Observation 2: RAN1 confirmed that SSB adaptation is supported for the SCell on the sync raster (B2’) and outside the sync raster (B3’) if the SSB is an NCD-SSB Observation 3: SSB adaption for SCell if the SSB is an CD-SSB is not yet agreed in RAN1. Proposal 2: RAN4 should start investigating requirements for SSB adaptation for UEs in connected mode that do not imply that the SSB is a CD-SSB. Observation 4: RAN4 already defines minimum requirements for the measurement procedures at transitions in the configurations. Change of the periodicity of SSB is a similar transition. Proposal 3: RAN4 should investigate what requirements are impacted by those periodicity transitions by SSB adaptation in time domain. Proposal 4: Only if clear benefits of measurement combining can be shown, RAN4 should further analyze how such requirements for measurement combining could be defined. Observation 5: A measurement period with transition in periodicity of the SSB implies that measurements with different periodicities need to be combined Observation 6: Instead of requesting the UE to combine measurements with different periodicities, the measurement cycle could also be defined based on maximum of the two periodicities P = max{P1, P2}. Proposal 5: RAN4 should further discuss which of the two options, i.e., combing of measurements or applying the maximum of the two periodicities, is more feasible and trade-off benefits versus impacts. Observation 7: No clear agreement has yet been made in RAN1 how Cell DTX relates to adaptation of common signals / channels. Proposal 6: RAN4 should delay any discussion related to Cell DTX to a later point in time. |
| R4-2415490 | Apple | Proposal 1: RAN4 doesn’t need to discuss SSB adaptation in IDLE mode at current stage. Proposal 2: minimum requirement upon SSB periodicity adaptation is needed: - When SSB periodicity changes upon SSB periodicity adaptation, the cell identification, measurement period and L1 requirements apply based on the longer delay before or after the adaptation. |
| R4-2415538 | vivo | Proposal 1 RAN4 may start to discuss RRM requirement impacts based on adaptation of SSB burst periodicity using one or more SSB burst periodicity value(s). RAN4 may further discuss RRM requirement impacts after adaptation based on two SSB configurations is supported in RAN1. Proposal 2 RAN4 prioritize the following RRM requirement impacts due to SSB adaptation on SCell. - L3 measurements, including cell identification and measurements - L1-RSRP/L1-SINR measurements - BFD Proposal 3 For SSB periodicity adaptation, RAN4 further check whether existing transition period requirements for L3 measurement can be re-used for BFD and L1/L3 measurements. |
| R4-2415700 | ZTE Corporation, Sanechips | Proposal 1: In time domain, the adaptation of NCD-SSB not on sync raster for PCell and SCell could be supported. Proposal 2: The impact caused by the SSB adaptation in time domain is to identify the relevant L1/L3 measurement requirements based on the adapted SSB periodicity. Larger periodicity could be allowed to achieve NW power saving gain. |
| R4-2415737 | China Telecom | Proposal 1: It’s proposed to apply legacy transition period requirements for L1 measurement/ L3 measurement to SSB periodicity adaptation. Proposal 2: For other issues, it’s proposed to start the discussion after RAN4 has sufficient information from RAN1. |
| R4-2415833 | Huawei, HiSilicon | Observation 1: There is no significant RRM requirements impacts for PRACH/Paging adaptation. Observation 2: Based on RAN1 agreements so far, it could be observed that: - At least adaptation of SSB periodicity is supported. - For Rel-19 NES-capable UE’s PCell (Connected mode), adaptation of CD-SSB on sync raster is not supported. - For Rel-19 NES-capable UE’s SCell, followings are supported: - NCD SSB on sync raster - NCD SSB off sync raster Proposal 1: After SSB adaptation, for SSB based measurement based on SSB periodicity, UE shall perform measurement based on “new SSB configurations” (e.g. new SSB periodicity) Proposal 2: After SSB adaptation, for SSB based measurement based on SMTC: - For L3 measurement in clause 9, UE is still expected to perform measurement based on configured SMTC. - For L1 and L3 measurement during SCell activation, UE shall perform SCell activation based on “new SSB configurations” (e.g. new SSB periodicity). Proposal 3: RAN4 to discuss whether and how to define requirements when SSB adaption happens during RRM procedures. |
| R4-2415955 | MediaTek inc. | Proposal 1: When the UE transitions when SSB periodicity changes, the L1/L3 measurement requirements apply based on the longer delay before or after the transition. |
| R4-2416024 | Ericsson | Proposal 1: RAN4 to define the NCD-SSB adaptation requirement for SCell first and wait RAN1’s conclusion on whether SSB adaptation can be applied to PCell. Proposal 2: RAN4 to focus on NW controlled SSB adaptation firstly. Proposal 3: In L1 measurement, UE shall combine the SSB measurements before and after SSB adaptation with different periodicities. Subsequent to the transition measurement period, the cell identification and measurement period requirements will follow the SSB periodicity after adaptation. Proposal 4: In L3 measurment, RAN4 to wait other WGs’ input on whether new SMTC after SSB adaptation will be applied. |
| R4-2416026 | LG Electronics Inc. | Observation 1: On-demand SSB and SSB adaptation has similar operation except for applied cell Proposal 1: For SSB adaptation requirements, we propose, - Consider the requirements of on-demand SSB as baseline at least for SCell - Specify the requirements of SSB adpatiation only for PCell |
| R4-2416116 | Samsung | Observation 1: Based on RAN1 agreements on SSB adaptation. - PCell and/or SCell (s). Rel-19 NES-capable UE’s PCell CD-SSB on sync raster is not supported, FFS: NCD-SSB or SSB not on sync raster; Rel-19 NES-capable UE’s SCell NCD-SSBs are supported, FFS: CD-SSB with > 20 ms periodicity - FFS: Whether to support Rel-19 NES-capable UE in idle/inactive mode - FFS: Additional SSB resources configuration/SSB transmission pattern for different adaptation mechanism Observation 2: Based on RAN1 agreements on PRACH adaptation. - DCI-based adaptation for additional PRACH resources. FFS: The details - Extend the RAN1#117 agreement on SSB-RO mapping rule to no time-domain overlap and time-domain overlap but no overlap in frequency domain cases - FFS: Additional PRACH resources configurations Observation 3: The following issues were discussed but without any clear conclusions in RAN1 - How NW to indicate SSB adaptation, by RRC/MAC-CE/DCI - How to adapt SSB transmission to reduce the SCell activation delay - Unified MAC-CE framework for on-demand SSB and adaptation SSB Observation 4：The whole procedure of the two options are the same, that is, the NES-capable UE gets the additional SSB configuration from RRC, and then gets the adaptation indication from L1/L2 signalling Observation 5: Option 2 can enable more scheduling flexibility, better UE experience and more NW energy saving gain, and it is one of the solutions to realize option 1 for some functionalities Observation 6: Based on RAN1 agreements, SSB adaptation triggered by NW is taken as a preferred solution no matter for idle mode UE or connected mode UE - For connected mode UE, the Alt.s can be RRC-reconfiguration, MAC-CE and DCI - For idle mode UE, the Alt.s can be SIB and DCI Observation 7: Based on RAN1 agreements, there are two applicable cases for intra-cell deployment. 1) Serving cell with only Rel-19 NES-capable UE; 2) Serving cell with both legacy and Rel-19 NES-capable UE Proposal 1: Rel-19 NES-capable UE’s SCell (s) are supported for adaptation of NCD-SSB in time-domain Proposal 2: In connected mode, the impact on the following RRM requirements can be studied on adaptation of SSB in time domain with high priority under the applicable scenario - L1 measurements including L1-RSRP/BFD/CBD/L1-SINR - L3 measurements including intra-frequency and inter-frequency - Measurement requirement of SCC with active SCell and the measurement requirement of SCC with deactivated SCell - FFS: Known and unknown SCell - SCell activation delay Proposal 3: RAN4 to wait for RAN1 progress whether: PCell are supported for adaptation mechanisms of SSB in time-domain Proposal 4: For Rel-19 NES-capable UE’s SCell - RAN4 to focus on MAC CE based SCell activation case as starting point for SSB adaptation SCell operation - RAN4 to study the SCell activation and deactivation delay impact. - Combine the RAN1 conclusions of on-demand SSB and SSB adaptation to define the corresponding SCell requirements Proposal 5: Suggest to wait for RAN1 progresseses and agreements on the applicable scenarios and functionalities of option 1 and option 2 for SSB adaptation Proposal 6: - Suggest RAN4 to consider legacy SSB burst periodicity as starting point to define the requirements - For discussion on RRM requirements impacts based on longer SSB adaptation periodicity can wait for RAN1 progress on whether longer SSB burst adaptation periodicity can be supported Proposal 7: - RAN4 to discuss the transition period requirements by SSB periodicity adaptation - FFS: Requirements on interrupts at state transitions Proposal 8: RAN4 to focus on NW controlled SSB adaptation as starting point Proposal 9: For adaptation of common signal/channel transmissions, RAN4 to discuss: - Which deployment assumption should be considered to define the new requirements (if any) - Intra-cell and/or inter-cell deployment - For the deployment assumption, which applicable case should be considered to define the new requirements (if any) - Serving cell with only Rel-19 NES-capable UE and/or serving cell with both legacy and Rel-19 NES-capable UE Proposal 10: Regarding UE behavior under NES scenario. - If NES only cell case is considered. The NES-capable UE shall measure the NES-based SSB resources always - If NES cell case is considered. The candidate UE behaviours can be - NES-capable UEs to measure these additional NES-based SSB resources with the highest priority - Both SSB resources (legacy SSBs and adapted SSBs) the NES-capble UEs can to measure. New measurement rules and the measurement results combination rules shall be discussed in RAN4 - Based on UE implementation |
| R4-2416438 | Nokia, Nokia Shanghai Bell | Observation #1: Options 1 and 2 of the RAN1#117 agreement are discussing the alternating between different SSB periodicities and the operation of up to two SSB configurations, respectively. Observation #2: For Option 1, only one SSB pattern is active at a time, and this may have different SSB burst periodicities. Observation #3: Option 4 of the RAN1#116 agreement is intended for Case-2 with always-ON periodic SSB transmission in SCell, where the NW triggers on-demand denser SSB transmission with shorter SSB periodicity from time instance A, i.e. when NW sends SCell activation command, and later NW starts sending SSB with longer periodicity after time instance B, i.e. when NW has received UE measurement report. Proposal 1: RAN4 to start working on the impact from SSB periodicity adaptation in time domain to RRM requirements. Proposal 2: RAN4 to study the RRM impact for the scenario of SSB adaptation in the PCell for NCD-SSB, regardless of whether on sync-raster or not. Proposal 3: RAN4 to study the RRM impact for the scenario of SSB adaptation in the SCell for NCD-SSB, regardless of whether on sync-raster or not. Proposal 4: RAN4 to start investigation of RRM impacts for SSB adaptation in time domain for a single active SSB pattern, due to usage of different SSB burst periodicities. Further scenarios can be considered based on further RAN1 agreements. Proposal 5: RAN4 to maintain the specified range for SSB burst periodicity i.e. 5ms to 160ms. Proposal 6: RAN4 to consider the impact of processing time for decoding the updated SSB burst periodicity. Proposal 7: RAN4 to consider the L3 measurement impact from SSB periodicity adaptation to NR intra- / inter-frequency cell detection delay and to NR intra- / inter-frequency measurement period, but not to inter-RAT NR cell detection delay or inter-RAT NR cell measurement delay when UE camps on E-UTRAN. Proposal 8: RAN4 to consider the L1 measurement impact from SSB periodicity adaptation both to L1-RSRP measurement delay for RLM, BFD and CBD, to TCI state switching delay and to L1-RSRP measurement restrictions as well as to L1-SINR for measurement restrictions. Proposal 9: RAN4 to investigate whether a transition period with no or relaxed RRM requirements can serve as starting point when changing SSB burst periodicity for defining the RRM impact. |

## Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 1-1 SSB adapation – Scenario

Issue 1-1-1: Scenario clarification based on RAN1 progress

Background: RAN1 agreements

Agreement Adaptation mechanism(s) of SSB in time-domain is supported at least for one of the following scenario(s): - For cell with both legacy UEs and Rel-19 NES-capable UEs - Rel-19 NES-capable UE’s PCell (Connected mode) - Study from the following options: - Option A1: adaptation for CD-SSB - Option A2: adaptation for SSB that is not CD-SSB - Option A3: adaptation for SSB not on sync raster - Rel-19 NES-capable UE’s SCell - Study from the following options: - Option B1: adaptation for CD-SSB - Option B2: adaptation for SSB that is not CD-SSB - Option B3: adaptation for SSB not on sync raster - FFS: Rel-19 NES-capable UE in idle/inactive mode - Note: Impact to idle/inactive UEs shall be minimized Agreement For indication of adaptation of SSB in time-domain, - Support at least SSB adaptation provided by gNB without UE trigger Agreement For adaptation of SSB in time-domain, Option 1 is supported - Option 1: Adaptation of SSB burst periodicity using one or more SSB burst periodicity value(s) - Note: Using Option 2 to realize Option 1 is not precluded - Option 2: Adaptation based on two SSB configurations [where up to two configurations can be active] - FFS: details of the differences between the two SSB configurations, e.g. two different periodicities - FFS: Details including applicable scenarios - FFS: Support of Cell DTX for connected mode UEs for SSB Agreement For adaptation mechanism(s) of SSB in time-domain, - For Rel-19 NES-capable UE’s PCell (Connected mode), adaptation of CD-SSB on sync raster is not supported - FFS: Adaptation for SSB that is not CD-SSB is supported (A2) - FFS: Adaptation for SSB not on sync raster is supported (A3) - For Rel-19 NES-capable UE’s SCell - Adaptation of SSB configured for the SCell is supported for the following cases - FFS: Adaptation for CD-SSB (B1) including UE impact compared to legacy operation where the SSB is configured with periodicity>20msec for SCell - Adaptation for SSB that is not CD-SSB on sync raster (B2’) - Adaptation for SSB that is not CD-SSB not on sync raster (B3’)

PCell & SCell

- Proposals

  - Option 1: RAN4 to discuss SSB adaptation for SCell as starting point, and wait for RAN1 progress on PCell. (CMCC, CATT, QC, Vivo, Ericsson, Samsung)

  - Option 2a: Specify requirements of SSB adaptation for PCell. (LGE)

  - Option 2b: RAN4 to study the RRM impact for the scenario of SSB adaptation in the PCell for NCD-SSB, regardless of whether on sync-raster or not. (Nokia)

  - Option 2c: In time domain, the adaptation of NCD-SSB not on sync raster for PCell and SCell could be supported. (ZTE)

CD & NCD SSB

- Proposals

  - Option 1: RAN4 to discuss SSB adaptation for NCD-SSB as starting point (CMCC, QC, Ericsson, Samsung, Nokia, ZTE)

  - Option 2: Consider the requirements of on-demand SSB as baseline at least for SCell (LGE)

IDLE & CONNECTED

- Proposals

  - Option 1: RAN4 doesn’t need to discuss SSB adaptation in IDLE mode at current stage (Apple)

Adaptation mechanism

- Proposals

  - Option 1: RAN4 to discuss SSB periodicity adaptation in time-domain as starting point. (CATT, Vivo, QC, Apple, CTC, Huawei, MTK, Nokia)

    - Option 1a: RAN4 to maintain the specified range for SSB burst periodicity i.e. 5ms to 160ms (Nokia, Samsung)

    - Option 1b: For discussion on RRM requirements impacts based on longer SSB adaptation periodicity can wait for RAN1 progress on whether longer SSB burst adaptation periodicity can be supported. (Samsung)

    - Option 1c: Larger periodicity could be allowed to achieve NW power saving gain. (ZTE)

  - Option 2: RAN4 to focus on NW controlled SSB adaptation firstly. (Ericsson, Samsung, OPPO)

  - Option 3: SSB burst periodicity using one or more SSB burst periodicity value. FFS adaptation based on two SSB configurations. (Vivo, Nokia)

  - Option 4: Suggest to wait for RAN1 progresses and agreements on the applicable scenarios and functionalities of option 1 and option 2 for SSB adaptation (Samsung)

- Moderator:

Based on the proposals from contributions, most of them are exactly the same as RAN1 agreements so far, in terms PCell/SCell, CD/NCD, etc. From moderator’s understanding, there is no intention to revert/update RAN1 agreements in RAN4 discussion. Thus, following WF is recommended.

- Recommended WF:

  - Per RAN1 agreements, RAN4 to take following as starting point for SSB adaption discussion:

    - CONNECTED mode

    - SSB adaptation for SCell

    - SSB adaptation for NCD-SSB

    - SSB periodicity adaptation in time-domain

Note: Above could be further updated based on further RAN1 progress (e.g. PCell, other adaptation mechanism)

Issue 1-1-2: Scenario clarification - others

- Proposals

  - Option 1: For adaptation of common signal/channel transmissions, RAN4 to discuss: (Samsung)

    - Which deployment assumption should be considered to define the new requirements (if any)

    - Intra-cell and/or inter-cell deployment

  - Option 2: For the deployment assumption, which applicable case should be considered to define the new requirements (if any): (Samsung)

    - Serving cell with only Rel-19 NES-capable UE and/or serving cell with both legacy and Rel-19 NES-capable UE

  - Option 4: RAN4 should delay any discussion related to Cell DTX to a later point in time. (QC)

- Recommended WF: Discuss above proposals

### Sub-topic 1-2 SSB adapation – Requirements impacts

Issue 1-2-1: Requirements need updating

- Proposals

  - Option 1: L3 measurement:

    - Option 1a: The current SSB based measurement requirements in 9.2, 9.3 and 9.4 [2, TS38.133] shall update the applicability for the adaptation SSB for PCell. (Xiaomi)

    - Option 1b: L3 measurement, including Cell identification and measurement. (Vivo, Huawei)

    - Option 1c: L3 intra-frequency and inter-frequency. (Samsung)

    - Option 1d: L3 measurement during SCell activation. (Huawei, Samsung)

    - Option 1e: RAN4 to consider the L3 measurement impact from SSB periodicity adaptation to NR intra- / inter-frequency cell detection delay and to NR intra- / inter-frequency measurement period, but not to inter-RAT NR cell detection delay or inter-RAT NR cell measurement delay when UE camps on E-UTRAN. (Nokia)

  - Option 2: L1 measurement:

    - Option 1a: L1-RSRP (Nokia, Vivo, Samsung)

    - Option 1b: L1-SINR (Nokia, Vivo, Samsung)

    - Option 1c: RLM (Nokia)

    - Option 1d: BFD (Nokia, Vivo, Samsung)

    - Option 1e: CBD (Nokia, Vivo, Samsung)

    - Option 1f: L1 during SCell activation (Huawei, Samsung)

    - Option 1g: L1 during TCI state switching (Nokia)

- Moderator:

From moderator’s understanding, all RRM procedures based on SSB could be impacted by SSB adaption no matter whether it is L1 measurement or L3 measurement. Thus, following WF is recommended:

- Recommended WF:

  - RAN4 to consider RRM requirements impacts by SSB adaption at least for:

    - L1-RSRP

    - L1-SINR

    - BFD

    - CBD

    - SCell activation

    - TCI state switching

    - NR intra-frequency measurement

    - NR inter-frequency measurement

Note: Above list could be further updated based on RAN4 discussion.

Issue 1-2-2: Whether and how to define requirements for “transition period”

- Proposals

  - Option 1: For the measurement requirement during SSB periodicity transitions period, the legacy transition requirement can be applied. (CTC, CMCC, CATT)

    - Option 1a: For SSB periodicity adaptation in time-domain, the existing minimum requirement at transitions for L1 measurement can still apply. (CATT)

  - Option 2: For SSB periodicity adaptation, RAN4 further check whether existing transition period requirements for L3 measurement can be re-used for BFD and L1/L3 measurements. (Vivo)

  - Option 3: RAN4 to discuss whether and how to define requirements when SSB adaption happens during RRM procedures (Huawei, Samsung)

    - Option 3a: FFS Requirements on interrupts at state transitions. (Samsung)

  - Option 4: When SSB periodicity changes upon SSB periodicity adaptation, the cell identification, measurement period and L1 requirements apply based on the longer delay before or after the adaptation. (Apple, MTK)

    - Option 4a: RAN4 to investigate whether a transition period with no or relaxed RRM requirements can serve as starting point when changing SSB burst periodicity for defining the RRM impact. (Nokia)

  - Option 5: In L1 measurement, UE shall combine the SSB measurements before and after SSB adaptation with different periodicities. (Ericsson)

  - Option 6: (QC)

    - RAN4 should investigate what requirements are impacted by those periodicity transitions by SSB adaptation in time domain

    - Only if clear benefits of measurement combining can be shown, RAN4 should further analyse how such requirements for measurement combining could be defined.

    - RAN4 should further discuss which of the two options, i.e., combing of measurements or applying the maximum of the two periodicities, is more feasible and trade-off benefits versus impacts.

  - Option 7: (Samsung) UE behavior under NES scenario based on two SSB configurations

    - If NES only cell case is considered. The NES-capable UE shall measure the NES-based SSB resources always

    - If NES cell case is considered. The candidate UE behaviours can be

      - NES-capable UEs to measure these additional NES-based SSB resources with the highest priority

      - Both SSB resources (legacy SSBs and adapted SSBs) the NES-capble UEs can to measure. New measurement rules and the measurement results combination rules shall be discussed in RAN4

      - Based on UE implementation

- Moderator:

Based on the contributions about “transition period” of SSB adaptation, some proposals are for both L1 and L3 measurement and some proposals are only for L1 measurement, while L3 measurement may related to SMTC. Thus, to facilitate the discussion, the issue is only to discuss the requirements impacts for “transition period” of SSB adaptation for L1 measurement, and L3 measurement will be discussed in separate issues.

- Recommended WF:

  - For L1 measurement requirements, RAN4 to define RRM requirements for “transition period” of SSB adaptation, considering following approach:

    - Option 1: Reuse exiting transition requirements

    - Option 2: longer delay before or after SSB adaption

    - Option 3: Combine SSB measurement before and after SSB adaptation

    - Other options are not precluded.

Issue 1-2-3: L1 Requirements after “transition period”

- Proposals

  - Option 1: After SSB adaptation, for SSB based measurement based on SSB periodicity, UE shall perform measurement based on “new SSB configurations” (e.g. new SSB periodicity). (Huawei, Ericsson)

    - Option 1a: In L1 measurement, UE shall combine the SSB measurements before and after SSB adaptation with different periodicities. Subsequent to the transition measurement period, the cell identification and measurement period requirements will follow the SSB periodicity after adaptation. (Ericsson)

    - Option 1b: The impact caused by the SSB adaptation in time domain is to identify the relevant L1/L3 measurement requirements based on the adapted SSB periodicity. (ZTE)

    - Option 1c: For SSB periodicity adaptation in time-domain, the existing L1/L3 measurement requirements can still apply. (CATT)

- Recommended WF:

  - Discuss above proposal

Issue 1-2-4: SCell activation requirement for SSB adaptation

- Proposals

  - Option 1: After SSB adaptation, For L1 and L3 measurement during SCell activation, UE shall perform SCell activation based on “new SSB configurations” (e.g. new SSB periodicity). (Huawei, OPPO)

    - Option 1b: Don’t pursue SMTC-based L3 measurement for SCell activation. (OPPO)

  - Option 2: RAN4 to study the SCell activation and deactivation delay impact. (Samsung)

    - RAN4 to focus on MAC CE based SCell activation case as starting point for SSB adaptation SCell operation

    - Combine the RAN1 conclusions of on-demand SSB and SSB adaptation to define the corresponding SCell requirements

- Recommended WF:

  - Discuss above proposal

Issue 1-2-5: L3 requirements for “transition period”

- Proposals

  - Option 1: For the measurement requirement during SSB periodicity transitions period, the legacy transition requirement can be applied. (CMCC, CTC)

  - Option 2: For SSB periodicity adaptation, RAN4 further check whether existing transition period requirements for L3 measurement can be re-used for BFD and L1/L3 measurements. (Vivo)

    - Option 2a: RAN4 to investigate whether a transition period with no or relaxed RRM requirements can serve as starting point when changing SSB burst periodicity for defining the RRM impact. (Nokia)

  - Option 3: When the UE transitions when SSB periodicity changes, the L1/L3 measurement requirements apply based on the longer delay before or after the transition. (MTK)

  - Option 4: In L3 measurement, RAN4 to wait other WGs’ input on whether new SMTC after SSB adaptation will be applied. (Ericsson)

  - Option 5: (OPPO)

    - don’t support L3 measurement after the SCell has been activated without relying on SMTC

    - RAN4 not to introduce new SMTC due to SSB adaptation

  - Option 5a: For L3 measurement in clause 9, UE is still expected to perform measurement based on configured SMTC. (Huawei)

  - Option 6: Deprioritize the minimum requirement at transitions for L3 measurement until RAN1 has further progress on SMTC configuration. (CATT)

  - Option 7: When defining the requirement with adaptation SSB, RAN4 needs only consider the case in which there is no any SSB adaptation period changed within one reporting interval. (Xiaomi)

  - Option 8: The impact caused by the SSB adaptation in time domain is to identify the relevant L1/L3 measurement requirements based on the adapted SSB periodicity. (ZTE)

- Moderator:

  - Based on the proposals above, the first question needs to be discussed is whether L3 measurement is still based on configured SMTC with SSB adaptation.

- Recommended WF:

  - Discuss following question:

    - Q1: Whether L3 measurement is still based on configured SMTC with SSB adaptation.

      - Q1-1: If the answer is “yes”, whether RAN4 needs to discuss new SMTC for SSB adaptation?

      - Q1-2: If the answer is “NO”, what is the detailed UE measurement behaviour?

Issue 1-2-6: L3 requirements after “transition period”

- Proposals

  - Option 1: For SSB periodicity adaptation in time-domain, the existing L1/L3 measurement requirements can still apply. (CATT)

    - Option 1a: (Huawei)

      - After SSB adaptation, for SSB based measurement based on SMTC:

      - For L3 measurement in clause 9, UE is still expected to perform measurement based on configured SMTC.

- Recommended WF:

  - Depends on the conclusion of issue 1-2-5 about UE measurement behaviour of L3 measurement.

Issue 1-2-7: Other requirements impacts by SSB adaptation

- Proposals

  - Option 1: RAN4 to consider the impact of processing time for decoding the updated SSB burst periodicity. (Nokia)

- Recommended WF:

  - Discuss above proposal