3GPP TSG RAN WG4 Meeting#112-bis                                                       R4-2415666

Hefei, CN, Oct 14 - Oct 18, 2024

Agenda item:			6.24.3

Source:	China Telecom

Title:	Topic summary for [112bis][219] NR_Mob_Ph4_Part2

Document for:	Information

Introduction

This summary includes the proposals from companies on RRM requirements for conditional LTM. This is the first RAN4 meeting of this part. The relevant objectives in the latest WID are duplicated here for information:

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint - Specify RRM requirements related to the above objectives as necessary [RAN4]

Recommended Topics to be treated online (in order of decreasing priority):

Issue 1-1-1: RAN4 scope of Conditional LTM

Issue 1-1-2: The applicable scenarios for conditional LTM

Issue 1-2-2: The starting point of conditional LTM delay

Issue 1-2-3: The ending point of conditional LTM delay

Issue 1-1-3: Measurement requirements on conditional LTM

Issue 1-2-4: The components of conditional LTM delay

Moderator’s recommendation is also provided under issue.

Topic #1: RRM discussion

Companies’ contributions summary

| TDoc | Source | Proposals |
| --- | --- | --- |
| R4-2414945 | China Telecom | Proposal 1: Conditional intra-CU LTM delay, TCI state activation for LTM candidate cell and the synchronization with the target cell should be discussed in the scope of conditional intra-CU LTM. Proposal 2: Conditional intra-CU LTM delay is the delay from the end of the last TTI containing the RRC command for conditional intra-CU LTM until the time the UE transmits the first UL message on the target cell. Proposal 3: DIntra-CLTM = TRRC + TEvent_DU + TTCI-state activate + Tmeasure + TIntra-CLTM_execution + Tinterrupt. Where: - TRRC is the RRC procedure delay. - TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional intra-CU LTM command until a condition exists at the measurement reference point which will trigger the conditional intra-CU LTM. - TTCI-state activate is the time for active the TCI state of candidate cell(s). - Tmeasure is defined from the end of TTCI-state activate until UE executes LTM to a target cell and interruption time starts. - TIntra-CLTM_execution is the UE execution preparation time for conditional intra-CU LTM and starts after UE realizes the condition of LTM is met and identity of the target cell is determined. - Tinterrupt is the time between when the UE starts to execute the conditional intra-CU LTM to the target cell and the time the UE starts to transmit the first UL message on the target cell. Further, the components of Tinterrupt could be defined as Tinterrupt = TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU - TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change. - Tfirst-RS is the time for fine time tracking and acquiring full timing information of the target cell. - TRS-proc is the time for SSB processing. - TLTM-IU is the interruption uncertainty during LTM cell switch. Proposal 4：Revisit conditional intra-CU LTM delay when other WGs achieve further conclusions. Proposal 5: - For early TCI state activate and DL-sync, we think R18 LTM early TCI state activate and DL-sync can be as baseline. - For UL-sync, we can consider some situations as follows, - Do not need to acquire the TA, i.e., the NTA of candidate cell(s) is/are same as serving cell or NTA = 0. - RAR-based TA acquisition. - UE based TA measurement. Proposal 6: Cover both RACH-based and RACH-less conditional intra-CU LTM. Proposal 7: The known cell condition for conditional intra-CU LTM can be revised as, during the last 5 seconds before the reception of the RRC command that contain conditional intra-CU LTM configuration……. Proposal 8: All the cell(s) configured as candidate cell(s) for conditional intra-CU LTM is/are known. |
| R4-2415067 | Xiaomi | Proposal 1: RAN4 to specify RRM requirements for the newly introduced conditional intra-CU LTM, while detailed discussions should be deferred until further progress is made in other working groups. |
| R4-2415147 | vivo | Proposal 1: RAN4 to define delay requirements respectively for conditional LTM and subsequent conditional LTM and suspend the detailed discussion until enough RAN2 progress on the supportive scenario and procedure is made |
| R4-2415156 | CATT | Proposal 1: For the scenario of Rel-19 conditional LTM, RAN4 will only define the related requirements for intra-CU and prioritize to consider non-DC. Proposal 2: For specification structure, the requirements for conditional LTM can be defined in the clauses of Rel-18 LTM. Proposal 3: At least the following applicable scenarios can be considered for conditional LTM: - PCell switch to a neighboring LTM candidate cell - FR1 cell to FR1 cell - FR1 cell to FR2 cell - FR2 cell to FR2 cell - FR2 cell to FR1 cell - PCell switch to an LTM candidate cell that is a serving SCell in MCG - FR1 cell to FR1 cell - FR2 cell to FR2 cell Observation 1: Compared to L3 HO delay requirements, the CHO delay requirement takes into account the delay uncertainty, the measurements time and the conditional execution preparation time in addition to the RRC procedure delay and the interruption time. Observation 2: The whole procedure will be discussed in parallel in RAN2, and the requirements for conditional LTM are related to the procedure. Proposal 4: The legacy requirments for L3 CHO can be a starting point, and RAN4 need to further study the LTM related features based on the legacy requirments. - Wait for RAN2 more progress. Proposal 5: The delay uncertainty, the measurements time and the conditional execution preparation time also need to be considered for Rel-19 conditional LTM Cell Switch delay. Proposal 6: For the interruption time in Rel-19 conditional LTM Cell Switch delay, the interruption time defined in Rel-18 LTM Cell Switch delay can be a baseline. Proposal 7: RAN4 should further clarify the scenario for conditional LTM including subsequent LTM. |
| R4-2415241 | CMCC | Proposal 1: it is proposed to define delay requirements for conditional LTM. Observation 1: for starting point for subsequent conditional LTM, there are two alternatives: - Alt 1: starting point is the time when UE receives RRC command triggering conditional LTM - Alt 2: starting point is the time when UE transmits RRCReconfigurationcomplete message for the previous cell switch Proposal 2: the starting point of conditional LTM is proposed as the time when UE transmits an RRCReconfigurationComplete message. Proposal 3: The cell switch delay for conditionl LTM is proposed as DLTM_Conditional = TEvent_DU + Tmeasure + TLTM_execution + TLTM-interrupt. - TEvent_DU is the delay uncertainty which is the time from when UE transmits RRCReconfigurationComplete message confirming the LTM configuration or the RRCReconfigurationcomplete message for the previous cell switch until a condition exists at the measurement reference point which will trigger the conditional LTM - Tmeasure is measurement time. - TLTM-interrupt is the interruption time as specified in Rel-18. - TLTM_execution is the UE execution preparation time for conditional LTM, and starts after UE realizes the condition of conditional LTM is met and identity of the target cell is determined |
| R4-2415350 | Qualcomm Incorporated | Proposal 1: For the topic of ‘Support of conditional LTM,’ RAN4 should wait for further progress to be made in other working groups. Proposal 2: If DL TODA-based UE autonomous TA estimation is found beneficial for the conditional LTM cell switch, RAN4 to define the requirements for DL TODA-based UE autonomous TA estimation. |
| R4-2415492 | Apple | Proposal 1: define conditional LTM cell switch delay requirements in this work item. Conditional LTM PSCell switch delay is not considered in current stage. Proposal 2: no new measurement requirements are needed under this agenda. Proposal 3: since L1-RSRP and LTM cell switch was decoupled in R18, L3 measurement shall also be considered in event configuration for conditional LTM. Proposal 4: inform RAN2 if P3 is agreeable. Proposal 5: CHO requirements framework can be used as baseline when discussing conditional LTM cell switch delay requirements. Proposal 6: conditional LTM cell switch delay is defined as: DCLTM = TRRC + TEvent_DU + Tmeasure + TLTM-interrupt + TCLTM_execution Where: TRRC is the RRC procedure delay defined in clause 12 in TS 38.331. TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional LTM cell switch command until a condition exists at the measurement reference point which will trigger the conditional LTM cell switch. Tmeasure is the measurements time. It is defined from the end of TEvent_DU until UE executes a LTM cell switch to a target cell and interruption time starts: - if SSB based L1-RSRP measurement is used in the event, Tmeasure is same as one measurement period of SSB based L1-RSRP measurement as introduced in R18. - if CSI-RS based L1-RSRP measurement is used in the event, Tmeasure is same as one measurement period of CSI-RS based L1-RSRP measurement which will be introduced in R19. - if L3 measurement is used in the event, Tmeasure is same as one measurement period of legacy L3 measurement. TCLTM_execution is the UE execution preparation time for conditional LTM cell switch, and starts after UE realizes the condition of CLTM is met and identity of the target cell is determined. TCLTM_execution can be up to [10ms]. TLTM-interrupt is the interruption time stated in clause 6.3.1.3, same as that defined in R18 LTM cell switch. |
| R4-2415554 | Samsung | Proposal 1: RAN4 should specify new RRM requirements including conditional cell switch for conditional Intra-CU LTM. Proposal 2: RAN4 can use Rel-18 LTM requirements and conditional handover requirements as the references to define the new requirements of conditional cell switching for conditional Intra-CU LTM. The start point may be when UE receives the command to config the conditional LTM. For further discussion on each component of the delay, need more RAN2 progress. |
| R4-2415604 | Apple | RAN4 kindly asks RAN2 to take above information into consideration in future design and introduce corresponding signaling, as necessary. |
| R4-2415842 | Huawei, HiSilicon | Proposal 1: The starting point of conditional LTM delay is the instance when the UE receives a RRC message implying conditional LTM. Proposal 2: The ending point of conditional LTM delay is the time the UE transmits the first UL message on the target cell (including RACH based and RACH less case). Proposal 3: TEvent_DU and Tmeasure for conditional LTM can wait for more progress in RAN2. Proposal 4: Figure out how/whether to support early RACH/TCI activation on candidate cells during conditional LTM and wait for more progress from other working group. Proposal 5: TCHO_execution may need update in conditional LTM. Proposal 6: The term Tinterrupt may induce ambiguity when define conditional LTM delay, as interruptions may also happen in early PRACH procedure (if supported in conditional LTM). Proposal 7: The starting point of the subsequent conditional LTM cell switch delay is the time UE transmits RRC Reconfigurationcomplete message for the previous PCell. Proposal 8: Apart from the starting point, the subsequent conditional LTM cell switch delay can be discussed based on the conclusion of conditional LTM delay. |
| R4-2416015 | Nokia | Observation 1: The conditions for triggering CLTM are up to RAN1/RAN2 to define. RAN4 to take into account such agreements once available. Observation 2: The design of early DL/UL sync in CLTM is up to RAN1/RAN2 discussion and agreements. It is foreseen that at least one of early DL and/or UL sync is to be defined as mandatory for CLTM to ensure optimization of CLTM interruption compared to CHO. Proposal 1: FFS whether updates are needed to the Rel-18 early TCI state activation and PDCCH ordered RACH delay requirements, or whether new requirements due to CLTM specific DL/UL sync procedures are needed. Proposal 2: RAN4 to define cell switch delay requirements for conditional PCell switch in intra-CU scenario. Proposal 3: Cell switch delay requirements shall cover also subsequent conditional PCell switch. FFS whether there is any difference in the delay for initial and subsequent CLTM cell switch. Proposal 4: The starting point for the CLTM delay is: DCLTM = TLTM_event_DU + TLTM_measure + TLTM-processing + TLTM-IU. FFS the exact definition of each component. Proposal 5: The need of TLTM-RRC-processing, Tfirst-RS, TRS-proc and any additional components in the cell switch delay depends on when DL sync and ASN.1 decoding are to happen, which may depend on RAN1/2 CLTM design. Proposal 6: The starting point of CLTM cell switch delay depends on when the condition evaluation begins, which is up to RAN1/RAN2. Proposal 7: The ending point of CLTM cell switch is the first UL transmission (RACH-based or RACH-less) similar to Rel-18 LTM. FFS whether both RACH-less and RACH-based CLTM are supported by RAN1/2. Proposal 8: RAN4 to discuss the starting point of the CLTM interruption, and the components included in TCLTM_interrupt. The goal shall be that CLTM interruption is shorter than CHO interruption. |
| R4-2416166 | Ericsson | • Proposal 1 : Rel-19 CLTM requirements are applicable for NR SA scenarios defined in Rel-18 LTM. • Proposal 2: Rel-18 UE capability for early ASN.1 decoding or fast RRC processing shall be supported for Rel-19 Conditional LTM. • Proposal 3: Rel-19 conditional LTM early DL/UL sync can wait for RAN2 further agreement on the framework. • Proposal 4: RAN4 to wait for RAN2 progress for defining the CLTM measurement requirements. • Proposal 5: RAN4 to wait for RAN2 progress for defining cell switch delay requirements for conditional LTM. |
| R4-2416385 | ZTE Corporation, Sanechips | Proposal 1: RAN4 should define conditional cell switch delay requirement, both intra-frequency and inter-frequency need to be considered: - NR FR1 – NR FR1 - NR FR1 – NR FR2 - NR FR2 – NR FR1 - NR FR2 – NR FR2 Proposal 2: RAN4 should defined RRM requirements for conditional LTM including subsequent LTM. Proposal 3: Wait for RAN2 input to further discuss RRM requirements. |

Open issues summary

Conditional LTM

Issue 1-1-1: RAN4 scope of Conditional LTM

- Candidate solutions:

  - Option 1: RAN4 to specify RRM requirements for the conditional intra-CU LTM. (Xiaomi, Samsung)

  - Option 2: RAN4 to define delay requirements for conditional LTM. (CMCC, Apple, CTC, vivo)

    - Option 2a: Conditional LTM PSCell switch delay is not considered in current stage. (Apple)

  - Option 3: Conditional LTM delay, TCI state activation for LTM candidate cell and the synchronization with the target cell should be discussed in the scope of conditional LTM. (CTC)

  - Option 4: The legacy requirements for L3 CHO can be a starting point, and RAN4 need to further study the LTM related features based on the legacy requirements. (CATT)

  - Option 5: Wait for other WGs input to further discuss detailed RRM requirements. (Xiaomi, vivo, CATT, QC, ZTE, CTC)

- Recommended WF

  - RAN4 to define delay requirements for conditional LTM.

  - Wait for other WGs input to further discuss detailed RRM requirements.

Issue 1-1-2: The applicable scenarios for conditional LTM

- Candidate solutions:

  - Option 1: For the scenario of Rel-19 conditional LTM, RAN4 will only define the related requirements for intra-CU and prioritize to consider non-DC. (CATT).

  - Option 2: (CATT, E///)

Rel-19 CLTM requirements are applicable for NR SA scenarios defined in Rel-18 LTM

- PCell switch to a neighboring LTM candidate cell

- FR1 cell to FR1 cell

- FR1 cell to FR2 cell

- FR2 cell to FR2 cell

- FR2 cell to FR1 cell

- PCell switch to an LTM candidate cell that is a serving SCell in MCG

- FR1 cell to FR1 cell

- FR2 cell to FR2 cell

  - Option 3: (ZTE)

RAN4 should define conditional cell switch delay requirement, both intra-frequency and inter-frequency need to be considered

- NR FR1 – NR FR1

- NR FR1 – NR FR2

- NR FR2 – NR FR1

- NR FR2 – NR FR2

- Recommended WF

  - Discussion is needed.

Issue 1-1-3: Measurement requirements on conditional LTM

- Candidate solutions:

  - Option 1: No new measurement requirements are needed under this agenda. (Apple)

  - Option 2: L3 measurement shall also be considered in event configuration for conditional LTM (Apple)

    - Option 2a: Inform RAN2 if Op2 is agreeable. (Apple)

  - Option 3: RAN4 to wait for RAN2 progress for defining the CLTM measurement requirements. (E///)

- Recommended WF

  - Discussion is needed.

Issue 1-1-4: Subsequent conditional LTM

- Candidate solutions:

  - Option 1: RAN4 to define delay requirements for conditional LTM and subsequent conditional LTM. (vivo, Nokia, ZTE, CATT)

    - Option 1a: FFS whether there is any difference in the delay for initial and subsequent CLTM cell switch. (Nokia)

    - Option 1b: RAN4 should further clarify the scenario for conditional LTM including subsequent LTM. (CATT)

  - Option 2: The starting point of the subsequent conditional LTM cell switch delay is the time UE transmits RRC Reconfigurationcomplete message for the previous PCell. (HW)

  - Option 3: Apart from the starting point, the subsequent conditional LTM cell switch delay can be discussed based on the conclusion of conditional LTM delay.(HW)

- Recommended WF

  - Discussion is needed.

Issue 1-1-5: The specification structure of the requirements for conditional LTM

- Candidate solutions:

  - Option 1: For specification structure, the requirements for conditional LTM can be defined in the clauses of Rel-18 LTM. (CATT)

- Recommended WF

  - Discussion is needed.

Conditional LTM delay

Issue 1-2-1: General aspects on Conditional LTM delay

- Candidate solutions:

  - Option 1: CHO requirements framework can be used as baseline. (Apple, Samsung, CTC, CATT)

    - Option 1a: Rel-18 LTM requirements framework can be used as references. (Samsung, CTC)

  - Option 2: Revisit conditional LTM delay when other WGs achieve further conclusions. (CTC, CATT)

  - Option 3: Wait for RAN2 progress for defining cell switch delay requirements for conditional LTM. (E///)

  - Option 4: RAN4 to define cell switch delay requirements for conditional PCell switch in intra-CU scenario. (Nokia)

  - Option 5: Rel-18 UE capability for early ASN.1 decoding or fast RRC processing shall be supported for Rel-19 Conditional LTM. (E///)

- Recommended WF

  - Discussion is needed.

Issue 1-2-2: The starting point of conditional LTM delay

- Candidate solutions:

  - Option 1: The starting point of conditional LTM delay is the end of the last TTI containing the RRC command for conditional LTM. (CTC, Samsung, HW)

  - Option 2: The starting point of conditional LTM delay is the time when UE transmits an RRCReconfigurationComplete message. (CMCC)

  - Option 3: The starting point of CLTM cell switch delay depends on when the condition evaluation begins, which is up to RAN1/RAN2. (Nokia)

- Recommended WF

  - Discussion is needed.

Issue 1-2-3: The ending point of conditional LTM delay

- Candidate solutions:

  - Option 1: The ending point of conditional LTM delay is the time when the UE transmits the first UL message on the target cell. (CTC, HW, Nokia)

- Recommended WF

  - The ending point of conditional LTM delay is the time when the UE transmits the first UL message on the target cell.

Issue 1-2-4: The components of conditional LTM delay

- Candidate solutions:

  - Option 1: DCLTM = TRRC + TEvent_DU + TTCI-state activate + Tmeasure + TCLTM_execution + Tinterrupt. (CTC)

  - Option 2: DLTM_Conditional = TEvent_DU + Tmeasure + TLTM_execution + TLTM-interrupt. (CMCC)

  - Option 3: DCLTM = TRRC + TEvent_DU + Tmeasure + TLTM-interrupt + TCLTM_execution. (Apple)

  - Option 4: DCLTM = TLTM_event_DU + TLTM_measure + TLTM-processing + TLTM-IU. (Nokia)

    - need of TLTM-RRC-processing, Tfirst-RS, TRS-proc and any additional components in the cell switch delay depends on when DL sync and ASN.1 decoding are to happen, which may depend on RAN1/2 CLTM design. (Nokia)

  - Option 5: At least the delay uncertainty (TEvent_DU), the measurements time (Tmeasure) and the conditional execution preparation time (TCLTM_execution) also need to be considered for Rel-19 conditional LTM Cell Switch delay. (CATT)

- Recommended WF

  - Discussion is needed.

Issue 1-2-5: TRRC

- Candidate solutions:

  - Option 1: TRRC is the RRC procedure delay defined in clause 12 in TS 38.331. (CTC, Apple)

- Recommended WF

  - Discussion is needed.

Issue 1-2-6: TEvent_DU

- Candidate solutions:

  - Option 1: TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional LTM command until a condition exists at the measurement reference point which will trigger the conditional LTM. (CTC, Apple)

  - Option 2: TEvent_DU is the delay uncertainty which is the time from when UE transmits RRCReconfigurationComplete message confirming the LTM configuration or the RRCReconfigurationcomplete message for the previous cell switch until a condition exists at the measurement reference point which will trigger the conditional LTM. (CMCC)

  - Option 3: Wait for more progress in RAN2. (HW)

- Recommended WF

  - Discussion is needed.

Issue 1-2-7: Tmeasure

- Candidate solutions:

  - Option 1: Tmeasure is measurement time. (CMCC)

    - Option 1a: Tmeasure is defined from the end of TTCI-state activate until UE executes LTM to a target cell and interruption time starts. (CTC)

    - Option 1b: Tmeasure is the measurements time. It is defined from the end of TEvent_DU until UE executes  LTM cell switch to a target cell and interruption time starts. (Apple)

- If SSB based L1-RSRP measurement is used in the event, Tmeasure is same as one measurement period of SSB based L1-RSRP measurement as introduced in R18.

- If CSI-RS based L1-RSRP measurement is used in the event, Tmeasure is same as one measurement period of CSI-RS based L1-RSRP measurement which will be introduced in R19.

- If L3 measurement is used in the event, Tmeasure is same as one measurement period of legacy L3 measurement.

  - Option 2: Wait for more progress in RAN2. (HW)

- Recommended WF

  - Discussion is needed.

Issue 1-2-8: TCLTM_execution / TLTM_execution

- Candidate solutions:

  - Option 1: TCLTM_execution / TCLTM_execution is the UE execution preparation time for conditional LTM and starts after UE realizes the condition of LTM is met and identity of the target cell is determined. (CTC, CMCC, Apple)

    - Option 1a: TCLTM_execution can be up to [10ms]. (Apple)

  - Option 2: TCHO_execution may need update in conditional LTM. (HW)

- Recommended WF

  - Discussion is needed.

Issue 1-2-9: Tinterrupt

- Candidate solutions:

  - Option 1: Tinterrupt is the time between when the UE starts to execute the conditional LTM to the target cell and the time the UE starts to transmit the first UL message on the target cell. (CTC)

  - Option 2: TLTM-interrupt is the interruption time as specified in Rel-18 LTM cell switch. (CMCC, Apple)

  - Option 3: Tinterrupt  may induce ambiguity when define conditional LTM delay, as interruptions may also happen in early PRACH procedure (if supported in conditional LTM). (HW)

  - Option 4: The interruption time defined in Rel-18 LTM Cell Switch delay can be a baseline. (CATT, CTC)

  - Option 5: RAN4 to discuss the starting point of the CLTM interruption, and the components included in TCLTM_interrupt. The goal shall be that CLTM interruption is shorter than CHO interruption. (Nokia)

- Recommended WF

  - Discussion is needed.

Issue 1-2-10: TTCI-state activate

- Candidate solutions:

  - Option 1: TTCI-state activate is the time for active the TCI state of candidate cell(s). (CTC)

- Recommended WF

  - Discussion is needed.

Issue 1-2-11: The components of Tinterrupt

- Candidate solutions:

  - Option 1: Tinterrupt = TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU. (CTC)

- TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change.

- Tfirst-RS is the time for fine time tracking and acquiring full timing information of the target cell.

- TRS-proc is the time for SSB processing.

- TLTM-IU is the interruption uncertainty during LTM cell switch.

- Recommended WF

  - Discussion is needed.

Early synchronization and TCI state activation

Issue 1-3-1: Early DL sync

- Candidate solutions:

  - Option 1: R18 LTM early DL-sync can be as baseline. (CTC)

  - Option 2: FFS whether new requirements due to CLTM specific DL/UL sync procedures are needed. (Nokia)

  - Option 3: Wait for RAN2 further agreement on the framework. (E///)

- Recommended WF

  - Discussion is needed.

Issue 1-3-2: Early UL sync

- Candidate solutions:

  - Option 1: For UL-sync, we can consider some situations as follows,(CTC)

- Do not need to acquire the TA, i.e., the NTA of candidate cell(s) is/are same as serving cell or NTA = 0.

- RAR-based TA acquisition.

- UE based TA measurement.

  - Option 2: How/whether to support early RACH on candidate cells during conditional LTM. (HW)

  - Option 3: Whether updates are needed to the Rel-18 PDCCH ordered RACH delay requirements. (Nokia)

  - Option 4:  If DL TODA-based UE autonomous TA estimation is found beneficial for the conditional LTM cell switch, RAN4 to define the requirements for DL TODA-based UE autonomous TA estimation. (QC)

  - Option 5: Wait for more progress from other WGs. (HW, E///)

- Recommended WF

  - Discussion is needed.

Issue 1-3-3: Early TCI state activation

- Candidate solutions:

  - Option 1: How/whether to support early TCI activation on candidate cells during conditional LTM. (HW)

  - Option 2: FFS whether updates are needed to the Rel-18 early TCI state activation delay requirements. (Nokia)

  - Option 3: R18 LTM early TCI state activation can be as baseline. (CTC)

  - Option 4: Wait for more progress from other WGs. (HW, Ericsson)

- Recommended WF

  - Discussion is needed.

Others

Issue 1-4-1: Whether cover both RACH-based and RACH-less conditional LTM

- Candidate solutions:

  - Option 1: Cover both RACH-based and RACH-less conditional LTM. (CTC, HW)

  - Option 2: FFS whether both RACH-less and RACH-based CLTM are supported by RAN1/2. (Nokia)

- Recommended WF

  - Discussion is needed.

Issue 1-4-2: Whether to cover both known and unknown cell for conditional LTM

- Candidate solutions:

  - Option 1: The known cell condition for conditional LTM can be revised as, during the last 5 seconds before the reception of the RRC command that contain conditional LTM configuration……. (CTC)

  - Option 2: All the cell(s) configured as candidate cell(s) for conditional LTM is/are known. (CTC)

- Recommended WF

  - Discussion is needed.

Topic #2: LS out

Issue 2-1-1: LS to RAN2 on conditional LTM cell switch

- Candidate solutions:

  - Option 1: RAN4 kindly asks RAN2 to take the information that L3 RRM measurement can be used to trigger conditional LTM cell switch into consideration in future design and introduce corresponding signaling, as necessary. (Apple)

- Recommended WF

  - Discussion is needed.