3GPP TSG-RAN WG4 Meeting #112bis				    					                    R4-2415670

Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:			6.27.4

Source:	Moderator (MediaTek inc.)

Title:	Topic summary for [112bis][223] IoT_NTN_Ph3

Document for:	Information

# Introduction

Briefly introduce background, the scope of this summary (e.g. list of treated agenda items).

This document is the topic summary for RRM requirements for R19 IoT (Internet of Things) NTN (non-terrestrial network) phase 3, including the following topics covered

- Topic#1: RRM core requirements (AI 6.27.3)

- Topic#2: LS reply for R1-2407548

Recommended issues for online discussion:

- Issue 2-1, 2-2, 1-2-1

# Topic #1: RRM core requirements (AI 6.27.3)

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2415353 | Qualcomm Incorporated | Store&Forward satellite operation Proposal 1: RAN4 to postpone the discussion on the topic of Store&Forward satellite operation until the group can get more clarity on the impact of the topic on RRM requirements. Capacity enhancements for uplink Proposal 2: RAN4 to further wait for RAN2’s progress on Msg3-solo based EDT. Broadcast of PWS messages for NB-IoT Proposal 3: RAN4 to postpone the discussion on the topic of Broadcast of PWS messages for NB-IoT until the group can get more clarity on the impact of the topic on RRM requirements. |
| R4-2415957 | MediaTek inc. | Proposal 3: Clarify the NTN timing requirement also covers R19 enhanced RACH-less EDT procedure (Msg3 transmission without Msg1/Msg2) |
| R4-2416074 | Ericsson | Observation 1: UE is able to know whether its serving satellite is currently operating in S&F via System Information broadcast, but the UE could not predict when its serving satellite will operate in S&F in advance since no early indication by NW. Observation 2: It’s likely that UE is able to know when its serving satellite will resume normal operating from S&F operating with respect to the indication by NW. Observation 3: Other than retaining being served by a S&F mode satellite, UE may re-attempt the Attach/TAU procedure over some satellites with respect to the information indicated by NAS messages, it implies that measurement and cell selection/reselection to such satellites may be impacted if RAN2 takes these messages into account and provides them to UE. Proposal 1: RAN4 to study the measurement behavior in S&F mode, which may cover the below cases: - Measurement when the feeder link is unavailable - Measurement when the service link is unavailable Proposal 2: During the time span when the feeder link is unavailable, UE shall keep SSB measurement since the UE shall monitor paging for the possible DL/UL transmission if the UE has data to be received/transmitted in S&F mode. Proposal 3: During the time span when the feeder link is unavailable, the UE may skip measurement or relax measurement in case of no (more) dedicated data transmission. Proposal 4: During the time span when the feeder link is unavailable, RAN4 to study whether cell reselection is valid or needs to be updated. Proposal 5: During the time span when the service link is unavailable, the measurement behavior is identical to the behavior in discontinuous coverage in Rel-18 IoT NTN. |

## Open issues summary

Before f2f meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-Topic 1-1: EDT enhancement

#### Issue 1-1-1: RRM impact from EDT enhancement

Proposals:

- Proposal 1 (Qualcomm): RAN4 to further wait for RAN2’s progress on Msg3-solo based EDT.

- Proposal 2 (MTK): Clarify the NTN timing requirement also covers R19 enhanced RACH-less EDT procedure (Msg3 transmission without Msg1/Msg2)

Recommended WF: Further discussion

### Sub-Topic 1-2: RRM impact from other objectives

Background:

- Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore:

  - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2]

    - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3]

Note: Strive to minimise UE impact.

Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required

….

- Support broadcast of PWS messages for NB-IoT re-using the LTE mechanisms [RAN2]

Note: The solutions will be developed for NTN but can be applicable to TN (if possible). Specific NB-IoT TN optimizations will not be considered as part of this WI.

Note: Support for indication of intended service area for ETWS will be considered based on NR NTN discussions/outcome.

The latest WID RP-242397

#### Issue 1-2-1: Store&Forward (S&F) satellite operation

Proposals:

- Option 1: RAN4 to postpone the discussion on the topic of Store&Forward satellite operation until the group can get more clarity on the impact of the topic on RRM requirements. (Qualcomm)

- Option 2 (Ericsson):

  - RAN4 to study the measurement behavior in S&F mode, which may cover the below cases:

    - Measurement when the feeder link is unavailable

    - Measurement when the service link is unavailable

  - During the time span when the feeder link is unavailable, UE shall keep SSB measurement since the UE shall monitor paging for the possible DL/UL transmission if the UE has data to be received/transmitted in S&F mode.

  - During the time span when the feeder link is unavailable, the UE may skip measurement or relax measurement in case of no (more) dedicated data transmission.

  - During the time span when the feeder link is unavailable, RAN4 to study whether cell reselection is valid or needs to be updated.

  - During the time span when the service link is unavailable, the measurement behavior is identical to the behavior in discontinuous coverage in Rel-18 IoT NTN.

Moderator’s Note: RAN4 is not included for this objective in WID.

Recommended WF: Further discussion on the RRM impact.

#### Issue 1-2-2: broadcast of PWS messages

Proposals:

- Option 1: RAN4 to postpone the discussion on the topic of Broadcast of PWS messages for NB-IoT until the group can get more clarity on the impact of the topic on RRM requirements. (Qualcomm)

Moderator’s Note: RAN4 is not included for this objective in WID.

Recommended WF: RAN4 to postpone the discussion on the topic of Broadcast of PWS messages for NB-IoT.

# Topic #2: LS reply for R1-2407548

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2415829 | Huawei, HiSilicon | Observation 1: Note 2 in RAN1 LS [6] is valid as replied by RAN4 in last meeting. Proposal 1: From RAN4 perspective, it is valid to assume NTA is equal to zero if there is no other procedure for NTA determination introduced by other WG |
| R4-2415957 | MediaTek inc. | Proposal 1: Regarding Note 1, RAN4 assumed that NTA_Ref is not configured by the network for MSG3 transmission without Msg1/Msg2, and RAN4 will follow the conclusion of the other working groups in the future, if any. Proposal 2: Confirm Note 2 in LS R1- 2407548. |
| R4-2416188 | Nokia | Observation 1: RAN4 has previously indicated that in our initial investigation, the value of the network controlled component of the timing advance was not initialized when MSG 3 was transmitted without MSG1/2. Observation 2: In the absence of the network controlled component of the timing advance, the UE is capable of fulfilling the UE transmit timing requirements from its point of view, but because the reference point has changed, the UL transmission might not be aligned in time in the UL synchronization point. Proposal 1: Reply to RAN1 that the exact definition of NTA is up to RAN1, but RAN4 has assumed to this point that the non-initalized value of NTA is equal to 0. Proposal 2: Confirm to RAN1 that NOTE 2 in R1-2407548 is valid. |
| R4-2416075 | Ericsson | Observation 1: LS reply R1-2407548 is aligned with LS reply R4-2414114, RAN4 doesn’t need to raise further LS reply. |

## Open issues summary

Before f2f meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

// The LS R4 replied to R2 in the last meeting (R4-2414114)

RAN4’s feedback:

For Msg3 transmission without Msg1/Msg2, UE can meet UE UL transmit timing requirements defined for IoT NTN in TS 36.133[attached in Annex] from UE RRM requirement definition point of view.

Note 1: RAN4 assumed that NTA_Ref is not configured by the network for MSG3 transmission without Msg1/Msg2. RAN4 will follow the conclusion of the other working groups in the future, if any.

Note 2: For NB-IoT with 15kHz SCS, the timing error may be larger than half CP.

// The LS received from R1 (R1-2407548)

RAN1’s response:

From RAN1 perspective, assuming Note 1 and Note 2 are satisfied, an RRC Idle UE with a pre-compensated TA (i.e., the one used for Msg1 transmission during random access for IoT NTN) can satisfy requirement for Msg3 transmission/reception without Msg1/Msg2 at least in the following cases:

- NB-IoT with 3.75kHz SCS.

- eMTC CE mode A.

Note 1: RAN1 assumes NTA=0 for Msg3 transmission without Msg1/Msg2

Note 2: RAN1 assumes that the RAN4 UL synchronization requirements specified in Table 7.20A.2-1 and Table 7.24A.2-1 of TS 36.133 apply to the Msg3 transmission without Msg1/Msg2 in NB-IoT over NTN and eMTC over NTN, respectively.

Note 3: RAN1 may further discuss the cases of NB-IoT with 15kHz SCS and eMTC CE mode B

2. Actions:

….

To RAN4:

ACTION: 	RAN1 respectfully asks RAN4 to confirm whether RAN1’s assumptions in Note 1 and Note 2 are valid.

Background:

### Issue 2-1: Reply regarding the Note 1

Proposals:

- Option 1a: From RAN4 perspective, it is valid to assume NTA is equal to zero if there is no other procedure for NTA determination introduced by other WG (Huawei)

- Option 1b: RAN4 assumed that NTA_Ref is not configured by the network for MSG3 transmission without Msg1/Msg2, and RAN4 will follow the conclusion of the other working groups in the future, if any. (MTK)

- Option 1c: Reply to RAN1 that the exact definition of NTA  is up to RAN1, but RAN4 has assumed to this point that the non-initalized value of NTA is equal to 0. (Nokia)

- Option 2: do not reply the LS (Ericsson)

Recommended WF:

- Reply to RAN1 that RAN4 assumed that N_TA is equal to 0, if there is no other procedure for NTA determination introduced by other WG, and RAN4 will follow the conclusion of the other working groups in the future, if any.

### Issue 2-2: Reply regarding the Note 2

Proposals:

- Option 1: confirm Note 2 (Nokia, MTK, Huawei)

- Option 2: do not reply the LS (Ericsson)

Recommended WF:

- Confirm to RAN1 that NOTE 2 in R1-2407548 is valid.