# 3GPP TSG-RAN WG4 Meeting #112bis	R4-2416524

# Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:			6.11.5

Source:	Moderator (Huawei)

Title:	Topic summary for [112bis][303] NR_BS_RF_Part2_CLTA

Document for:	Information

# Introduction

Focus of this thread is co-location test antenna (CLTA) topic of the NR_BS_RF_req_evo work item. CLTA-related objectives were extracted from the WID in RP-242010 for reference:

- Core:

  - Investigate whether the BS/IAB OTA co-location reference antenna definition need be improved for FR1, and if feasible, update the definition.

- Perf:

  - Investigate whether the existing test approach can be improved, and if feasible, improve test methods for BS/IAB OTA co-location requirements and tests for AAS-based test specifications for FR1.

# Topic #1: CLTA

## Companies’ contributions summary

Numner of contributions were submitted to this meeting, to address open issues identified for CLTA in the WF in R4-2413509.

The following contributions were identifed as reated to CLTA topic:

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2415674 | Nokia | On OTA test enhancements - CLTA Observation 1: The power level of the spurs in these measurements are fluctuating in time. Observation 2: More measurements are needed to ensure the usability of wideband horn antennas as CLTAs. Observation 3: Beam steering is a technology of paramount importance for AAS. Observation 4: Beam steering has a significant effect on the coupling level. Observation 5: Side-by-side can still be expected as a typical scenario due to site constraints Proposal 1: Co-location scenarios for higher FR1 frequencies need to consider beam steering. |
| R4-2415971 | Ericsson | On the topic of aspects related to BS-to-BS co-location Observation 1: The assumed BS-to-BS isolation of 30 dB have direct impact on required RF filter suppression for any base station implementation. For AAS BS operating above 3500 MHz the filter suppression has direct impact on the physical size of the RF filter and corresponding routing losses, volume, weight and cost. Since the current assumption was derived for GSM operating at 900 MHz it would be reasonable to further study the relevance for AAS base stations operating above 3500 MHz. Observation 2: Using a fixed value for BS-to-BS isolation would risk to over specify spurious emission requirements for the situation where the frequency offset is large between the aggressor base station and victim base station. For AAS base stations it would be reasonable to further study statistical properties due beamforming and how they affect BS-to-BS isolation. Also, aspects related to victim/aggressor frequency offset would be relevant to study. It could be transmission mismatch, different physical size of victim and aggressor antenna aperture that have impact on BS-to-BS isolation. Observation 3: Based on statistical simulation results RAN4 need to establish criteria for requirements derivation. Observation 4: RAN4 need to study more relevant and accurate models to evaluate BS-to-BS isolation for both coordinated, uncoordinated and intermediate co-location scenarios. Observation 5: When evaluating BS-to-BS isolation from simulation results and measurement results, it is essential to be clear on isolation definition and the purpose of the result. The requirement derivation for different requirement may depend on different isolation definitions. Proposal 1: Since the technical background for 30 dB BS-to-BS isolation is obsolete for large array AAS base station operating above 3500 MHz RAN4 should create a study with the intention to better describe relevant co-location scenarios and evaluate expected BS-to-BS isolation based on simulations. |
| R4-2416238 | ZTE Corporation, Sanechips | Observation 1: if two NR AAS BS are placed with vertical separation which is quite typical for the mast installation, then the coupling loss would be larger than 50dB with10cm separation distance. Observation 2: if two NR BS are placed with horizontal separation which is typical for roof based installation, the coupling loss is still larger than 30dBc and the coupling loss among different antenna ports are varying from 31dB to 46dBc. Observation 3: the side by side co-location deployment should be still typical for the roof based installation Observation 4: according to the AAU/RRU installation guidance, the minimum horizontal separation distance should be 0.6m and minimum vertical separation distance should be 0.2m which are both larger than the specified 0.1m separation distance. The co-location requirement itself in the specification is very stringent requirement compared with the practical deployment. Observation 5: it might be very difficult to have Tx and Rx installed in the different heights of BS mast or tower according to the legacy deployment practice; Observation 6: These two received gain for passive antenna of CLTA and AAS sub-array would be quite different. In other words, the measured spurious emission or transmitter OFF power from CLTA cannot be reused or applicable for AAS BS co-locating with AAS BS at sub-array level. Observation 7: These two passive antenna and AAS BS pattern would be quite different, from Tx intermodulation and co-location OOBB requirement, it seems that the legacy CLTA requirement would provide more stringent conformance testing for it. |
| R4-2416276 | Huawei, HiSilicon | Discussion on CLRA/CLTA Proposal: for CLTA the introduction of new antenna alternatives is ok as long as they are in parallel to the existing CLTA method rather than replacing it. |
| R4-2416279 | Huawei, HiSilicon | TX IMD test for high frequency band in FR1 Observation: A min isolation of approx. 40dB is probably enough to remove the reverse IMD requirement. For band n104 even with 30dB isolation the reverse IMD requirement can probably be removed. Proposal: It is proposed to remove TX IMD CLTA test for high FR1 band, e.g. frequency bands above 4.2 GHz. |

## Open issues summary

### Sub-topic 1-1: CLTA definition

This topic is identified as related to the following WID objective: “Investigate whether the BS/IAB OTA co-location reference antenna definition need be improved for FR1, and if feasible, update the definition.”

Open issues and candidate options before meeting:

- Proposals

  - Option 1: for CLTA the introduction of new antenna alternatives is ok as long as they are in parallel to the existing CLTA method rather than replacing it. (Huawei, R4-2416276)

- Recommended WF

  - Check online whether Option 1 is agreeable; collect comments.

  - Moderator: suggest to clarify whether this proposal is applicable to the whole FR1.

### Sub-topic 1-2: BS-to-BS isolation analysis

Discussion on BS-to-BS isolation analyses were provided in multiple contributions: R4-2415971 (Ericsson), R4-2416238 (ZTE), R4-2416279 (Huawei).

- Proposals

  - Option 1: Since the technical background for 30 dB BS-to-BS isolation is obsolete for large array AAS base station operating above 3500 MHz RAN4 should create a study with the intention to better describe relevant co-location scenarios and evaluate expected BS-to-BS isolation based on simulations. (Ericsson, R4-2415971)

- Recommended WF

  - Check online whether Option 1 is agreeable; collect comments.

  - Moderator:

    - While Rel-19 work item is ongoing, it is suggested to rather focus on technical analyses, as potential Rel-19 SI decision is outside RAN4 scope.

### Sub-topic 1-3: Beam stering consideration for co-location scenario

Open issues and candidate options before meeting:

- Proposals

  - Option 1: Co-location scenarios for higher FR1 frequencies need to consider beam steering. (Nokia, R4-2415674)

- Recommended WF

  - Check online whether Option 1 is agreeable; collect comments.

  - Moderator: suggest to clarify applicable frequency range within FR1 for this proposal.

### Sub-topic 1-4: TX IMD co-location

While BS-to-BS isolation analysis was captured in sub-topic 1-2, this sub-topic 1-4 looks into a specific requirement (i.e. TX IMD) with the proposal motivated by the BS-to-BS isolation analysis as well.

Open issues and candidate options before meeting:

- Proposals

  - Option 1: It is proposed to remove TX IMD CLTA test for high FR1 band, e.g. frequency bands above 4.2 GHz. (R4-2416279, Huawei)

- Recommended WF

  - Check online whether Option 1 is agreeable; collect comments.