3GPP TSG-RAN WG4 Meeting #112bis                                                          R4-2416527

Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:			6.20.4

Source:	Moderator (Samsung)

Title:	Topic summary for [112bis][306] NR_duplex_evo_General

Document for:	Information

# Introduction

In RAN#102, the work item on evolution of NR duplex operation (SBFD) has been approved [RP-234035], with WID further revised in the follow-up RAN plenary [RP-240789]. According to the objectives in WID, RAN1 is tasked to specify the mechanisms to support SBFD, including semi-static indication of time/frequency location, random access in SBFD symbols, and other transmission, reception and measurement behavior and procedures for SBFD aware UE. Furthermore, the enhancement for CLI handing, including gNB-to-gNB and UE-to-UE CLI handling, will also be specified in RAN1. Accordingly, from RAN4 perspective, it is tasked to “Specify BS RF requirements for SBFD operation at gNB [RAN4]”.

Over RAN4#112, there was extensive discussion on SBFD general aspects and potential new requirements for SBFD operation for FR1 and FR2-1, which results in good progress captured in the approved WF [R4-2413514].

This document is provided for the moderator summary of thread [306] on Rel-19 work item on evolution of NR duplex operation (SBFD) for general aspect and potentially new requirements for SBFD, in which the following highlighted agenda items are supposed to be covered specifically:

- Evolution of NR duplex operation: Sub-band full duplex (SBFD) [NR_duplex_evo] - General aspects (including RAN4 aspects for SBFD system parameters) [NR_duplex_evo-Core] - BS RF requirements [NR_duplex_evo-Core] - Potentially new requirements for SBFD operation for FR1 and FR2-1 [NR_duplex_evo-Core] - Modification of existing Tx requirements for FR1 and FR2-1 [NR_duplex_evo-Core] - Modification of existing Rx requirements for FR1 and FR2-1 [NR_duplex_evo-Core] - RRM core requirements [NR_duplex_evo-Core] - Moderator summary and conclusions [NR_duplex_evo]

# Topic #1: General aspects (including RAN4 aspects for SBFD system parameters)

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2415021 | CMCC | Proposal 1: for RF spec structure of SBFD requirements, it’s better to create new sub-clauses in TS 38.104 which capture all new and existing RF requirements that applies for SBFD. Proposal 2: The channel bandwidth should be equal or larger than 20MHz. Proposal 3: To keep the flexibility of SBFD, option 2(with the middle flexibility) could be used as baseline. Proposal 4: The maximum value of the guard band size should be specified in the spec. |
| R4-2415042 | Tejas Network Limited | Proposal 1: To consider the option-3 with the highest flexibility i.e., no restriction on the subband sizes, i.e., RF requirement shall be specified to enable all DL and UL subband configurations (and implicitly guard band configurations). Observation 1: If the channels are discontinuous, then SBFD in these channels will not be useful; instead, we can use the supplementary uplink channel to increase uplink throughput. As an example, if the operator owns continuous 4x10 MHz channels, then SBFD can be used; otherwise, it will add up complexity without much advantage. Observation 2: From the frequency allocation to the Indian operator, a minimum 20 MHz BW is allocated in TDD bands. Observation 3: If the configured BW is less than 20 MHz then there is a significant drop in the DL throughput as compared to 20 MHz. Proposal 2: A minimum 20 MHz BW in FR-1 is required for a reasonable SBFD operation; otherwise, the throughput will be reduced significantly. |
| R4-2415100 | CATT | For specification, Observation 1: Option 2a in the WF[1] may be the better choice in recent meetings. How to write the spec can be discussed when most of the requirements are agreed. For Band/CBW support for SBFD, Observation 2: Unnecessary restrictions on the CBW support for SBFD feature is not preferred from the specification perspective. Proposal 1: Use adding SBFD entry to the table to introduce the SBFD bands. For sub-band configuration and guard band size, Proposal 2: Option-2b in the WF[1] is agreed to handle the sub-band configuration for SBFD. Proposal 3: Option 3 in WF [1] is agreed for the GB size for SBFD. Proposal 4: The following approach can be considered to handle the maximum GB size for DL/UL GB. – The maximum allowed number of RBs for GB is separated to DL SB GB and UL SB GB. – The maximum GB restriction uses the GB size in current specification if it can be found. – If SB CBW can’t be found in current specification, 10% GB size is used as the maximum GB restriction. |
| R4-2415272 | CableLabs, Charter Communications | Proposal 1: The total bandwidth of the guard band(s) reduces the transmission bandwidth, and such a reduction percentage should be considered as a reference when deciding the minimum channel bandwidth for SBFD operating bands. Proposal 2: The transmission bandwidth reduction, R, is suggested between 2% and 5%, and the value of R could be further discussed. Proposal 3: Based on proposals 1 and 2 and an assumed 1.8 MHz inter-subband guard bandwidth (five RBs), to maintain the spectrum efficiency, SBFD should operate in bands with the minimum channel bandwidth of 50 MHz for the DU configuration and 100 MHz for the DUD configuration. |
| R4-2415467 | Qualcomm Germany | Observation 1: In RAN4#112, proposed value of X for SBFD applicability for existing TDD bands was between 10 MHz and 50 MHz. Proposal 1: RAN4 to adopt X = 20 MHz and to include the following text in Clause 5.2 in TS 38.104: subband full duplex can be applied to TDD bands with channel bandwidth >= X MHz, given in Table 5.2-1. Observation 2: During study item phase, 5 PRBs were generally considered by several companies when studying the feasibility of wide area SBFD deployments. However, for different SBFD configuration and different cancellation techniques, that value of the guardband will vary. Observation 3: By setting the UL subband configuration to be selected from the existing values of BS transmission bandwidth configuration, as provided TS 38.104, while letting the DL subband size(s) and guardband size(s) depends on BS implementation, RAN4 would provide flexibility for implementation aspects that might differ from one vendor to another. Observation 4: An agnostic configuration approach in relation to the subband configuration should be considered in order not to restrict the normative work to a set of subband configurations. Proposal 2: RAN4 to agree on selecting UL subband configuration to be selected from the existing values of BS transmission bandwidth configuration, as provided TS 38.104, while letting the DL subband size(s) and guardband size(s) depends on BS implementation. Proposal 3: To enhance the readability of the spec, it is preferable to have standalone sub-clauses in core specs for SBFD-specific existing or new gNB RF requirements. |
| R4-2415587 | Charter, CableLabs | Observation 1: Figure 1 shows that the overhead of the 100 MHz guard band is smaller relative to the 50 MHz by a factor of 2. Observation 2: This means that the 100 MHz SBFD carrier will have always a higher and better spectral efficiency than the 50 MHz carrier. For this reason, 100 MHz becomes our 1st priority relative to 50 MHz. Observation 3: Figure 2 shows that there are no guard bands between the AMBIT band and the CBRS band, as well as there are no guard bands between the CBRS band and the C-band. Observation 4: Figure 2 shows that both the AMBIT band and C-band can transmit at 75 dBm/10MHz EIRP, but the CBRS band is limited to 30 dBm/10MHz only. This means that the CBRS band EIRP is 45 dB smaller than band n77. Therefore, legacy TDD operations in the USA CBRS band will suffer much higher risk from adjacent-band SBFD deployments. Proposal 1: Priority 1: The channel bandwidth of a Subband Full Duplex carrier should be no less than 100 MHz Priority 2: The channel bandwidth of a Subband Full Duplex carrier should be no less than 50 MHz Proposal 2: To include the following text in Clause 5.2 in TS 38.104: Sub-band full duplex can be applied to the following TDD bands given in Table 5.2-1: n38, n40, n41, n46, n50, n78, n79, n90, n96, n102, n104 for FR1, excluding n48 and n77. Proposal 3: To include the following text in Clause 5.2 in TS 38.104: Sub-band full duplex can be applied to the following TDD bands given in Table 5.2-1: n38, n40, n41, n46, n48, n50, n77, n78, n79, n90, n96, n102, n104 for FR1, except for the following bands in the USA: n48 and n77. |
| R4-2415678 | Nokia | Observation 1: SBFD operation below 50MHz BW would result in a very narrow UL subband, and it introduces some challenges in terms of accomodating reference signals and channels such as SSB and CORESET 0 within the DL subband. Observation 2: Analog filters require large guardband which makes the operation of SBFD in narrow channel bandwidths either unfeasible or not attractive from performance point of view. Proposal 1: RAN4 to not revisit RF requirements pending on RAN1 progress on the gNB-to-gNB CLI handling work. Proposal 2: Limit X to be no less than 50MHz. Proposal 3: For FR1, RAN4 to prioritize SBFD requirements for BS carrier bandwidth of 100MHz first and 50 MHz secondly. Proposal 4: For FR1 SBFD requirements, from base station perspective, use the following configuration: a. For FR1 DUD configuration in 100 MHz channel and 30 kHz SCS: < ND, NU, NG > = <106, 51, 5> b. For FR1 DU or UD configuration in 100 MHz channel and 30 kHz SCS: < ND, NU, NG > = <217, 51, 5> Proposal 5: For FR2-1 SBFD requirements, from base station perspective, the following assumptions for guardbands and UL and DL subbands can be used as starting point: a. For FR2-1 DUD configuration in 100 MHz channel and 120 kHz SCS: < ND, NU, NG > = <25, 14, 1> b. For FR2-1 DUD configuration in 200 MHz channel and 120 kHz SCS: < ND, NU, NG > = <52, 26, 1> |
| R4-2415748 | vivo | Proposal 1：Channel bandwidth no less than 20MHz for SBFD can be a starting and further check if RSIC can be achievable for 20 MHz channel bandwidth especially for FR1 WA and MR BS. Proposal 2：Option 2b (UL subband shall be selected from BS transmission channel bandwidth) can be used as a starting point for subband configuration. Proposal 3: No restriction on maximum/minimum guardband size if UL subband size shall be selected from BS transmission channel bandwidth and DL subband size shall be declare based. |
| R4-2416134 | Ericsson | Observation 1 Wide guardband can consume valuable bandwidth. When the channel bandwidth is low, after excluding the guardband, the remaining PRBs for both DL and UL subbands may become insufficient. Such a situation is not practical in real-world deployments. Observation 2 SBFD configurations shall be captured in Rel-19 RAN4 specification and restricted to a limited set. Observation 3 15 kHz SCS is only supported by channel bandwidth no larger than 50 MHz, which are too narrow to be considered for SBFD. Proposal 1 A general principle should be applied when specifying SBFD: The channel bandwidth should be no less than 50 MHz. Proposal 2 To include the following text in Clause 5.2 in TS 38.104: subband full duplex can be applied to the following TDD bands given in Table 5.2-1: n38, n40, n41, n46, n48, n50, n77, n78, n79, n90, n96, n102, n104 for FR1. Proposal 3 The SBFD capable BS product shall remain fixed in its configuration during real operation, i.e., it shall not be reconfigurable to a different configuration/DL/UL/guardband setting other than those that have been declared and tested. Proposal 4 For one supported channel bandwidth, only one fixed UL subband size is specified, which is around 20% of the supported channel bandwidth. Proposal 5 Only symmetric DUD configurations are specified, as there is no foreseeable benefit from flexible placement of the UL subband. Proposal 6 Select Option 1 to specify DL/UL subband configuration in BS RF requirements, due to interoperability issues between different vendors. Proposal 7 Reuse the existing channel bandwidths for SBFD UL subbands. Proposal 8 SBFD-capable BS channel bandwidths are down selected to 50 MHz, 60 MHz, 70 MHz, 80 MHz, 90 MHz and 100 MHz for FR1. Proposal 9 For FR1, use 30 kHz as the SCS for SBFD slots. Proposal 10 For WA BSs, wider guardbands than existing ones are needed for SBFD. Proposal 11 RAN4 define SBFD subband configurations for WA BS as shown in Table 2 for FR1. Proposal 12 Introduce Transmission bandwidth configuration for SBFD DUD, DU and UD patterns, as shown in Figure 1-3 below. Figure 1: Transmission bandwidth configuration for SBFD DUD pattern for one NR channel Figure 2: Transmission bandwidth configuration for SBFD DU pattern for one NR channel Figure 3: Transmission bandwidth configuration for SBFD UD pattern for one NR channel Proposal 13 RAN4 create new specifications for SBFD-capable BS. |
| R4-2416185 | Huawei, HiSilicon | Observation 1: The size of guard band for SBFD is suitable to be manufacturer declaration based as it can vary depending on implementation for different BS type or frequency band, thus having limitation on the minimum/maximum value of the size of guard band would be a great loss on implementation flexibility. Proposal 1: To specify DL/UL subband configuration in BS RF requirements: • UL subband configuration shall be selected from the existing values of BS transmission bandwidth configuration, as provided in Table 5.3.2-1 for FR1 and Table 5.3.2-2 for FR2-1. • DL subband size(s) and guardband size(s) depends on BS implementation. Proposal 2: For single carrier operation, the supported BS CBW for SBFD should be no less than 60MHz. Proposal 3: For multi-carriers operation, the SBFD can be supported on only one of the carriers, and the supported CBW for this carrier should be no less than 60MHz and applying 100MHz as maximum. • For instance, 100MHz CC for SBFD + N*100MHz CC, where N could be e.g. 1, 2, and 3 Proposal 4: Add SBFD entry in Table 5.2-1 in Clause 5.2 in TS 38.104 for the TDD bands with channel bandwidth >= 60MHz. Proposal 5: Embed the corresponding SBFD RF requirements in the corresponding section of TS 38.104 as much as possible (e.g. RAN4 already agreed to embed the supporting band of SBFD in clause 5) and consider new clauses or new suffix for SBFD for the RF requirements that are necessary. |
| R4-2416248 | ZTE Corporation, Sanechips | Proposal 1: propose to adopt the 20MHz as the minimum carrier bandwidth for SBFD operation in FR1 TDD bands and also 50MHz as minimum carrier bandwidth for SBFD operation in FR2-1 TDD bands. Proposal 2: propose to add the clarification figure for the SBFD BS PRB utilization in the specification Proposal 3: propose to adopt the option -2b to specify the DL/UL subband configuration. Proposal 4:all UL sub-band configuration (e..g.reusing from the existing carrier bandwidth, 20~90MHz) should be specified for SBFD receiver requirements together with FRC and power levels. The detailed MCS, reference signal pattern for licensed band could align with WF R4-1801031. For unlicensed bands, the interleaved PRB pattern should be also configured. Proposal 5: propose to guard band between DL and UL sub-band is up to the declaration from the implementation perspective. Proposal 6: propose to have the transition period between non-SFBD symbols and SBFD symbols located within SBFD symbols. |
| R4-2416497 | Samsung | Proposal 1: For gNB support of SBFD for a certain TDD band, it is based on gNB vendor’s declaration. For the SBFD-supported band, the channel bandwidth(s) supported for SBFD shall follow the below principles: - The supported channel bandwidth(s) is declared by gNB vendors - The supported channel bandwidth(s) is from existing channel bandwidth defined in TS 38.104 - The supported channel bandwidth shall be no less than X=20 MHz Proposal 2: By following the principle of “The supported channel bandwidth shall be no less than X=20 MHz”, TDD bands n34, n51, n53, n54 and n101 shall be precluded from SBFD deployment. Proposal 3: RAN4 shall introduce the following text in Clause 5.2: - “Subband full duplex can be applied to all FR1 TDD bands given in Table 5.2-1, except bands n34, n51, n53, n54 and n101, and all FR2 bands given in table 5.2-2.” Proposal 4: Except channel bandwidth values smaller than 20MHz, there is no necessity to preclude other supported channel bandwidth values for each band from SBFD deployment given in Table 5.3.5-1. Proposal 5: RAN4 shall introduce the following note in Table 5.3.5-1: - “Note 9: Subband full duplex can be applied to all supported channel bandwidth values except the ones smaller than 20MHz.” Proposal 6: The following figure is introduced for the illustrations of BS channel bandwidth, BS transmission bandwidth configuration for SBFD NRB,SBFD, transmission bandwidth configuration for SBFD DL subband NRB,SBFD,DL, transmission bandwidth configuration for SBFD UL subband NRB,SBFD,UL, and guardband between SBFD UL/DL subbands NRB,SBFD,GB for SBFD operation: Figure 1: Definition of channel bandwidth and transmission bandwidth configurations for one NR channel for SBFD operation Observation 1: To specify DL/UL subband configuration in BS RF requirements, Option-1 (i.e., only a number of selected DL/UL subband configurations are specified) is impracticable for standardization and against RAN1 agreements. Proposal 7: To specify DL/UL subband configuration in BS RF requirements, the following Option-2b is adopted: - Option-2b: - UL subband configuration shall be selected from the existing values of BS transmission bandwidth configuration, as provided in Table 5.3.2-1 for FR1 and Table 5.3.2-2 for FR2-1. - DL subband size(s) and guardband size(s) depends on BS implementation - Example (FR1 DU or UD in 100MHz and 30kHz SCS, 80-20MHz case): As long as BS transmission configuration for SBFD shall be 273 (NRB,SBFD for 100MHz and 30kHz SCS) and the value of NRB,SBFD,UL is selected from Table 5.3.2-1 (according to the 30kHz SCS), from BS RF requirement perspective, no limitation on the values NRB,SBFD,DL and NRB,SBFD,GB. Proposal 8: For guardband size, no restriction shall be introduced on maximum/minimum number of RBs. Proposal 9: A new 5.3B clause is added to capture the BS channel bandwidth for SBFD (including Transmission bandwidth configuration, guardband and subband configuration for SBFD), with the following text proposal to be considered as baseline skeleton for triggering discussion: =================== Start of Text Proposal =================== ## 5.3B BS channel bandwidth for SBFD ### 5.3B.1 General <To add the illustrative definition of channel bandwidth, transmission bandwidth configuration, uplink and downlink subbands and guardband within one NR channel for SBFD operation> ### 5.3B.2 Transmission bandwidth configuration for SBFD <To be added> ### 5.3B.3 Guardband and Subband configuration for SBFD <To be added> =================== End of Text Proposal =================== Proposal 10: For introducing BS RF requirements for SBFD-capable BS, RAN4 shall adopt the way of creating new and standalone sub-clauses in TS 38.104 by setting up new sub-clauses with suffix-B. |

## Open issues summary

### Sub-topic 1-1: SBFD frequency domain issues

##### Issue 1-1-1: Minimum channel bandwidth for SBFD (X MHz)

- [Moderator] Based on the discussion in RAN4#111, SBFD was recognized as “a feature with requirements of which can potentially be applied to all TDD band”, and the declaration-based method for the SBFD-support in a particular TDD band was agreed in RAN4#112. Particularly, the following agreements was provided:

<From RAN4#112> - Agreement: - Declaration-based method, and SBFD is possible to be used on any TDD band, with the following principle: - The supported channel bandwidth(s) is declared by gNB vendors - The supported channel bandwidth(s) is from existing channel bandwidth defined in TS 38.104 - The supported channel bandwidth shall be no less than X MHz - X = [20] - For next meeting, we will further discuss the potential to increase the value of X beyond X = [20]. Companies are encouraged to bring t-docs to discuss the benefits or drawback to increase this value. - FFS for some specific band if this principle rules out this band, which is required for SBFD by operators.

- [Moderator] It is agreed the supported channel bandwidth shall be no less than X MHz, while X = [20] is the baseline for further study:

- The value of X

  - Option 1 (CMCC, Samsung, Tejas, Qualcomm, ZTE): X = 20 is confirmed

    - Proposal 1-1 (Samsung): By following the principle of “The supported channel bandwidth shall be no less than X=20 MHz”, TDD bands n34, n51, n53, n54 and n101 shall be precluded from SBFD deployment.

  - Option 1a (vivo): X = 20 as starting point, and further check if RSIC can be achievable for 20 MHz channel bandwidth especially for FR1 WA and MR BS

  - Option 2 (Nokia, Ericsson): X = 50

    - Proposal 2-1 (Ericsson): A general principle should be applied when specifying SBFD: The channel bandwidth should be no less than 50 MHz.

    - Observation 2-1 (Nokia): SBFD operation below 50MHz BW would result in a very narrow UL subband, and it introduces some challenges in terms of accomodating reference signals and channels such as SSB and CORESET 0 within the DL subband.

    - Observation 2-2 (Nokia): Analog filters require large guardband which makes the operation of SBFD in narrow channel bandwidths either unfeasible or not attractive from performance point of view.

  - Option 3 (CableLabs, Charter): X = 50MHz for DU and X = 100MHz for DUD

    - Proposal 3-1 (CableLabs): The total bandwidth of the guard band(s) reduces the transmission bandwidth, and such a reduction percentage should be considered as a reference when deciding the minimum channel bandwidth for SBFD operating bands. The transmission bandwidth reduction, R, is suggested between 2% and 5%, and the value of R could be further discussed.

    - Proposal 3-2 (CableLabs): Based on proposal 3-1 and an assumed 1.8 MHz inter-subband guard bandwidth (five RBs), to maintain the spectrum efficiency, SBFD should operate in bands with the minimum channel bandwidth of 50 MHz for the DU configuration and 100 MHz for the DUD configuration.

    - Proposal 3-3 (Charter): The channel bandwidth for a Subband Full Duplex carrier should be no less than 100 MHz.  (100 MHz becomes 1st priority relative to 50 MHz)

  - Option 4 (Huawei): X = 60

    - Proposal 4-1 (Huawei): For single carrier operation, the supported BS CBW for SBFD should be no less than 60MHz.

    - Proposal 4-2 (Huawei): For multi-carriers operation, the SBFD can be supported on only one of the carriers, and the supported CBW for this carrier should be no less than 60MHz and applying 100MHz as maximum.

      - For instance, 100MHz CC for SBFD + N*100MHz CC, where N could be e.g. 1, 2, and 3

- Moderator Recommendation:

  - Further discussion on the above options.

##### Issue 1-1-2: SBFD as a Band-specific feature

- [Moderator] In RAN4 #112, it is agreed that SBFD is possible to be used on any TDD band, in other words, SBFD is not a band-specific feature. On the other hand, Charter has the following observations and proposals, specifically for US band n48 and n77:

- Observations/Proposals for SBFD as a band-specific feature:

  - Observation 1 (Charter): Figure 2 shows that there are no guard bands between the AMBIT band and the CBRS band, as well as there are no guard bands between the CBRS band and the C-band.

  - Observation 2 (Charter): Figure 2 shows that both the AMBIT band C-band can transmit at 75 dBm /10MHz EIRP, but the CBRS band is limited to 30 dBm/10MHz only. This means that the CBRS band EIRP is 45 dB smaller than band n77. Therefore, legacy TDD operations in the USA CBRS band will suffer much higher risk from adjacent-band SBFD deployments.

Figure 2: Band n48 and n77 spectrum allocation in the USA and EIRP requirements

  - Proposal 1 (Charter): To include the following text in Clause 5.2 in TS 38.104: Sub-band full duplex can be applied to the following TDD bands given in Table 5.2-1: n38, n40, n41, n46, n50, n78, n79, n90, n96, n102, n104 for FR1, excluding n48 and n77.

  - Proposal 2 (Compromised Proposal from Charter): To include the following text in Clause 5.2 in TS 38.104: Sub-band full duplex can be applied to the following TDD bands given in Table 5.2-1: 
n38, n40, n41, n46, n48, n50, n77, n78, n79, n90, n96, n102, n104 for FR1, 
except for the following bands in the USA:  n48 and n77.

- Moderator Recommendation:

  - It should be noted that:

    - It is agreed that “is agreed that SBFD is possible to be used on any TDD band”, and in general, we shall discuss a general principle to handle this, rather than discussing band by band.

    - Even with the restriction of “channel BW no less than X=20MHz”, some of TDD bands are precluded, which is not far from Charter’s proposal

##### Issue 1-1-3: Specification implementation for band/channel bandwidth support for SBFD

- [Moderator] There are two options listed for how to implement in the specification for band/channel bandwidth support for SBFD

- FFS how to implement in the spec: - Option 1: To include the following text in Clause 5.2 in TS 38.104: subband full duplex can be applied to TDD bands with channel bandwidth >= X MHz, given in Table 5.2-1. - Option 2: Add SBFD entry in Table 5.2-1 in Clause 5.2 in TS 38.104 for the TDD bands with channel bandwidth >= X MHz.

- •	FFS how to implement in the spec

  - Option 1 (Samsung, Qualcomm, Ericsson): To include the following text in Clause 5.2 in TS 38.104: subband full duplex can be applied to TDD bands with channel bandwidth >= X MHz, given in Table 5.2-1.

    - Proposal 1 (Samsung): Text proposal added Clause 5.2: “Subband full duplex can be applied to all FR1 TDD bands given in Table 5.2-1, except bands n34, n51, n53, n54 and n101, and all FR2 bands given in table 5.2-2.”

    - Proposal 2 (Samsung): Table 5.3.5-1 (BS channel bandwidths and SCS per operating band in FR1), introduce the note “Note 9: Subband full duplex can be applied to all supported channel bandwidth values except the ones smaller than 20MHz.”

    - Proposal 3 (Ericsson): To include the following text in Clause 5.2 in TS 38.104: subband full duplex can be applied to the following TDD bands given in Table 5.2-1: n38, n40, n41, n46, n48, n50, n77, n78, n79, n90, n96, n102, n104 for FR1.

  - Option 2 (Huawei): Add SBFD entry in Table 5.2-1 in Clause 5.2 in TS 38.104 for the TDD bands with channel bandwidth >= X MHz

    - Option 2a (CATT): Use adding SBFD entry to the table to introduce the SBFD bands

- Moderator Recommendation:

  - Further discussion on the above options.

##### Issue 1-1-4: Transmission bandwidth configuration for SBFD

- [Moderator] The following agreement is provided in last meeting:

Agreement: - BS channel bandwidth for SBFD-capable BS: - BS channel bandwidth for SBFD-capable BS: RF bandwidth supporting a single NR RF carrier with the transmission bandwidth configured in the uplink or downlink or SBFD operation. Transmission bandwidth configuration for SBFD Agreement: - BS transmission bandwidth configuration for SBFD: - BS transmission bandwidth configuration for SBFD NRB,SBFD: set of resource blocks located within the BS channel bandwidth which may be used for transmitting or/and receiving by the BS. - The transmission bandwidth configuration for SBFD NRB,SBFD for each BS channel bandwidth and subcarrier spacing shall reuse the values for BS transmission bandwidth configuration in table 5.3.2-1 for FR1 and table 5.3.2-2 for FR2-1. - FFS a corresponding figure for SBFD is needed

- Follow-up proposals for the corresponding figure and other spec. implementation issue:

  - Proposal 1 (Samsung): The following figure is introduced for the illustrations of BS channel bandwidth, BS transmission bandwidth configuration for SBFD NRB,SBFD, transmission bandwidth configuration for SBFD DL subband NRB,SBFD,DL, transmission bandwidth configuration for SBFD UL subband NRB,SBFD,UL, and guardband between SBFD UL/DL subbands NRB,SBFD,GB for SBFD operation:

Figure 1: Definition of channel bandwidth and transmission bandwidth configurations for one NR channel for SBFD operation

  - Proposal 2 (Nokia): Illustrative figure is provided:

  - Proposal 3 (Ericsson): Introduce Transmission bandwidth configuration for SBFD DUD, DU and UD patterns, as shown in Figure 1-3 below.Figure 1: Transmission bandwidth configuration for SBFD DUD pattern for one NR channelFigure 2: Transmission bandwidth configuration for SBFD DU pattern for one NR channelFigure 3: Transmission bandwidth configuration for SBFD UD pattern for one NR channel

  - propose to add the clarification figure for the SBFD BS PRB utilization in the specification

- Moderator Recommendation:

  - Further discussion on the above illustrative figures.

##### Issue 1-1-5: DL/UL subband configurations support for SBFD

- [Moderator] In RAN4#112, progress has been achieved in clarifying the definition of transmission bandwidth configuration for SBFD DL subband NRB,SBFD,DL, transmission bandwidth configuration for SBFD UL subband NRB,SBFD,UL.

Agreement: - Transmission bandwidth configuration for SBFD UL/DL subband: - NRB,SBFD,DL: Transmission bandwidth configuration for SBFD DL subband, expressed in resource blocks. - NRB,SBFD,UL: Transmission bandwidth configuration for SBFD UL subband, expressed in resource blocks. - Guardband between SBFD UL and DL subband: - NRB,SBFD,GB: The size of guardband between SBFD UL subband and DL subband, expressed in resource blocks.

Based upon previous RAN4 discussion, it is “within RAN4 scope to study/specify the limitation or restriction on the size of subband/guardband”. In RAN4#112, the following three options with different levels of configuration flexibilities have been provided:

Agreement: - To specify DL/UL subband configuration in BS RF requirements, the following 3 options for DL/UL subband configuration are identified: - Option-1 (with the least flexibility): - Only a number of selected DL/UL subband configurations are specified for RAN4 BS RF requirement. - Example (given in Nokia Tdoc R4-2413238): - For FR1 DUD configuration in 100 MHz channel and 30 kHz SCS: < NRB,SBFD,DL, NRB,SBFD,UL, NRB,SBFD,GB > = <106, 51, 5> - For FR1 DU or UD configuration in 100 MHz channel and 30 kHz SCS: < NRB,SBFD,DL, NRB,SBFD,UL, NRB,SBFD,GB > = <217, 51, 5> - Option-2 (with the middle flexibility): - Option-2a: - DL subband configuration shall be selected from the existing values of BS transmission bandwidth configuration, as provided in Table 5.3.2-1 for FR1 and Table 5.3.2-2 for FR2-1. - UL subband size and guardband size(s) depends on BS implementation - Example (FR1 DU or UD in 100MHz and 30kHz SCS, 80-20MHz case): As long as BS transmission configuration for SBFD shall be 273 (NRB,SBFD for 100MHz and 30kHz SCS) and NRB,SBFD,DL is selected from Table 5.3.2-1 (according to the 30kHz SCS), from BS RF requirement perspective, no limitation on the values NRB,SBFD,UL and NRB,SBFD,GB. - Option-2b: - UL subband configuration shall be selected from the existing values of BS transmission bandwidth configuration, as provided in Table 5.3.2-1 for FR1 and Table 5.3.2-2 for FR2-1. - DL subband size(s) and guardband size(s) depends on BS implementation - Example (FR1 DU or UD in 100MHz and 30kHz SCS, 80-20MHz case): As long as BS transmission configuration for SBFD shall be 273 (NRB,SBFD for 100MHz and 30kHz SCS) and the value of NRB,SBFD,UL is selected from Table 5.3.2-1 (according to the 30kHz SCS), from BS RF requirement perspective, no limitation on the values NRB,SBFD,DL and NRB,SBFD,GB. - Option-3 (with the highest flexibility): - No restriction on the subband sizes, i.e., RF requirement shall be specified to enable all DL and UL subband configurations (and implicitly guardband configurations). - Example (FR1 DU or UD in 100MHz and 30kHz SCS): From BS RF requirement perspective, no limitation on the values of NRB,SBFD,DL, NRB,SBFD,UL, NRB,SBFD,GB - FFS how to implement BS RF requirements based on the above options for different levels of flexibility.

- Clarification on declaration and flexibility:

  - Proposal 1 (Ericsson): The SBFD capable BS product shall remain fixed in its configuration during real operation, i.e., it shall not be reconfigurable to a different configuration/DL/UL/guardband setting other than those that have been declared and tested.

- Companies’ preference on 3 identified options:

  - Option-1 (with the least flexibility): Nokia, Ericsson

    - Proposal 1 (Nokia): For FR1, RAN4 to prioritize SBFD requirements for BS carrier bandwidth of 100MHz first and 50 MHz secondly.

    - Proposal 2 (Nokia): For FR1 SBFD requirements, from base station perspective, use the following configuration:

      - For FR1 DUD configuration in 100 MHz channel and 30 kHz SCS: < ND, NU, NG > = <106, 51, 5>

      - For FR1 DU or UD configuration in 100 MHz channel and 30 kHz SCS: < ND, NU, NG > = <217, 51, 5>

    - Proposal 3 (Nokia): For FR2-1 SBFD requirements, from base station perspective, the following assumptions for guardbands and UL and DL subbands can be used as starting point

      - For FR2-1 DUD configuration in 100 MHz channel and 120 kHz SCS: < ND, NU, NG > = <25, 14, 1>

      - For FR2-1 DUD configuration in 200 MHz channel and 120 kHz SCS: < ND, NU, NG > = <52, 26, 1>

    - Proposal 4 (Ericsson): For one supported channel bandwidth, only one fixed UL subband size is specified, which is around 20% of the supported channel bandwidth.

    - Proposal 5 (Ericsson): Only symmetric DUD configurations are specified, as there is no foreseeable benefit from flexible placement of the UL subband.

    - Proposal 6 (Ericsson): Reuse the existing channel bandwidths for SBFD UL subbands.

    - Proposal 7 (Ericsson): SBFD-capable BS channel bandwidths are down selected to 50 MHz, 60 MHz, 70 MHz, 80 MHz, 90 MHz and 100 MHz for FR1.

    - Proposal 8 (Ericsson): For FR1, use 30 kHz as the SCS for SBFD slots.

    - Proposal 9 (Ericsson): For WA BSs, wider guardbands than existing ones are needed for SBFD.

    - Proposal 10 (Ericsson): RAN4 define SBFD subband configurations for WA BS as shown in Table 2 for FR1.

Table 2: Transmission bandwidth configuration NRB for SBFD for FR1

|  |  | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | D | 20 | 22.5 | 27.5 | 30 | 35 | 40 |
|  | U | 10 | 15 | 15 | 20 | 20 | 20 |
|  | D | 20 | 22.5 | 27.5 | 30 | 35 | 40 |
| DUD (SCS 30 kHz) | NRB,SBFD,DL | 49/50 | 5857 | 70/71 | 78 | 92 | 106 |
|  | NRB,SBFD,UL | 24 | 38 | 38 | 51 | 51 | 51 |
|  | NRB,SBFD,GB | 5 | 45 | 5 | 5 | 5 | 5 |
|  | D | 40 | 45 | 55 | 60 | 70 | 80 |
|  | U | 10 | 15 | 15 | 20 | 20 | 20 |
| DU/UD (SCS 30 kHz) | NRB,SBFD,DL | 104 | 119 | 146 | 161 | 189 | 217 |
|  | NRB,SBFD,UL | 24 | 38 | 38 | 51 | 51 | 51 |
|  | NRB,SBFD,GB | 5 | 5 | 5 | 5 | 5 | 5 |

  - Option-2 (with the middle flexibility): CMCC, Samsung, CATT, Qualcomm, vivo, Huawei, ZTE

    - Option-2b: Samsung, CATT, Qualcomm, vivo, Huawei, ZTE

      - Observation 1 (Huawei): The size of guard band for SBFD is suitable to be manufacturer declaration based as it can vary depending on implementation for different BS type or frequency band, thus having limitation on the minimum/maximum value of the size of guard band would be a great loss on implementation flexibility.

      - Proposal 1 (ZTE): All UL sub-band configuration (e..g.reusing from the existing carrier bandwidth, 20~90MHz) should be specified for SBFD receiver requirements together with FRC and power levels. The detailed MCS, reference signal pattern for licensed band could align with WF R4-1801031. For unlicensed bands, the interleaved PRB pattern should be also configured.

  - Option-3 (with the highest flexibility): Tejas

- Moderator Recommendation:

  - Discuss on Ericsson’s proposal for “Clarification on declaration and flexibility” firstly.

  - Still need more discussion on three options.

##### Issue 1-1-6: Restriction on guardband size for SBFD

- [Moderator] Based on last meeting’s agreement, the following options for restriction on guardband size are identified:

Agreement: - Restriction on guardband size for SBFD: - Option 1: Definition based on existing guardbands requirement in TS38.104 (i.e., minimum guardband) - Option 2: Typical values of guardband defined for requirement, but the guardband should not be restricted to those in RAN4 specifications. - Option 3: No restriction (manufacturer declaration-based), but with restriction on maximum allowed number of RBs for guardband: - Option 3a: No restriction (manufacturer declaration-based), and no restriction on maximum/minimum number of RBs for guandband:

- Companies’ preference:

  - Option 2: Ericsson, Nokia

  - Option 3: CMCC, CATT

    - Proposals for maximum guardband size (CATT):

      - The maximum allowed number of RBs for GB is separated to DL SB GB and UL SB GB.

      - The maximum GB restriction uses the GB size in current specification if it can be found.

      - If SB CBW can’t be found in current specification, 10% GB size is used as the maximum GB restriction.

  - Option 3a: Samsung, vivo, ZTE.

- Moderator Recommendation:

  - Further discussion on the above options, which are dependent on Issue 1-1-4.

##### Issue 1-1-7: Specification implementation issue for BS channel BW for SBFD

- [Moderator] Text proposal is given for specification implementation for BS channel bandwidth for SBFD.

  - Proposal 1 (Samsung): A new 5.3B clause is added to capture the BS channel bandwidth for SBFD (including Transmission bandwidth configuration, guardband and subband configuration for SBFD), with the following text proposal to be considered as baseline skeleton for triggering discussion:

=================== Start of Text Proposal ===================

5.3B	BS channel bandwidth for SBFD

5.3B.1	General

<To add the illustrative definition of channel bandwidth, transmission bandwidth configuration, uplink and downlink subbands and guardband within one NR channel for SBFD operation>

5.3B.2	Transmission bandwidth configuration for SBFD

<To be added>

5.3B.3	Guardband and Subband configuration for SBFD

<To be added>

=================== End of Text Proposal ===================

- Moderator Recommendation:

  - It is welcomed to see other companies’ constructive comments for how the spec shall be organized.

### Sub-topic 1-2: Other general issues for SBFD requirements

##### Issue 1-2-1: BS RF Specification structure for SBFD requirements

- [Moderator] Based on RAN4#112 discussion, the follow way forward is achieved. Accordingly, BS RF specification structure for SBFD requirements shall be further discussed in this meeting.

<From RAN4#112> - Way forward: - RAN4 shall study how to introduce BS RF new requirements for SBFD-capable BS: - Option 1: Embedding the corresponding SBFD RF requirements in the corresponding section of TS 38.104 (similar as NB-IoT RF requirement in TS36.104) - Option 2: Creating new sub-clauses in TS 38.104 for RF requirements that applies for SBFD - Option 2a: details (like new suffix or not) can be FFS when RF requirements are stable. - Option 2b: Set up new sub-clause with suffix-B for modified existing RF requirements and SBFD-dedicated new RF requirements - Option 3: A new specification for SBFD BS RF requirements

- Options of how to introduce BS RF new requirements for SBFD-capable BS:

  - Option 2 (CMCC, Samsung, Qualcomm, Nokia): Creating new sub-clauses in TS 38.104 for RF requirements that applies for SBFD

    - Option 2a (Nokia): details (like new suffix or not) can be FFS when RF requirements are stable.

    - Option 2b (Samsung): Set up new sub-clause with suffix-B for modified existing RF requirements and SBFD-dedicated new RF requirements

    - Option 2c (Huawei): Embed the corresponding SBFD RF requirements in the corresponding section of TS 38.104 as much as possible (e.g. RAN4 already agreed to embed the supporting band of SBFD in clause 5) and consider new clauses or new suffix for SBFD for the RF requirements that are necessary.

  - Option 3 (Qualcomm, Ericsson): A new specification for SBFD BS RF requirements

- Moderator Recommendation:

  - Based on majority view, can we firstly agree on Option 2 or 2a, firstly. For detailed requirement implementation, we can FFS when detailed requirements are stable, as suggestion in Option 2a.

##### Issue 1-2-2: CLI handling impact on RF requirement

- [Moderator] Based on RAN4#112 discussion, the follow way forward is achieved.

<From RAN4#112> - Agreement: - RAN4 shall define RF requirements for the frequency outside of wanted carrier based on no gNB-to-gNB CLI handling as baseline - FFS Revisit particular RF requirements pending on RAN1 progress on the gNB-to-gNB CLI handling work.

- Further proposals are provided:

  - Proposal 1 (Nokia): RAN4 to not revisit RF requirements pending on RAN1 progress on the gNB-to-gNB CLI handling work.

- Moderator Recommendation:

  - Further discussion on the above proposal.

# Topic #2: Potentially new requirements for SBFD

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2415024 | CMCC | Proposal 1: In-channel adjacent sub-band leakage ratio should be defined or reflect by other requirements. Proposal 2: Both sub-band selectivity and blocking requirements should be defined. |
| R4-2415043 | Tejas Network Limited | Observation 1: The transient time should, in general, be the same for both full TDD switching and switching related to SBFD, as the entire TX switching time budget must be the same for both switching the complete UL/DL and the SBFD RBs. In the case of DL-SBFD UL and SBFD-DL, the existing TDD switching time can be used since during this transition the power amplifier (PA) will be always on. Furthermore, since the UE is in half duplex, the transition from non-SBFD DL to SBFD UL will have to consider the PA switching time only. Therefore, no new requirement from the base station [BS] side. Proposal 1: The location of the transient period between SBFD and non-SBFD shall be within the SBFD slots. |
| R4-2415109 | CATT | Proposal 1: Location of transient period between SBFD and non-SBFD shall be located within the SBFD slot. Proposal 2: The following new requirements are not defined for SBFD BS. – In-channel adjacent subband leakage ratio – In-channel adjacent subband selectivity – In-channel adjacent subband blocking |
| R4-2415468 | Qualcomm Germany | Observation 1: The in-channel subband sub-band Tx leakage, selectivity, and blocking depend on the self-interference, inter-site and inter-sector interference considerations, thus, they are implementation dependent. Proposal 1: RAN4 to agree that no new requirements for in-channel leakage, selectivity, and blocking are defined for Rel-19 SBFD-capable gNBs. |
| R4-2415677 | Nokia | Observation 1: Separate sub-clauses are the clearest way to introduce new BS RF requirements for SBFD in core specification TS 38.104. Observation 2: Creating a new separate specification will complex BS specifications and increase efforts to maintenance both core and test specifications, and thus it should be avoided. Observation 3: The OTA sensitivity requirement does not capture the effects from co-site inter-sector and inter-site interference. Observation 4: In-channel adjacent subband leakage ratio, in-channel adjacent subband blocking and in-channel adjacent subband selectivity requirements cannot be guaranteed implicitly by the OTA sensitivity requirement, since the methods used for self-interference cancellation, might not be available for cancelling interference from other sectors and gNBs, especially when considering a multi-vendor deployment. Observation 5: Even though RAN4 has not agreed on a reference implementation for SBFD operation, minimum requirements can still be defined to enable proper operation considering self-interference, co-site inter-sector and inter-site interference. Proposal 1: For introduction of new BS RF requirements for SBFD operation, solution with creating new sub-clauses is proposed. Proposal 2: Details (like new suffix or not) of the new sub-clauses can be left to decision when the RF requirements are stable. Proposal 3: RAN4 to define in-channel adjacent sub-band leakage ratio requirements within SBFD time slots considering co-site inter-sector interference and inter-site interference. Existing ACLR requirements could be used as baseline depending on the ratio between the bandwidths of the DL and the UL subbands. Proposal 4: RAN4 to define in-channel adjacent sub-band selectivity requirement, with the existing ACS requirement would be used as baseline. |
| R4-2416135 | Ericsson | Proposal 1 Place the transient period outside of both SBFD and non-SBFD slots. Proposal 2 Define a requirement on TX sub-band ACLR similar to the ACLR requirement and use existing ACLR requirement as baseline. Proposal 3 Define a requirement on RX sub-band ACS similar to the ACS requirement and use existing ACS requirement as baseline. |
| R4-2416186 | Huawei, HiSilicon | Observation: For in-channel adjacent subband leakage ratio derivation, it is difficult for RAN4 to agree on a reference scheme for self-interference suppression. While the REFSENS for SBFD will reflect such suppression to some extent. Proposal 1: The location of transient period between SBFD and non-SBFD shall be located within the SBFD slot/symbol. Proposal 2: Prefer not to introduce new requirement for in-channel adjacent subband leakage ratio. Proposal 3: Prefer not to introduce new requirement for in-channel adjacent subband selectivity or blocking. |
| R4-2416249 | ZTE Corporation, Sanechips | Proposal 1: for the co-site inter-sector, in-channel blocking, in-channel selectivity and in-channel sub-band leakage, this could be left up to the vendor declaration without defining any specific power or freq offset of the corresponding requirement. Proposal 2: for the inter-site scenario, not to define in-channel blocking, in-channel selectivity and in-channel sub-band leakage and leave this up to the implementation. |
| R4-2416498 | Samsung | Observation 1: The transient period for SBFD operation is different from TDD operation in terms of: - Transmitter OFF-to-ON and ON-to OFF happen in part(s) of frequency resources, and - Transmitter OFF-to-ON and ON-to OFF happen in part(s) of antenna elements. Observation 2: The transient period for SBFD operation could be further summarized as below, by elaborating the four different cases for different SBFD antenna configuration: Frequency regions for transition; SBFD antenna configuration Option-1 (Same panel area as TDD); SBFD antenna configuration Option-2&3 (Twice panel area as TDD); BS transmitter behavior during transition. Case-A (SBFD symbol to DL symbol); SBFD UL subband and guardband(s); SBFD RX antenna panel; N/A; BS transmitter OFF-to-ON. Case-B (SBFD symbol to UL symbol); SBFD DL subband; SBFD TX antenna panel; SBFD TX antenna panel; BS transmitter ON-to-OFF. Case-C (DL symbol to SBFD symbol); SBFD UL subband and guardband(s); SBFD RX antenna panel; N/A; BS transmitter ON-to-OFF. Case-D (UL symbol to SBFD symbol); SBFD DL subband; SBFD TX antenna panel; SBFD TX antenna panel; BS transmitter OFF-to-ON. Proposal 1: The following text proposal is given for the clause of transmitter transient period for SBFD: =================== Start of Text Proposal =================== ### 6.4.2B Transmitter transient period for SBFD #### 6.4.2.1B General Transmitter transient period for SBFD requirements apply only to SBFD operation of the BS. The transmitter transient period for SBFD is the time period during which the transmitter is changing from the transmitter OFF period to the transmitter ON period or vice versa. The transmitter transient period is illustrated in figure 6.4.2.1-1. The transmitter transient period for SBFD shall be measured in the SBFD TX and RX antenna penal and transmission bandwidth configuration for SBFD DL subband, UL subband and guardband, depending on different SBFD antenna configuration: Table 6.4.2.1B-1: Transmitter transient period for SBFD Frequency regions for transition; SBFD antenna configuration Option-1 (Same panel area as TDD); SBFD antenna configuration Option-2&3 (Twice panel area as TDD); BS transmitter behavior during transition. Case-A (SBFD symbol to DL symbol); SBFD UL subband and guardband(s); SBFD RX antenna panel; N/A; BS transmitter OFF-to-ON. Case-B (SBFD symbol to UL symbol); SBFD DL subband; SBFD TX antenna panel; SBFD TX antenna panel; BS transmitter ON-to-OFF. Case-C (DL symbol to SBFD symbol); SBFD UL subband and guardband(s); SBFD RX antenna panel; N/A; BS transmitter ON-to-OFF. Case-D (UL symbol to SBFD symbol); SBFD DL subband; SBFD TX antenna panel; SBFD TX antenna panel; BS transmitter OFF-to-ON. For BS type 1-C this requirement shall be applied at the antenna connector supporting transmission in the operating band. For BS type 1-H this requirement shall be applied at each TAB connector supporting transmission in the operating band. =================== End of Text Proposal =================== Proposal 2: No need to discuss and decide whether the transmitter transient period shall be located within the SBFD symbol or not. Observation 3: It is difficult for RAN4 to agree on a reference scheme for self-interference suppression implemented to derive the potential new requirement in-channel adjacent subband leakage ratio. Observation 4: All the gNB-to-gNB CLI handling schemes likely to be introduced in RAN1 (including beam nulling, beam pairing and non-transparent UL resource muting) will greatly mitigate the gNB-to-gNB CLI, and make it be hard to have a criterion for a “proper” interference level or in-channel adjacent subband leakage ratio. Proposal 3: There is no necessity to introduce new requirement for in-channel adjacent subband leakage ratio. Proposal 4: There is no necessity to introduce new requirement for in-channel adjacent subband blocking and adjacent subband selectivity. |

The moderator can suggest a limited number of papers which could be presented.

## Open issues summary

Before f2f meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 2-1: Requirements for Transient period between SBFD and non-SBFD

#### Issue 2-1-1: Transient period requirement applicability on frequency/antenna

- [Moderator] In RAN4#112, the following agreement is achieved for the requirement for transient period between SBFD and non-SBFD:

- Agreement: - RAN4 shall use existing transient period requirement as baseline for the following four cases identified for SBFD: Frequency regions for Transition; BS transmitter behavior during transition. Case-A (SBFD to DL); SBFD UL subband and guardband(s); BS transmitter OFF-to-ON. Case-B (SBFD to UL); SBFD DL subband; BS transmitter ON-to-OFF. Case-C (DL to SBFD); SBFD UL subband and guardband(s); BS transmitter ON-to-OFF. Case-D (UL to SBFD); SBFD DL subband; BS transmitter OFF-to-ON.

- In this meeting, the following observation/proposal are given to further clarify in which frequency range and antenna panel the transient period is required:

  - Observation 1 (Samsung): The transient period for SBFD operation could be further summarized as below, by elaborating the four different cases for different SBFD antenna configuration:

|  | Frequency regions for transition | SBFD antenna configuration Option-1 (Same panel area as TDD) | SBFD antenna configuration Option-2&3 (Twice panel area as TDD) | BS transmitter behavior during transition |
| --- | --- | --- | --- | --- |
| Case-A (SBFD symbol to DL symbol) | SBFD UL subband and guardband(s) | SBFD RX antenna panel | N/A | BS transmitter OFF-to-ON |
| Case-B (SBFD symbol to UL symbol) | SBFD DL subband | SBFD TX antenna panel | SBFD TX antenna panel | BS transmitter ON-to-OFF |
| Case-C (DL symbol to SBFD symbol) | SBFD UL subband and guardband(s) | SBFD RX antenna panel | N/A | BS transmitter ON-to-OFF |
| Case-D (UL symbol to SBFD symbol) | SBFD DL subband | SBFD TX antenna panel | SBFD TX antenna panel | BS transmitter OFF-to-ON |

  - Proposal 1 (Samsung): The following text proposal is given for the clause of transmitter transient period for SBFD:

=================== Start of Text Proposal ===================

##### 6.4.2B	Transmitter transient period for SBFD

###### 6.4.2.1B	General

Transmitter transient period for SBFD requirements apply only to SBFD operation of the BS.

The transmitter transient period for SBFD is the time period during which the transmitter is changing from the transmitter OFF period to the transmitter ON period or vice versa. The transmitter transient period is illustrated in figure 6.4.2.1-1.

The transmitter transient period for SBFD shall be measured in the SBFD TX and RX antenna penal and transmission bandwidth configuration for SBFD DL subband, UL subband and guardband, depending on different SBFD antenna configuration:

Table 6.4.2.1B-1: Transmitter transient period for SBFD

|  | Frequency regions for transition | SBFD antenna configuration Option-1 (Same panel area as TDD) | SBFD antenna configuration Option-2&3 (Twice panel area as TDD) | BS transmitter behavior during transition |
| --- | --- | --- | --- | --- |
| Case-A (SBFD symbol to DL symbol) | SBFD UL subband and guardband(s) | SBFD RX antenna panel | N/A | BS transmitter OFF-to-ON |
| Case-B (SBFD symbol to UL symbol) | SBFD DL subband | SBFD TX antenna panel | SBFD TX antenna panel | BS transmitter ON-to-OFF |
| Case-C (DL symbol to SBFD symbol) | SBFD UL subband and guardband(s) | SBFD RX antenna panel | N/A | BS transmitter ON-to-OFF |
| Case-D (UL symbol to SBFD symbol) | SBFD DL subband | SBFD TX antenna panel | SBFD TX antenna panel | BS transmitter OFF-to-ON |

For BS type 1-C this requirement shall be applied at the antenna connector supporting transmission in the operating band.

For BS type 1-H this requirement shall be applied at each TAB connector supporting transmission in the operating band.

=================== End of Text Proposal ===================

- Moderator Recommendation:

  - Further discussion on the above proposal.

#### Issue 2-1-2: Location of transient period between SBFD and non-SBFD

- [Moderator] In RAN4#112, the following agreement is achieved for “Location of transient period between SBFD and non-SBFD”:

- Agreement: - Location of transient period between SBFD and non-SBFD in RAN4 RF requirement: - FFS the location of transient period between SBFD and non-SBFD shall be located within the SBFD slot/symbol if it needs to be decided.

- Location of transient period between SBFD and non-SBFD:

  - Option 1 (Samsung): No need to discuss and decide whether the transmitter transient period shall be located within the SBFD symbol or not.

  - Option 2 (Ericsson): Place the transient period outside of both SBFD and non-SBFD slots.

  - Option 3 (Tejas/CATT/Huawei/ZTE): The location of the transient period between SBFD and non-SBFD shall be within the SBFD symbol).

- Moderator Recommendation:

  - Based on last meeting’s agreement, “When specifying RAN4 requirements for SBFD-capable BS, SBFD symbols (instead of SBFD slots/symbols) shall be used to designate the time domain duration in which SBFD related requirements is applied.”, “SBFD symbol” is used for above options, instead of “SBFD slot/symbol”.

  - Although Option 1 and Option 2 are not the same, but both are based on the analysis for TDD transient period which are located in the “guard period”, and from detailed RAN4 requirement for TDD transient period, it is not relevant to the actual location which justifies Option 1.

### Sub-topic 2-2: In-channel adjacent subband leakage ratio requirements

#### Issue 2-2-1: Necessity of in-channel adjacent subband leakage ratio requirements

- [Moderator] The necessity of the requirement for in-channel adjacent subband leakage ratio requirements has not yet been confirmed, based upon previous meeting discussion. In RAN4#112, the following WF is approved:

- Way forward: - FFS the necessity of in-channel adjacent subband leakage ratio requirements - Option 1: No necessity to introduce new requirement for in-channel adjacent subband leakage ratio. - Option 2: New requirement for in-channel adjacent subband leakage ratio should be defined. - Define a requirement on TX sub-band ACLR similar to the ACLR requirement and use existing ACLR requirement as baseline.

- Proposals:

  - Option 1 (Samsung/CATT/Qualcomm/Huawei): no necessity to introduce new requirement for in-channel adjacent subband leakage ratio.

  - Option 2 (Nokia, CMCC, Ericsson): New requirement for in-channel adjacent subband leakage ratio should be defined.

    - Proposal 1 (Nokia, Ericsson): Existing ACLR requirements could be used as baseline depending on the ratio between the bandwidths of the DL and the UL subbands.

  - Option 2a (CMCC): In-channel adjacent sub-band leakage ratio should be defined or reflect by other requirements.

  - Option 3 (ZTE):

    - For the co-site inter-sector scenario: in-channel adjacent subband leakage could be left up to the vendor declaration without defining any specific power or freq offset of the corresponding requirement.

    - For the inter-site scenario: no in-channel adjacent subband leakage requirement and leave up to implementation.

- Moderator Recommendation:

  - The group firstly check Option 3 is a reasonable compromise to move forward or not.

### Sub-topic 2-3: In-channel adjacent subband selectivity/blocking requirements

#### Issue 2-3-1: Necessity of in-channel adjacent subband selectivity requirements

- [Moderator] The necessity of the requirement for in-channel adjacent subband selectivity requirements has not yet been confirmed, based upon previous meeting discussion. In RAN4#112, the following WF is approved:

- Way forward: - FFS the necessity of in-channel adjacent subband selectivity requirements - Option 1: No necessity to introduce new requirement for in-channel adjacent subband selectivity. - Option 2: New requirement for in-channel adjacent subband selectivity should be defined. - Define a requirement on RX sub-band ACS similar to the ACS requirement and use existing ACS requirement as baseline.

- Proposals:

  - Option 1 (Samsung/CATT/Qualcomm/Huawei): no necessity to introduce new requirement for in-channel adjacent subband selectivity.

  - Option 2 (CMCC/Ericsson/Nokia): New requirement for in-channel adjacent subband selectivity should be defined.

    - Define a requirement on RX sub-band ACS similar to the ACS requirement and use existing ACS requirement as baseline.

  - Option 3 (ZTE):

    - For the co-site inter-sector scenario: in-channel adjacent subband selectivity could be left up to the vendor declaration without defining any specific power or freq offset of the corresponding requirement.

    - For the inter-site scenario: no in-channel adjacent subband selectivity requirement and leave up to implementation.

- Moderator Recommendation:

  - The group firstly check Option 3 is a reasonable compromise to move forward or not.

#### Issue 2-3-2: Necessity of in-channel adjacent subband blocking requirement

- [Moderator] The necessity of the requirement for in-channel adjacent subband blocking requirements has not yet been confirmed, based upon previous meeting discussion. In RAN4#112, the following WF is approved:

- Way forward: - FFS the necessity of in-channel adjacent subband blocking requirements - Option 1: No necessity to introduce new requirement for in-channel adjacent subband blocking. - Option 2: New requirement for in-channel adjacent subband blocking should be defined.

- Proposals:

  - Option 1 (Samsung/CATT/Qualcomm/Huawei/Ericsson/Nokia): no necessity to introduce new requirement for in-channel adjacent subband blocking.

    - Proposal 1 (Nokia/Ericsson): defining/testing both blocking and ACS requirements is notnecessary but only one of them would suffice.

  - Option 2 (CMCC): New requirement for in-channel adjacent subband blocking should be defined.

  - Option 3 (ZTE):

    - For the co-site inter-sector scenario: in-channel adjacent subband blocking could be left up to the vendor declaration without defining any specific power or freq offset of the corresponding requirement.

    - For the inter-site scenario: no in-channel adjacent subband blocking requirement and leave up to implementation.

- Moderator Recommendation:

  - The group can agree on Option 1 firstly.