3GPP TSG-RAN WG4 Meeting # 112bis														R4-2416533

Hefei, China, 14th – 18th October 2024

Agenda item:			6.8.5

Source:	Moderator (Nokia)

Title:	Topic summary for [112bis][312] NR_IoT_NTN_less_than_5MHz_BSRF

Document for:	Information

# Introduction

Briefly introduce background, the scope of this email discussion (e.g. list of treated agenda items) and provide some guidelines for email discussion if necessary.

Summary for contributions submitted under agenda items 6.8.3.3 for SAN RF core requirements of Less than 5MHz for NTN.

List of candidate target of discussion for 1st round and 2nd round:

- 1st round: Discussion and agreement on open issues listed below.

- 2nd round: Continue discussion and agreement on open issues listed below.

# Topic #1: Core requirements

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2415090 | CATT | Proposal 1: To use the proposals in Table 2.1 for NTN SAN RF requirements for NR NTN support 3 MHz channel bandwidth. Table 2.1: SAN RF requirements impact by 3 MHz NR NTN SAN RF Tx / Rx requirements; NR TN BS spec affected by 3 MHz; Proposal for NTN 3 MHz. Radiated transmit power; Channel bandwidth agnostic; No specification impact. OTA SAN output power; Channel bandwidth agnostic; No specification impact. OTA output power dynamics; NR TN BS Type 1-O requirements is specified the same as NR TN BS Type 1-H requirements; No specification impact. OTA transmit ON/OFF power; Not applicable for NTN; Not applicable. OTA transmitted signal quality; Channel bandwidth agnostic; No specification impact. OTA unwanted emissions; NR TN BS Type 1-O requirements is specified the same as NR TN BS Type 1-H requirements; No specification impact. OTA transmitter intermodulation; Not applicable for NTN; Not applicable. OTA sensitivity; CBW for OSDD is reused from type 1-H BS; No specification impact. OTA reference sensitivity level; No 3 MHz CBW for TN; Same approach as introducing 3 MHz CBW for NTN conducted requirements. OTA dynamic range; No 3 MHz CBW for TN; Same approach as introducing 3 MHz CBW for NTN conducted requirements. OTA In-band selectivity and blocking; No 3 MHz CBW for TN; Same approach as introducing 3 MHz CBW for NTN conducted requirements. OTA Out-of-band blocking; EISminSENS related CBW is reused from type 1-H BS; No specification impact. OTA receiver spurious emissions; Not applicable for NTN; Not applicable. OTA receiver intermodulation; Not applicable for NTN; Not applicable. OTA In-channel selectivity; No 3 MHz CBW for TN; Same approach as introducing 3 MHz CBW for NTN conducted requirements. Observation 1: |
| R4-2415485 | Nokia | Proposal 1: The radiated Tx/Rx requirement for SAN type 1-H and SAN type 1-O should also be defined for less than 5MHz. Observation 1: AAS type implementation is more common for currently defined NTN bands. |
| R4-2415691 | Samsung | Proposal 1: For 10.3 OTA Reference sensitivity level (SAN 1-O), new requirements need to be specified for 3MHz by reusing TN FRC “G-FR1-A1-7”. Proposal 2: For 10.4 OTA Dynamic range (SAN 1-O), new requirements need to be specified for 3MHz by reusing TN FRC “G-FR1-A1-15”. Proposal 3: For 10.5 ACS (SAN 1-O), new requirements need to be specified for 3MHz Proposal 4: For 10.9 ICS (SAN 1-O), new requirements need to be specified for 3MHz by reusing TN FRC “G-FR1-A1-20”. Observation 1: |
| R4-2415821 | ZTE Corporation, Sanechips | Proposal 1: DraftCR to TS38.108 Introduction of less than 5MHz channel bandwidth for FR1-NTN. Observation 1: |
| R4-2416489 | Ericsson India Private Limited | Proposal 1: Regarding the total power dynamic range requirement, the minimum requirement should be calculated according to the transmission bandwidth configuration: specify 11.7dB in Table 6.3.3.2-1, which is the same value as for 3MHz CBW for TN. Proposal 2: For the ACLR requirement, 3 MHz CBW should be added to Tables 6.6.3.2-1 and 6.6.3.2-2, where the ACLR limit should remain the same and the assumed adjacent channel carrier should be an NR of same BW. Proposal 3: For the ACS requirement, 3 MHz CBW shall be added. The same PREFSENS + 8dB wanted signal mean power as for TN 3 MHz CBW should be specified in Table 7.4.1.2-1. In Table 7.4.1.2-2, the same type of interfering signal and the interfering signal center frequency offset could be reused from 3 MHz BS CBW for TN: ±1.5075 MHz frequency offset for 3 MHz DFT-s-OFDM NR signal with 15 kHz SCS and 15 RBs. Proposal 4: Agree with the tentative analysis on the impact on NR SAN RF radiated Tx/Rx requirements. The requirements to be defined for 3 MHz CBW are the following: • OTA reference sensitivity level • OTA dynamic range • OTA adjacent channel selectivity • OTA in-band blocking • OTA in-channel selectivity |

## Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 1-1

Sub-topic description:

Open issues and candidate options before meeting:

Issue 1-1: Radiated Tx/Rx requirement for SAN

- Proposals

  - Option 1: Radiated Tx/Rx requirement for SAN type 1-H and SAN type 1-O should be defined for less than 5MHz. (CATT, Nokia, Samsung, ZTE, Ericsson)

- Recommended WF

  - Option 1: Radiated Tx/Rx requirement for SAN type 1-H and SAN type 1-O should be defined for less than 5MHz.

### Sub-topic 1-2

Sub-topic description:

Open issues and candidate options before meeting:

Issue 1-2: Reference sensitivity compared to 5MHz channel bandwidth with 15kHz SCS

- Proposals

  - Option 1: -2.2 dB (Samsung).

  - Option 2: -1.9 dB (ZTE, Nokia).

- Recommended WF

  - TBD

### Sub-topic 1-3

Sub-topic description:

Open issues and candidate options before meeting:

Issue 1-3: Draft CR

- Proposals

  - Option 1: Endorse draft CR in R4-2415821.

  - Option 2: Revise draft CR in R4-2415821.

  - Option 3: Note draft CR in R4-2415821.

- Recommended WF

  - TBD

# Recommendations for Tdocs

| Tdoc number | Recommendation | Comments |
| --- | --- | --- |
| R4-2415090 | Noted |  |
| R4-2415485 | Noted |  |
| R4-2415691 | Noted |  |
| R4-2415821 | Noted |  |
| R4-2416489 | Noted |  |

…