3GPP TSG-RAN WG4 Meeting #112-bis	R4-2414913

Hefei, Anhui, China, 14th – 18th October, 2024

3GPP TSG RAN WG2 Meeting 127		TDoc R2-2407601

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Title:	LS on LTM L1 intra and inter-frequency measurements capabilities

Response to:	R2-2406227/ R4-2410303 Reply LS on LTM L1 intra and inter-frequency measurements

Release:	Rel-18

Work Item:	Further NR mobility enhancements (NR_Mob_enh2-Core)

Source:	RAN WG2

To:	RAN WG1, RAN WG4

Contact person:	Sudeep Palat

sudeep.k.palat@intel.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

Subsequent to the previous RAN2 LS (R2-2404014) on LTM L1 intra and inter-frequency measurements from R2-126bis and the RAN4 LS response (R2-2406227/R4-2410303), RAN2-127 discussed further RAN1 and RAN4 feature lists associated with Rel-18 NR_Mob_enh2 and would like to inform RAN1/4 of the following agreements.

- Intra-frequency LTM RAN2 capabilities for MCG LTM and SCG LTM are defined per band and the UE sets each of them consistently across all FDD-FR1 bands, and respectively across all TDD-FR1 bands, across all TDD-FR2-1 bands and across all TDD-FR2-2 bands.

- An additional capability bit (per UE) is introduced to indicate inter-frequency LTM (UE indicating this capability shall also support intra-frequency MCG or SCG LTM i.e., the default indicated by the per band bits).

- No dependency between LTM and L1 measurements is captured in RAN2 specs (TS 38.306, TS 38.331, TS 38.300) for both FR1 and FR2.

- Band Combination (BC) of the Intra and Inter-frequency L1 measurements is the BC of serving cells (Option 1 of RAN4 LS response to Q2).

  - RAN2 is further discussing whether to introduce another capability bit to indicate that UE supports inter-frequency L1 measurements only in the band combinations containing the bands of the serving cells

- RAN2 understands only UEs supporting LTM fast processing (R4 39-6) indicate the components associated with the feature (maxNumberStoredConfigCells-r18 and maxNumberConfigs-r18).

- UE support of R4 39-4, 39-4a and 39-5 can be indicated independently.

Additionally, RAN2 would like to check with RAN1:

Given the RAN2 agreement that RAN2 has defined separate capabilities for intra and inter frequency LTM cell switch (agreement 1 above), RAN2 would like to check with RAN1 if the UE support of RAN1 features 45-3a/4a, 45-5/5a/6 should also be indicated separately for intra and inter-frequency LTM cell switch.

# 2	Actions

To RAN1 and RAN4:

ACTION: 	RAN2 respectfully asks RAN1 and RAN4 to take note of the RAN2 agreements and provide feedback if any.

To RAN1:

ACTION: 	RAN2 respectfully asks RAN1 if UE support of RAN1 features 45-3a/4a, 45-5/5a/6 should also be indicated separately for intra and inter-frequency LTM cell switch considering that the RAN2 has defined separate capabilities for intra and inter-frequency LTM cell switch.

# 3	Dates of next TSG RAN WG2 meetings

TSG RAN WG2 Meeting #127bis	14 - 18 Oct 2024 	Hefei, China

TSG RAN WG2 Meeting #128	18 - 22 Nov 2024 	Orlando, USA