3GPP TSG-RAN WG4 Meeting #112-bis                                   	R4-2417547

# Orlando, USA, 18th Nov 2024 – 22nd Nov 2024

Agenda item:			7.9.3

Source:	Eutelsat Group, European Space Agency, Airbus, Intelsat

Title:	Ku Band Numerology analysis

Document for:	Information

# Introduction

The Ku band WID [1] requires a choice between FR1 or FR2 numerology to be made by RAN#106 (December 2024). During RAN4 #112 in Maastricht it was agreed [2] that the following technical criteria will be used to determine the feasibility of FR1 and FR2 numerologies to be used for Ku band:

- UL timing synchronization

- Phase noise

- Efficient channel utilization

- Beam hopping granularity

- Link budget UL/DL – as a function of SCS and channel bandwidth

This contribution provides further analysis of the above five criteria beyond what was provided at RAN4 #112bis in [3].

The following FR1 and FR2 options will be considered:

Table 1 FR1 and FR2 numerology / SCS combinations under consideration

| Sub-Carier Spacing | Numerology |  |
| --- | --- | --- |
|  | FR1 | FR2 |
| SCS of uplink signals | 15 kHz, 30 kHz, 60 kHz | 60 kHz, 120 kHz |
| SSB SCS | 15 kHz, 30 kHz | 120 kHz, 240 kHz |

# Discussion

## Uplink timing synchronization aspects

In this section the timing requirements specified in [4] are considered.

Section 7.1C requires that:

The UE initial transmission timing error shall be less than or equal to Te_NTN where the timing error limit value Te_NTN.

Te_NTN is specified in Table 7.1C.2-1 for FR1-NTN.

Te_NTN is specified in Table 7.1C.2-2 and Table 7.1C.2-3 for VSAT UE in FR2-NTN.

This requirement applies:

-	when it is the first transmission in a DRX cycle for PUCCH, PUSCH and SRS, or it is the PRACH transmission, or it is the msgA transmission.

Table 7.1C.2-1: Te_NTN Timing Error Limit

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te_NTN |
| --- | --- | --- | --- |
| FR1-NTN | 15 | 15 | 29*64*Tc |
|  |  | 30 | 24*64*Tc |
|  |  | 60 | N/A |
|  | 30 | 15 | 24*64*Tc |
|  |  | 30 | 22*64*Tc |
|  |  | 60 | N/A |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

Table 7.1C.2-2: Te_NTN Timing Error Limit for fixed VSAT is served by GSO and fixed VSAT is served by NGSO

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te_NTN |
| --- | --- | --- | --- |
| FR2-NTN | 120 | 60 | 13*64*Tc |
|  |  | 120 | 7.5*64*Tc |
|  | 240 | 60 | 13*64*Tc |
|  |  | 120 | 7.5*64*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

Table 7.1C.2-3: Te_NTN Timing Error Limit for mobile VSAT is served by GSO

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te_NTN |
| --- | --- | --- | --- |
| FR2-NTN | 120 | 60 | 13*64*Tc |
|  |  | 120 | [10]*64*Tc |
|  | 240 | 60 | 13*64*Tc |
|  |  | 120 | [10]*64*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

Working assumptions for this feasibility analysis

In [5], an analysis of Te_NTN for FR2 is provided. The following observations were made:

Observation 1: For Mobile VSAT served by GSO using 120 kHz SCS (with SSB SCS of 120 kHz or 240 kHz), the value for Te_NTN contains figures in square brackets. Observation 2: For Mobile VSAT served by GSO using 120 kHz SCS, the value of [10] for Te_NTN causes the guard period Tg to extend 27.8 % beyond the half CP limit which is not acceptable for reliable system operation. Observation 3: The guard period is defined as worst case assuming all the errors are uncorrelated and add up linearly. There is an opportunity to consider a more pragmatic analysis should it be necessary. Observation 4: From R4-2317275 [8], when setting the requirements for fixed VSAT served by GSO and NGSO, the UE and SAN positioning errors were assumed to be X (UE) = 30 m and Y (SAN) = 15 m. Observation 5: For fixed VSAT served by GSO and NGSO at 120 kHz SCS, the defined value of 7.5 for Te_NTN implies a maximum one-way path length uncertainty of 31.6 m which is 18.47 m less than the assumption used in R4-2317275 [8] of 45 m. Observation 6: For fixed VSAT served by GSO and NGSO at 120 kHz SCS, operating with the assumed (X + Y) one-way positioning error of 45 m results in a guard period Tg of -2.77. This represents a 30.8 % incursion beyond the half CP limit and predicts loss of system performance. Observation 7: For mobile VSAT served by GSO and NGSO at 120 kHz SCS, the defined value of [10] for Te_NTN implies a maximum one-way path length error of 43.7 m which is 1.3 m less than the assumption used in R4-2317275 [8] of 45 m for the easier SAN + Fixed VSAT case. Observation 8: For mobile VSAT served by GSO and NGSO at 120 kHz SCS, operating with a derived (X + Y) one-way positioning error of 43.7 m results in a guard period Tg of -2.5. This represents a 27.8 % incursion beyond the half CP limit and predicts loss of system performance. Observation 9: The conclusion from observations 5 and 6 is that for Fixed VSAT the current specification of Te_NTN = 7.5 is inconsistent with the assumptions made for the worst-case SAN + UE positioning error of 45 m. Operating with such an error would result in the guard period exceeding the 0.5 CP limit by 30.8 %. Observation 10: The conclusion from observations 1, 7 and 8 is that for Mobile VSAT served by GSO, the current specification of Te_NTN = [10] is inconsistent with the assumptions made for the worst-case SAN + UE positioning error of 45 m used for the easier Fixed VSAT case and despite this, would result in the guard period exceeding the 0.5 CP limit by 27.8 %. Proposal 1: To resolve the current issues with Table 7.1C.2-3 the following changes are proposed: Table 7.1C.2-3: Te_NTN Timing Error Limit for mobile VSAT is served by GSO Frequency Range; SCS of SSB signals (kHz); SCS of uplink signals (kHz); Te_NTN. FR2-NTN; 120; 60; 13*64*Tc. 120; 7.5*64*Tc. 240; 60; 13*64*Tc. 120; 7.5*64*Tc. Note 1: Tc is the basic timing unit defined in TS 38.211 [6]. Also correct Table 7.3C.2.2-2. Table as presented does not define the accuracy as double sided as in Table 7.3.2.2-1 for TN. This should be corrected. Proposal 2: To resolve the current issues with Table 7.3C.2.2-2 the following changes are proposed: Table 7.3C.2.2-2: UE Timing Advance adjustment accuracy for VSAT UE in FR2-NTN UL Sub Carrier Spacing(kHz); 60; 120. UE Timing Advance adjustment accuracy; ±128 Tc; ±32 Tc. NOTE 1: VSAT UE are defined in TS 38.101-5 [43].

Given that there are open issues on UL timing for FR2-NTN it is not possible to properly evaluate the feasibility of FR2 numerology for Ku band until those issues are resolved. These issues are being address as a Ka band maintenance issue in [5] at this meeting. To make progress on the numerology study, it will be assumed here that the proposals to correct 38.133 above are accepted. This would mean that for Mobile VSAT served by GSO, Te_NTN for 120 kHz SCS will be aligned with the 7.5 figure used for Fixed VSAT served by GSO and NGSO. This assumption does not mean that Mobile VSAT positioning is easier than Fixed VSAT positioning but does assume that the maximum allowable value of Te_NTN should ensure that the guard period Tg does not go below zero. This also implies that the maximum allowable value for Te_NTN, is defined by the numerology, and not by assumptions made about the positioning accuracy of specific device types.

Observation 1: It is the working assumption of this paper that the maximum value specified for Te_NTN for any numerology, should be bounded by the requirement that the guard period Tg should not become negative as this would predict inter-symbol interference (ISI) and unstable performance. The current value of Te_NTN in 38.133 for Mobile VSAT to GSO does not meet this expectation.

Observation 2: It is the working assumption of this paper that to have a stable basis for analyzing the feasibility of FR2 numerology, that the proposals to define Te_NTN =7.5 in [5] for Mobile VSAT to GSO does is approved.

Observation 3: It is the assumption of this paper that if the open issues with UL timing for Mobile VSAT to GSO are not resolved then it is not possible to state that FR2 numerology is feasible for  Mobile VSAT served by GSO.

Using the above as a starting point for this feasibility analysis we can investigate the feasibility of all the numerologies listed in Table 1.

Current error model for defining Te_NTN

From [5] we have:

Tg =  0.5*Tcp – (Td + Tp + Tr + Ta + Tf + Tm)

Where:

Tg = The guard period

TCP = The cyclic prefix length

Td = Downlink synchronization error = (1/(2*SSB BW))/2

TP = Round trip positioning error of the UE and SAN (Tp,ue + Tp,san)

Tr = Timing advance command resolution error

Ta = Timing advance accuracy

Tf = Frequency drift between SSB

Tm = Any other factors

Of the six error terms that are used to calculate the system guard period, Ta is specified separately for the UE in Table 7.3C.2.2-2 of [4] and Tr is a resolution error Affecting the SAN receiver and is not a function of the UE. The remaining four error terms are then used in evaluating Te_NTN as:

Te_NTN = Td + Tp + Tf + Tm

Tg =  0.5*Tcp – (Te_NTN + Tr + Ta)

Tp =  0.5*Tcp – (Tg + Td + Tr + Ta + Tf + Tm)

By setting Tg to zero we can evaluate the maximum allowable path length error:

Max_Tp =  0.5*Tcp – (Td + Tr + Ta + Tf + Tm)

All T values are in units of κ = 64 * Tc where Tc = 1/(480 kHz * 4096) = 0.5086 ns.

Positioning margin analysis based on the  current error model

Using the current error model described above it is possible to analyze the guard period Tg and the maximum allowable positioning error Tp for the current specifications. It is noted that in cases where Tg > 0 it is possible to define a larger Tp. Included in the analysis is the assumptions made for the one-way positioning error of the UE (X) and the one -way positioning error of the SAN (Y).

Table 2. Analysis of current Te_NTN

| 38.133 Tables 7.1C.2-1/2/3 |  |  |  | Te_NTN |  |  |  | Other terms |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency Range | SCS of SSB kHz | SCS of UL (kHz) | Te_NTN | Td | TP | Tf | Tm | Tr | Ta | Tg |
| FR1-NTN | 15 | 15 | 29 | 4.27 | 24.24 | 0.49 | 0.00 | 8.00 | 4.00 | 31.00 |
|  |  | 30 | 24 | 4.27 | 19.24 | 0.49 | 0.00 | 4.00 | 4.00 | 4.00 |
|  |  | 60 | N/A* |  |  |  |  |  |  |  |
|  | 30 | 15 | 24 | 2.13 | 21.38 | 0.49 | 0.00 | 8.00 | 4.00 | 36.00 |
|  |  | 30 | 22 | 2.13 | 19.38 | 0.49 | 0.00 | 4.00 | 4.00 | 6.00 |
|  |  | 60 | N/A* |  |  |  |  |  |  |  |
| FR2-NTN (Fixed VSAT) | 120 | 60 | 13 | 0.53 | 11.98 | 0.49 | 0.00 | 2.00 | 2.00 | 1.00 |
|  |  | 120 | 7.5 | 0.53 | 6.48 | 0.49 | 0.00 | 1.00 | 0.50 | 0.00 |
|  | 240 | 60 | 13 | 0.27 | 12.24 | 0.49 | 0.00 | 2.00 | 2.00 | 1.00 |
|  |  | 120 | 7.5 | 0.27 | 6.74 | 0.49 | 0.00 | 1.00 | 0.50 | 0.00 |
| FR2-NTN (Mobile VSAT to GSO) | 120 | 60 | 13 | 0.53 | 11.98 | 0.49 | 0.00 | 2.00 | 2.00 | 1.00 |
|  |  | 120 | [10] | 0.53 | 8.98 | 0.49 | 0.00 | 1.00 | 0.50 | -2.50 |
|  | 240 | 60 | 13 | 0.27 | 12.24 | 0.49 | 0.00 | 2.00 | 2.00 | 1.00 |
|  |  | 120 | [10] | 0.27 | 9.24 | 0.49 | 0.00 | 1.00 | 0.50 | -2.50 |

*The main reason why no specifications exist for FR1-NTN at 60 kHz UL is that FR1 antennas are omnidirectional, and the expected delay spread due to terrestrial clutter would significantly impact Tm. See [6].

Table 3. Analysis of assumptions and system potential

| 38.133 Tables 7.1C.2-1/2/3 |  |  |  | As defined |  |  |  |  | Potential with Tg = 0 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency Range | SCS of SSB kHz | SCS of UL (kHz) | Te_NTN | TP | Tg | 0.5*CP Mar gin % | X+Y from Tp m | X+Y Assu med m | Tg | Max (X+Y) m | Max_Tp | Max_ Te_NTN |
| FR1-NTN | 15 | 15 | 29 | 24.24 | 31.00 | 43.06 | 118.29 | 50 + 30 | 0 | 269.55 | 55.24 | 60.00 |
|  |  | 30 | 24 | 19.24 | 4.00 | 11.11 | 93.89 | 50 + 30 | 0 | 113.41 | 23.24 | 28.00 |
|  |  | 60 | N/A* |  |  |  |  |  |  |  |  |  |
|  | 30 | 15 | 24 | 21.38 | 36.00 | 50.00 | 104.30 | 50 + 30 | 0 | 279.96 | 57.38 | 60.00 |
|  |  | 30 | 22 | 19.38 | 6.00 | 16.67 | 94.54 | 50 + 30 | 0 | 123.82 | 25.38 | 28.00 |
|  |  | 60 | N/A* |  |  |  |  |  |  |  |  |  |
| FR2-NTN (Fixed VSAT) | 120 | 60 | 13 | 11.98 | 1.00 | 5.56 | 58.43 | 45.00 | 0 | 63.31 | 12.98 | 14.00 |
|  |  | 120 | 7.5 | 6.48 | 0.00 | 0.00 | 31.60 | 45.00 | 0 | 31.60 | 6.48 | 7.50 |
|  | 240 | 60 | 13 | 12.24 | 1.00 | 5.56 | 59.73 | 45.00 | 0 | 64.61 | 13.24 | 14.00 |
|  |  | 120 | 7.5 | 6.74 | 0.00 | 0.00 | 32.90 | 45.00 | 0 | 32.90 | 6.74 | 7.50 |
| FR2-NTN (Mobile VSAT to GSO) | 120 | 60 | 13 | 11.98 | 1.00 | 5.56 | 58.43 | 45.00 | 0 | 63.31 | 12.98 | 14.00 |
|  |  | 120 | [10] | 8.98 | -2.50 | -27.78 | 43.79 | 45.00 | 0 | 31.60 | 6.48 | 7.50 |
|  | 240 | 60 | 13 | 12.24 | 1.00 | 5.56 | 59.73 | 45.00 | 0 | 64.61 | 13.24 | 14.00 |
|  |  | 120 | [10] | 9.24 | -2.50 | -27.78 | 45.09 | 45.00 | 0 | 32.90 | 6.74 | 7.50 |

It can be seen from Table 3 that some numerologies allow for a large uncertainty in (X+Y) while others are very restrictive. Specifically, For FR1-NTN at 15 kHz SSB SCS and 30 kHz UL SCS, there is an 11.11 % margin to 0.5 CP, with an allowed (X+Y) of 93.89 m for an assumed 80 m error. The max allowed (X+Y) could be increased to 113.41 m using a Te_NTN of 28. This indicates FR1 numerology is very stable from a timing perspective.

Observation 4: For VSAT using FR1-NTN numerology the system can operate with a positive guard period assuming a combine a combined UE and SAN positioning error of 93.89 m which could be extended to 113.41 m by setting Tg to zero.

For FR2-NTN, the figures are much tighter and, in some cases, drop below an acceptable margin for reliable operation. For Fixed VSAT, the figures for UL 60 kHz provide a 5.56 % margin with 58.43 m X + Y allowance. This could be increased to 63.31 m using a Te_NTN of 14. This calculated allowance for X + Y is larger than the 45 m assumption, although that was optimistic as will be discussed later.

Observation 5: For Fixed VSAT using FR2-NTN numerology with 60 kHz UL SCS, the system can operate with a positive guard period assuming a combined UE and SAN positioning error of 58.43 m which could be extended to 63.31 m by setting Tg to zero.

For Fixed VSAT UL 120 kHz the 7.5 value for Te_NTN has been set to the maximum possible to sure the guard period Tg remains positive. This will ensure reliable operation, however, the calculated value for X + Y is 31.6 m, which are well below the optimistic assumption of 45 m. This indicates that for Fixed VSAT, FR2-NTN numerology, only very accurately positioned UE and SAN can guarantee to work reliably.

Observation 6: For Fixed VSAT using FR2-NTN numerology with 120 kHz UL SCS, the system can operate with a positive guard period assuming a combined UE and SAN positioning error of 31.6 m. This is significantly smaller than the 45 m X + Y assumption and would result in unreliable operation for larger combined positioning errors.

For mobile VSAT to GSO, the situation is worse, since the current specification for Te_NTN of [10] is predicting a negative Tg, which extends 27.78 % beyond the half CP limit. The maximum allowed X + Y to ensure a positive Tg is the same as for the Fixed VSAT case i.e. 31.6 m, but it was already expected that the 45 m assumption for Fixed VSAT would degrade for Mobile VSAT, so the probability of Mobile VSAT to GSO operating reliably is less than for the Fixed VSAT case.

Observation 7: For Mobile VSAT using FR2-NTN numerology with 120 kHz UL SCS, the system can operate with a positive guard period assuming a combined UE and SAN positioning error of 31.6 m. This is significantly smaller than the 45 m X + Y assumption used for the Fixed VSAT case, and it was expected Mobile VSAT + SAN would have a worse positioning accuracy than for the Fixed VSAT case. This implies Mobile VSAT to GSO operation would be less reliable than Fixed VSAT to GSO.

Re-evaluating the  error model

From Table 2, it is eident that the main component of Te_NTN for the problematic case of FR2-NTN at 120 kHz UL: SCS is TP which represents 6.48 out of a budget of 7.5. TP is given as:

TP = Tp,ue + Tp,san

Where Tp,ue is the round-trip path length uncertainty (in units of κ) of the US (VSAT) and Tp,san is the round-trip path length uncertainty for the SAN.

The round-trip Path length uncertainty is double the one-way path length uncertainty, If X is the UE position uncertainty and Y is the SAN position uncertainty then:

TP = Tp,ue + Tp,san = 2 * (X + Y) * c

Where c is the speed of light = 2.9979 * 108 m/s.

The error model that calculates Tg is based on a linear addition of all the error terms:

Tg =  0.5*Tcp – (Td + Tp + Tr + Ta + Tf + Tm)

TP is further assumed to be the linear addition of Tp,ue and Tp,san. This is a worst-case assumption that is open for further analysis.

The assumptions for calculating Tg are based on TN geometry with additions for the SAN positing uncertainty. The original TN model for calculating UL timing requirements is shown in Figure 1.

Max UE positioning uncertainty

Static

Figure 1. Positioning uncertainty in a TN

The base station is static and the UEs in the cell are of unknown position and distributed randomly to the cell edge. The time delay between the edge of the cell and the centre has been used to set the guard period for initial access (RACH, PRACH) such that an UL transmission with zero timing advance, arrives within a limited window. After this, the base station signals a timing advance for the UE such that subsequent transmissions on other channels are synchronized between all UEs in the cell, eliminating possible inter symbol interference (ISI) between users and minimizing wasted gaps in scheduling.

If we consider a TN where the base station was not static (e.g. a HAPS), then the base station positioning uncertainty would add linearly to the positioning uncertainty of the UEs. This is the error model that has been assumed for NTN analysis. This is illustrated in Figure 2.

Max system positioning uncertainty

Base station position uncertainty

Figure 2. Positioning uncertainty in a TN with non-static base station

In this case, the overall positioning uncertainty would be the linear sum of the UE and base station positioning uncertainties.

If we then take Figure 2 and apply NTN assumptions and geometry, we get Figure 3.

N_TA common

Tp,san

Tp,ue

Figure 3. Positioning uncertainty in an NTN

The first thing that is evident is that for NTN, all the VSAT are located on one side of the SAN. Secondly, the perceived distance of the SAN from the ground will be almost identical for all VSATs. The worst-case correlate error seen by the SAN due to its position error as seen by two UEs on the ground is when the one UE located at the nadir (directly below the SAN) and the other is at the edge of the largest cell, and the SAN orbital height is at a minimum.

600 km

UE2

UE1

55 km

602.52 km

SAN

For  LEO at 600 km height with 55 km cell diameter, we get (assuming a flat earth :-0) :

Figure 4. Worst case geometry for correlated Tp,san

If we assume Tp,san is spherical, it can be easily seen that thwe worst correlation will occur with a lateral positioning error further than UE1 since an orbital height error is more correlated.

For the ideal case with zero SAN positioning error:

UE1 path length = 600,000 m

UE2 path length = sqrt(600,0002 + 55,0002) = 602,515.56 m

The differential path length is 602,515.56 - 600,000 = 2,515.56 m

If we assume  50 m lateral positioning error away from UE1 then the new path lengths are:

UE1 path length = sqrt(600,0002 + 502) = 600,000.0021 m

UE2 path length = sqrt(600,0002 + 55,0502) = 602,520.1262 m

The new differential path length is 602,520.1262 - 600,000.0021 = 2,520.1241

The change in path length is 2,520.1241 – 2,515.56 = 4.56 m

Therfore the correlated error is 4.56 / 50 = 9.13 %. This means that the worst-case correlated error of the SAN is only 9.13 % of the absolute uncertanity..

Observation 8: For the worst-case geometry of NGSO 600 km orbital height with a 55 km diameter cell, only 9.13 % of the SAN positioning uncertainty is correlated between one UE at the cell edge and the nadir with another UE at the opposite cell edge.

The equivalent analysis for a GSO with cell diameter of 110 km and orbital height of 35,786 km is is as follows:

UE1 path length = 35,786,000 m

UE2 path length = sqrt(35,786,0002 + 110,0002) = 35,786,169.06 m

The differential path length is 35,786,042.265 - 35,786,000 = 169.06 m

If we assume  50 m lateral positioning error away from UE1 then the new path lengths are:

UE1 path length = sqrt(35,786,0002 + 502) = 35,786,000.000035 m

UE2 path length = sqrt(35,786,0002 + 110,0502) = 35,786,169.214 m

The new differential path length is 35,786,169.214 - 35,786,000.000035 = 169.214 m

The change in path length is 169.214 – 169.06 = 0154 m

Therfore the correlated error is 0.154 / 50 = 0.31 %. This means that the worst-case correlated error of Tp,san is only 0.31 % of Tp,san.

Observation 9: For the worst-case geometry of GSO 35,786 km orbital height with a 110 km diameter cell, only 0.31 % of the SAN positioning uncertainty is uncorrelated between one UE at the cell edge and the nadir with another UE at the opposite cell edge.

Since it can be shows that most or all fo the Tp,san positioning uncertainty is correlated between UEs in the same cell on the ground, it is also the case that the correlated part of the path length uncertianty will not result in a worsening of ISI at the SAN, which is the primary goal of controlling UL timing.  For the system to operate effectively with this assumption, it will be necessary for the SAN to offset its receive window by an amount equal to its correlated positioning uncertianty. This additional correlated timing shift does not need to be signalled to the UE.

Observation 10: The correlated timing error of the UE at the SAN due to SAN positioning error does not cause degraded ISI and can be mitigated with a shift in SAN timing without need to signal the UE.

In additon to removing the correlated part of Tp,san from the error model there are also opportunites to further optimize other elements of the model such as random vs. systemtic errors, however, the bulk of the opportunity is achieved with removing most of Tp,san. Further analsis can be done if necessary later.

Positioning margin analysis based on the  new error model

Using the observaitons from the previous section it is possible to re-state the allowable path length errors as follows:

For NGSO: TP = 2*(X + 0.0913 Y)

For GSO: TP = 2*(X + 0.0031 Y)

This modification in the erro model does not increase the overall positiong margin for the numerology but it does mean that the available amrgin can be met using (X + 0.0913 Y) for NGSO and (X + 0.0031 Y) for GSO. The available positioning margin is thefore almost fully determined by the UE path length uncertianty which increases the range of VSAT device types that can operate reliably for any given numerology.

Analysis of achievable VSAT and SAN positioning uncertainties

The assumption made during the development of the FR1-NTN UL timing requirements appear to be X = 50 m for the UE [7] and Y = 30 m (explicit reference not found) for the SAN. Using these figures there was still a large operating margin so further debate on the positioning figures was not pursued.

For the development of the FR2-NTN UL timing requiremnts, there was condierably less operating margin and so there was substatial debate on what figures to use for X and Y. It can be seen from Table 4 which summarizes company views in [9].

Table 4. Error margin analysis in units of κ = 64 * Tc

| Option | X (UE) | Y (SAN) | X + Y | Implied X+Y | Te_NTN | Company |
| --- | --- | --- | --- | --- | --- | --- |
| Case 1 Fixed to GSO |  |  |  |  |  |  |
| 1-4 | 5 | 10 |  | 15 |  | Qualcomm |
| 1-2 |  |  | 15 |  |  | ZTE |
| 1-6 | 15 | 5 |  | 20 |  | Samsung |
| 1-5 |  |  | 30 |  |  | CATT |
| 1-9 | 15 | 15 |  | 30 | 9.64*Ts | Apple |
| 1-3 |  |  |  | 39 | 8*Ts | Nokia |
| 1-1 | 15 | 30 |  | 45 |  | Xiaomi, ZTE |
| 1-8 |  |  |  | 58.5 | 12*Ts | Ericsson |
| 1-7 |  |  | 60 |  |  | Vivo |
| Case 2 Fixed to LEO |  |  |  |  |  |  |
| 2-4 | 5 | >10 |  | >15 |  | Qualcomm |
| 2-6 | 15 | 15 |  | 30 |  | Samsung |
| 2-2, 2-5 |  |  | 30 |  |  | Huawei, CATT |
| 2-3 |  |  |  | 39 | 8*Ts | Nokia |
| 2-1 | 15 | 30 |  | 45 |  | Xiaomi, ZTE, Apple |
| 1-9 | 15 | 30 |  | 45 | 12.72*Ts | Apple |
| 2-8 |  |  |  | 58.5 | 12*Ts | Ericsson* |
| 2-7 |  |  | 60 |  |  | Vivo |
| Case 3 Mobile to GSO |  |  |  |  |  |  |
| 3-4 | >5 | >10 |  | >15 |  | Qualcomm |
| 3-5 |  |  | 30 | 30 |  | CATT |
| 3-1 | 15 | 30 |  | 45 |  | Xiaomi, ZTE |
| 3-3 |  |  |  | 39 | 8*Ts | Nokia |
| 3-2 |  |  | 45 | 45 |  | Huawei |
| 3-6 | 30 | 5 |  | 35 |  | Samsung |
| 3-8 |  |  |  | 58.5 | 12*Ts | Ericsson |
| 3-9 | 30 | 15 |  | 45 | 12.72*Ts | Apple |
| 3-7 |  |  | 60 | 60 |  | Vivo |

* Ericsson’s proposal for 12*Ts was later modifeid to 7.5 Ts in [10]

In reality, GSO positioning errors can be up to 300 m radial (altitude) and 3 km track (east west). This is mainly because GSO do not hae good sight of the GNSS and rely on side lobes. Howver, NGSO positionng uncertainty is much closer to the values in Table 4.

Observation 11: Practical GSO positining uncertainty can be up to 300 m altitude and 3 km lateral. This fa exceeds 3GPP assumptions. The main reason is the GSO can only se the side lobes of th GNSS far below which are pointing at the earth.

For the UE, values ranged from 5 m to 30 m and for the SAN from 15 m to 60 m. The figure assumed for X + Y when developing the UL timing requiremnets in [8] appears to have been 45 m although it was not explicit prior to discussion moving to Te_NTN values.

The subject of positioning accuracy for NTN is not within 3GPP scope. Figure 5 shows some of the complexity of the systems invovled.

Figure 5. Timing estimation in an NTN

For the SAN, [11] gives examples of high-end positioning accuracy. This indicates that the best military grade SAN using multi orbit, multi freuqency L5 (wideband codeword) systems can achive around 25 m absolute acuracy. This is very close to the 30 m figure assemed in 3GPP. It should be expected that communciataions grade SAN will not necessarily have state of the art positioning systmes deployed, and so it should be expected that commercial SAN positioning accuracy will not necessarily achieve 25 m. That said, the argument is made in this paper that Tp,san is largely correlated across UEs, so its value is not critical to reliable system operation.

For the UE, [12] provides information on performance standards for GPS. It is clear from this that the 3GPP assumption of 15 m, although achievable, is by no means typical of GPS positioning accuracy. There are many factors that are involved in degerming a VSAT’s position. Table 5 shows how the factors of positioning technology, modem propagator quality and antenna size combine to influence achievable positioning accuracy for different device applications.

Table 5. VSAT device types and parameters impacting positioning accuracy

| Positioning accuracy | Modem quality (propagator accuracy) | Small antenna (-3 dB SNR) | Large antenna (12 dB SNR) |
| --- | --- | --- | --- |
| Low accuracy (L1 GNSS) | Low accuracy | Not a likely device type | Enterprise broadband (fixed VSAT) |
|  | High accuracy | ? | Aircraft |
| High accuracy (L5 multi-orbit, multi-frequency) | Low accuracy | Automotive | ? |
|  | High accuracy | Automotive | Maritime |

It should not be the goal of 3GPP to debate what NTN positioning accuracy is achievable but rather to specify the positioning tolerances that particular numerologies require for reliable operation. How combinations of VSAT device types and SAN meet those limits can be deferred to commercial decisions.

Summary of FR1 and FR2 numerology feasibility for Ku band

A summary of the feasibility of different numerologies is provided in Table 6.

Table 6. Feasibility of different numerologies

| Frequency Range / VSAT type | SAN type | SCS of SSB kHz | SCS of UL (kHz) | Feasible? | Side condition (assume 2 sigma, 95 %) | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| FR1-NTN Fixed and mobile VSAT | NGSO | 15 | 15 | Yes | X + Y < 269.55 m |  |
|  |  |  | 30 | Yes | X + Y < 113.41 m |  |
|  |  | 30 | 15 | Yes | X + Y < 279.96 m |  |
|  |  |  | 30 | Yes | X + Y < 123.82 m |  |
|  | GSO | 15 | 15 | Yes | X + 0.0031* Y < 269.55 m | Assumes the new error model |
|  |  |  | 30 | Yes | X + 0.0031* Y < 113.41 m |  |
|  |  | 30 | 15 | Yes | X + 0.0031 * Y < 279.96 m |  |
|  |  |  | 30 | Yes | X + 0.0031 * Y < 123.82 m |  |
|  | GSO |  |  | No | X + Y < 269.55 m | Assumes the worst-case error model GSO positioning uncertainty is still 3,000 m |
|  |  |  |  | No | X + Y < 113.41 m |  |
|  |  |  |  | No | X + Y < 279.96 m |  |
|  |  |  |  | No | X + Y < 123.82 m |  |
| Assuming current error model (X + Y) |  |  |  |  |  |  |
| FR2-NTN Fixed VSAT | NGSO | 120 | 60 | Marginal | X + Y < 63.31 m |  |
|  |  |  | 120 | Unlikely | X + Y < 31.6 m |  |
|  |  | 240 | 60 | Marginal | X + Y < 64.61 m |  |
|  |  |  | 120 | Unlikely | X + Y < 32.9 m |  |
|  | GSO | 120 | 60 | No | X + Y < 63.31 m | GSO Lateral positioning uncertainty is 3,000 m |
|  |  |  | 120 | No | X + Y < 31.6 m |  |
|  |  | 240 | 60 | No | X + Y < 64.61 m |  |
|  |  |  | 120 | No | X + Y < 32.9 m |  |
| FR2-NTN Mobile VSAT | GSO | 120 | 60 | No |  | Assumes Ku does not adopt change proposed in 38.133 to replace Te_NTN = [10] with 7.5 |
|  |  |  | 120 | No |  |  |
|  |  | 240 | 60 | No |  |  |
|  |  |  | 120 | No |  |  |
|  | GSO | 120 | 60 | No | X + Y < 63.31 m | Assumes Ku adopts change to current Ka 38.133 to replace Te_NTN = [10] with 7.5 GSO positioning uncertainty is still 3,000 m |
|  |  |  | 120 | No | X + Y < 31.6 m |  |
|  |  | 240 | 60 | No | X + Y < 64.61 m |  |
|  |  |  | 120 | No | X + Y < 32.9 m |  |
| Assuming new error model NGSO (X + 0.0913 * Y), GSO (X + 0.0031 * Y) |  |  |  |  |  |  |
| FR2-NTN Fixed VSAT | GSO and NGSO | 120 | 60 | Yes | X + 0.09 * Y < 63.31 m |  |
|  |  |  | 120 | Yes | X + 0.09 * Y < 31.6 m | Assumes wide sky view (down to 30 deg elevation) or manual map-based installation |
|  |  | 240 | 60 | Yes | X + 0.09 * Y < 64.61 m |  |
|  |  |  | 120 | Yes | X + 0.09 * Y < 32.9 m | Assumes wide sky view (down to 30 deg elevation) or manual map-based installation |
| FR2-NTN Mobile VSAT | GSO and NGSO | 120 | 60 | Yes | X + 0.0031 * Y < 63.31 m |  |
|  |  |  | 120 | Platform dependent | X + 0.0031 * Y < 31.6 m | Assumes Ku adopts change to current Ka 38.133 to replace Te_NTN = [10] with 7.5 |
|  |  | 240 | 60 | Yes | X + 0.0031 * Y < 64.61 m |  |
|  |  |  | 120 | Platform dependent | X + 0.0031 * Y < 32.9 m | Assumes Ku adopts change to current Ka 38.133 to replace Te_NTN = [10] with 7.5 |

For FR1 numerology:

Observation 12: For Fixed and Mobile VSAT using FR1-NTN numerology served by NGSO the system is feasible and can operate with a positive guard period assuming a combined UE and SAN one-way path length uncertainty of 93.89 m which could be extended to 113.41 m by setting Tg = 0.

Observation 13: For Fixed and Mobile VSAT using FR1-NTN numerology served by GSO and the worst-case error model, reliable system operation is not feasible due to the 3,000 m SAN lateral positioning error.

Observation 14: For Fixed and Mobile VSAT using FR1-NTN numerology served by GSO and the alternative error model, the system is feasible and can operate with a positive guard period assuming a combined UE and 0.0031 * SAN one-way path length uncertainty of 93.89 m which could be extended to 113.41 m by setting Tg = 0.

For FR2 numerology Using the current worst-case error model:

Observation 15: For Fixed and Mobile VSAT using FR2-NTN numerology served by NGSO with 60 kHz UL SCS the system is marginally feasible dependent on platform implementation with a positive guard period assuming a combined UE and SAN one-way path length uncertainty of 63.31 m and Te_NTN set to the maximum of 14.

Observation 16: For Fixed and Mobile VSAT using FR2-NTN numerology served by NGSO with 120 kHz UL SCS the system is unlikely to be feasible with a positive guard period assuming a combined UE and SAN one-way path length uncertainty of 31.6 m and Te_NTN set to the maximum of 7.5.

Observation 17: For Fixed and Mobile VSAT using FR2-NTN numerology served by GSO the system is not feasible with a positive guard period since the SAN positioning uncertainty is 3,000 m.

For FR2 numerology Using the alternative error model:

Observation 18: For Fixed VSAT using FR2-NTN numerology served by NGSO and GSO with 60 kHz UL SCS the system is feasible and can operate with a positive guard period assuming a combined UE and SAN one-way path length uncertainty of 63.31 m and Te_NTN set to the maximum of 14.

Observation 19: For Fixed VSAT using FR2-NTN numerology served by NGSO and GSO with 120 kHz UL SCS the system is feasible and can operate with a positive guard period assuming a combined UE and SAN one-way path length uncertainty of 31.6 m and Te_NTN set to the maximum of 14. This feasibility is platform dependent and assumes either wide sky view (down to 30 deg elevation) or manual map-based installation.

Observation 20: For Mobile VSAT using FR2-NTN numerology served by GSO with 60 kHz UL SCS the system is feasible and can operate with a positive guard period assuming a combined UE and SAN one-way path length uncertainty of 63.31 m and Te_NTN is set to the maximum of 14.

Observation 21: For Mobile VSAT using FR2-NTN numerology served by GSO with 120 kHz UL SCS the system is not generally feasible but very dependent on the platform in order to operate with a positive guard period assuming a combined UE and SAN one-way path length uncertainty of 31.6 m and Te_NTN is set to the maximum of 7.5 as proposed in [5] for Ka band.

## Phase noise [and residual Doppler]

Phase noise needs to be considered as a contributor to the overall performance. Specifications for 5G NR introduced enhancements to mitigate the effects of phase noise, especially in FR2 (24 GHz and above). Higher SCS and Phase Tracking Reference Signals (PT-RS) can be used to mitigate the effects of phase noise and are especially useful at higher frequencies where high order modulation is used. PT-RS are specified for FR1 and FR2 whereas the higher SCS, above 60 kHz, are only available in FR2. PT-RS increase the overhead so should only be used when a net benefit will result. The specifications include the provision for UE to indicate their preference for PT-RS to the network – UE with better (cleaner) oscillators can request the network uses sparser PT-RS than used for UE with less good phase noise characteristics.

The graphs below, from [13] illustrate the potential benefits of PT-RS.

Figure 1: Simulation results, at 30 GHz using 60 kHz SCS and 16-QAM code rate 3/4 and freq. density 2, Figure 5 in [13]

Note that the higher edge of the Ku band Downlink frequency is less than half the RF carrier frequency (i.e., Ku band DL 12.7 GHz maximum vs. 30 GHz) and modelled 60 kHz SCS is double compared with FR1 30 kHz SCS; these factors will tend to cancel each other out and we may, therefore, expect results to be comparable.

The extent to which PT-RS might be an advantage in the NTN Ku band is uncertain since NTN operate under very different conditions to TN FR2 deployments (that is, typically, short range, high SNR, narrow beams supporting high order modulation schemes).

In contrast with FR2 TN, it is unlikely that NTN in Ku band will operate at very high SNR. It was agreed in [14] that for the purposes of analyzing phase noise, the maximum SNR should be 12 dB and the maximum modulation depth 16 QAM.

Furthermore, for the purposes of analysis, we could assume the uplink (earth to space) is likely to be limited to around 2 bits/s/Hz due to practical constraints of transmitter power and antenna size. Terminals employing smaller antenna apertures, such as 20 x 20 cm which has been proposed for cars, will operate at significantly lower SNR (likely including negative SNR values).

Observation 22: NTN Ku band VSATs are expected to typically operate at around 12 SNR or below with maximum 16 QAM modulation. Higher SNR are possible only with larger antennas, smaller antennas will operate at negative SNR.

Residual Doppler shift (i.e., an uplink frequency shift due to imperfectly corrected Doppler) will have a detrimental impact for multiple user operation it will also cause uplink orthogonality to be impaired or, in the limit, completely lost. For multi-user uplink different shifts cannot be compensated at the base station. Unlike TN, Doppler spread is likely to be minimal (very little multipath when directional VSAT antennas are used) typically two or more users will have different residual Doppler shift. The effect of any given amount of Doppler shift will be inversely proportional to SCS (e.g., a system using 15 kHz SCS will be more vulnerable to this effect than 120 kHz SCS).

Observation 23: Current proprietary commercial NTN systems based on LTE technology are operating successfully using 15 kHz UL SCS. This indicates that phase noise has not been a determining factor. The availability of PT-RS in NTN shows that additional performance gains are possible at 12 dB SNR which offer the potential to relax phase noise specifications of existing proprietary systems. Without any additional link level analysis, it is possible to conclude from current systems that FR1 numerology is feasible for Ku band. Fr2 numerology would have further benefits for relaxing phase noise requirements.

## Efficient channel utilization

The ability to efficiently utilize 250 MHz (downlink) and 125 MHz (uplink) transponders is critically important. Support of legacy transponder bandwidths (such as 36 MHz) and wider bandwidths (such as 500 MHz) is also important to some Ku band operators. #

For the uplink we assume DFT-S-OFDM will be used to maximise power efficiency and note that multi-carrier operation may result in requiring transmitter power back-off.

In populating Table 7, for FR1 we assume 30 kHz SCS, resulting in a maximum bandwidth of 100 MHz (nominal) and for FR2 we assume 60 kHz or 120 kHz SCS are supported.

As Carrier Aggregation (CA) is not yet specified for any NTN band (and is outside the scope of the present Ku band WID) we only consider the ability to fully utilize the transponder bandwidths with one or more carriers.

Table 7: FR1/FR2 efficient transponder utilization examples.

| #### Transponder Bandwidth | #### FR1 Options | #### FR2 Options | #### Comments |
| --- | --- | --- | --- |
| #### 125 MHz (Uplink) | #### 100 MHz + [20 MHz or 25 MHz] | #### 100 MHz | #### Assuming FR1 30 kHz SCS. |
| #### 250 MHz (Downlink or Uplink) | #### 100 MHz x 2 + 50 MHz | #### 200 MHz + 50 MHz |  |
| #### 36 MHz (Bi-directional) | #### 35 MHz | #### ? | #### Example legacy GEO DVB transponder bandwidth. #### FR2 may use part of a 50 MHz FR2 carrier. |
| #### 500 MHz (Downlink) | #### 100 MHz x 5 | #### 400 MHz + 100 MHz |  |

For transponder bandwidths above 50 MHz selecting 15 kHz SCS would limit the maximum bandwidth per RF carrier to 50 MHz (nominal) and require a larger number of RF carriers to fully utilize the available bandwidth.

Once the FR1/FR2 down selection is made, RAN4 can then consider which CA combinations are of most practical use to operators.

Observation 24: FR1 numerology provides more flexibility in efficiently utilizing transponder bandwidths at the expense of requiring slightly higher numbers of RF carriers per transponder for wider bandwidths.

Observation 25: If FR1 numerology is selected, 30 kHz SCS is preferred over 15 kHz SCS for transponder bandwidths above 50 MHz.

[Need some criteria for feasibility]

## Downlink beam hopping granularity

In this section we discuss the relative downlink (satellite to VSAT) coverage with FR1 or FR2 [15] numerology under the assumption of Rel-18, that is, without considering extended SSB periodicity under consideration in RAN1 for Rel-19.

Satellite resources are limited, especially power and, for higher frequencies, the number of RF analogue beamformers and thus, onboard resources need to be used efficiently. It is not practical to serve many beam footprints simultaneously to cover all, or most of, the satellite coverage footprint. Downlink Beam Hopping (BH) enables the sharing of data transmission capacity (power) between different geographical areas ("beam footprints") in which UEs with active traffic are located. From the UE perspective, the downlink is not always present, but a maximum revisit time (when the beam footprint is illuminated) is preserved.

For the purpose of this simplified evaluation, we consider the coverage that can be provided from a single analogue beamformer (i.e., number of beam footprints that can be serviced per beamformer) assuming equal sharing of the capacity across the considered beam footprints.

Beam Hopping (BH) cycle represents the maximal scheduling latency expected for an active UE. Two options - 5ms and 10ms - are considered here, 5ms being required for low latency services. The BH cycle can be seen as the maximum "revisit time" for a given beam footprint.

Table 8 shows how many ‘swaps’ can be performed during a BH cycle, depending on the number of OFDM slots available during the BH cycle (assuming one hop per slot), which gives a maximum average capacity that can be served for a given beam footprint.

Table 8: Example Beam Hopping configuration for 5 ms and 10 ms revisit periodicity.

| BH cycle (ms) | UL SCS (kHz) | Numerology | Slot Duration (ms) | Beam footprints serviced per BH Cycle [Note 1] |
| --- | --- | --- | --- | --- |
| 5 | 15 | FR1 | 1 | 5 |
| 5 | 30 | FR1 | 0.5 | 10 |
| 5 | 60 | FR1, FR2 | 0.25 | 20 |
| 5 | 120 | FR2 | 0.125 | 40 |
| 10 | 15 | FR1 | 1 | 10 |
| 10 | 30 | FR1 | 0.5 | 20 |
| 10 | 60 | FR1, FR2 | 0.25 | 40 |
| 10 | 120 | FR2 | 0.125 | 80 |

Note 1: In practice one slot per cycle can be allocated to non-active cells. This "loss" of slot is ignored in the above table.

For Ku band, under the assumption that analogue SAN beamforming is employed, the total number of beam footprints that can be served will be given by the number of beam footprints per cycle x number of beamformers. If digital beamforming and wider bandwidths are employed, the above granularity constraints do not apply.

Observation 26: For digital beamforming at the SAN, beam hopping granularity constraints do not apply. For analogue SAN beamforming, both FR1 and FR2 numerologies support the use of BH. Higher SCS provides more scheduling flexibility/ finer granularity than lower SCS. There is a trade-off between maximum scheduling latency and number of ’beam footprints’ that can be served from a single beamformer. FR2 provides the finest granularity but FR1 is still feasible.

## Link budget

There should be no significant difference for bandwidths greater than the smallest number of RB for the highest SCS all else being equal (e.g., bandwidth allocated, modulation and coding scheme, maximum transmit power). That is, FR1 numerology only has advantages for uplink RB allocations of a few RB per UE; for example, two 15 kHz SCS RB can achieve a 3 dB higher power density (dBm/MHz) compared with allocating one RB using 60 kHz SCS. Although potentially a useful advantage for FR1 in ‘edge cases’ with low SNR, for typical VSAT applications the allocation of very small bandwidths is not considered a compelling reason to favour FR1 over FR2.

Observation 27: FR1 holds an advantage for very low uplink allocations; however, for most practical VSAT scenarios the link budgets for FR1 and FR2 numerologies are the same so both FR1 and FR2 should be considered feasible from a link budget perspective.

# Conclusions

The following observations are made:

Observation 1: It is the working assumption of this paper that the maximum value specified for Te_NTN for any numerology, should be bounded by the requirement that the guard period Tg should not become negative as this would predict inter-symbol interference (ISI) and unstable performance. The current value of Te_NTN in 38.133 for Mobile VSAT to GSO does not meet this expectation.

Observation 2: It is the working assumption of this paper that to have a stable basis for analyzing the feasibility of FR2 numerology, that the proposals to define Te_NTN =7.5 in [5] for Mobile VSAT to GSO does is approved.

Observation 3: It is the assumption of this paper that if the open issues with UL timing for Mobile VSAT to GSO are not resolved then it is not possible to state that FR2 numerology is feasible for  Mobile VSAT served by GSO.

Observation 4: For VSAT using FR1-NTN numerology the system can operate with a positive guard period assuming a combine a combined UE and SAN positioning error of 93.89 m which could be extended to 113.41 m by setting Tg to zero.

Observation 5: For Fixed VSAT using FR2-NTN numerology with 60 kHz UL SCS, the system can operate with a positive guard period assuming a combined UE and SAN positioning error of 58.43 m which could be extended to 63.31 m by setting Tg to zero.

Observation 6: For Fixed VSAT using FR2-NTN numerology with 120 kHz UL SCS, the system can operate with a positive guard period assuming a combined UE and SAN positioning error of 31.6 m. This is significantly smaller than the 45 m X + Y assumption and would result in unreliable operation for larger combined positioning errors.

Observation 7: For Mobile VSAT using FR2-NTN numerology with 120 kHz UL SCS, the system can operate with a positive guard period assuming a combined UE and SAN positioning error of 31.6 m. This is significantly smaller than the 45 m X + Y assumption used for the Fixed VSAT case, and it was expected Mobile VSAT + SAN would have a worse positioning accuracy than for the Fixed VSAT case. This implies Mobile VSAT to GSO operation would be less reliable than Fixed VSAT to GSO.

Observation 8: For the worst-case geometry of 600 km orbital height with a 55 km diameter cell, only 9.13 % of the SAN positioning uncertainty is correlated between one UE at the cell edge and the nadir with another UE at the opposite cell edge.

Observation 9: For the worst-case geometry of 35,786 km orbital height with a 110 km diameter cell, only 0.31 % of the SAN positioning uncertainty is uncorrelated between one UE at the cell edge and the nadir with another UE at the opposite cell edge.

Observation 10: The correlated timing error of the UE at the SAN due to SAN positioning error does not cause degraded ISI and can be mitigated with a shift in SAN timing without need to signal the UE.

Observation 11: Practical GSO positining uncertainty can be up to 300 m altitude and 3 km lateral. This fa exceeds 3GPP assumptions. The main reason is the GSO can only se the side lobes of th GNSS far below which are pointing at the earth.

Observation 12: For Fixed and Mobile VSAT using FR1-NTN numerology served by NGSO the system is feasible and can operate with a positive guard period assuming a combined UE and SAN one-way path length uncertainty of 93.89 m which could be extended to 113.41 m by setting Tg = 0.

Observation 13: For Fixed and Mobile VSAT using FR1-NTN numerology served by GSO and the worst-case error model, reliable system operation is not feasible due to the 3,000 m SAN lateral positioning error.

Observation 14: For Fixed and Mobile VSAT using FR1-NTN numerology served by GSO and the alternative error model, the system is feasible and can operate with a positive guard period assuming a combined UE and 0.0031 * SAN one-way path length uncertainty of 93.89 m which could be extended to 113.41 m by setting Tg = 0.

For FR2 numerology Using the current worst-case error model:

Observation 15: For Fixed and Mobile VSAT using FR2-NTN numerology served by NGSO with 60 kHz UL SCS the system is marginally feasible dependent on platform implementation with a positive guard period assuming a combined UE and SAN one-way path length uncertainty of 63.31 m and Te_NTN set to the maximum of 14.

Observation 16: For Fixed and Mobile VSAT using FR2-NTN numerology served by NGSO with 120 kHz UL SCS the system is unlikely to be feasible with a positive guard period assuming a combined UE and SAN one-way path length uncertainty of 31.6 m and Te_NTN set to the maximum of 7.5.

Observation 17: For Fixed and Mobile VSAT using FR2-NTN numerology served by GSO the system is not feasible with a positive guard period since the SAN positioning uncertainty is 3,000 m.

For FR2 numerology Using the alternative error model:

Observation 18: For Fixed VSAT using FR2-NTN numerology served by NGSO and GSO with 60 kHz UL SCS the system is feasible and can operate with a positive guard period assuming a combined UE and SAN one-way path length uncertainty of 63.31 m and Te_NTN set to the maximum of 14.

Observation 19: For Fixed VSAT using FR2-NTN numerology served by NGSO and GSO with 120 kHz UL SCS the system is feasible and can operate with a positive guard period assuming a combined UE and SAN one-way path length uncertainty of 31.6 m and Te_NTN set to the maximum of 14. This feasibility is platform dependent and assumes either wide sky view (down to 30 deg elevation) or manual map-based installation.

Observation 20: For Mobile VSAT using FR2-NTN numerology served by GSO with 60 kHz UL SCS the system is feasible and can operate with a positive guard period assuming a combined UE and SAN one-way path length uncertainty of 63.31 m and Te_NTN is set to the maximum of 14.

Observation 21: For Mobile VSAT using FR2-NTN numerology served by GSO with 120 kHz UL SCS the system is not generally feasible but very dependent on the platform in order to operate with a positive guard period assuming a combined UE and SAN one-way path length uncertainty of 31.6 m and Te_NTN is set to the maximum of 7.5 as proposed in [5] for Ka band.

Observation 22: NTN Ku band VSATs are expected to typically operate at around 12 SNR or below with maximum 16 QAM modulation. Higher SNR are possible only with larger antennas, smaller antennas will operate at negative SNR.

Observation 23: Current proprietary commercial NTN systems based on LTE technology are operating successfully using 15 kHz UL SCS. This indicates that phase noise has not been a determining factor. The availability of PT-RS in NTN shows that additional performance gains are possible at 12 dB SNR which offer the potential to relax phase noise specifications of existing proprietary systems. Without any additional link level analysis, it is possible to conclude from current systems that FR1 numerology is feasible for Ku band. Fr2 numerology would have further benefits for relaxing phase noise requirements.

Observation 24: FR1 numerology provides more flexibility in efficiently utilizing transponder bandwidths at the expense of requiring higher numbers of RF carriers per transponder for wider bandwidths.

Observation 25: If FR1 numerology is selected, 30 kHz SCS is preferred over 15 kHz SCS for transponder bandwidths above 50 MHz.

Observation 26: For digital beamforming at the SAN, beam hopping granularity constraints do not apply. For analogue SAN beamforming, both FR1 and FR2 numerologies support the use of BH. Higher SCS provides more scheduling flexibility/ finer granularity than lower SCS. There is a trade-off between maximum scheduling latency and number of ’beam footprints’ that can be served from a single beamformer. FR2 provides the finest granularity but FR1 is still feasible.

Observation 27: FR1 holds an advantage for very low uplink allocations; however, for most practical VSAT scenarios the link budgets for FR1 and FR2 numerologies are the same so both FR1 and FR2 should be considered feasible from a link budget perspective.

Observation 28: This Tdoc has more observations than any previous Tdoc ;-0

# References

[1] RP-241690, “New WID on Introduction of Ku Band for NR NTN” (Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat) RAN #104 Jun 2024

[2] R4-2413519, Way Forward for [112][314] NR_NTN_Ku_Band_General, Moderator (Eutelsat Group) RAN WG4 #112 Aug 2024

[3] R4-2415284 “Feasibility analysis of FR1 and FR2 Numerology” (Eutelsat Group) RAN WG4 #112bis Oct 2024

[4] TS 38.133 v18.7.0. Requirements for support of radio resource management (Release 18)

[5] R4-2417511 “Required corrections to UE transmit timing for Satellite Access” (Eutelsat Group) RAN WG4 #113 Nov 2024

[6] R1-2118269 “UE positioning and timing requirements” (Ericsson) RAN WG4 101e Nov 2021

[7] R4-2207062 “Email discussion summary for [102-e][221] NR_NTN_solutions_RRM_2” (Moderator (Xiaomi) RAN WG4 #102e Feb 2022

[8] R4-2317275 “Ad-hoc minutes for [222] NR_NTN_enh” (Ad-hoc chair (Samsung) RAN WG4 108bis Oct 2023

[9] R4-2321330 “Ad-hoc minutes for [109][226] NR_NTN_enh” (Ad-hoc chair (Samsung) RAN WG4 #109 Nov 2023

[10] R4-2318821 “RRM requirements in above 10 GHz bands” (Ericsson) RAN WG4 #109 Nov 2023

[11] R1-200886 “Satellite Position Accuracy” (Eutelsat Group RAN WG1 #103e Oct 2023

[12] GLOBAL POSITIONING SYSTEM STANDARD POSITIONING SERVICE PERFORMANCE STANDARD https://www.gps.gov/technical/ps/2020-SPS-performance-standard.pdf

[13] R1-1701161, On DL and UL phase noise tracking RS (PTRS), Ericsson

[14] R4-2416593, Way Forward for [112bis][313] NR_NTN_Ku_Band_General, Moderator (Eutelsat Group) RAN WG4 #112bis Oct 2024

[15] R1-2401470, An operator view on Downlink Coverage Enhancements for FR2-NTN, Eutelsat Group