3GPP TSG-RAN WG4 Meeting #113	R4-2417613

Orlando, US, 18th – 22th November, 2024

Agenda Item:	7.17.4.1

Source:	CAICT

Title:	Discussion on RRM core requirements for beam management

Document for:	Discussion

1	Introduction

This contribution provides discussion on following issue from last meeting.

- Issue 2-5: Requirements for beam prediction

2	Discussion

2.1 Issue 2-5: Requirements for beam prediction

In last RAN4 meeting, we had following agreement on this issue.

Issue 2-5: Requirements for beam prediction Agreement: - Requirements to be defined: - Measurement period for prediction (how long will set B be measured before making a prediction) - FFS on whether the legacy measurement period can be reused or not. - Prediction accuracy - FFS how/what to define depending on the chosen KPI (e.g. predicted RSRP, beam ID, Top-K/1 or Top 1/K, etc) - TCI state known/unknown - FFS on whether the other requirements are needed

Regarding to the first FFS bullet, we think the principle to define the measurement period TL1-RSRP_Measurement_Period_SSB for FR2 in current specification TS 38.133 could be reused, as following.

The detailed value of {M, N, P} in the definition needs further study.

- The value of M depends on the minimum number of required measured instances as model input for inference. The value would be varied for different patterns of Set B.

- The value of N depends on UE Rx beam assumption, and whether Rx beam sweeping is enabled for inference.

- The value of P depends on the periodicity of SSB, SMTC, measurement gap and other related configurations. Current spec could be taken as starting point.

Proposal 1: The principle to define the measurement period TL1-RSRP_Measurement_Period_SSB for FR2 in current specification TS 38.133 could be reused. The detailed values of {M, N, P} needs further study.

Regarding to the last FFS bullet, “whether the other requirements are needed”,

1) with the introduction of AI-based beam prediction, NW is expected to only configure CSI resources for beams in set B. This would cause different periodicities for beams in set B and beams out of set B. Therefore, the performance requirements with L1-RSRP related measurements on UE side may be impacted. For example, the requirements for SSB based beam failure detection, “UE shall be able to evaluate whether the downlink radio link quality on the configured SSB resource in set q0 estimated over the last Tevaluate_BFD_SSB ms period becomes worse than a threshold”. In the definition of Tevaluate_BFD_SSB, whether existing value sets of {N, P} are able to meet certain requirements with different Tssb requires verification.

Proposal 2: RAN4 to identify the impact of reduced resource configuration (only for beams in Set B) on existing RAN4 requirements with L1-RSRP related measurements. Table 1 could be considered as starting list.

Table 1

| RRM Req. Category | RRM procedure |
| --- | --- |
| Signaling characteristics | SSB /CSI-RS based Radio Link Monitoring |
|  | Link recovery procedures (SSB/CSI-RS based beam failure indication, L1 indication, Candidate beam detection) |
|  | Active TCI state switching delay |
|  | Uplink spatial relation switch delay |
|  | Pathloss reference signal switching delay |
|  | Active downlink TCI switching delay for unified TCI |
|  | Active uplink TCI switching delay for unified TCI |
| Measurement procedure | NR intra-frequency measurements (Number of cells and number of SSB, measurement reporting requirements, etc) |
|  | L1-RSRP measurements for reporting |
| Measurement performance requirements – NR measurements | Intra-frequency RSRP accuracy requirements for FR1/FR2 |
|  | RSRP measurement report mapping |
|  | L1-RSRP accuracy requirements for FR1/FR2 |

2) Based on current agreements in RAN1, the beam measurement report would be enhanced compare to legacy specification. Specifically, reporting more than 4 beams in L1 signaling would be supported for BM-case 1, and reporting inference results of multiple time instances would be supported for BM-case 2. Discussion on related requirements shall wait for other WGs to determine detailed reporting mechanism.

Agreement (RAN1 #116) For NW-sided model, for inference, in a beam report initiated by network, based on one measurement resource set, support the report of more than 4 beam related information in L1 signaling - Note: Purpose, such as above “For NW-sided model, for inference”, will not be specified in RAN 1 specifications - FFS on the report content for beam related information - FFS on max number of reported beam related information in one report Agreement (RAN1 #116bis) For UE-side AI/ML model inference, for BM-Case2, support to report inference results of N(N>=1, FFS on N) future time instance(s) in one report - wherein information of inference results of one time instance is as in one report for BM-Case 1 - Note: overhead reduction is not precluded - FFS on details

Proposal 3: Postpone discussion on reporting related requirements until other WGs define detailed reporting mechanism.

3) In RAN4 #111, we had following agreement on RRM requirements for LCM procedures.

Agreement (RAN4 #111): - RAN4 considers defining RRM requirements for LCM procedures based on the specific use cases (beam management and positioning), and candidates are: - Model identification - Selection - Activation - Deactivation - Switching - fallback to non-AI operation - performance monitoring - Others are not precluded

In our understanding, RRM core requirements for LCM procedures is mainly to define delay requirements with specified signaling. Although the signaling aspects of LCM procedures has not been finalized in other WGs, RAN4 can initiate the discussion on delay components of these procedures. For example, model switching delay is analogous to active BWP switch delay, thus the delay components may include, such as

- UE processing time of the signaling with model switching command

- UE processing time of switching model

- UE processing time of transmitting UL acknowledgement, if any

- Signaling of RRC-reconfiguration for the target model, if any

- Whether UE has the associated ID and/or other additional conditions for the target model.

Proposal 4: RAN4 to discuss the delay components of LCM procedures, which may include such as,

- UE processing time of the signaling with model switching command

- UE processing time of switching model

- UE processing time of transmitting UL acknowledgement, if any

- Signaling of RRC-reconfiguration for the target model, if any

- Whether UE has the associated ID and/or other additional conditions for the target model.

Conclusion

In this contribution, RRM core requirements for beam management are discussed with following proposals:

Proposal 1: The principle to define the measurement period TL1-RSRP_Measurement_Period_SSB for FR2 in current specification TS 38.133 could be reused. The detailed values of {M, N, P} needs further study.

Proposal 2: RAN4 to identify the impact of reduced resource configuration (only for beams in Set B) on existing RAN4 requirements with L1-RSRP related measurements. Table 1 could be considered as starting point.

Table 1

| RRM Req. Category | RRM procedure |
| --- | --- |
| Signaling characteristics | SSB /CSI-RS based Radio Link Monitoring |
|  | Link recovery procedures (SSB/CSI-RS based beam failure indication, L1 indication, Candidate beam detection) |
|  | Active TCI state switching delay |
|  | Uplink spatial relation switch delay |
|  | Pathloss reference signal switching delay |
|  | Active downlink TCI switching delay for unified TCI |
|  | Active uplink TCI switching delay for unified TCI |
| Measurement procedure | NR intra-frequency measurements (Number of cells and number of SSB, measurement reporting requirements, etc) |
|  | L1-RSRP measurements for reporting |
| Measurement performance requirements – NR measurements | Intra-frequency RSRP accuracy requirements for FR1/FR2 |
|  | RSRP measurement report mapping |
|  | L1-RSRP accuracy requirements for FR1/FR2 |

Proposal 3: Postpone discussion on reporting related requirements until other WGs define detailed reporting mechanism.

Proposal 4: RAN4 to discuss the delay components of LCM procedures, which may include such as,

- UE processing time of the signaling with model switching command

- UE processing time of switching model

- UE processing time of transmitting UL acknowledgement, if any

- Signaling of RRC-reconfiguration for the target model, if any

- Whether UE has the associated ID and/or other additional conditions for the target model.