3GPP TSG-RAN WG4 Meeting #113	R4-2417633

Orlando, FL, US, Nov. 2024

Source	:	Qualcomm Incorporated

Title	:	On SAR solution for Rel-19 HPUE

Agenda item	:	7.1.1.1.1

Document for	:	Approval

# 1.	Introduction

The remainder of the Rel-19 HPUE discussion on core requirements focus mainly on MSD aspects and a duty cycle based SAR solution. In this contribution we share our view on the latter.

# 2. 	Discussion

In [2], the following was identified as direction for the WI:

## 4.2 Duty cycle solution for SAR compliance Agreement: - For intra-band CA and inter-band CA PC1.5, duty cycle solution for SAR will be discussed. - For EN-DC PC1.5 and PC2, duty cycle solution for SAR will discussed for the scenario without new IE introduction.

For the inter-band case, the exposure criteria are agnostic to whether it is triggered by a CA configuration or a DC configuration. As such, similar solutions must apply to both. If for EN-DC, the inter-band duty cycle solution for SAR ‘will discussed for the scenario without new IE introduction’ [2], such a solution must be pursued for CA also

Proposal 1: Both inter-band CA and EN-DC PC1.5 shall have a similar capability solution, for example both get a new IE, or neither gets a new IE.

From a WI completion perspective, given the general understanding that a duty cycle based scheme is not feasible for implementation at the gNB, the necessity to have a complete solution is not clear. In fact, a related proposal has been made previously [3] and is worth establishing as common understanding. Reproduced from [3]:

Proposal 2: If no consensus on the SAR solution, leave the issue to UE implementation (i.e., P-MPR).

# 3.	References

[1] 	RP-240828, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, RAN#103, March 18th – March 21st , 2024

[2] 	R4-2410749, ‘WF on power domain enhancements’, Huawei, RAN4#111, May 2024

[3] 	R4-2415465, ‘Discussion on the general aspects for HPUE CA’, Huawei, RAN4#112-Bis, Oct. 2024