3GPP RAN WG4 Meeting #113	R4-2417700

Orlando, USA, November 18th – November 22nd, 2024

Title: 	Discussion on general aspects for AIML for NR air

Source: 	CATT

Agenda item:	7.17.1

Document for:	Discussion

Introduction

In RAN4#112bis meeting, RAN4 continued discussing the general aspects of Rel-19 AI/ML for NR air interface and the agreements were captured in [1]. Some open issues summarized in [2] need further discussions and we would like to present our views on some of the open issues in this contribution.

Discussion

Post deployment testing

In last meeting, a concrete summary including issues that are considered for post deployment testing and potential options was captured in way forward [1]. In brief, the following two issues are the targets for post deployment testing:

•	Issue 1: Update or fine tuning or addition/removal of models, if applicable, resulting in a change of functionality which may impact functionality performance.

•	Issue 2: Data drift / mismatch between the conditions encountered by the UE in the field and the training data, which may impact functionality performance.

To resolve issue 1, two options are proposed:

•	Option 1: Conduct the validation of a change in AI functionality before its deployment/activation in already deployed UEs (in the field or in lab conditions).

•	Option 2: Using performance monitoring and LCM procedures.

Issue 1 can be divided into different scenarios according to the types of changes. For the changes referring to update or fine tuning, conducting the validation in lab conditions cannot ensure a consistent performance between field and post deployment testing, since the parameters are updated or fine-tuned are highly scenario-dependent. Even the updated functionality passes the post deployment testing, its performance in the field cannot be guaranteed. If the validation is conducted in the field, from our perspective, it is the same as the LCM procedures, i.e., performance monitoring.

Observation 1: For changes due to update or fine tuning of models,

- Validation in lab conditions cannot ensure a consistent performance between field and post deployment testing, because the updates or fine tuning are scenario-dependent.

- Validation in the field is the same as the performance monitoring procedure.

For the changes referring to addition/removal of models, the discussion needs to be based on RAN2 agreements on the definitions of functionality and procedures. The related RAN2 agreements are reproduced below [3]:

RAN2#127: Agreements on definitions 1 Supported functionalities refer to functionalities that UE can indicate by using UE capability information (via RRC/LPP signalling) 2 Applicable functionalities refers to functionalities that the UE is ready to apply for inference 3 Activated functionalities refers to functionalities already enabled for performing inference Agreements on procedures - “Step 3”: Following configurations are provided from NW to UE: 1) UE is allowed to do UAI reporting via OtherConfig. 2) Network may provide NW-side additional condition. FFS on the RRC signalling and whether it is mandatory or optional. 3) FFS on configuration (e.g. inference configuration) of supported functionalities. FFS on the content of configuration. - UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can considered by UE (e.g. inference configuration). FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3.

As highlighted contents show, after addition/removal of models is completed and before corresponding functionalities can start to perform inference, UE needs to decide the applicability of functionalities. Therefore whether a functionality impacted by addition/removal of models can be activated or not can rely on the applicability checking procedure. At least at current stage, we do not see necessary validations of functionalities impacted by changes after UE is deployed.

Observation 2: For changes due to addition/removal of models, applicability checking procedure of functionality can handle the potential impacts on functionality performance.

For issue2, the impacts mainly come from data drift / mismatch, resulting from inconsistent scenarios between field and the training phase. In our understanding, the purpose of the post deployment testing for this issue is trying to identify the reasons why AI/ML functionality performs badly in the field. Firstly, it is already beyond the purpose of RAN4 performance testing which tests AI/ML capability and its basic performance. Secondly, even such post deployment testing is carried out and the reason is found out, what UE can do is, for example, to switch functionality or fallback to non-AI functions, which is also a part of LCM procedures to us.

Observation 3: For impacts due to data drift/mismatch, the post deployment testing is actually trying to identify the reasons why AI/ML functionality performs badly in the field, which is beyond the scope of RAN4 performance testing and belongs to management of AI/ML functionality.

Proposal 1: Impacts due to changes of models and data drift / mismatch be handled by at least the following procedures:

- Performance monitoring and LCM procedures

- applicability checking procedure of functionality

Potential LCM requirements

Several companies proposed to study various potential requirements for LCM procedures. Considering RAN2/1 is still working on the design of LCM procedures and some basic concepts, such as definition of functionality, it is premature to discuss detailed requirements for LCM procedures. Therefore we prefer to postpone LCM discussions until more RAN2/1 agreements are made.

Proposal 2: Postpone LCM discussions until more RAN2/1 agreements are made.

AI/ML Performance

Regarding AI/ML performance, there were some agreements achieved in last meeting:

Agreement: - For CSI prediction, AI/ML based performance requirement should not be worse than the legacy (non-AI/ML based) if a relevant equivalent legacy requirement/feature exists. - FFS which legacy requirement/feature are relevant - FFS whether AI/ML based performance should be better than the legacy - FFS on target for AI/ML based beam management

Performance of CSI prediction is discussed in our companion contribution [4].

For AI/ML based beam management, the test target depends on the way how the performance requirements are defined. When defining performance requirements and different levels of overhead reduction are taken into account for a given SINR. For example, predicting 64 beams based on measuring 8 beams or 16 beams respectively, leading to 87.5% and 75% overhead reduction, and different performance requirements are defined for both cases. Now assuming that the reduction ratio is 0, which means SetB is the same as SetA, and prediction is also performed. In this case, the performance requirements of AI/ML based BM shall not be worse than the legacy requirements given the same SINR, i.e., no performance degradation is not expected.

Then assuming reduction ratio increases, if lower RSRP results are allowed is the key point in this issue. If the legacy values are reused for all overhead reduction ratios, it means only one set of performance requirements will be defined and it is exactly the same as the legacy requirements. Option 2 is higher RSRP results are defined, but it is not the main purpose of this feature which is to reduce overhead rather than improving performance. Option 3 is lower RSRP results are allowed. Considering different overhead reduction ratios, larger margin values are defined for higher overhead reduction ratios. For instance, ±1 dB margin can be defined for overhead reduction ratio of 50% and ±2 dB margin for 75%.

Observation 4: Test target of AI/ML based BM depends on the way how performance requirements are defined.

Proposal 3: For AI/ML based beam management, two options can be considered for defining performance requirements:

- Option 1: The legacy performance requirements are reused, regardless of overhead reduction ratio.

- Option 2: The legacy performance requirements are used as baseline, and larger margin values are defined for higher overhead reduction ratios.

AI/ML test setup

Some proposals suggested introducing new test set up for performance comparison between AI/ML functions and non- AI functions.

- Proposals - Option 1: If new test set up is introduced for AI test - Define non-AI requirements under new test set up subject to AI. AI requirement has to be higher than that of non-AI. - Option 2: If new test set up is introduced for AI test - Define absolute requirement of AI without considering the comparison with non-AI. - Option 3: Discuss on a case by case basis - there is no need for non AI based requirements if the feature/use case is only enabled with AI

From our perspective, it is too early to discuss whether to introduce new test setups or not, considering the test purpose and performance requirements are not clear enough. Therefor RAN4 should postpone discussion on this issue. Besides, option 1 is not applicable sine it will define new requirements for non-AI functions, which is out of this WI.

Proposal 4: Postpone the discussion on if new test setups need to be introduced.

Number of tests

This issue depends on the definition of functionality and it is still under discussion in RAN1/2. RAN4 should postpone the discussion until the definition of functionality is specified.

Proposal 5: Postpone the discussion on number of test cases until the definition of functionality is specified.

Conclusions

This paper discussed the general issues related to AI/ML for NR air interface, and following observations and proposals are provided:

Observation 1: For changes due to update or fine tuning of models,

- Validation in lab conditions cannot ensure a consistent performance between field and post deployment testing, because the updates or fine tuning are scenario-dependent.

- Validation in the field is the same as the performance monitoring procedure.

Observation 2: For changes due to addition/removal of models, applicability checking procedure of functionality can handle the potential impacts on functionality performance.

Observation 3: For impacts due to data drift/mismatch, the post deployment testing is actually trying to identify the reasons why AI/ML functionality performs badly in the field, which is beyond the scope of RAN4 performance testing and belongs to management of AI/ML functionality.

Proposal 1: Impacts due to changes of models and data drift / mismatch be handled by at least the following procedures:

- Performance monitoring and LCM procedures

- applicability checking procedure of functionality

Proposal 2: Postpone LCM discussions until more RAN2/1 agreements are made.

Observation 4: Test target of AI/ML based BM depends on the way how performance requirements are defined.

Proposal 3: For AI/ML based beam management, two options can be considered for defining performance requirements:

- Option 1: The legacy performance requirements are reused, regardless of overhead reduction ratio.

- Option 2: The legacy performance requirements are used as baseline, and larger margin values are defined for higher overhead reduction ratios.

Proposal 4: Postpone the discussion on if new test setups need to be introduced.

Proposal 5: Postpone the discussion on number of test cases until the definition of functionality is specified.

References

[1] R4-2417212, WF on requirements for AI/ML air interface, Qualcomm Incorporated, RAN4#112bis.

[2] R4-2415231, Topic summary for [112-Bis][127] NR_AI/ML_air, Qualcomm Incorporated, RAN4#112bis.

[3] R2_127_ChairNotes_24-08-23_17-00_eom, RAN2#127.

[4] R4-2417702, Discussion on testability and interoperability issues for CSI prediction, CATT, RAN4#113.