3GPP TSG-RAN WG4 Meeting #113	R4-2417719

# Orlando , US, Nov 18 – 22, 2024

Title: 	Discussion on conditional Intra-CU LTM for Rel-19 mobility enhancements

Source: 	CATT

Agenda item:	7.24.2.3

Document for:	Discussion

- Introduction

A new RAN4 leading work item for Rel-19 mobility enhancements was approved in RAN#102 [1], and the latest revised version was approved in RAN#105[2].

This document will discuss the general RRM issues about conditional Intra-CU LTM, and give our understanding and proposals.

- Discussion

For legacy L3 mobility, conditional handover (CHO) and other conditional mobility procedures (CPAC, SCPAC) were developed to achieve high robustness by enabling the procedure to be executed without necessitating a signalling exchange with source cell beforehand.

However, LTM introduced in Rel-18 offers short interruption time but not with the same level of robustness as the conditional L3 mobility procedures. Therefore, enhancements should be specified so that the system can benefit from both the high robustness and short interruption in Rel-19.

- 2.1 Measurement requirements on conditional LTM

In the last meeting, whether L3 measurement based conditional LTM in FR1 can be supported was discussed in RAN4, and the agreement is copied as follows [3]:

< Agreement > - On whether L3 measurement based conditional LTM in FR1 can be supported - RAN4 can further discuss this issue in FR1 and make recommendation to RAN2 if RAN4 consensus can be reached. - As generic procedure for the objective led by RAN2, if any conclusion will be made in RAN2, RAN4 will follow the RAN2 conclusion unless any technical issue is identified.

In RAN2 #127bis meeting, RAN2 also discussed whether L3 measurement based conditional LTM is supported as R18 LTM, but it was still FFS, the related agreement is copied as follows:

Agreements on C-LTM - It is DU to generate the L1 execution condition. FFS on a case that L3 measurement is used.

Observation 1: RAN2 is also discussing L3 measurement based conditional LTM.

In our view, RAN4 can discuss this issue in parallel with RAN2. Since L1 measurement has been decoupled with LTM cell switch at least in FR1 in R18, so it is better to align with R18 LTM for R19 conditional LTM, as it may be unreasonable for some UEs that support Rel-18 LTM to not support conditional LTM. Therefore, we believe that RAN4 needs to focus on the issues that need to be addressed if L3 measurement based conditional LTM is supported.

Observation 2: It may be unreasonable for some UEs that support Rel-18 LTM to not support conditional LTM.

Proposal 1: RAN4 to discuss this issue in parallel with RAN2.

- It is better to align with R18 LTM for R19 conditional LTM to support L3 measurement based conditional LTM.

- RAN4 needs to focus on the issues that need to be addressed if L3 measurement based conditional LTM is supported.

- 2.2 Whether cover both RACH-based and RACH-less conditional LTM

In the last meeting, whether cover both RACH-based and RACH-less conditional LTM was FFS and the Way Forward is copied as follows [3]:

< Way Forward > FFS the following options: - Option 1: Cover both RACH-based and RACH-less conditional LTM. - Option 2: FFS whether both RACH-less and RACH-based CLTM are supported by RAN1/2.

It was too early to discuss this issue in RAN4’s last meeting, and it is related to the progress from other WGs. In RAN2 #127bis meeting, the relevant agreements reached are as follows:

Agreements on C-LTM - RACH-less Conditional intra-CU LTM is supported. - RACH based conditional intra-CU LTM is supported.

According to the above RAN2’s agreements, both RACH-less and RACH based conditional intra-CU LTM are supported, so RAN4 should follow RAN2’s conclusion, and both RACH-less and RACH based conditional intra-CU LTM should be considered when defining RRM requirements.

Observation 3: RAN2 agreed to support both RACH-less and RACH based conditional intra-CU LTM in the last meeting.

Proposal 2: Both RACH-less and RACH based conditional intra-CU LTM should be considered when defining RRM requirements.

- 2.3 The ending point of conditional LTM delay

In the last meeting, the ending point of conditional LTM delay was still FFS and the Way Forward is copied as follows [3]:

< Way Forward > FFS the following options: - Option 1: - The ending point of conditional LTM delay is the time when the UE transmits the first UL message on the target cell. - FFS the support of RACH-based and/or RACH-less, which is up to RAN1/2 agreement. - It can be revisited if any technical issue will be identified considering the progress in other WGs. - Other option is not precluded.

In addition, the relevant agreement reached in RAN2 #127bis meeting is copied as follows:

Agreements on C-LTM - The LTM completion defined for Rel-18 intra-CU LTM is reused for conditional LTM.

Generally speaking, option 1 is ok for us. Since both RACH-less and RACH based conditional intra-CU LTM are supported in RAN2 according to the last issue, so the ending point of conditional LTM delay can be further clarified. For RACH-based conditional LTM, the ending point is the time when the UE transmits PRACH on the target cell. For RACH-less conditional LTM, the ending point is the time when UE performs the first UL transmission on the indicated beam of the target cell.

Proposal 3: For the ending point of conditional LTM delay, the legacy definition in R18 LTM can be reused.

- For RACH-based conditional LTM, the ending point is the time when the UE transmits PRACH on the target cell.

- For RACH-less conditional LTM, the ending point is the time when UE performs the first UL transmission on the indicated beam of the target cell.

- 2.4 The components of (Subsequent)conditional LTM delay

In the last meeting, the components of conditional LTM delay was still FFS and the Way Forward is copied as follows [3]:

< Way Forward > FFS the following options: - Option 1: DCLTM = TRRC + TEvent_DU + TTCI-state activate + Tmeasure + TCLTM_execution + Tinterrupt. - Option 2: DLTM_Conditional = TEvent_DU + Tmeasure + TLTM_execution + TLTM-interrupt. - Option 3: DCLTM = TRRC + TEvent_DU + Tmeasure + TLTM-interrupt + TCLTM_execution. - Option 4: DCLTM = TLTM_event_DU + TLTM_measure + TLTM-processing + TLTM-IU. - need of TLTM-RRC-processing, Tfirst-RS, TRS-proc and any additional components in the cell switch delay depends on when DL sync and ASN.1 decoding are to happen, which may depend on RAN1/2 CLTM design. - Option 5: At least the delay uncertainty (TEvent_DU), the measurements time (Tmeasure) and the conditional execution preparation time (TCLTM_execution) also need to be considered for Rel-19 conditional LTM Cell Switch delay.

In detail, the legacy L3 CHO delay (DCHO) was defined as follows in TS 38.133 [4]:

DCHO = TRRC + TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution

Compared to L3 HO delay requirements, the CHO delay requirement takes into account the delay uncertainty, the measurements time and the conditional execution preparation time in addition to the RRC procedure delay and the interruption time. Therefore, for Rel-19 conditional LTM, we believe that the legacy DCHO can be a baseline, so the conditional LTM delay can be:

DCLTM = TRRC + TEvent_DU + Tmeasure + TLTM-interrupt + TCLTM_execution.

For the interruption time in conditional LTM Cell Switch delay, the interruption time defined in Rel-18 LTM Cell Switch delay can be a baseline, and RAN4 can further discuss it according to more progress from RAN1/RAN2.

Proposal 4: The legacy requirments for L3 CHO can be a baseline, that is, DCLTM = TRRC + TEvent_DU + Tmeasure + TLTM-interrupt + TCLTM_execution.

Proposal 5: RAN4 need to wait for RAN1/RAN2 more progress and further study the LTM related features based on the above requirments.

- For the interruption time in Rel-19 conditional LTM Cell Switch delay, the interruption time defined in Rel-18 LTM Cell Switch delay can be a baseline.

For subsequent conditional LTM delay, it should be discussed based on conditional LTM delay, we believe that at least TRRC will not be considered in subsequent conditional LTM delay.

Proposal 6: At least TRRC will not be considered in subsequent conditional LTM delay.

- 2.5 Early UL sync

In the last meeting, early UL sync related issues was FFS and the Way Forward is copied as follows [3]:

< Way Forward > FFS the following options: - Option 1: For UL-sync, we can consider some situations as follows, - Do not need to acquire the TA, i.e., the NTA of candidate cell(s) is/are same as serving cell or NTA = 0. - RAR-based TA acquisition. - UE based TA measurement. - Option 2: How/whether to support early RACH on candidate cells during conditional LTM. - Option 3: Whether updates are needed to the Rel-18 PDCCH ordered RACH delay requirements. - Option 4: If DL TODA-based UE autonomous TA estimation is found beneficial for the conditional LTM cell switch, RAN4 to define the requirements for DL TODA-based UE autonomous TA estimation. - Option 5: Wait for more progress from other WGs.

It was too early to discuss this issue in RAN4’s last meeting, and it is related to the progress from other WGs. In RAN2 #127bis meeting, the relevant agreements reached are as follows:

Agreements on C-LTM - UE based TA measurement mechanism is supported for conditional intra-CU LTM. - PDCCH ordered early TA acquisition is supported for conditional LTM.

According to the above RAN2’s agreements, both UE based TA measurement mechanism and PDCCH ordered early TA acquisition are supported for conditional intra-CU LTM, so RAN4 should follow RAN2’s conclusion.

For UL-sync, the situation of UE based TA measurement should be considered, so RAN4 need to define RRM requirements for UE based TA measurement for conditional intra-CU LTM.

For PDCCH ordered early TA acquisition, whether updates are needed compared to the Rel-18 PDCCH ordered RACH delay requirements should be discussed, and RAN4 should wait for more progress from other WGs.

Observation 4: RAN2 agreed to support UE based TA measurement mechanism for conditional intra-CU LTM in the last meeting.

Observation 5: RAN2 agreed to support early TA acquisition for conditional intra-CU LTM in the last meeting.

Proposal 7: RAN4 need to define RRM requirements for UE based TA measurement for conditional intra-CU LTM.

Proposal 8: RAN4 to define PDCCH ordered RACH related requirements, and whether updates are needed compared to the Rel-18 should be discussed.

- RAN4 should wait for more progress from other WGs.

- 2.6 Early TCI state activation

In the last meeting, early TCI state activation related issues was FFS and the Way Forward is copied as follows [3]:

< Way Forward > FFS the following options: - Option 1: How/whether to support early TCI activation on candidate cells during conditional LTM. - Option 2: FFS whether updates are needed to the Rel-18 early TCI state activation delay requirements. - Option 3: R18 LTM early TCI state activation can be as baseline. - Option 4: Wait for more progress from other WGs.

It was too early to discuss this issue in RAN4’s last meeting, and it is related to the progress from other WGs. In RAN2 #127bis meeting, the relevant agreements reached are as follows:

Agreements on C-LTM - Rel-18 Early candidate TCI State activation/deactivation is supported for conditional intra-CU LTM.

According to the above RAN2’s agreements, Rel-18 early candidate TCI state activation/deactivation is supported for conditional intra-CU LTM, so RAN4 should follow RAN2’s conclusion. Early TCI state activation for LTM candidate cell should be considered when defining RRM requirements, and whether updates are needed compared to the Rel-18 early TCI state activation delay requirements should wait for more progress from other WGs.

Observation 6: RAN2 agreed to support candidate TCI state activation/deactivation for conditional intra-CU LTM in the last meeting.

Proposal 9: RAN4 to define early TCI state activation related requirements, and whether updates are needed compared to the Rel-18 should be discussed.

- RAN4 should wait for more progress from other WGs.

- Conclusion

This document discussed preliminary the issues for conditional Intra-CU LTM and presented following proposals and observations:

Observation 1: RAN2 is also discussing L3 measurement based conditional LTM.

Observation 2: It may be unreasonable for some UEs that support Rel-18 LTM to not support conditional LTM.

Proposal 1: RAN4 to discuss this issue in parallel with RAN2.

- It is better to align with R18 LTM for R19 conditional LTM to support L3 measurement based conditional LTM.

- RAN4 needs to focus on the issues that need to be addressed if L3 measurement based conditional LTM is supported.

Observation 3: RAN2 agreed to support both RACH-less and RACH based conditional intra-CU LTM in the last meeting.

Proposal 2: Both RACH-less and RACH based conditional intra-CU LTM should be considered when defining RRM requirements.

Proposal 3: For the ending point of conditional LTM delay, the legacy definition in R18 LTM can be reused.

- For RACH-based conditional LTM, the ending point is the time when the UE transmits PRACH on the target cell.

- For RACH-less conditional LTM, the ending point is the time when UE performs the first UL transmission on the indicated beam of the target cell.

Proposal 4: The legacy requirments for L3 CHO can be a baseline, that is, DCLTM = TRRC + TEvent_DU + Tmeasure + TLTM-interrupt + TCLTM_execution.

Proposal 5: RAN4 need to wait for RAN1/RAN2 more progress and further study the LTM related features based on the above requirments.

- For the interruption time in Rel-19 conditional LTM Cell Switch delay, the interruption time defined in Rel-18 LTM Cell Switch delay can be a baseline.

Proposal 6: At least TRRC will not be considered in subsequent conditional LTM delay.

Observation 4: RAN2 agreed to support UE based TA measurement mechanism for conditional intra-CU LTM in the last meeting.

Observation 5: RAN2 agreed to support early TA acquisition for conditional intra-CU LTM in the last meeting.

Proposal 7: RAN4 need to define RRM requirements for UE based TA measurement for conditional intra-CU LTM.

Proposal 8: RAN4 to define PDCCH ordered RACH related requirements, and whether updates are needed compared to the Rel-18 should be discussed.

- RAN4 should wait for more progress from other WGs.

Observation 6: RAN2 agreed to support candidate TCI state activation/deactivation for conditional intra-CU LTM in the last meeting.

Proposal 9: RAN4 to define early TCI state activation related requirements, and whether updates are needed compared to the Rel-18 should be discussed.

- RAN4 should wait for more progress from other WGs.

- Reference

[1]     RP-234036, New WID: NR mobility enhancements Phase 4, Intel

[2]     RP-242356, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom

[3]     R4-2416863, WF on RRM requirements for NR_Mob_Ph4_Part2, China Telecom

[4]     3GPP TS 38.133, NR Requirements for support of radio resource management