3GPP TSG-RAN WG4 Meeting #113                                                                                             R4-2417813

Orlando, US, Nov. 18 –Nov. 22, 2024

Source: 	CATT

Title: 	Discussion on UE RF requirements for NTN support spectrum less than 5 MHz

Agenda item:	7.8.3.2

Document for:	Approval

- Introduction

In RAN4 #112-bis meeting, a WF [1] on NR NTN support spectrum less than 5 MHz UE RF requirements was agreed. However, there are still some remaining issues need to be discussed in the next meeting.

In this contribution, we would like to share some views on UE RF requirements for NTN support spectrum less than 5 MHz.

- Discussion

The open issues on UE RF requirements for NR NTN support spectrum less than 5 MHz was discussed during the previous meetings, as shown below:

Issue 2-1-3: A-MPR - Way forward - RAN4 to evaluate A-MPR for 3MHz channel BW for NS_02N, NS_03N, NS_04N, NS_05N and NS_24; Issue 2-1-9: General spurious emission - Way forward - FFS Issue 2-1-10: Additional spurious emissions - Way forward - Option 1: New requirements need define for NS_02N, 03N, 04N, 05N - Option 2: For the additional spurious emissions requirements, add 3 MHz CBW into - Table 6.5.3.3.2-1: Additional requirements for "NS_02N"; - Table 6.5.3.3.3-1: Additional out-of-band requirements for "NS_03N"; - Table 6.5.3.3.4-1: Additional out-of-band requirements for "NS_04N" and "NS_05N". - Add Table 6.5.3.3.5-1 Additional out-of-band requirements for "NS_24" - Option 3: Introduce 3 MHz channel bandwidth for all network signalling values Issue 2-2-1: Reference sensitivity - Way forward - Option 1: 2.2dB improvement compared with 5 MHz CBW; - Option 2: 1.7dB improvement compared with 5 MHz CBW; Issue 2-2-2: Maximum input level - Way forward - FFS on the requirements for 5MHz, 10MHz, 15MHz and 20MHz of NTN FR1 UE (-40dBm) Issue 2-2-3: ACS - Way forward - Option 1: Reuse the requirements for 5MHz, 10MHz of NTN FR1 UE. - Option 2: 3 MHz shall be added to table 7.5-1 into the same column as for 5 and 10 MHz channel BW. For tables 7.5-2 and 7.5-3, 3 MHz channel BW shall also be added but considering a 3 MHz interferer BW and 3/-3 interferer offset.

Table 2.1 provides all UE RF Tx and Rx requirements that might be affected by the introduction of 3 MHz channel bandwidth.

Table 2.1: UE RF requirements impact by 3 MHz

| NR NTN UE RF Tx / Rx requirements | Proposal for NTN 3 MHz |
| --- | --- |
| UE additional maximum output power reduction | No further relaxation on A-MPR values compared to existing values. |
| General spurious emissions | Reuse the TN UE approach for 3 MHz channel bandwidth |
| Additional spurious emissions | Introduce 3 MHz channel bandwidth for NS_02, NS_03, NS_04_NS_05, NS_24 |
| Reference sensitivity power level | Introduce 3 MHz channel bandwidth 2.2 dB scaling could be reused from TN approach |
| Maximum input level | Reuse the TN UE requirements for 3 MHz channel bandwidth |
| Adjacent channel selectivity (ACS) | 3 MHz shall be added to table 7.5-1 into the same column as for 5 and 10 MHz channel BW. For tables 7.5-2 and 7.5-3, 3 MHz channel BW shall also be added but considering a 3 MHz interferer BW and 3/-3 interferer offset. |

Therefore, we believe that RAN 4 could use the above table as the starting point for introducing 3 MHz channel bandwidth.

Proposal 1: To use the proposals in Table 2.1 for NTN UE RF requirements for NR NTN support 3 MHz channel bandwidth.

- Conclusion

This contribution provides our views on UE RF requirements for NTN support spectrum less than 5 MHz. The following proposals are concluded as follows:

Proposal 1: To use the proposals in Table 2.1 for NTN UE RF requirements for NR NTN support 3 MHz channel bandwidth.

- References

- R4-2416532, WF on [112-bis][311] NR_IoT_NTN_less_than_5MHz_UERF, Xiaomi, RAN4 #112-bis