3GPP TSG-RAN WG4 Meeting #113                                                                                             R4-2417815

Orlando, US, Nov. 18 –Nov. 22, 2024

Source: 	CATT

Title: 	Discussion on UE Rx requirement for NTN support HPUE

Agenda item:	7.8.2.2.2

Document for:	Approval

- Introduction

In RAN4 #112-bis meeting, a WF [1] on UE Rx requirement for NTN support HPUE was agreed. However, there are still some remaining issues need to be discussed in the next meeting.

In this contribution, we would like to share some views on UE Rx requirement for NTN support HPUE for NR NTN HPUE.

- Discussion

The open issues on UE Rx requirements for NTN support HPUE was achieved during the previous meetings, as shown below:

Issue 3-2: RSD requirements for PC2 Agreement: - For n256: [0dB] RSD for PC2 NR-NTN for 1Tx and 2Tx - FFS on the duplexer isolation and PA interference Issue 3-3: RSD requirements for other PC (PC1.5 and PC1) For PC1.5: - Option 1-1: For power class 1.5, more study on RSD is needed for NR-NTN, depending on the conclusion on the feasibility and agreed architecture. (vivo) - Option 1-2: If PC1.5 2Tx RSD is studied, it should be based on handheld UE, and can also applied to non-handheld. (vivo) For PC 1: - Option 2: For PC1 1Tx for NR-NTN, which is only considered feasible for non-handheld, the RSD is proposed to be 0dB. Whether and how to make it clear in the spec that this is for non-handheld only is FFS. (vivo) Way forward: - FFS RSD for PC1/PC1.5 for applicable UE architectures.

Since FR1 NR NTN UE and FR1 NR TN UE are implemented in a similar approach, the study of TN HPUE operating in single band FDD mode could be reused for NR NTN HPUE. Referring to the RSD study TR 38.861 and TR 38.896, the 2Tx architecture causes more severe interference compared to the 1Tx architecture, but the larger Tx-Rx separation prevents potential interference from the Tx chain.

For HPUE operating at the frequency band with larger Tx-Rx separation, the 2Tx architecture does not introduce severe interference compared to the 1Tx architecture, and therefore the RSD value is 0 dB, as the agreements made during the previous meeting for band n256. Considering that the NR NTN PC2 HPUE operated in band n254 has larger Tx-Rx separation, the self-interference will be more negligible than in the n256 band, we believed that for NR NTN PC2 HPUE in band n254, the RSD requirements could be 0 dB.

Proposal 1: For NR NTN PC 2 HPUE, the RSD value of band n254 could be 0 dB.

For PC1.5 NTN HPUE, the Tx architectures has not yet been determined, and the architecture of the HPUE will have a significant impact on the conclusions of the RSD. For non-handheld PC 1.5 NTN HPUE, we believed that the RSD evaluation could be lower priority than that of handheld UE (due to the hardware capabilities). It is important to determine whether the PC1.5 NTN HPUE supports the 2Tx architecture and the corresponding RSD value could be derived. After determining the RSD for the handheld PC1.5 NTN HPUE, it can be discussed whether to apply the conclusions to non-handheld PC1.5 NTN HPUE.

Proposal 2: For NR NTN PC 1.5 handheld HPUE, the RSD value depends on the architecture of HPUE.

- Conclusion

This contribution provides our initial views on how to conduct the UE RF requirements for NR NTN HPUE. The following proposals are concluded as follows:

Proposal 1: For NR NTN PC 2 HPUE, the RSD value of band n254 could be 0 dB.

Proposal 2: For NR NTN PC 1.5 handheld HPUE, the RSD value depends on the architecture of HPUE.

- References

- R4-2417122, WF on UE RF requirements for NTN HPUE, Samsung, RAN4 #112-bis