3GPP TSG RAN WG4 Meeting #113		R4-2417871

Orlando, USA, Nov 18th – 22nd , 2024

Title: 	On remaining issues on UE RF requirements for inter-band NR CA/DC with 3MHz CBW

Agenda Item:	7.4.2

Source: 	CATT

Document for:	Discussion

# 1 Introduction

In RAN4#112bis, almost all left issues have been resolved [1], with an LS sent to RAN2 [2], and post-meeting bigCR [3]. In particular, the LS to RAN2 clearly states that BCS4 and 5 could be indicated by a UE:

- For BCS signalling the following framework is used: - Traditional BCSs (i.e., not BCS4 or BCS5): UE is expected to support 3MHz channel bandwidth in the band in the band combination if it indicates a corresponding BCS, which includes 3MHz channel bandwidth in RAN4 specifications. No signalling changes are expected by RAN4. - BCS4: if UE indicates support for BCS4 including a band that it also supports 3 MHz channel bandwidth for, then it shall support 3 MHz for CA for that band in the band combination. No signalling changes are expected by RAN4. - BCS5: if UE indicates support for BCS5 and minimum channel bandwidth of 3MHz as a part of BCS signalling, then it shall support 3 MHz for CA for that band in the combination. The existing BCS5 signalling framework needs to be modified to allow indication of 3 MHz as minimum channel bandwidth.

However, BCS4/5 is not defined in the bigCR. In this contribution, we present two options on how to handle this.

# 2 Discussion

As RAN4 agreed to put square brackets around 5MHz channel bandwidth for band n100, only BCS 0 is introduced according to current bigCR [3]:

##### Table 5.5A.3.1-1o Table 5.5A.3.1-1o: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands) NR CA configuration; Uplink CA configuration or single uplink carrier10; NR Band; Channel bandwidth (MHz) (NOTE 3); Bandwidth combination set. CA_n100A-n101A; CA_n100A-n101A; n100; 3, [5]; 0. n101; 5, 10.

Fig. 1, NR band n100 and n101

As could be seen from Fig. 1, the full possible set of channel bandwidths in band n100 is 3 and 5MHz, and 5 and 10MHz for band n101. Hence, one option is to just introduce BCS 4/5 for CA_n100A-n101A, as shown below:

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n100A-n101A | CA_n100A-n101A | n100 | 3, [5] | 4 or 5 |
|  |  | n101 | 5, 10 |  |

Another option is to limit BCS 0 to the case where only 3MHz channel bandwidth is supported for band n100, and define BCS4/5 to capture all possible channel bandwidth combinations.

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n100A-n101A | CA_n100A-n101A | n100 | 3, [5] | 0 |
|  |  | n101 | 5, 10 |  |
| CA_n100A-n101A | CA_n100A-n101A | n100 | 3, [5] | 4 or 5 |
|  |  | n101 | 5, 10 |  |

Proposal: RAN4 to decide which of the following two options to define BCS for band combination CA_n100A-n101A:

- Option 1: just introduce BCS 4/5 for CA_n100A-n101A.

- Option 2: limit BCS 0 to the case where only 3MHz channel bandwidth is supported for band n100, and define BCS4/5 to capture all possible channel bandwidth combinations.

# 3 Conclusion

In this contribution, we have the following proposal for the remaining issues for inter-band NR CA/DC with 3MHz channel bandwidth:

Proposal: RAN4 to decide which of the following two options to define BCS for band combination CA_n100A-n101A:

- Option 1: just introduce BCS 4/5 for CA_n100A-n101A.

- Option 2: limit BCS 0 to the case where only 3MHz channel bandwidth is supported for band n100, and define BCS4/5 to capture all possible channel bandwidth combinations.

# Reference

[1] R4-2417192, “WF on UE RF requirements for Rel-19 NR channel BW less than 5MHz for FR1 Phase 2”, Intel

[2] R4-2417119, “LS on Rel-19 NR channel BW less than 5MHz for FR1”, Intel

[3] R4-2416083, “Draft Big CR for less than 5MHz UE RF requirements in Rel-19”, Intel