3GPP TSG RAN WG4 Meeting #113		R4-2417882

Orlando, USA, Nov 18th – 22nd , 2024

Title: 	On power domain enhancements for single carrier

Agenda Item:	7.1.1.2.1

Source: 	CATT

Document for:	Discussion

# 1 Introduction

In RAN4#112bis, discussion continued for power domain enhancements for single carrier with an approved WF [1]. A unified solution accommodating both scenario 1 and 2 to convert outer RB allocation to inner RB allocation is intended. Part of open issues are shown below:

Issue 1-2-2: Approaches of converting outer RB allocation to inner RB allocation - WF - The approach should consider the following aspects - unified solution could be applicable for both scenarios 1 and scenario 2 - unified solution considering the spectrum allocation status of operators, e.g multi-CC case from NW perspective, to better leverage the feature of MPR reduction - Solution could benefit RedCap and non-RedCap with UE CBW less or even identical to BS CBW - Solution could include both symmetric and asymmetric CBW extension - Whether the extended UE CBW could exceed the BS CBW for different scenarios - Signalling overhead is considered for the proposed approach(s) #### Issue 1-2-3: Where to use IBE in the larger BS CBW - WF - IBE should be used between edges of UE CBW and extended UE CBW - FFS whether IBE could be used in the guard band of UE CBW - FFS the full RB allocation scenario - FFS the impact to testability #### Issue 1-2-4: Boundary to apply ACLR and SEM - Proposals - Proposal 1: ACLR and SEM should be applicable from the edge of extended UE CBW instead of the BS CBW. - Proposal 2: ACLR, SEM and spurious emissions would be defined based on BS channel bandwidth. - WF - FFS in next meeting - ITU regulation of 250% necessary bandwidth should be considered in further analysis #### Issue 1-2-5: Boundary to apply SE - Proposals - Proposal 1: The application range of SE should be altered with the shifting of the edge of the UE CBW. - Proposal 2: SE is applied at BS channel bandwidth. - WF - FFS in next meeting - ITU regulation of 250% necessary bandwidth should be considered in further analysis #### Issue 1-2-6: Which CBW is utilized as the basis for the OOBE requirement and the applicable boundary - Proposals - Proposal 1: The OOBE requirement and applicable boundary should be based on UE CBW. - Proposal 2: The OOBE requirement and applicable boundary is based on BS CBW. - WF - FFS in next meeting #### Issue 1-2-7: Ratio of extended CBW to UE CBW and to larger BS channel BW - WF - To discuss the following aspects for the extended UE CBW: - Whether the extended CBW with new boundary of FOOB should comply with the recommendation by ITU-R on necessary bandwidth in terms of SE for the original UE BW - Whether default or fixed extension, e.g. 1/2 UE CBW could be stipulated in the spec or the extension ratio could be a UE capability to fulfil the conversion of outer to inner - Whether the extended UE CBW could belong to the regular defined channel bandwidth - Whether some premise should be established for the extension size, e.g. - minimum extended CBW compared to original UE CBW - minimum BS CBW compared to UE CBW - frequency separation of the RB allocation to the BS edge - Other identified issues are not precluded #### Issue 1-2-8: Asymmetrical extended CBW approach - WF - Asymmetrical extension is considered, FFS the solution. #### Issue 1-2-10: Signaling aspects - WF - To further discuss the signalling aspects in conjunction with the solution to be adopted for MPR reduction. - Companies are encouraged to provide thinking on signalling impact together with the proposed solution

In this contribution, we present our views on part of these open issues.

# 2 Discussion

- Symmetric extension

- Asymmetric extension

Fig. 1, Extended UE CBW, BS CBW and boundaries

As illustrated in Fig. 1(a), for the symmetric case, the extended UE channel bandwidth should not exceed the ITU-R OOB boundary in all cases, hence a shrinking factor α (<1.0) is required, and it can be calculated as following:

2αB+αB+5≤2.5B          (1)

Then we could obtain α as:

α≤2.5B-53B=56-53B              (2)

If the minimum UE channel bandwidth which allows for the extension is 10MHz, then the minimum α is (5/6-1/6)=2/3, which means that the extended UE channel bandwidth is extended 1/6 of the UE channel bandwidth on each side, and the total bandwidth is 4/3 of the UE channel bandwidth.

Similarly in Fig. 1(b), for the asymmetric case, the extended UE channel bandwidth should not exceed the BS boundary.

Hence, the unified approach for extending UE channel bandwidth can be defined as the common part between BS channel bandwidth and the virtual bandwidth where 1/6 of the UE channel bandwidth is extended on each side with a total bandwidth as 4/3 of the UE channel bandwidth.

Proposal 1: The extended UE channel bandwidth should not exceed BS channel bandwidth.

Proposal 2: The absolute minimum UE channel bandwidth to allow the extension is 10MHz.

Proposal 3: To avoid violating the ITU-R OOB boundary, RAN4 should define the unified extended UE channel bandwidth as the overlap between the BS channel bandwidth and a virtual bandwidth. This virtual bandwidth extends by 1/6 of the UE channel bandwidth on each side, totaling 4/3 of the original UE channel bandwidth.

To calculate the minimum BS channel bandwidth, we can see that 2/3*BUE <= ½ BBS, which means that BS channel bandwidth should be at least larger than 4/3 of UE channel bandwidth, and for the convenience, the threshold can be set to 1.5.

Proposal 4: RAN4 to set the minimum ratio between BS channel bandwidth and UE channel bandwidth as 1.5 to allow the extension.

# 3 Conclusion

In this contribution, we have the following proposals for power domain enhancements for single carrier:

Proposal 1: The extended UE channel bandwidth should not exceed BS channel bandwidth.

Proposal 2: The absolute minimum UE channel bandwidth to allow the extension is 10MHz.

Proposal 3: To avoid violating the ITU-R OOB boundary, RAN4 should define the unified extended UE channel bandwidth as the overlap between the BS channel bandwidth and a virtual bandwidth. This virtual bandwidth extends by 1/6 of the UE channel bandwidth on each side, totaling 4/3 of the original UE channel bandwidth.

Proposal 4: RAN4 to set the minimum ratio between BS channel bandwidth and UE channel bandwidth as 1.5 to allow the extension.

# Reference

[1] R4-2417204, “WF on power domain enhancements”, Huawei