3GPP TSG RAN WG4 #113	R4-2418023

Orlando, USA, Nov 18 – 22, 2024

Agenda item:		7.20.2.1

Source:	Nokia

Title:	On potentially new BS RF requirements for SBFD operation

Document for:		Approval

# Introduction

A Way Forward was agreed in RAN4#112bis [1] regarding evolution of NR duplex operation. There were some agreements made that have impact on the potentially new BS RF requirements for SBFD operation. Also, the open items are listed in the WF. This contribution displays Nokia’s view on the potentially new BS RF requirements for SBFD operation.

# Discussion

## BS RF Specification structure for SBFD requirements

The following was agreed in RAN4#112bis:

Way Forward: - FFS options of how to introduce BS RF new requirements for SBFD-capable BS: - Option-2: Creating new sub-clauses in TS 38.104 for RF requirements that applies for SBFD - Option-3: A new specification for SBFD BS RF requirements

#### How to introduce BS RF new requirements for SBFD-capable BS has also been under discussion during the last meetings. The WF presents two options and we consider as best solution to choose option-3. SBFD would be better handled in a completely new specification as is already done for example for IAB, repeaters and satellite access nodes. SBFD is not only just an enhancement to TDD but it is also combining the full duplexing of FDD systems. SBFD-capable FR1 WA BS must use separate antenna panels for transmitting and receiving and there is no agreed implementation for SBFD-capable BS. SBFD system also requires highly advanced and complex interference cancellation mechanisms and isolation materials to be used between antenna panels and sectors to protect the receiver from excessive interference. This makes it very crucial to ensure there is no mix-up in the requirements between TDD and SBFD. SBFD has also limitations where it is feasible to use and there are discussions ongoing for different RF requirements for different scenarios which might be very confusing to the reader. One could not be sure what requirements are valid for which systems and possibility for error would be highly increased. Therefore, we see that the best option for SBFD would be to have a new specification for it. There should not be bigger effort with maintenance of new specifications, as with separate specifications all SBFD related requirements will be in one place, maintenance even may be easier with separate specifications.

For introduction of new BS RF requirements for SBFD operation, solution with creating new specification is proposed (option 3).

## Transient period requirements

The following was agreed in RAN4#112bis:

Way Forward: - FFS the text proposal for the clause of transmitter transient period for SBFD, e.g., the example provided in R4-2416498.

We think a text proposal should be first agreed once there is an agreement on whether SBFD would have a new dedicated specification for SBFD-capable BS RF requirements or sub-clauses for TS 38.104 would be used.

Nevertheless, on the example provided in R4-2416498, we think the potential TP should adhere to the agreement reached in RAN4#112 where four transition cases were identified as shown in Table 1. We do not see any need to list detailed antenna configuration options and/or describing on which specific antenna panel the transition occurs unless further agreements take place in RAN4. Note that SBFD antenna configuration option 1, 2, 3 mentioned in R4-2416498 were discussed by RAN1/RAN4 in Rel-18 for the purposes of performance evaluation, but no agreement on normative specification impact has been made.

Table 1: RAN4-agreed cases for transient period requirements

|  | Frequency regions for Transition | BS transmitter behavior during transition |
| --- | --- | --- |
| Case-A (SBFD to DL) | SBFD UL subband and guardband(s) | BS transmitter OFF-to-ON |
| Case-B (SBFD to UL) | SBFD DL subband | BS transmitter ON-to-OFF |
| Case-C (DL to SBFD) | SBFD UL subband and guardband(s) | BS transmitter ON-to-OFF |
| Case-D (UL to SBFD) | SBFD DL subband | BS transmitter OFF-to-ON |

On transient period requirements, that SBFD antenna configuration option 1, 2, 3 mentioned in R4-2416498 were discussed in Rel-18 by RAN1/RAN4 for the purposes of performance evaluation, but no agreement on normative specification impact has been made.

Agreement on a text proposal on transmitter transient period for SBFD should wait for the discussion on whether SBFD would have a new dedicated specification for SBFD-capable BS RF requirements or sub-clauses for TS 38.104 would be used.

Potential text proposal on transmitter transient period for SBFD should adhere to the agreement reached in RAN4#112 where four transition cases were identified as shown in Table 1, without detailing gNB antenna configuration options.

## In-channel adjacent subband leakage ratio

The following was agreed in RAN4#112 and agreed as FFS in RAN4#112bis:

- Way forward: - FFS the necessity of in-channel adjacent subband leakage ratio requirements - Option 1: No necessity to introduce new requirement for in-channel adjacent subband leakage ratio. - Option 2: New requirement for in-channel adjacent subband leakage ratio should be defined. - Define a requirement on TX sub-band ACLR similar to the ACLR requirement and use existing ACLR requirement as baseline.

SBFD introduces new types of interference, some of which are highlighted in the figure below. From the BS perspective, the new cases of interference come from: 1) self-interference, 2) co-site inter-sector interference, and 3) inter-site interference.

Figure 1: Types of interference in SBFD deployment

During meetings, it has been discussed whether new requirements should be defined for the adjacent subbands, such as:

- In-channel adjacent subband blocking and selectivity

- In channel adjacent subband leakage

Below, we repeat the need to introduce the different requirements to ensure the SBFD operation.

In our view, new requirements are needed to ensure interoperability between base stations from different vendors in the same network. They cannot be implicitly guaranteed by OTA sensitivity requirement. It is easier to cancel the effects of interference inside your base station, whereas from other base stations there might be no cancellation options. Inter-site and co-site inter-sector BS interference should be taken into consideration in the definition of the requirements for SBFD operation.

The OTA sensitivity requirement does not capture the effects from co-site inter-sector and inter-site interference.

As RAN4 has not agreed on a reference implementation for the gNB, it is difficult to define additional requirements. Though we agree that depending on the gNB implementation the adjacent channel leakage, for example, can be different, in our view, RAN4 is responsible to define minimum requirements which should be achievable for reasonable implementations to guarantee the minimum performance.

In-channel adjacent subband leakage ratio, in-channel adjacent subband blocking and in-channel adjacent subband selectivity requirements cannot be guaranteed implicitly by the OTA sensitivity requirement, since the methods used for self-interference cancellation, might not be available for cancelling interference from other sectors and gNBs, especially when considering a multi-vendor deployment.

Even though RAN4 has not agreed on a reference implementation for SBFD operation, minimum requirements can still be defined to enable proper operation considering self-interference, co-site inter-sector and inter-site interference.

For the options listed in the WF, we see that option 2 is needed to be selected here. As shown in Figure 1, the leakage from the transmission in the DL subband can cause self-interference, co-site inter-sector interference and inter-site interference. The effects of the adjacent sub-band leakage into the gNBs own receiver can be cancelled to some extent, and we agree that this requirement can be implicitly guaranteed by the OTA sensitivity when considering self-interference only. However, the OTA sensitivity requirement does not guarantee that the adjacent sub-band leakage to other nodes in the network is sufficiently low, too. Therefore, we believe that a new requirement is needed. The exact value of the requirement needs more discussion, since in our view it would depend on the ratio between the bandwidths of the DL and the UL subbands but the baseline value should be the existing ACLR requirement.

RAN4 to define in-channel adjacent sub-band leakage ratio requirements within SBFD time slots considering co-site inter-sector interference and inter-site interference. Existing ACLR requirements could be used as baseline depending on the ratio between the bandwidths of the DL and the UL subbands.

## Necessity of in-channel adjacent subband selectivity/blocking requirements

The following was agreed in RAN4#112:

- Way forward: - FFS the necessity of in-channel adjacent subband selectivity requirements - Option 1: No necessity to introduce new requirement for in-channel adjacent subband selectivity. - Option 2: New requirement for in-channel adjacent subband selectivity should be defined. - Define a requirement on RX sub-band ACS similar to the ACS requirement and use existing ACS requirement as baseline. - Way forward: - FFS the necessity of in-channel adjacent subband blocking requirements - Option 1: No necessity to introduce new requirement for in-channel adjacent subband blocking. - Option 2: New requirement for in-channel adjacent subband blocking should be defined.

The following was agreed in RAN4#112bis:

Way Forward: - FFS based on the way forward approved in RAN4#112. - Consider the possibility of specifying RX requirement to cover both in-channel adjacent subband selectivity/blocking requirements

For the options listed in the WF, we see that option 2 is needed to be selected here. Currently, the adjacent channel selectivity (ACS) is defined as a measure of the receiver's ability to receive a wanted signal at its assigned channel frequency in the presence of an adjacent channel signal with a specified centre frequency offset of the interfering signal to the band edge of a victim system. The in-band blocking is defined as: a measure of the receiver's ability to receive a wanted signal at its assigned channel in the presence of an unwanted interferer, which is an NR signal for general blocking or an NR signal with one resource block for narrowband blocking.

In case of SBFD, the sub-band selectivity and blocking would be measured considering the locations of the UL subband and the DL subbands.

As mentioned before, the gNB operating in SBFD would suffer from self-interference, co-site inter-sector interference and inter-site interference. Therefore, there should be a requirement for the in-channel adjacent sub-band selectivity or blocking. Again, it can be argued that the in-channel adjacent sub-band selectivity can be captured in the OTA sensitivity requirements. However, as in the in-channel adjacent sub-band leakage ratio case, this test does not capture the selectivity in the presence of co-site inter-sector and inter-site interference. Furthermore, there might be base stations from different vendors in the same network. That is why these requirements are needed to ensure interoperability between these base stations.

Regarding whether both the in-channel adjacent sub-band selectivity and blocking requirements should be created, it is important to discuss how the existing adjacent channel and in-band blocking requirements are defined and tested. The current tests differ in terms of:

- the frequency offset between the RF bandwidth edge and the center frequency of the unwanted signal

- the unwanted signal power level

In case of assessment of the in-channel adjacent sub-band selectivity or blocking test, the frequency offset should the same, since it depends on the configuration of the UL and DL sub-bands. Therefore, we see that this new requirement should not be named selectivity or blocking but it would require a new name to not cause confusion. Both blocking and selectivity requirements need to be studied and more stringent should be selected as the limit of the requirement to cover the worst case.

RAN4 to define in-channel adjacent sub-band RX requirement to cover both selectivity and blocking requirement.

# Conclusion

In this contribution, our views on the potentially new SBFD BS RF requirements are presented. There were observations and proposals as listed below.

- On transient period requirements, that SBFD antenna configuration option 1, 2, 3 mentioned in R4-2416498 were discussed in Rel-18 by RAN1/RAN4 for the purposes of performance evaluation, but no agreement on normative specification impact has been made.

- The OTA sensitivity requirement does not capture the effects from co-site inter-sector and inter-site interference.

- In-channel adjacent subband leakage ratio, in-channel adjacent subband blocking and in-channel adjacent subband selectivity requirements cannot be guaranteed implicitly by the OTA sensitivity requirement, since the methods used for self-interference cancellation, might not be available for cancelling interference from other sectors and gNBs, especially when considering a multi-vendor deployment.

- Even though RAN4 has not agreed on a reference implementation for SBFD operation, minimum requirements can still be defined to enable proper operation considering self-interference, co-site inter-sector and inter-site interference.

- For introduction of new BS RF requirements for SBFD operation, solution with creating new specification is proposed (option 3).

- Agreement on a text proposal on transmitter transient period for SBFD should wait for the discussion on whether SBFD would have a new dedicated specification for SBFD-capable BS RF requirements or sub-clauses for TS 38.104 would be used.

- Potential text proposal on transmitter transient period for SBFD should adhere to the agreement reached in RAN4#112 where four transition cases were identified as shown in Table 1, without detailing gNB antenna configuration options.

RAN4 to define in-channel adjacent sub-band leakage ratio requirements within SBFD time slots considering co-site inter-sector interference and inter-site interference. Existing ACLR requirements could be used as baseline depending on the ratio between the bandwidths of the DL and the UL subbands.

RAN4 to define in-channel adjacent sub-band RX requirement to cover both selectivity and blocking requirement.

# References

- R4-2416584 “Way Forward for [112bis][306] NR_duplex_evo_BSRF”, RAN4#113, Hefei, China, 14th-18th October, 2024