3GPP TSG-RAN WG4 Meeting #113 	R4-2418062

Orlando, Florida, USA, 18th – 22nd November, 2024

Source:	T-Mobile USA

Title:	TP for TR38.746 for DL CA_n25-n77 with 3Tx 2BUL

Agenda item:	6.8.3

Document for:	Approval

- Introduction

This contribution is a text proposal to introduce PC2 2BDL/2BUL for CA_n25-n77.

The power class 1.5 MSD is calculated by calculating the interference power for PC2 IMD MSD, increasing the interference by 3 dB * IMD order, and then calculating the resulting MSD for PC1.5. The equations are included.

- Reference

[1] RP-242003, Revised WID: Rel-19 High power UE (power class 1.5 or 2) for NR intra-band Carrier Aggregation (CA) or NR inter-band CA/Dual connectivity (DC) band combinations with/without NR Supplementary uplink (SUL)

- Text Proposal

<Beginning of Text Proposal>

## 5.x	CA_n25-n77 3Tx

### 5.x.1	Operating bands for CA and Configurations

Table 5.x.1-1:  CA band combination of band n25+n77

| NR CA Band combination | NR Band |
| --- | --- |
| CA_n25-n77 | n25, n77 |

Table 5.x.1-2:  NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier6 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n25A-n77A |  | n258 n778,9 CA_n25A-n77A8,13,14 |  | n25 |  | 5, 10, 15, 20 |  | 0 |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |
|  |  |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 |  | 1 |  |
|  |  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |
|  |  |  |  | n25 |  | n25 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |  |
|  |  |  |  | n77 |  | n77 channel bandwidths in Table 5.3.5-1 |  |  |  |

### 5.x.2	Maximum output power for inter-band CA

Table 5.x.2-1: UE Power Class for 3Tx inter-band CA with UL MIMO

| NR UL CA Configuration | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n25A-n77A | 293 | +2/-3 | 262 | +2/-3 | 23 | +2/-3 |
| NOTE 1: (Void in this TR) NOTE 2: The UE supports PC3 in FDD band and PC3 or PC2 with UL MIMO in TDD band. NOTE 3: The UE supports PC3 in FDD band and PC1.5 with UL MIMO in TDD band. |  |  |  |  |  |  |

Table 5.x.2-2: UE Power Class for 3Tx inter-band CA with TxD

| NR UL CA Configuration | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n25A-n77A | 293 | +2/-3 | 262 | +2/-3 | 23 | +2/-3 |
| NOTE 1: (Void in this TR) NOTE 2: The UE supports PC3 in FDD band and PC2 with Tx Diversity in TDD band. NOTE 3: The UE supports PC3 in FDD band and PC1.5 with Tx Diversity in TDD band. |  |  |  |  |  |  |

### 5.x.3	MSD scenario studies

Table 5.x.3-1 and 5.x.3-2 summarizes frequency ranges where harmonic mixing, cross band leakage, IMD interferences occur for CA_n77-n85.

Table 5.x.3-1: MSD scenarios for CA_n77-n85 with total power class 2

|  | Aggressor Tx | Victim Rx | Whether 2Tx requirements exists |
| --- | --- | --- | --- |
| Harmonic mixing | n77 (UL1/DL2) | n25 | Yes |
| Cross band leakage | n77 | n25 | Yes |
| IMD2 | f n77 – f n25 | n25 | Yes |
| IMD4 | 3*f n25 – f n77 | n25 | Yes |

### 5.x.4	REFSENS requirements

#### 5.x.4.1	REFSENS requirements for total power class 2

- The MSD due to harmonic mixing PC2 is already specified in 38.101-1.

- The MSD due to cross band leakage PC2 is already specified in 38.101-1.

- The MSD due to IMD2 for PC2 is already specified in 38.101-1.

- The MSD due to IMD4 for PC2 is already specified in 38.101-1.

<End of Text Proposal>