3GPP TSG-RAN WG4 Meeting #113		R4-2418108

Orlando, US, 18th – 22nd November, 2024

Agenda item:		7.8.3.4

Source:		Samsung

Title:	Discussion on RRM requirements of Rel-19 NR-NTN in less than 5MHz

Document for:		Discussion

- Introduction

In RAN4#112bis meeting, we have already discussed all the RRM requirements and achieved agreements in WF [1]. There are still some FFS in [1]. In this contribution, we continue to discuss the FFS parts of RRM core requirements for less than 5MHz for NR-NTN.

- Discussion

In RAN4#112bis meeting, RAN4 achieved lots of agreements in [1]. There are two open issues: one is the CHO delay with L3 measurement. The second one is for the X/Y values in L3 measurement.

CHO delay requirements

In RAN4#112bis meeting, the CHO delay requirement without L3 measurement has been agreed to extend T∆. For CHO with L3 measurement, it is still FFS which is listed in WF [1] as below:

#### Issue 1-2-1-2 CHO delay Agreements/ Way-forward: - for CHO without L3 measurement: - T∆ in CHO requirement for NTN in exiting TS38.133 shall be extended to [3 Trs] - FFS on for CHO with L3 measurement: - Tmeasure which is same as the definition for Tidentify_intra_with_index and Tidentify_inter_with_index in issue 1-2-4-2 will be extended - T∆ in CHO requirement for NTN in exiting TS38.133 shall be extended to [3 Trs]

In legacy CHO with L3 measurement in NTN system:

When the UE receives a RRC message implying conditional handover the UE shall be ready to start the transmission of the new uplink PRACH channel or PUSCH channel within DCHO seconds from the end of the last TTI containing the RRC command. DCHO = TRRC + TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution

In the delay requirement, the SSB measurement are including the two yellow parts above.

Tmeasure is the measurement time delay is defined from the end of TEvent_DU until UE executes a handover to a target cell and interruption time starts.

This time delay is for the L3 measurement for neighbor cell before UE executes a HO. The time is related to the L3 measurement including SSB time index detection. Therefore, it will be extended due to additional time because SSB is punctured to 3MHz. The real times of Tmeasure are different for 12 PRB SSB and 20 PRB SSB. But for the spec drafting, this sub-clause is referred to section 9.x directly. No need to update in 6.1C.2.

For Tinterrupt, as the same discussion for other HO requirements, extension is needed for 3MHz NTN.

T∆ is time for fine time tracking and acquiring full timing information of the target cell. T∆ = Trs for both known and unknown target cells operating with 20 PRB SSB BW. T∆ = 3*Trs for both known and unknown target cells operating with 12 PRB SSB BW.

Although both two parts contain SSB measurement, they have different motivations and effects and in different time periods. Tmeasure is for UE to perform neighbor cell measurement before HO action. T∆ in Tinterrupt is when UE executes CHO and time and frequency tracking for new cell which UE will handover to. Thus, we don’t think they can be considered together. Both are needed for extension.

Proposal 1: For CHO with L3 measurement:

- DCHO = TRRC + TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution

- Tmeasure: the real times of Tmeasure are different for 12 PRB SSB and 20 PRB SSB. But for the spec drafting, this sub-clause is referred to section 9.x directly. no need to update in 6.1C.2.

- Tinterrupt, extend T∆ for 3MHz NTN system.

  - T∆ = 3*Trs for both known and unknown target cells operating with 12 PRB SSB BW.

L3 measurement

In RAN4#112bis meeting, the L3 measurement requirements for NTN less than 5MHz are agreed in high level with FFS of the values:

#### Issue 1-2-4-2: How to define the new requirements for SSB time index detection Agreements/Wayforward - The number of SSB samples for Rel-18 NTN measurement requirements (Tidentify_xxx_with_index) will be extended by X and Y more samples for intra-frequency and inter-frequency cases respectively. - For X and Y, the values from TN less than 5MHz as starting point.

In current TN less than 5MHz, for SSB based L3 measurement, for time period for time index detection of both intra-frequency measurement without MG or with MG, 4 additional samples are needed. For inter-frequency measurement without MG or with MG, 3 additional samples are added. For NTN 3MHz, there are no further difference for the simulation parameters except the channel condition. In last meeting, the principle was agreed above. However, some companies who want to check whether X and Y are different from that in TN less than 5MHz. By our understanding, the system parameters in physical layer are the same in NTN and TN. In addition, the side condition in NTN and TN are also the same. There is no further reason to set different values in NTN and TN. Therefore, we propose to reuse 4/3 additional samples for intra-frequency measurement and inter-frequency measurement, respectively.

Proposal 2: For time index detection requirement, reuse same 4/3 additional samples for intra-frequency measurement and inter-frequency measurement, respectively.

Proposal 3: For Intra-frequency measurements without measurement gaps, the time index detection requirements for 3MHz FR1-NTN should be defined:

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | max(120ms, ceil( 7 x Kp x Klayer1_measurement) x Kmulti_SMTC x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max(120ms, ceil (1.5 x 7 x Kp x Klayer1_measurement) x Kmulti_SMTC x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | Ceil(7 x Kp x Klayer1_measurement) x Kmulti_SMTC x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

For Intra-frequency measurements with measurement gaps, the time index detection requirements for 3MHz FR1-NTN should be defined:

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | max(120ms, 7 x Kgap x Kmulti_SMTC x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320ms | max(120ms, ceil(1.5 x 7) x Kgap x Kmulti_SMTC x max(MGRP, SMTC period,DRX cycle) x CSSFintra) |
| DRX cycle>320ms | 7 x Kgap x Kmulti_SMTC x max(MGRP, DRX cycle) x CSSFintra |

For Inter-frequency measurements without measurement gaps, the time index detection requirements for 3MHz FR1-NTN should be defined:

| DRX cycle | TSSB_time_index_inter |
| --- | --- |
| No DRX | max(120ms, ceil( 6 x Kp ) x SMTC period)Note 1 x CSSFinter Ksatellite |
| DRX cycle≤ 320ms | max(120ms, ceil (1.5 x 6 x Kp) x max(SMTC period,DRX cycle)) x CSSFinter Ksatellite |
| DRX cycle>320ms | Ceil(6 x Kp) x DRX cycle x CSSFinter Ksatellite |
| NOTE 1: SMTC period is the SMTC period in SMTC configuration which is associated with the target cell to be measured configured in SSB-MTC4List-r17. |  |

For Inter-frequency measurements with measurement gaps, the time index detection requirements for 3MHz FR1-NTN should be defined:

| Condition NOTE1 | TSSB_time_index_inter |
| --- | --- |
| No DRX | Max(120ms, Ceil(6 x Kgap) Max(MGRP, SMTC period NOTE2)) CSSFinter K_satellite |
| DRX cycle ≤ 320ms | Max(120ms, Ceil(6 1.5 x Kgap) Max(MGRP, SMTC period, DRX cycle)) CSSFinter K_satellite |
| DRX cycle > 320ms | Ceil(6 x Kgap) DRX cycle CSSFinter K_satellite |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: SMTC period is the SMTC period in SMTC configuration which is associated with the target cell to be measured configured in SSB-MTC4List-r17. |  |

- Conclusion

In this contribution, we provide our proposals for RRM core requirements for less than 5MHz for NR-NTN:

Proposal 1: For CHO with L3 measurement:

- DCHO = TRRC + TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution

- Tmeasure: the real times of Tmeasure are different for 12 PRB SSB and 20 PRB SSB. But for the spec drafting, this sub-clause is referred to section 9.x directly. no need to update in 6.1C.2.

- Tinterrupt, extend T∆ for 3MHz NTN system.

  - T∆ = 3*Trs for both known and unknown target cells operating with 12 PRB SSB BW.

Proposal 2: For time index detection requirement, reuse same 4/3 additional samples for intra-frequency measurement and inter-frequency measurement, respectively.

Proposal 3: For Intra-frequency measurements without measurement gaps, the time index detection requirements for 3MHz FR1-NTN should be defined:

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | max(120ms, ceil( 7 x Kp x Klayer1_measurement) x Kmulti_SMTC x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max(120ms, ceil (1.5 x 7 x Kp x Klayer1_measurement) x Kmulti_SMTC x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | Ceil(7 x Kp x Klayer1_measurement) x Kmulti_SMTC x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

For Intra-frequency measurements with measurement gaps, the time index detection requirements for 3MHz FR1-NTN should be defined:

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | max(120ms, 7 x Kgap x Kmulti_SMTC x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320ms | max(120ms, ceil(1.5 x 7) x Kgap x Kmulti_SMTC x max(MGRP, SMTC period,DRX cycle) x CSSFintra) |
| DRX cycle>320ms | 7 x Kgap x Kmulti_SMTC x max(MGRP, DRX cycle) x CSSFintra |

For Inter-frequency measurements without measurement gaps, the time index detection requirements for 3MHz FR1-NTN should be defined:

| DRX cycle | TSSB_time_index_inter |
| --- | --- |
| No DRX | max(120ms, ceil( 6 x Kp ) x SMTC period)Note 1 x CSSFinter Ksatellite |
| DRX cycle≤ 320ms | max(120ms, ceil (1.5 x 6 x Kp) x max(SMTC period,DRX cycle)) x CSSFinter Ksatellite |
| DRX cycle>320ms | Ceil(6 x Kp) x DRX cycle x CSSFinter Ksatellite |
| NOTE 1: SMTC period is the SMTC period in SMTC configuration which is associated with the target cell to be measured configured in SSB-MTC4List-r17. |  |

For Inter-frequency measurements with measurement gaps, the time index detection requirements for 3MHz FR1-NTN should be defined:

| Condition NOTE1 | TSSB_time_index_inter |
| --- | --- |
| No DRX | Max(120ms, Ceil(6 x Kgap) Max(MGRP, SMTC period NOTE2)) CSSFinter K_satellite |
| DRX cycle ≤ 320ms | Max(120ms, Ceil(6 1.5 x Kgap) Max(MGRP, SMTC period, DRX cycle)) CSSFinter K_satellite |
| DRX cycle > 320ms | Ceil(6 x Kgap) DRX cycle CSSFinter K_satellite |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: SMTC period is the SMTC period in SMTC configuration which is associated with the target cell to be measured configured in SSB-MTC4List-r17. |  |

- Reference

[1] R4-2416851, WF on RRM requirements for [112bis][207] NR_IoT_NTN_req_test_enh, Xiaomi