3GPP TSG-RAN WG4 Meeting #113		R4-2418110

Orlando, US, 18th – 22nd November, 2024

Agenda item:		7.19.3.2

Source:		Samsung

Title:	Discussion on RRM requirements of other aspects in Rel-19 MIMO phase 5

Document for:		Discussion

- Introduction

The latest revised WID [1] has been approved in the RAN plenary RAN#105 meeting. The objectives of Rel-19 MIMO Phase 5 are described in [1] including:

The detailed objectives are as follows: RAN1: - Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting - Specify CSI support for up to 128 CSI-RS ports, targeting FR1 - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s) - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design - SRS port grouping and its association to the two codewords for the 6/8Rx low complexity receiver supporting more than 4 layers, with legacy codebook - No enhancement on codeword-to-layer mapping, DL resource allocation, CSI feedback, and DCI format - Note: Whether to support 6Rx with more than 4 layers is to be decided in RAN4 Rel-19 RF enhancements WI - Specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD - Inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH - Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, enhancement(s) to enable 3T6R SRS antenna switching, as well as UE capability signaling for 3T3R antenna switching and 3-antenna-port non-codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource Note: UL full power transmission mode 1 and 2 are not supported. Note: Other than UE capability signaling, no other enhancement is specified for 3T3R SRS antenna switching. - Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP. - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources Acronyms: - sTRP: single TRP (transmit-receive point) - mTRP: multiple TRP (transmit-receive point) - CJT: coherent joint transmission - DU: distributed unit - CRI: CSI-RS resource indicator RAN2: Specify higher layer support of the enhancements listed above. RAN4: Specify necessary core requirements for the enhancements listed above.

In last RAN4 meeting, we analyzed the RRM requirements/impacts of objectives#2/3/4/5 and achieved some agreements in [2].

In this contribution, we continue to discuss the RRM requirements for objective#2, #3 and #5 in Rel-19 MIMO phase 5.

- Discussion

2.1 CSI enhancement

In RAN4#112bis meeting, the RRM impact of new added SRS port grouping is still open in [2]:

Issue 1-1: Whether RRM impacts exist by the added SRS port grouping for CSI enhancement? In last RAN plenary meeting RAN#105, another sub-bullet was approved for CSI enhancement (Objective#2): - SRS port grouping and its association to the two codewords for the 6/8Rx low complexity receiver supporting more than 4 layers, with legacy codebook - No enhancement on codeword-to-layer mapping, DL resource allocation, CSI feedback, and DCI format - Note: Whether to support 6Rx with more than 4 layers is to be decided in RAN4 Rel-19 RF enhancements WI. Way forward: - Option 1: Previous RAN4 agreement still apply “No RRM impact by CSI enhancement to support up to 128 ports”. - Option 2: Wait for RAN1.

In previous agreement, we agreed that there are no RRM impacts on:

a. Type-I codebook refinement to support 128 CSI-RS ports

b. Type-II codebook refinement to support 128 CSI-RS ports

c. Extension of CRI(s)-based CSI reporting to supports 128 CSI-RS ports

In last RAN plenary meeting RAN#105, another sub-bullet was approved as:

- SRS port grouping and its association to the two codewords for the 6/8Rx low complexity receiver supporting more than 4 layers, with legacy codebook - No enhancement on codeword-to-layer mapping, DL resource allocation, CSI feedback, and DCI format - Note: Whether to support 6Rx with more than 4 layers is to be decided in RAN4 Rel-19 RF enhancements WI

For this sub-bullet, in last meeting, some companies want to check further discussion in RAN1 for SRS group. By our understanding, as we present in last meeting, it introduces the SRS group definition. The UE receiver antennas are divided into two antenna groups. And each group corresponds to each CSI acquisition. In addition, for uplink, the UE antennas for SRS ports are also grouped such as two groups. It might have RAN4 impact for Demod performance. However, for RRM requirements, we only focus on the SRS usage of antenna switching. We defined the interruption time for it. By this sub-bullet, SRS port grouping is some processing part of TDD CSI procedure.

Proposal 1: No RRM impact by added SRS port grouping to support more than 4 layers.

2.2 UE reporting enhancement for CJT calibration

In RAN4#112bis meeting, RAN4 agreed to perform further simulation for CJTC reporting.

Issue 2-1: Whether RRM core requirements impacts exist by UE reporting enhancement for CJT calibration? Agreement: - RAN4 to simulate the performance of offsets for phase, time delay and frequency, based on one-shot measurement and multiple-sample measurement, and discuss the impact on RRM core requirement later. - Further discuss the simulation assumptions offline during this meeting. Way forward: - Companies are encouraged to bring simulation assumption/results in the next meeting. Simulation assumption in Appendix A can be used as a starting point for information only.

Before the simulation results collection, RAN4 needs to agree the simulation assumption. The initial version is attached in WF [2] to further discuss:

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | TDD, OFDM |
| Carrier Frequency | 3.5 GHz |
| Channel Model | TDL-C |
| Delay Spread | 300ns |
| antenna configuration | 1X2 |
| TRP number | 2 |
| TRS Bandwidth | 48RB, 106 RB |
| Numerology | 14 OFDM symbol per slot, 30kHz SCS |
| UE speed | 3km/h, 30kmph, 72kmph, 108kmph |
| SNR | -3dB, 0dB, 3dB |
| Frequency offset between TRP (only for freq. offset simulations) | 0.1ppm. |
| Delay difference between TRP (only for delay offset simulations) | 0.5CP, CP |
| Number of samples for measurement | 1, 2, 3, 4, 5, 10 |
| CSI-RS periodicity | 20ms / 40 slots |
| CSI-RS offset | Slot 20, 21 |

In high level, we are fine for the simulation assumption. But what to add minor modification.

Proposal 2: RAN4 to approve simulation assumption with minor modification as:

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | TDD/FDD, OFDM |
| Carrier Frequency | 3.5 GHz |
| Channel Model | TDL-C |
| Delay Spread | 300ns |
| antenna configuration | 1X2 |
| TRP number | 2 |
| TRS Bandwidth | 48RB, 106 RB |
| Numerology | 14 OFDM symbol per slot, 30kHz SCS, 15kHz SCS |
| UE speed | 3km/h, 30kmph, 72kmph, 108kmph |
| SNR | -3dB, 0dB, 3dB |
| Frequency offset between TRP (only for freq. offset simulations) | 0.1ppm./0.2ppm |
| Delay difference between TRP (only for delay offset simulations) | 0.5CP, CP |
| Number of samples for measurement | 1, 2, 3, 4, 5, 10 |
| CSI-RS config | Cjtc-d and cjtc-f: periodic TRS (‘CSI-RS for tracking’) Cjtc-P: single-port CSI-RS(s) for CQI |
| CSI-RS periodicity | 20ms / 40 slots |
| CSI-RS offset | Slot 20, 21 |

2.3 Asymmetric DL sTRP/UL mTRP

In RAN4#112bis meeting, it is agreed to update the RRM requirement for 2TA enhancement for asymmetric DL sTRP/UL mTRP in [2]:

Issue 4-1: Whether RRM impacts exist by 2TA enhancement? Agreement: - RAN4 to update the RRM requirements for 2TA enhancement [for asymmetric DL sTRP/ UL mTRP] - As baseline, the Rel-18 framework can be considered as starting point, and the update is needed to accommodate the new scenario in this WID. - Further discuss the details in the next meeting.

The detailed requirements are still not defined.

In RAN1#118bis meeting, RAN1 agreement for 2TA extension are shown as:

Agreement Support 2TA for the asymmetric DL sTRP/UL mTRP deployment scenarios: - Remove the restriction that coresetPoolIndex needs to be configured for the 2TA feature. - One downlink reference timing is supported and applied to both TAGs. - (FFS) Note: UE autonomous TA adjustment is only applicable to the first TAG - One single n-TimingAdvanceoffset is configured and applied to both TAGs. - Any of the TCI states can be associated with any one of the two TAGs. - The RAR carrying TA adjustment for those 2 TAGs is reused for Rel-19 2TA - The MAC CE based TA adjustment for 2 TAGs is reused for Rel-19 2TA. - Introduce the optional UE capability of “Overlapping UL transmission reduction” for Rel-19 2TA - If UE does not report this UE capability, UE does not expect two UL transmissions associated with different TAGs are overlapped. - FFS: UE does not expect that in intra-slot TDM PUSCH type-B repetition transmission, two consecutive repetitions associated with different TAGs are overlapped.

Firstly, it removed the restriction of that coresetPoolIndex needs to be configured for the 2TA feature.

In Rel-18, two TAs for m-DCI and m-TRP are introduced. RAN4 define such RRM requirements including: MRTD/MTTD, DL reference timing, other RRM requitements which have impacts such as L1-RSRP and TCI state switching delay requitements.

The association between coresetPoolIndex and TCI state are shown as:

38.821 The Unified TCI States Activation/Deactivation MAC CE is identified by a MAC subheader with eLCID as specified in Table 6.2.1-1b. It has a variable size consisting of following fields: - CORESET Pool ID: This field indicates that the mapping between the activated TCI states and the codepoint of the DCI Transmission Configuration Indication set by field TCI state ID is specific to the ControlResourceSetId configured with CORESET Pool ID as specified in TS 38.331 [5]. This field set to 1 indicates that the TCI states are specified to CORESET pool ID equal to 1, otherwise the TCI states are specified to CORESET pool ID equal to 0. If no more than one value for the coresetPoolIndex is configured for any CORESET in the BWP, the R bit is present instead;

For one unified TCI state, it is associated with CORESET Pool ID.

For joint/UL TCI state, it is associated with either TAG1 or TAG2, as shown in:

TCI-State ::= SEQUENCE { tci-StateId TCI-StateId, qcl-Type1 QCL-Info, qcl-Type2 QCL-Info OPTIONAL, -- Need R ..., [[ additionalPCI-r17 AdditionalPCIIndex-r17 OPTIONAL, -- Need R pathlossReferenceRS-Id-r17 PathlossReferenceRS-Id-r17 OPTIONAL, -- Cond JointTCI1 ul-powerControl-r17 Uplink-powerControlId-r17 OPTIONAL -- Cond JointTCI ]], [[ tag-Id-ptr-r18 ENUMERATED {n0,n1} OPTIONAL -- Cond 2TA ]] } TCI-UL-State-r17 ::= SEQUENCE { tci-UL-StateId-r17 TCI-UL-StateId-r17, servingCellId-r17 ServCellIndex OPTIONAL, -- Need R bwp-Id-r17 BWP-Id OPTIONAL, -- Cond CSI-RSorSRS-Indicated referenceSignal-r17 CHOICE { ssb-Index-r17 SSB-Index, csi-RS-Index-r17 NZP-CSI-RS-ResourceId, srs-r17 SRS-ResourceId }, additionalPCI-r17 AdditionalPCIIndex-r17 OPTIONAL, -- Need R ul-powerControl-r17 Uplink-powerControlId-r17 OPTIONAL, -- Need R pathlossReferenceRS-Id-r17 PathlossReferenceRS-Id-r17 OPTIONAL, -- Cond Mandatory ..., [[ tag-Id-ptr-r18 ENUMERATED {n0,n1} OPTIONAL -- Cond 2TA ]] }

If follow the principle of that in Rel-18, the uplink shall be timing advanced before a downlink reference point.

In Rel-19 asymmetric DL sTRP/UL mTRP, the scenarios are: UE can receive DL transmission and may transmit UL transmission from the TRP#1 while transmit UL transmission from TRP#2 (TRP#2 might be a UL-TRP with PL-RS offset configured).

In Rel-18 two TA mechanism, two uplink transmission timing are related to the reception of the downlink reference point for each TAG, which means for each TAG, there can be dedicated downlink reference point.

In Rel-19 asymmetric DL sTRP/UL mTRP, if two TRPs are from the same serving cell (intra-cell), for both two TAGs, they are corresponding to the same downlink reference point.

In addition, RAN1 support ICBM in asymmetric DL sTRP/UL mTRP scenarios as agreed in RAN1#117:

RAN1#117: Conclusion For the asymmetric DL sTRP/UL mTRP deployment scenario, reuse the rel-17 unified TCI/ICBM and rel-18 unified TCI framework: - When rel-17 unified TCI/ICBM is configured: - For FR1: one joint TCI state or {one DL TCI state + one UL TCI state} can be applied. - For FR2: {one DL TCI state + one UL TCI state} can be applied. - When rel-18 unified TCI is configured: - For FR1: up to two joint TCI states or {one DL TCI state + up to two UL TCI state} can be applied. - For FR2: {one DL TCI state + up to two UL TCI states} can be applied.

In Rel-19 asymmetric DL sTRP/UL mTRP, if all TRPs are from the serving cell and additional PCI cell (inter-cell), at the same moment, the inter-cell is a single TRP transmission, which is the same as Rel-17 ICBM. The downlink reference points and side condition can be the same as Rel-17 ICBM.

Proposal 3: RAN4 to specify RRM requirements for two TAs in asymmetric DL sTRP/UL mTRP. Rel-18 uplink timing requirements can be used as start point. In Rel-19 asymmetric DL sTRP/UL mTRP, if all TRPs are from the same serving cell (intra-cell), for both two TAGs, they are corresponding to the same downlink reference point. If all TRPs are from the serving cell and additional PCI cell (inter-cell), The downlink reference points and side condition can be the same as Rel-17 ICBM.

For the TCI state switching delay requitements of Rel-19 asymmetric DL sTRP/UL mTRP scenarios, it is still open and the options are listed in [2] as below:

Issue 4-2: RRM core impacts of Active uplink TCI state switching delay for unified TCI? Way forward: - Option 1: No RRM impacts on UL TCI state switching delay requirement for unified TCI states. - Option 2: RAN4 to specify UL TCI state switching requirements. - Option 3: FFS.

Let’s divided this issue into two levels.

Proposal 4: Separate the discussion into:

    - RAN4 to discussion whether to specified active uplink TCI state switching delay when the source RS is SRS.

    - RAN4 to discussion whether RRM impacts for active uplink TCI state switching delay

Firstly, for FR2, it seems the common understanding in group is that the UL TCI state can be associated to SRS only.

According to RAN1 agreement, joint TCI states is not supported in FR2. Only separate TCI UL state can be supported in FR2.

If the UL TCI state switching requirements are needed to be defined for SRS source, let’s discuss what can be defined.

If the target TCI state is associated with SRS, there is no time for measurement and UL beam refinement. In other word, if there is the delay, the delay is corresponding to the MAC-CE decoding time.

The other thing is the known condition. For uplink TCI state, the only thing is the spatial info of the beams.

In uplink TCI, there are two types of RS. One if referenceSignal which we called source RS (SRS in FR2 above). The other is pathlossReferenceRS. In UL-TRP, the power control is using PL-RS offset + PL-RS in DL/UL TRP.

If the known condition of target TCI is defined, it can contain: known condition of source RS as SRS. known condition of PL-RS in another TRP and relationship with PL-RS offset.

In legacy Rel-17 feMIMO discussion, the known condition of SRS cannot be defined. By our understanding, it is the reason that why current requirements “There is no requirement when the UE is requested to switch to a TCI state with the higher layer parameter TCI-UL-State associated to SRS” and why we proposed to follow no requirements when UE is requested to switch to a TCI state with the higher layer parameter TCI-UL-State associated to SRS in this asymmetric DL sTRP/UL mTRP scenarios.

Proposal 5: For FR2, no joint TCI state is supported. Only separate TCI UL state can be supported. Only the UL TCI states can only be associated to SRS.

Observation 1: For UL TCI states switching delay if the UL TCI state is associated to SRS, no RAN4 requirement in Rel-17/Rel-18.

Proposal 6: RAN4 to decide: If UL TCI states can only be associated to SRS

    - Alt1: Follow same principle in Rel-17 which is no requirement for UE is requested to a separate UL TCI state.

    - Alt2: The delay is MAC-CE processing time if define the known condition of target TCI state.

The second level is how to handle joint TCI state switching FR1.

If it is joint TCI in FR1, no matter one joint TCI or two joint TCIs, when joint TCI#1 is changed to joint TCI#2, the source RS is only applicable for DL reception which the source RS can be the DL RS which is only from DL/UL-TRP. And the PL-RS is only available from DL/UL-TRP, UE is indicated by PL-offset and perform UL transmission on UL-TRP.

If joint TCI is switching in FR1 mentioned above, there is no difference from legacy, the joint TCI state switching is the same as legacy.

Proposal 7: If joint TCI is switching in FR1 mentioned above, there is no difference from legacy, the joint TCI state switching is the same as legacy.

- Conclusion

In this contribution, we provide our proposals for RRM requirements for objective#2, #3 and #5 in Rel-19 MIMO phase 5:

Proposal 1: No RRM impact by added SRS port grouping to support more than 4 layers.

Proposal 2: RAN4 to approve simulation assumption with minor modification:

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | TDD/FDD, OFDM |
| Carrier Frequency | 3.5 GHz |
| Channel Model | TDL-C |
| Delay Spread | 300ns |
| antenna configuration | 1X2 |
| TRP number | 2 |
| TRS Bandwidth | 48RB, 106 RB |
| Numerology | 14 OFDM symbol per slot, 30kHz SCS, 15kHz SCS |
| UE speed | 3km/h, 30kmph, 72kmph, 108kmph |
| SNR | -3dB, 0dB, 3dB |
| Frequency offset between TRP (only for freq. offset simulations) | 0.1ppm./0.2ppm |
| Delay difference between TRP (only for delay offset simulations) | 0.5CP, CP |
| Number of samples for measurement | 1, 2, 3, 4, 5, 10 |
| CSI-RS config | Cjtc-d and cjtc-f: periodic TRS (‘CSI-RS for tracking’) Cjtc-P: single-port CSI-RS(s) for CSI |
| CSI-RS periodicity | 20ms / 40 slots |
| CSI-RS offset | Slot 20, 21 |

Proposal 3: RAN4 to specify RRM requirements for two TAs in asymmetric DL sTRP/UL mTRP. Rel-18 uplink timing requirements can be used as start point. In Rel-19 asymmetric DL sTRP/UL mTRP, if all TRPs are from the same serving cell (intra-cell), for both two TAGs, they are corresponding to the same downlink reference point. If all TRPs are from the serving cell and additional PCI cell (inter-cell), The downlink reference points and side condition can be the same as Rel-17 ICBM.

Proposal 4: Separate the discussion into:

    - RAN4 to discussion whether to specified active uplink TCI state switching delay when the source RS is SRS.

    - RAN4 to discussion whether RRM impacts for active uplink TCI state switching delay

Proposal 5: For FR2, no joint TCI state is supported. Only separate TCI UL state can be supported. Only the UL TCI states can only be associated to SRS.

Observation 1: For UL TCI states switching delay if the UL TCI state is associated to SRS, no RAN4 requirement in Rel-17/Rel-18.

Proposal 6: RAN4 to decide: If UL TCI states can only be associated to SRS

    - Alt1: Follow same principle in Rel-17 which is no requirement for UE is requested to a separate UL TCI state.

    - Alt2: The delay is MAC-CE processing time if define the known condition of target TCI state.

Proposal 7: If joint TCI is switching in FR1 mentioned above, there is no difference from legacy, the joint TCI state switching is the same as legacy.

- Reference

[1] RP-242394, WID revision: NR MIMO Phase 5, Samsung

[2] R4-2416857, WF on RRM requirements for NR_MIMO_Ph5_Part2, MediaTek