3GPP TSG-RAN WG4 Meeting #113		R4-2418114

Orlando, US, 18th – 22nd November, 2024

Agenda item:		7.26.4.1

Source:		Samsung

Title:	Discussion on RRM requirements of RedCap in Rel-19 NTN phase 3

Document for:		Discussion

- Introduction

In RAN4#112bis meeting, we have already discussed all the RRM requirements of RedCap in NTN and achieved some agreements in [1]. There are still some FFS in [1]. In this contribution, we continue to discuss the FFS parts of RRM requirements for (e)RedCap UE with NR NTN operating in FR1-NTN bands.

- Discussion

In RAN4#112bis meeting, RAN4 achieved lots of agreements in [1]. We will discuss remaining open issues as below.

#### Issue 2-2-1: Further discussion on cell reselection requirement for (e)RedCap UE with FR1-NTN Online agreement <Agreement>: - The existing cell reselection requirements in RRC_IDLE/RRC_INACTIVE state defined for Rel-17 and Rel-18 (e)NTN can be the baseline for (e)RedCap 2Rx UE with FR1-NTN bands. - Both time-based and location-based cell reselection measurement criteria will be defined. - For both earth fixed and earth moving scenario - Both NTN-NTN and NTN-TN (from NTN to TN and from TN to NTN) cell reselection requirements will be defined. - FFS for the measurement requirements for inter-RAT (NTN to E-UTRA TN) measurement.

In Rel-18 NTN, the measurements of inter-RAT E-UTRAN cells with TN carrier has been supported for normal UEs. From the real scenario, the inter-RAT for co-existence between NTN FR1 and LTE are also possible for operation. From the requirements view, for 2Rx (e)RedCap UE, there is no such difference between 2Rx RedCap UE and Rel-17 and Rel-18 handheld UEs.

In addition, for Redcap UE, the UE shall measure and detect E-UTRA cells as well as specified in 38.133:

The requirements in this clause apply to inter-RAT E-UTRAN FDD measurements and E-UTRA TDD measurements. When the measurement rules indicate that inter-RAT E-UTRAN cells are to be measured, the UE shall measure RSRP and RSRQ of detected E-UTRA cells in the neighbour frequency list at the minimum measurement rate specified in this clause.

The existing cell reselection requirements can be reused. The only difference is eDRX which can be discussed in eDRX issue.

Proposal 1: For the measurements of inter-RAT E-UTRAN cells with TN carrier, the existing cell reselection requirements in Rel-18 (e)NTN can be reused for (e)RedCap 2Rx UE with FR1-NTN bands. It can also be supported for (e)RedCap 1Rx UE. For 1Rx and eDRX, the relaxation and enhancement, discuss in separate issues.

#### Issue 2-2-2: Further discussion on L3 measurements for (e)RedCap UE with FR1-NTN In the last meeting, RAN4 has agreed to defined the requirements of NR intra-frequency L3 measurements and NR inter-frequency L3 measurements, the proposal is a further clarification on the supported types of L3 measurements for (e)RedCap UEs with FR1-NTN. <Way Forward>: - Proposal 1 (CATT): RAN4 to define the requirements of the following types of L3 measurements for (e)RedCap UEs with FR1-NTN: - Intra-frequency measurements without measurement gaps - Intra-frequency measurements with measurement gaps - Inter-frequency measurement without measurement gaps - Inter-frequency measurement with measurement gaps

In previous meeting, RAN4 has already agreed to define L3 measurement. As we proposed in last meeting, how to define such requirements:

| Measurement procedure: Intra-frequency measurement w/o MG Inter-frequency measurement w/o MG | For 2Rx, reuse the legacy FR1-NTN requirements For 1Rx, reuse the additional samples of 1Rx in TN in legacy FR1-NTN requirements. |
| --- | --- |

In legacy NTN L3 measurement, the L3 measurement are specified as below:

### 9.2C.5 Intra frequency measurements without measurement gaps ### 9.2C.6 Intra-frequency measurements with measurement gaps ### 9.3C.4 Inter-frequency measurement with measurement gaps ### 9.3C.7 Inter frequency measurements without measurement gaps

Proposal 2: For L3 measurements for (e)RedCap UE with FR1-NTN, specify the requirements for:

- Intra frequency measurements without measurement gaps

- Intra-frequency measurements with measurement gaps

- Inter-frequency measurement with measurement gaps

- Inter frequency measurements without measurement gaps

For detailed requirements, need further conclusion of whether parallel measurement can be support or not.

For 2Rx RedCap UE, reuse the legacy FR1-NTN requirements in high level (FFS on whether parallel measurement is allowed for RedCap UE)

For 1Rx RedCap, requirements difference comparation to 2Rx UE, reuse the same principle in TN system for RedCap UE which means additional samples for extension. Use the same number of additional samples as that in TN.

#### Issue 2-2-3: The Requirements to be further determined Online agreement <Agreement>: - Define NR Rel-17 and Rel-18 Conditional Handover requirements. - Include location-based CHO and time-based CHO. - Support earth moving cell scenario - Define RACH-less Handover requirements. - Define SDT requirements (including CG-SDT and RA-SDT) in RRC_INACTIVE state. - FFS TA validation based on the RSRP change criterion for RedCap in TN shall be removed and the requirements may refer to the PUR requirements for IoT for NTN. - FFS: MDT requirements in RRC_IDLE and RRC_INACTIVE state.

From LTE and NR, the driving testing has been introduced. The typical use scenario is to use it for network optimization.

In Rel-16 NR, MDT has been introduced as an optional feature.

For (e)Redcap UE, it can support to log such measurement in RRC_IDLE/RRC_INACTIVE case. However, typically, the log especially for long time log for example: time-stamped RSRP/RSRQ will cause the UE power consumption. That is why we think (e)RedCap UE can use for MDT but not typical usage. That is also why no RAN4 requirements for Rel-17/Rel-18 (e)RedCap UE for MDT requirements in TN in Rel-17/Rel-18.

Proposal 3: Do not specify Minimization of Drive Tests (MDT) requirements in RRC_IDLE state and RRC_INACTIVE state for (e)RedCap UE in NTN FR1 as the same situation for (e)RedCap UE in TN system.

#### Issue 2-3-1: RedCap R17 relaxed measurement Online agreement <Agreement>: - FFS: R17 NR NTN relaxations in GSO (corresponding to the TN R16 relaxation) for normal UE are applicable for (e)RedCap UE with FR1-NTN bands. - FFS: RAN4 to define Rel-17 RedCap relaxed measurement requirements for cell reselection for (e)RedCap UE with FR1-NTN bands, and the corresponding requirements defined for RedCap UE can be the baseline. - If defined, the requirements apply provided that target cell is GSO

In legacy RAN4 and RAN2 specs, there are multiple criteria for relaxed measurements:

R16 relaxed measurement: UE with low mobility and/or UE not at cell edge

There are two criterions: lowMobilityEvaluation-r16 cellEdgeEvaluation-r16

R17/R18 RedCap relaxed measurement:

4.2B.2.9.2 Measurements for UE fulfilling stationary criterion 4.2B.2.9.3 Measurements for a UE fulfilling not-at-cell edge while stationary criterion 4.2B.2.9.3A Measurements for a UE fulfilling stationary and not-at-cell-edge criteria 4.2B.2.9.4 Measurements for a UE fulfilling low mobility and stationary criteria 4.2B.2.9.5 Measurements for a UE fulfilling low mobility and not-at-cell-edge while stationary criteria 4.2B.2.9.6 Measurements for a UE fulfilling not-at-cell edge and not-at-cell edge while stationary criteria 4.2B.2.9.7 Measurements for a UE fulfilling low mobility and not-at-cell edge criteria and not-at-cell-edge while stationary criteria 4.2B.2.9.8 Measurements for a UE fulfilling low mobility, not-at-cell edge and stationary criterion 4.2B.2.9.9 Measurements for UE fulfilling low mobility criterion 4.2B.2.9.10 Measurements for UE fulfilling not-at-cell edge criterion 4.2B.2.9.11 Measurements for UE fulfilling low mobility and not-at-cell edge criteria

In RedCap, although stationary criteria and not-at-cell-edge while stationary criteria are similar with lowMobility criteria and not-at-cell-edge criteria. However, they still are different criterion and different signallings.

[[ relaxedMeasurement-r16 SEQUENCE { lowMobilityEvaluation-r16 SEQUENCE { s-SearchDeltaP-r16 ENUMERATED { dB3, dB6, dB9, dB12, dB15, spare3, spare2, spare1}, t-SearchDeltaP-r16 ENUMERATED { s5, s10, s20, s30, s60, s120, s180, s240, s300, spare7, spare6, spare5, spare4, spare3, spare2, spare1} } OPTIONAL, -- Need R cellEdgeEvaluation-r16 SEQUENCE { s-SearchThresholdP-r16 ReselectionThreshold, s-SearchThresholdQ-r16 ReselectionThresholdQ OPTIONAL -- Need R } OPTIONAL, -- Need R combineRelaxedMeasCondition-r16 ENUMERATED {true} OPTIONAL, -- Need R highPriorityMeasRelax-r16 ENUMERATED {true} OPTIONAL -- Need R } OPTIONAL -- Need R ]], [[ cellEquivalentSize-r17 INTEGER(2..16) OPTIONAL, -- Cond HSDN relaxedMeasurement-r17 SEQUENCE { stationaryMobilityEvaluation-r17 SEQUENCE { s-SearchDeltaP-Stationary-r17 ENUMERATED {dB2, dB3, dB6, dB9, dB12, dB15, spare2, spare1}, t-SearchDeltaP-Stationary-r17 ENUMERATED {s5, s10, s20, s30, s60, s120, s180, s240, s300, spare7, spare6, spare5, spare4, spare3, spare2, spare1} }, cellEdgeEvaluationWhileStationary-r17 SEQUENCE { s-SearchThresholdP2-r17 ReselectionThreshold, s-SearchThresholdQ2-r17 ReselectionThresholdQ OPTIONAL -- Need R } OPTIONAL, -- Need R combineRelaxedMeasCondition2-r17 ENUMERATED {true} OPTIONAL -- Need R } OPTIONAL -- Need R ]]

In the “lowMobilityEvaluation” and “stationaryMobilityEvaluation” and “cellEdgeEvaluationWhileStationary” criteria, all depend on the RSRP/RSRQ variation which means depends on the RSRP measurement.

As summarized in 38.821:

In TN scenarios, UE can measure RSRP and use the results to determine roughly that UE is near a cell center or at a cell edge because the RSRP are clear different in such different areas.

However, in NTN scenarios, the RSRP level are quite flat and with small difference in a large area, due to near-far-effect.

Proposal 4: It is very hard and meaningless to distinguish low mobility and stationary mobility by RSRP variation in NTN due to near-far-effect. Even for NW implementation, it’s hard to determine different RSRP variation by different “s-SearchDeltaP”/” s-SearchDeltaP-Stationary”/” t-SearchDeltaP”/” t-SearchDeltaP-Stationary”

Proposal 5: Use unified set of requirements for “lowmobility/stationary” and/or “not-at cell edge” relaxation.

#### Issue 2-3-2: eDRX configuration Online agreement <Agreement>: - For GSO: - Define eDRX related requirements in RRC_Idle / RRC_Inactive mode for (e)RedCap UE with FR1-NTN. - For GSO, All DRX and eDRX cycle can be supported, and the DRX and eDRX related requirements for RedCap UE in TN are re-used. - Further discuss for LEO.

For GSO scenario, eDRX has already been agreed in last meeting. For LEO scenario, there can be two cases:

- Earth moving beams

- Earth fixed beams

For earth-moving beams, when satellite is moving, the rough time is considered by footprint diameter and largest elevation angle.

For earth-moving beam LEO deployment, even 2.56s DRX cannot be applied in Rel-17 FR1 NTN for intra-frequency measurement and inter-frequency measurement as:

Note 2: The UE is not required to meet the requirements for 2.56s DRX cycle length for earth-moving LEO deployment.

For RedCap UEs, the smallest eDRX is 2.56s:

Table 4.2B.2.3-2: Tdetect,NR_Intra_RedCap, Tmeasure,NR_Intra_RedCap and Tevaluate,NR_Intra_RedCap for UE configured with eDRX_IDLE cycle (Frequency range FR1)

| eDRX_IDLE cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 1.28s periods) | Tdetect,NR_Intra_RedCap [s] (number of DRX cycles or eDRX cycles Note 3) | Tmeasure,NR_Intra_RedCap [s] (number of DRX cycles or eDRX cycles Note 3) | Tevaluate,NR_Intra_RedCap [s] (number of DRX cycles or eDRX cycles Note 3) |
| --- | --- | --- | --- | --- | --- |
| 2.56 | - | - | 58.88 (23) | 2.56 (1) | 7.68 (3) |
| 5.12 | - | - | 117.76 (23) | 5.12 (1) | 10.24 (2) |
| 10.24 | - | - | 235.52 (23) | 10.24 (1) | 20.48 (2) |
| 20.48 ≤ eDRX_IDLE cycle length ≤10485.76 | 0.32 | ≥1.28 (1) | eDRX_cycle_length×23PTW/DRX_cycle_length (23) | 0.32 x M2 (1 x M2) | 0.64 x M2 (2 x M2) |
|  | 0.64 | ≥1.28 (1) |  | 0.64 (1) | 1.28 (2) |
|  | 1.28 | ≥2.56 (2) |  | 1.28 (1) | 2.56 (2) |
|  | 2.56 | ≥5.12 (4) |  | 2.56 (1) | 5.12 (2) |
| Note 1: The number of DRX cycles in this table corresponds to the DRX cycles within PTWs, when PTW is configured. Note 2: The eDRX_IDLE cycle lengths are as specified in Section 10.5.5.32 of TS 24.008 [42]. Note 3: Number of eDRX cycles when eDRX_IDLE cycle length equals 2.56s, 5.12s and 10.24s. Otherwise, number of DRX cycles. Note 4: The lower bound of PTW length is derived based on Tevaluate,NR_Intra_RedCap*DRX_cycle1.28*1.28. Note 5: M2 = 2 if SMTC periodicity of measured intra-frequency cell > 20 ms; otherwise M2=1. |  |  |  |  |  |

Table 4.2B.2.4-2: Tdetect,NR_Inter_RedCap, Tmeasure,NR_Inter_RedCap and Tevaluate,NR_Inter_RedCap for UE configured with eDRX_IDLE cycle (Frequency range FR1)

| eDRX_IDLE cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 1.28s periods) | Tdetect,NR_Inter_RedCap [s] (number of DRX cycles or eDRX cycles Note 3) | Tmeasure,NR_Inter_RedCap [s] (number of DRX cycles or eDRX cycles Note 3) | Tevaluate,NR_Inter_RedCap [s] (number of DRX cycles or eDRX cycles Note 3) |
| --- | --- | --- | --- | --- | --- |
| 2.56 | - | - | 58.88 (23) | 2.56 (1) | 7.68 (3) |
| 5.12 | - | - | 117.76 (23) | 5.12 (1) | 10.24 (2) |
| 10.24 | - | - | 235.52 (23) | 10.24 (1) | 20.48 (2) |
| 20.48 ≤ eDRX_IDLE cycle length ≤10485.76 | 0.32 | ≥1.28 (1) | eDRX_cycle_length×23PTW/DRX_cycle_length (23) | 0.32 x 1.5 (1 x 1.5) | 0.64 x 1.5 (2 x 1.5) |
|  | 0.64 | ≥1.28 (1) |  | 0.64 (1) | 1.28 (2) |
|  | 1.28 | ≥2.56 (2) |  | 1.28 (1) | 2.56 (2) |
|  | 2.56 | ≥5.12 (4) |  | 2.56 (1) | 5.12 (2) |

Proposal 6: For earth-moving beam LEO deployment, all eDRX cycle cannot be applied. Not define requirements for eDRX in earth-moving beam LEO deployment.

For earth-fixed beam LEO deployment, most important factor is the dwell time. In previous discussion, the typical dwell time is about 1~10min.

For 2.56s eDRX, the longest detection time is up to 58.88s. Compare to dwell time, it is still feasible.

For 5.12s eDRX, the longest detection time is up to 117.76s. However, Kcarrier_RedCap is multiplied to detection time for inter-frequency measurement. Consider some deployment, it is still feasible while others may not.

For 10.24s eDRX, the longest detection time is up to 235.52s. Compare to longer dwell time 10min=600s, it is hard to guarantee the performance.

For eDRX even larger than 10.24s, the situation goes worse.

Proposal 7: For earth-fixed beam LEO deployment, 10.24s eDRX cycle and larger than 10.24s eDRX cycle cannot be applied. RedCap UEs are not required to meet the requirements. For 2.56s eDRX, define requirements for RedCap UEs. For 5.12s eDRX, prefer to define requirements for RedCap UEs to make more flexible/different network deployments.

#### Issue 2-3-3: Multiple SMTCs <Way Forward>: - Proposal 1 (HW): RAN4 to discuss support of multiple SMTCs for (e)Redcap UE with FR1-NTN bands. - Option 1: make it optional for RedCap UE to support 2 SMTC per MO. - Option 2: change the requirements so that measurement for different SMTCs are not parallel. - Proposal 2 (ZTE): Multiple SMTCs shall be reused for RedCap UEs in NTN scenario. - Proposal 2a (Ericsson): RedCap for NTN supports (at the least) 2 SMTC. - Proposal 3 (OPPO): RAN4 to not consider the support of multiple SMTCs for (e)RedCap UE with FR1-NTN bands. - Proposal 4 (QC): NTN RRM requirements defined for simultaneous measurements on multiple satellites are not applicable for RedCap NTN UE.

In Rel-17 NTN, 2 SMTC is default and mandatory for UE. 4 SMTC-s are optional UE capability.

#### Issue 2-3-4: Concurrent gap <Way Forward>: - Proposal 1 (CMCC, HW): Consider concurrent gap for Redcap over NTN. - Proposal 1a (HW): Concurrent gaps are supported for (e)Redcap UE with FR1-NTN bands, and existing requirements for normal UE in NTN are re-used as baseline. - Proposal 2 (CATT, QC): NTN RRM requirements defined for concurrent measurement gaps are not applicable for RedCap NTN UE. - Proposal 3 (Ericsson): RedCap for NTN supports (at the least) 1 MG.

In Rel-17 NTN, concurrent gap is supported. In Rel-17/18 RedCap UEs, only per-UE and per-FR MG are supported.

From technical view, multiple SMTC and concurrent gap can be supported absolutely. The key concern for RedCap UE is the complexity and power consumption. From UE perspective, deprioritize parallel SMTC measurement and concurrent gap for RedCap UE in NTN.

Proposal 8: Deprioritize parallel SMTC measurement and concurrent gap for RedCap UE in NTN.

For 1RX RedCap UE discussion,

Issue 3-2-1: RRC_IDLE/RRC_INACTIVE mode mobility for 1Rx/2Rx (e)RedCap UEs with FR1-NTN <Way Forward>: - For 2Rx (e)RedCap UEs with FR1-NTN bands, reuse the legacy FR1-NTN requirements for normal UEs. - For 1Rx (e)RedCap UEs with FR1-NTN bands, firstly to discuss the following proposals one by one: - Proposal 1: RAN4 to consider the offset for the reletive cell-specific RSRP thresholds for 1Rx (e)RedCap UEs with FR1-NTN bands, including Qrxlevmin and Qqualmin: - -1 dB offset can be reused. - Proposal 2: RAN4 to reduce the number of NR inter-frequency carriers that UE shall at least be capable of monitoring for 1Rx (e)RedCap UEs with FR1-NTN bands. - 6 NR inter-frequency carriers can be reused. - Proposal 3: The paremeters of Nserv_RedCap, Tdetect,NR_Intra/Inter_RedCap, Tmeasure,NR_Intra/Inter_RedCap and Tevaluate,NR_Intra/Inter_RedCap defined for cell re-selection requirements will be not distinguished between 1Rx and 2 Rx (e)Redcap UE, and the requirements will be the same. - Proposal 4: For the measurements of intra- and inter-frequency NR cells, - +1 dB offset on the threshold of absThreshSS-BlocksConsolidation can be reused as a baseline for 1Rx(e)RedCap UEs with FR1-NTN bands. - For the cases of rangeToBestCell is not configured and rangeToBestCell is configured, +1 dB offset can be reused for better cell ranked for 1Rx (e)RedCap UEs with FR1-NTN bands. - Proposal 5: In RRC_IDLE / RRC_INACTIVE state, +1 dB offset can be reused for a margin in FR1 for reselections when UE evaluating whether a newly detectable inter-frequency cell meets the reselection criteria for 1Rx (e)RedCap with FR1-NTN: - For 1 Rx RedCap by a margin of at least - 6 dB in FR1 for reselections based on ranking or - 7 dB in FR1 for SS-RSRP reselections based on absolute priorities or - 5 dB in FR1 for SS-RSRQ reselections based on absolute priorities. - Further discuss SDT requirements. - MDT requirements are related to issue 2-2-3, RAN4 to discuss them based on the conclusions on issue 2-2-3 later.

As we proposed in last meeting:

For 2Rx, reuse the legacy FR1-NTN requirements

For 1Rx, the ranking should be 1dB relaxed.

The proposals above are almost the same although the wording is a little different. For 1RX, reuse the same principle in TN system which is 1dB relaxed in multiple power comparation.

For detailed requirements and whether to reuse the existing FR1-NTN requierements, need further conclusion of whether parallel measurement can be support or not.

Proposal 9: For cell re-selection,

- For 2Rx, reuse the legacy FR1-NTN requirements in high level (FFS on whether parallel measurement is allowed for RedCap UE)

- For 1Rx, requirements difference comparation to 2Rx UE, reuse the same principle in TN system for RedCap UE which means 1dB offset/difference applied.

#### Issue 3-2-2: NR HO requirements for 1Rx/2Rx (e)RedCap UEs with FR1-NTN <Way Forward>: - For NR RACH-based HO requirements, - For 2Rx (e)RedCap UEs with FR1-NTN bands, reuse the legacy FR1-NTN RACH-based HO requirements for normal UEs. - For 1Rx (e)RedCap UEs with FR1-NTN bands, RACH-based handover delays should be extended compared to those defined for 2Rx (e)RedCap UEs. - Define the relaxation on Tsearch for unknown intra/inter-frequency cell, and the same relaxation for 1 Rx RedCap UE in TN network can be reused. - Relax by 1 sample: Tsearch = 2*Trs for intra-frequency HO - Relax by 2 samples: Tsearch = 5* Trs for inter-frequency HO - Further discuss the requirements for RACH-less HO, Time/location-based CHO with/without L3 measurement and Satellite switching with re-sync.

As we proposed in last meeting: For 2Rx, reuse the legacy FR1-NTN requirements, For 1Rx, reuse the same principle for RedCap UEs in TN system.

Proposal 10: For handover requirements for RedCap UEs in FR1-NTN:

- For 2Rx, reuse the legacy FR1-NTN requirements

- For 1Rx, reuse the same principle for RedCap 1Rx UE in TN for 1 additional sample for intra-frequency and 2 additional samples for inter-frequency.

Issue 3-2-3: RRC Re-establishment requirements for 1Rx/2Rx (e)RedCap UEs with FR1-NTN <Way Forward>: - For 2Rx (e)RedCap UEs with FR1-NTN bands, reuse the legacy FR1-NTN RRC re-establishment delay requirements for normal UEs. - For 1Rx (e)RedCap UEs with FR1-NTN bands, UE RRC re-establishment delay (TUE_re-establish_delay) should be extended compared to those defined for 2Rx (e)RedCap UEs. - Define the relaxation on Tidentify for known and unknown intra/inter-frequency cell, and the same relaxation for 1 Rx RedCap UE in TN network can be reused. - Relax the Tidentify_intra_NR and Tidentify_inter_NR, i by 1 more samples for FR1 known and unknown intra/inter-frequency cell.

In Rel-17 NTN requirements, it is related to the multi-SMTC.

Table 6.2C.1.2.1-1: Time to identify target NR cell for RRC connection re-establishment to NR intra-frequency cell Serving cell; FR of target NR; Tidentify_intra_NR [ms]. SSB Ês/Iot (dB); cell; Known NR cell; Unknown NR cell. ≥ -8; FR1; MAX (200 ms, 5 x TSMTC); Kmulti_SMTC * MAX (800 ms, 10 x TSMTC). < -8; FR1; N/A; k * 800 ms Note1, 3. Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC > 20 ms and serving cell SSB Ês/Iot < -8 dB. Note 2: Kmulti_SMTC is defined in clause 9.2C.5.1. Note 3: k = 1 if the cells on the target frequency are served by GEO. k = (N+1) if the cells on the target frequency are served by LEO, where N is the number of different satellites associated to the list of configured neighbor cells in ntn-NeighCellConfigList and ntn-NeighCellConfigListExt.. Table 6.2C.1.2.1-2: Time to identify target NR cell for RRC connection re-establishment to NR inter-frequency cell Serving cell SSB Ês/Iot (dB); FR of target NR cell; Tidentify_inter_NR, i [ms]. Known NR cell; Unknown NR cell. ≥ -8; FR1; MAX (200 ms, 6 x TSMTC, i); K_satellite * MAX (800 ms, 13 x TSMTC, i). < -8; FR1; N/A; k * 800 ms Note1, 3. Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC,i > 20 ms and serving cell SSB Ês/Iot < -8 dB. Note 2: K_satellite is defined in clause 9.3C.4. Note 3: k = 1 if the cells on the target frequency are served by GEO. k = (N+1) if the cells on the target frequency are served by LEO, where N is the number of different satellites associated to the list of configured neighbor cells in ntn-NeighCellConfigList and ntn-NeighCellConfigListExt..

Proposal 11: For RRC Re-establishment requirements, it need the agreement for another issue for multi-SMTC. From high level perspective, for 1Rx UE, reuse the same principle for RedCap 1Rx UE in TN for X additional sample for intra-frequency and inter-frequency. Use the same additional numbers as 1Rx RedCap in TN. (In TN system, not finished yet, [] are still in the spec).

#### Issue 3-2-4: RRC Connection Release with redirection requirements for 1Rx/2Rx (e)RedCap UEs with FR1-NTN <Way Forward>: - For 2Rx (e)RedCap UEs with FR1-NTN bands, reuse the legacy FR1-NTN RRC Connection Release with redirection requirements for normal UEs. - For 1Rx (e)RedCap UEs with FR1-NTN bands, the delay of RRC connection release with redirection to NR (Tconnection_release_redirect_NR) should be extended compared to those defined for 2Rx (e)RedCap UEs. - Define the relaxation on Tidentify-NR for FR1, and the same relaxation for 1 Rx RedCap UE in TN network can be reused. - Relax the Tidentify-NR by 1 more samples for FR1.

For RRC connection Release with redirection requirements, it is similar at RRC Re-establishment. It is related to whether UE support multiple satellites or not.

Table 6.2C.3.2.1-1: Time to identify target NR cell for RRC connection release with redirection to NR FR of target NR cell; Tidentify-NR. FR1; K_satellite * MAX (680 ms, 11 x Trs). Note 1: If the UE has been provided with higher layer signaling of smtc2 specified in TS 38.331 [2] prior to the redirection command, Trs follows smtc1 or smtc2 according to the physical cell ID of the target cell. Note 2: K_satellite is defined in clause 9.3C.4..

Proposal 12: For RRC connection Release with redirection requirements, it need the agreement for other issue. From high level perspective, for 1Rx UE, reuse the same principle for RedCap 1Rx UE in TN for 1 additional sample.

#### Issue 3-3-1: Timing requirements for 1Rx/2Rx (e)RedCap UEs with FR1-NTN <Way Forward>: - The existing timing requirements defined for NTN can be reused for (e)RedCap UEs with FR1-NTN bands. - UE transmit timing - UE timer accuracy - Timing Advance - The timing requirements defined for 1Rx and 2Rx (e)RedCap UEs with FR1-NTN bands will be the same, and there is no need to differentiate 1Rx/2Rx requirements.

For timing requirements, it cannot be observed any difference for 1Rx and 2Rx UEs.

Proposal 13: For timing requirements, for both 1Rx and 2Rx RedCap UEs, reuse the Rel-17 FR1-NTN requirements.

#### Issue 3-4-1: RLM/BFD/CBD requirements for 1Rx/2Rx (e)RedCap UEs with FR1-NTN <Way Forward>: - For 2Rx (e)RedCap UEs with FR1-NTN bands, reuse the legacy FR1-NTN RLM/BFD/CBD requirements for normal UEs. - For 1Rx (e)RedCap UEs with FR1-NTN bands, - For SSB based and CSI-RS based RLM/BFD requirements, some of hypothetical PDCCH parameters including aggregation level (CCE) and bandwidth (PRBs) are defined as follows: - Aggregation level (CCE) of 16. - Bandwidth (PRBs) of 48. - For SSB based and CSI-RS based RLM/BFD requirements, SCS=60kHz is not applicable for (e)RedCap UEs with FR1-NTN. - The SSB and CSI-RS based RLM OOS and BFD evaluation periods should be extended for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs. - For RLM evaluation periods, - RAN4 not to extend the TEvaluate_in_SSB/CSI-RS for FR1. - RAN4 to extend the TEvaluate_out_SSB/CSI-RS by factor 2 for FR1. - For BFD evaluation periods, - RAN4 to extend the TEvaluate_BFD_SSB/CSI-RS by factor 2 for FR1.

As we proposed in last meeting:

For 2Rx, reuse the legacy FR1-NTN requirements

For 1Rx, reuse the same principle in TN system for RedCap UE.

The proposals above are almost the same although the wording is a little different. For 1RX, reuse the same principle in TN system.

Proposal 14: For RLM/BFD/CBD requirements:

- Need further conclusion for whether parallel measurement can be support or not.

- From high level:

- For 1Rx/2Rx requirements difference, reuse the same principle in TN system for RedCap UE.

  - Different Hypothetical PDCCH parameters for 1Rx RedCap: Aggregation level (CCE), bandwidth

  - Extended RLM/BFD evaluation period for 1Rx. The factors are the same as 1Rx UE in TN.

#### Issue 3-5-1: Measurement requirements for 1Rx/2Rx (e)RedCap UEs with FR1-NTN - Proposals - Proposal 1 (vivo): - For Measurement Procedure requirement for Red-cap NTN, for 2Rx, RAN4 to reuse the existing Measurement Procedure requirements for NTN. For 1Rx, the corresponding 1Rx requirements defined in Red-cap can be reused. - Proposal 2 (CATT): - The time period for PSS/SSS detection defined for intra/inter-frequency measurement requirements should be relaxed for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs. - Reuse the same extensions for 1Rx RedCap UE in TN network. - Increase the number of attempts by 2 SSB samples for TPSS/SSS_sync_intra and TPSS/SSS_sync_inter. - Not to extend the lower boundary. - The time period for SSB index identification (PBCH decoding) defined in intra/inter-frequency measurement requirements should be relaxed for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs. - Reuse the same extensions for 1Rx RedCap UE in TN network. - Increase number of attempts by 3 SSB samples for TSSB_time_index_intra and TSSB_time_index_inter. - Extend the lower boundary from ‘120 ms’ to ‘160ms’. - The measurement period defined in inter-frequency measurement with measurement gaps requirements should be relaxed for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs. - Reuse the same extensions for 1Rx RedCap UE in TN network. - Extend the lower boundary from ‘200 ms’ to ‘400ms’ - Keep the same number of samples for T SSB_measurement_period_inter. - The measurement period defined in intra-frequency measurement with/without gap and inter-frequency measurement without gaps requirements for 1Rx and 2Rx (e)RedCap UEs with FR1-NTN bands will be the same. - The SSB and CSI-RS based L1-RSRP measurement requirements for 1Rx and 2Rx (e)RedCap UEs with FR1-NTN bands will be the same. - Proposal 3 (QC, Nokia): - Extended delays for PSS/SSS detection and SSB index identification (PBCH decoding) for 1Rx (e)RedCap - Proposal 4 (Samsung): For Intra-frequency w/o MG and inter-frequency w/o MG measurement requirements, - For 2Rx, reuse the legacy FR1-NTN requirements - For 1Rx, reuse the additional samples of 1Rx in TN in legacy FR1-NTN requirements. - Proposal 5 (HW): - L3 and L1 measurement for 1RX: relaxation for delay and accuracy - Proposal 6 (MTK): - Measurement delay - PSS/SSS detection delay is extended by 2-3 samples, based on whether it is intra-f/inter-f and whether it is with MG or not. - SSB index identification (PBCH decoding) delay is extended by 2 samples. - Proposal 7 (Xiaomi, OPPO): - For 1RX (e)RedCap UE with FR1-NTN bands, support to relax measurement requirements by extending the measurement delays with more measurement samples. - Proposal 8 (ZTE): - RAN4 shall define time period of PSS/SSS detection and time index detection. The requirements for 1Rx and 2Rx RedCap UE in NTN scenario shall reuse the legacy RedCap requirements. - The RedCap UEs with 1Rx and 2Rx in NTN shall own the same measurement period requirements as legacy RedCap UE. <Way Forward>: - For 2Rx (e)RedCap UEs with FR1-NTN bands, reuse the legacy FR1-NTN measurement requirements for normal UEs. - For 1Rx (e)RedCap UEs with FR1-NTN bands, to discuss the following proposals one by one: - The time period for PSS/SSS detection defined for intra-/inter-frequency measurement requirements should be relaxed for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs. - Reuse the same extensions for 1Rx RedCap UE in TN network. - Increase the number of attempts by 2 SSB samples for TPSS/SSS_sync_intra and TPSS/SSS_sync_inter. - Not to extend the lower boundary. - The time period for SSB index identification (PBCH decoding) defined in intra-/inter-frequency measurement requirements should be relaxed for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs. - Reuse the same extensions for 1Rx RedCap UE in TN network. - Increase number of attempts by 3 SSB samples for TSSB_time_index_intra and TSSB_time_index_inter. - Extend the lower boundary from ‘120 ms’ to ‘160ms’. - The measurement period defined in inter-frequency measurement with measurement gaps requirements should be relaxed for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs. - Reuse the same extensions for 1Rx RedCap UE in TN network. - Extend the lower boundary from ‘200 ms’ to ‘400ms’ - Keep the same number of samples for T SSB_measurement_period_inter. - The measurement period defined in intra-frequency measurement with/without gap and inter-frequency measurement without gaps requirements for 1Rx and 2Rx (e)RedCap UEs with FR1-NTN bands will be the same. - The SSB and CSI-RS based L1-RSRP measurement requirements for 1Rx and 2Rx (e)RedCap UEs with FR1-NTN bands will be the same.

It is similar as other issues above:

Firstly, it is related to whether UE support parallel measurement as Rel-17 NTN.

The proposals above are almost the same although the wording is a little different. For 1RX, reuse the same principle in TN system.

For the difference between 2Rx and 1Rx, reuse the same principle as that in TN which means additional samples for extension.

Proposal 15: For Measurement requirements:

- Need further conclusion for whether parallel measurement can be support or not.

- From high level:

- For 1Rx/2Rx requirements difference, reuse the same principle in TN system for RedCap UE which mean additional samples for extension. Use the same number of additional samples as that in TN.

#### Issue 3-6-1: Measurement accuracy relaxation for 1Rx/2Rx (e)RedCap UEs with FR1-NTN - Proposals - Proposal 1 (CATT, Samsung): For 2Rx (e)RedCap UEs with FR1-NTN bands, RAN4 to reuse the accuracy requirements defined in NTN. - Proposal 2 (Xiaomi, CATT, QC, Apple, Samsung, HW, OPPO, Nokia): For 1Rx (e)RedCap UEs with FR1-NTN bands, support to relax the measurement accuracy for SS-RSRP, SS-RSRQ, SS-SINR and L1-RSRP compared to those defined for 2Rx (e)RedCap UEs. - Proposal 2a (CATT, Samsung, MTK): The same relaxation defined in TN for 1Rx (e)RedCap UEs can be reused. - SS-RSRP, SS-RSRQ, SS-SINR: relax by 1dB. - L1-RSRP: relax by 3dB. - Proposal 3 (CATT): RAN4 to reuse the conditions defined in NTN for a corresponding Band, and the values for SSB_RP and SSB Ês/Iot are the same for 1Rx (e)RedCap UEs and 2Rx (e)RedCap UEs with FR1-NTN bands. <Way Forward>: - For 2Rx (e)RedCap UEs with FR1-NTN bands, RAN4 to reuse the measurement accuracy requirements defined in NTN. - For 1Rx (e)RedCap UEs with FR1-NTN bands, support to relax the measurement accuracy for SS-RSRP, SS-RSRQ, SS-SINR and L1-RSRP compared to those defined for 2Rx (e)RedCap UEs. - The same relaxation defined in TN for 1Rx (e)RedCap UEs can be reused. - SS-RSRP, SS-RSRQ, SS-SINR: relax by 1dB. - L1-RSRP: relax by 3dB. - RAN4 to reuse the conditions defined in NTN for a corresponding Band, and the values for SSB_RP and SSB Ês/Iot are the same for 1Rx (e)RedCap UEs and 2Rx (e)RedCap UEs with FR1-NTN bands.

Proposal 16: For accuracy requirements:

- 2Rx UE, reuse Rel-17 NTN requirements.

- 1Rx UE, reuse the same relaxation for SS-RSRP, SS-RSRQ, SS-SINR and L1-RSRP in TN.

The last aspect is HD-FDD impact.

In RAN1 discussion, there are still many open issues. It is not clear for RRM impacts of HD-FDD RedCap UEs.

Proposal 17: For HD-FDD related impact, wait further RAN1 progress.

- Conclusion

In this contribution, we provide our proposals for RRM requirements for (e)RedCap UE with NR NTN operating in FR1-NTN bands:

Proposal 1: For the measurements of inter-RAT E-UTRAN cells with TN carrier, the existing cell reselection requirements in Rel-18 (e)NTN can be reused for (e)RedCap 2Rx UE with FR1-NTN bands. It can also be supported for (e)RedCap 1Rx UE. For 1Rx and eDRX, the relaxation and enhancement, discuss in separate issues.

Proposal 2: For L3 measurements for (e)RedCap UE with FR1-NTN, specify the requirements for:

- Intra frequency measurements without measurement gaps

- Intra-frequency measurements with measurement gaps

- Inter-frequency measurement with measurement gaps

- Inter frequency measurements without measurement gaps

For detailed requirements, need further conclusion of whether parallel measurement can be support or not.

For 2Rx RedCap UE, reuse the legacy FR1-NTN requirements in high level (FFS on whether parallel measurement is allowed for RedCap UE)

For 1Rx RedCap, requirements difference comparation to 2Rx UE, reuse the same principle in TN system for RedCap UE which means additional samples for extension. Use the same number of additional samples as that in TN.

Proposal 3: Do not specify Minimization of Drive Tests (MDT) requirements in RRC_IDLE state and RRC_INACTIVE state for (e)RedCap UE in NTN FR1 as the same situation for (e)RedCap UE in TN system.

Proposal 4: It is very hard and meaningless to distinguish low mobility and stationary mobility by RSRP variation in NTN due to near-far-effect. Even for NW implementation, it’s hard to determine different RSRP variation by different “s-SearchDeltaP”/” s-SearchDeltaP-Stationary”/” t-SearchDeltaP”/” t-SearchDeltaP-Stationary”

Proposal 5: Use unified set of requirements for “lowmobility/ stationary” and/or “not-at cell edge” relaxation.

Proposal 6: For earth-moving beam LEO deployment, all eDRX cycle cannot be applied. Not define requirements for eDRX in earth-moving beam LEO deployment.

Proposal 7: For earth-fixed beam LEO deployment, 10.24s eDRX cycle and larger than 10.24s eDRX cycle cannot be applied. RedCap UEs are not required to meet the requirements. For 2.56s eDRX, define requirements for RedCap UEs. For 5.12s eDRX, prefer to define requirements for RedCap UEs to make more flexible/different network deployments.

Proposal 8: Deprioritize parallel SMTC measurement and concurrent gap for RedCap UE in NTN.

Proposal 9: For cell re-selection,

- For 2Rx, reuse the legacy FR1-NTN requirements in high level (FFS on whether parallel measurement is allowed for RedCap UE)

- For 1Rx, requirements difference comparation to 2Rx UE, reuse the same principle in TN system for RedCap UE which means 1dB offset/difference applied.

Proposal 10: For handover requirements for RedCap UEs in FR1-NTN:

- For 2Rx, reuse the legacy FR1-NTN requirements

- For 1Rx, reuse the same principle for RedCap 1Rx UE in TN for 1 additional sample for intra-frequency and 2 additional samples for inter-frequency.

Proposal 11: For RRC Re-establishment requirements, it need the agreement for another issue for multi-SMTC. From high level perspective, for 1Rx UE, reuse the same principle for RedCap 1Rx UE in TN for X additional sample for intra-frequency and inter-frequency. Use the same additional numbers as 1Rx RedCap in TN. (In TN system, not finished yet, [] are still in the spec).

Proposal 12: For RRC connection Release with redirection requirements, it need the agreement for other issue. From high level perspective, for 1Rx UE, reuse the same principle for RedCap 1Rx UE in TN for 1 additional sample.

Proposal 13: For timing requirements, for both 1Rx and 2Rx RedCap UEs, reuse the Rel-17 FR1-NTN requirements.

Proposal 14: For RLM/BFD/CBD requirements:

- Need further conclusion for whether parallel measurement can be support or not.

- From high level:

- For 1Rx/2Rx requirements difference, reuse the same principle in TN system for RedCap UE.

  - Different Hypothetical PDCCH parameters for 1Rx RedCap: Aggregation level (CCE), bandwidth

  - Extended RLM/BFD evaluation period for 1Rx. The factors are the same as 1Rx UE in TN.

Proposal 15: For Measurement requirements:

- Need further conclusion for whether parallel measurement can be support or not.

- From high level:

For 1Rx/2Rx requirements difference, reuse the same principle in TN system for RedCap UE which mean additional samples for extension. Use the same number of additional samples as that in TN.

Proposal 16: For accuracy requirements:

- 2Rx UE, reuse Rel-17 NTN requirements.

- 1Rx UE, reuse the same relaxation for SS-RSRP, SS-RSRQ, SS-SINR and L1-RSRP in TN.

Proposal 17: For HD-FDD related impact, wait further RAN1 progress.

- Reference

[1] R4-2416865, WF on RRM requirements for NR_NTN_Ph3_Part1, CATT