3GPP TSG-RAN WG4 Meeting #113	R4-2418159

Orlando, Florida,  18 – 22 Nov 2024

Source:	Charter Communications, Cable Labs

Title:	SBFD minimum channel bandwidth

Agenda item:	7.20.2	[NR_duplex_evo-Core]

Document for:	approval

Introduction

In TSG-RAN WG4 Meeting #112 bis, in way forward R4-2416584, it was agreed that the minimum channel bandwidth of any SBFD carrier shall be X = 40 MHz, but for band n39, X = 20MHz is used instead.

Discussion

In this contribution we propose to add to TS 38.104, Table 5.3.2-1:

Note: The channel bandwidth for a Subband Full Duplex carrier shall be no less than X = 40 MHz.
•	For band n39, X = 20MHz is used instead.

### 5.3.2	Transmission bandwidth configuration

The transmission bandwidth configuration NRB for each BS channel bandwidth and subcarrier spacing is specified in table 5.3.2.-1 for FR1 and table 5.3.2-2 for FR2.

Table 5.3.2-1: Transmission bandwidth configuration NRB for FR1

| SCS (kHz) | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 15 | 25 | 52 | 79 | 106 | 133 | 160 | 188 | 216 | 242 | 270 | N/A | N/A | N/A | N/A | N/A |
| 30 | N/A | 11 | 24 | 38 | 51 | 65 | 78 | 92 | 106 | 119 | 133 | 162 | 189 | 217 | 245 | 273 |
| 60 | N/A | N/A | 11 | 18 | 24 | 31 | 38 | 44 | 51 | 58 | 65 | 79 | 93 | 107 | 121 | 135 |

Note: The channel bandwidth for a Subband Full Duplex carrier shall be no less than X = 40 MHz.
•	For band n39, X = 20MHz is used instead.

Conclusion

The following proposal was made:

Proposal:   Add this note to Table 5.3.2-1:  
Note: The channel bandwidth for a Subband Full Duplex carrier shall be no less than X = 40 MHz.
•	For band n39, X = 20MHz is used instead.

- References

- R4-2416584 Way Forward for [112][307][308] NR duplex evo, Samsung

- TS 38.104