# 3GPP TSG-RAN WG4 Meeting #113	R4-2418193

Orlando, US, 18th – 22nd November, 2024

Source: 	vivo

Title: 	Discussion on FR2-NTN inclusion to specifications

Agenda Item:	8.1

Document for:	Approval

Introduction

An incoming LS [1] was received for Rel-18 NR_NTN_enh-Core. RAN1 asked if the note related to FR2-NTN needs to be updated. In this contribution, we tried to give our views on this issue and provided reply LS to RAN1.

Discussion

In the latest TS 38.101-5, NTN frequency ranges are defined as in Table 5.1-1.

Table 5.1-1: Definition of NTN frequency ranges

| Frequency range designation | Corresponding frequency range |
| --- | --- |
| - FR1-NTN (Note 1) | - 410 MHz – 7125 MHz |
| - FR2-NTN (Note 2) | - 17300 MHz – 30000 MHz |
| NOTE 1: NTN bands within this frequency range are regarded as a FR1 band when references from other specifications. NOTE 2: NTN bands within this frequency range are regarded as a FR2 band when references from other specifications. |  |

In LS [1], RAN1 asked if Note 2 in Table 5.1-1 needs to be updated. It can be observed that the brackets have been removed for NOTE 1 and 2. However, we believe it is necessary to update the notes to make the specification clear.

In NR, the frequency ranges are defined as Table 5.1-1 in TS 38.101-2.

Table 5.1-1: Definition of frequency ranges

| Frequency range designation |  | Corresponding frequency range |
| --- | --- | --- |
| FR1 |  | 410 MHz – 7125 MHz |
| FR2 | FR2-1 | 24250 MHz – 52600 MHz |
|  | FR2-2 | 52600 MHz – 71000 MHz |

It can be observed that FR2-NTN are not entirely covered in FR2-1, and the low bound of FR2-NTN 17300MHz is out of FR2-1 range. However, we prefer to treat FR2-NTN as FR2-1 bands when references from other specifications. Firstly, the SCS and CHBW defined for FR2-NTN are aligned with FR2-1. Secondly, other specification (e.g. TS 38.213) can simply reuse the FR2-1 procedures or requirements for FR2-NTN. It saves a lot of trouble of adding FR2-NTN procedures as in the CR in the attachment of LS [1].

In the LS [1], RAN1 also mentioned that some RRC parameters (e.g., subcarrierSpacing in BWP) and UE capabilities (e.g., channelBWs-DL, configuredUL-GrantType1-v1650) may have the same issue in RAN2 spec, which only refer to the term FR2-1. In our understanding, RAN2 spec may lack of clear clarification for FR2-NTN. The following are some examples cited from TS 38.306 and 38.331:

TS 38.306 clause 4.2.7.2: channelBWs-DL Indicates for each subcarrier spacing the UE supported channel bandwidths. Absence of the channelBWs-DL (without suffix) for a band or absence of specific scs-XXkHz entry for a supported subcarrier spacing means that the UE supports the channel bandwidths among [5, 10, 15, 20, 25, 30, 40, 50, 60, 80, 100] and [50, 100, 200] that were defined in clause 5.3.5 of TS 38.101-1 version 15.7.0 [2] and TS 38.101-2 version 15.7.0 [3] for the given band or the specific SCS entry. For IAB-MT, to determine whether the IAB-MT supports a channel bandwidth of 100 MHz, the network checks channelBW-DL-IAB-r16. For FR1, the bits in channelBWs-DL (without suffix) starting from the leading / leftmost bit indicate 5, 10, 15, 20, 25, 30, 40, 50, 60 and 80MHz. For FR2, the bits in channelBWs-DL (without suffix) starting from the leading / leftmost bit indicate 50, 100 and 200MHz. The third / rightmost bit (for 200MHz) shall be set to 1. For IAB-MT the third / rightmost bit (for 200MHz) is ignored. To determine whether the IAB-MT supports a channel bandwidth of 200 MHz, the network checks channelBW-DL-IAB-r16. For FR1, the leading/leftmost bit in channelBWs-DL-v1590 indicates 70MHz, the second leftmost bit indicates 45MHz, the third leftmost bit indicates 35MHz, the fourth leftmost bit indicates 100MHz and all the remaining bits in channelBWs-DL-v1590 shall be set to 0. The fourth leftmost bit (for 100MHz) is not applicable for bands n41, n48, n77, n78, n79 and n90 as defined in TS 38.101-1 [2]. For each band, (e)RedCap UEs shall indicate supporting the maximum of those channel bandwidths that are less than or equal to 20 MHz for FR1 and less than or equal to 100 Mhz for FR2, taking restrictions in TS 38.101-1 [2] and TS 38.101-2 [3] into consideration. For each band, NTN capable UEs shall indicate the supported channel bandwidths for FR1, taking restrictions in TS 38.101-5 [34] into consideration. This feature is applicable only for FR1 and FR2-1 band, otherwise it is absent.

TS 38.331 clause 6.3.2: BWP ::= SEQUENCE { locationAndBandwidth INTEGER (0..37949), subcarrierSpacing SubcarrierSpacing, cyclicPrefix ENUMERATED { extended } OPTIONAL -- Need R } subcarrierSpacing Subcarrier spacing to be used in this BWP for all channels and reference signals unless explicitly configured elsewhere. Corresponds to subcarrier spacing according to TS 38.211 [16], table 4.2-1. The value kHz15 corresponds to µ=0, value kHz30 corresponds to µ=1, and so on. Only the following values are applicable depending on the used frequency: FR1: 15, 30, or 60 kHz FR2-1: 60 or 120 kHz FR2-2: 120, 480, or 960 kHz

In a summary, we need to update the notes for FR2-NTN when other specs refer to RAN4 spec:

NOTE 2:	NTN bands within this frequency range are regarded as a FR2-1 band when references from other specifications.

In other RAN4 specification, we suggest to apply the change to TS 38.133/TS 38.108 since the same note still exists. As an example shown below, there is a similar description specified as NOTE 2 in Table 8.1C.1-2 in TS 38.133.

Table 8.1C.1-2: Maximum number of RLM-RS resources NRLM

| Carrier frequency range of PCell |  | Maximum number of RLM-RS resources, NRLM |
| --- | --- | --- |
| FR1, ≤ 3 GHzNote | 4 | 2 |
| FR1, > 3 GHzNote | 8 | 4 |
| [FR2-NTN]Note 2 | 64 | 8 |
| NOTE: For unpaired spectrum operation with Case C - 30 kHz SCS, 3GHz is replaced by 1.88GHz, as specified in clause 4.1 in TS 38.213 [3]. NOTE 2: [NTN bands within this frequency range are regarded as a FR2 band when references from other specifications.] |  |  |

Proposal 1: Reply to RAN1 and RAN2 that the notes in NTN frequency ranges table need to be updated for TS 38.101-5/38.133/38.108 :

NOTE 2:	NTN bands within this frequency range are regarded as a FR2-1 band when references from other specifications.

Conclusion

This contribution discusses incoming LS [1] for FR2-NTN inclusion to specifications. The following proposal is made:

Proposal 1: Reply to RAN1 and RAN2 that the notes in NTN frequency ranges table need to be updated for TS 38.101-1/38.133/38.108:

NOTE 2:	NTN bands within this frequency range are regarded as a FR2-1 band when references from other specifications.

- References

[1] R1-2407406, LS on FR2-NTN inclusion to specifications, RAN1#118.