3GPP TSG-RAN WG4 Meeting #113	R4-2418343

# Orlando, US, 18th – 22nd November, 2024

Title:	On remaining issues of FR2 STxMP testing

Source:	Samsung

Agenda item:	7.13.2

Document for:	Discussion

1.	Introduction

In last RAN4 meeting there were much progress on test method of FR2 simultaneous transmission of multi-panels, the default AoA pair was confirmed, the testing skip rule for EIRPmax and TRPmax was agreed upon, the measurement grid was also concluded. The remaining issues are mainly about how to select the AoA pair other than default pair, and a polarization issue for 2AoA case.

In this contribution, we further discuss the above remaining issues.

2. 	Discussion

It was big progress that the default AoA pair selection was agreed in last meeting [1, R4-2416580] which leverages the legacy beam peak as one of the AoA [1, R4-2413534]:

Issue 1-1-1: Default AoA separation and UE orientation for EIRP PUMAX,f,c,k testing - Agreements: - RAN4 confirms the baseline multi-RX/STxMP system is suitable to guarantee the same relative angular separations between specific UE test orientations and probes. - The following test procedure can be considered as the solution to select default AoA separation and UE orientation EIRP PUMAX,f,c,k testing: - AoA1 direction for STxMP testing should re-use the legacy beam peak direction (from single carrier test) for the declared UE orientation (selected from any of the approved STxMP device orientations - For AoA1, no new BP search is necessary for device orientations used for STxMP test that have not been used for single carrier test - AoA2 direction is offset by AoA1 by the declared offset from set of {30deg, 60deg, 90deg, 120deg, 150deg} in xz plane of test system

AoA1 direction is crystal clear, but AoA2 direction seems a little ambiguous. Assume UE declared offset is 90deg, it is not very clear which AoA2 should be exactly selected:

- Option 1: AoA2 = AoA1 + 90deg

- Option 2: AoA2 = AoA1 – 90deg

- Option 3: testing both.

In Multi-Rx testing there is no such ambiguity because the Multi-Rx testing include both AoA+ pair and AoA- pair in test procedure. Given RAN4 has confirmed that the baseline multi-RX/STxMP system is suitable to guarantee the same relative angular separations between specific UE test orientations and probes, the above different options are all feasible, however, from UE declaration perspective, either Option 1 or Option 2 aligns with the real panel design under specific UE orientation. In extreme case that panel1 and panel2 have 90deg angular separation but with exact same performance, the AoA1 direction is possible from either panel1 boresight or panel2 boresight randomly depending on measurement uncertainty, in such case, both Option 1 and Option 2 are possible.

It seems that if AoA1 direction is not declared, RAN4 need to further discuss how to eliminate the ambiguity.

Proposal 1:	if AoA1 direction is not declared, RAN4 to further discuss how to eliminate the ambiguity in AoA2 selection between AoA+offset and AoA-offset in EIRP PUMAX,f,c,k testing, and EIRPmax testing as well.

Because AoA2 direction is non-optimal if AoA1 reusing legacy beam peak direction, it was agreed that AoA1 direction can be different from the legacy beam peak [1]:

Issue 1-1-2: Whether a test direction other than the beam peak direction of single CC operation without STxMP can be used for first AoA - Agreements: - The test direction other than the beam peak direction of single CC operation without STxMP can be used for the first AoA if the test results based on the default AoA separation fail the limits in the spec. The following options can be considered: - Option 1: A local scan in xz plane is conducted. As long as test results do not pass the spec yet during scan, the local scan does not stop until AoA1 EIRP fail the legacy beam peak EIRP limit in both directions. FFS on the measurement grid of local scan. - Option 2: Alternative test directions on top of the default AoA separation can be declared by UE. FFS on the allowed offset of alternative test directions to the default test directions. - Option 1 and Option 2 only apply for EIRP PUMAX,f,c,k testing and will not have the impact on the EIRPmax and TRPmax test directions selection. - RAN4 to make down selection from Option 1 and Option 2 in next meeting

In last meeting companies showed concern on the local scan, instead a declaration of AoA1 is more preferred. AoA1 declaration is good way to avoid complicated local scan.

If AoA1 direction is declared within a range w.r.t the measured beam peak direction, the aforementioned AoA2 ambiguity still exist in case the two panels have similar performance at boresight. But if AoA1 declaration is decoupled from the measured beam peak direction, it is beneficial to eliminate the AoA2 ambiguity.

Observation 1:	AoA1 declaration decoupled from the measured beam peak direction is beneficial to eliminate the AoA2 ambiguity

Given the default AoA pair has bias on the performance at AoA2, it is fair for UE vendors to declare AoA1 direction with high flexibility to compensate such bias.

Observation 2:	High flexibility should be given to UE vendors in the AoA1 declaration to compensate the AoA2 performance bias.

Based on above, it is proposed that the declaration of AoA1 direction is totally up to UE vendor, it can be any direction without reference to the measured beam peak direction.

Proposal 2:	the declaration of AoA1 direction can be any direction.

When the vendor declares the AoA1 direction, it is probably that the default AoA pair is not good enough. It would be a waste to test the default AoA pair firstly and then test the declared AOA pair.

Proposal 3:	for EIRP PUMAX,f,c,k testing, testing default AoA pair if there is no declaration of AoA1 direction, otherwise testing the declared AoA pair.

One more remaining issue is about polarization issue. In the legacy single AoA testing there are four EIRP component testing (2 link pol x 2 measurement pol). The 2 measurement polarizations correspond to  and  EIRP component which can not be reduced. The 2 link polarizations are measured in order to get the max total EIRP between the two link pol.

When EIRP comes to 2AoA case, the whole EIRP measurements would be 4 link pol combinations x 4 EIRP components = 16 EIRP testing. It is suggested to limit the polarization combinations to simplify the test, it is possible to down-select between same link polarization (‘’ and ‘’) and crossed link polarization (‘’ and ‘’).

In [2, R4-2413534] it was assumed up to [40] dB power difference between the rank2 PUSCH transmissions at the test probes:

Issue 1-1-1: Signal level condition for measuring/distinguishing EIRP per TCI - Agreement - RAN4 confirms the feasibility of measuring EIRP per TCI with the configuration of rank 2 PUSCH transmission under SDM scheme. - Up to a power difference of [40dB] is assumed. - Detailed MU analysis can be left to RAN5 if needed, taking into account the exact Test Case parameters.

To guarantee the power difference as high as possible, crossed link polarization may be better choice.

Proposal 4:	It is proposed to limit the link polarization combinations for the 2AoA, and crossed link polarization (‘’ and ‘’) can be considered for the sake of lower interference.

3. 	Conclusion

Proposal 1:	if AoA1 direction is not declared, RAN4 to further discuss how to eliminate the ambiguity in AoA2 selection between AoA+offset and AoA-offset in EIRP PUMAX,f,c,k testing, and EIRPmax testing as well.

Observation 1:	AoA1 declaration decoupled from the measured beam peak direction is beneficial to eliminate the AoA2 ambiguity

Observation 2:	High flexibility should be given to UE vendors in the AoA1 declaration to compensate the AoA2 performance bias.

Proposal 2:	the declaration of AoA1 direction can be any direction.

Proposal 3:	for EIRP PUMAX,f,c,k testing, testing default AoA pair if there is no declaration of AoA1 direction, otherwise testing the declared AoA pair.

Proposal 4:	It is proposed to limit the link polarization combinations for the 2AoA, and crossed link polarization (‘’ and ‘’) can be considered for the sake of lower interference.

4. 	References

- R4-2416580    “WF on [112bis][325] NR_FR2_OTA”, Qualcomm Incorporated

- R4-2413534    “WF on Rel-19 NR_FR2_OTA”, Qualcomm Incorporated