# 3GPP TSG RAN WG4 Meeting#113	R4-2418437

# Orlando, US, Nov 18 – Nov 22, 2024

Source: 	Qualcomm Incorporated

Title: 	Discussion on the demodulation performance requirements for 6Rx devices

Agenda item:	7.1.1.3.2

Document for:	Discussion

# 1. Introduction

During the last meeting, RAN4 discussed issues pertaining to performance requirements for 6Rx devices as well as agreed on an updated set of simulation assumptions for feasibility evaluations [1]. In this paper, we share our views and link-level simulation results for MIMO layer evaluation for the 6Rx device. Additionally, we share data regarding number of PDSCH layers seen in field scenarios for a commercial 6Rx device.

2. Field Tests

In [2] we shared already projected throughput for 6 Rx antennas and 6 layers for a test device in a commercial network. It was shown that large throughput gains of 35% for SSB RSRP > -80 dBm and of 45% between -100dBm and -80dBm can be achieved. Advanced receivers help to increase the gain by 6Rx antennas and 6 layers further.

In this paper, we share additional field data for an commercial Snapdragon X80 device that is equipped with 6Rx antennas, see Snapdragon X80 5G Modem-RF System | Qualcomm. The data has been collected in a commercial network in San Diego in band n41 in an outdoor urban scenario. The velocity was between 30 and 60 miles per hour.

In Fig. 1, the PDSCH number of layers is shown as a function as SSB SNR. Fig. 2 provides the number of layers as a function of SSB RSRP. It is mentioned again that this commercial modem is equipped with 6Rx antennas since this helps to increase the number of layers.

The plots show that 4 layers are seen with probability between 10% and 20% between SNR = 0dB and SNR = 10dB. At SNR > 20 dB, the percentage of four layers even increases to up to 50%. This reveals that a large number of layers is not only achievable at high SNR but can also occur at medium to low SNR if the environment provides rich scattering. At high SNR > 20 dB, the likelihood that four layers are observed with 4Rx antennas is very high.

In the second plot it is seen that percentage of four MIMO layers is between 10% and 20% for RSRP between -100dBm and -80dBm. This confirms the analysis in [2] where it was also shown that in this RSRP region four layers are feasible, and more than four layers can be expected in mid-cell scenarios. Even at cell edge, four layers have been observed. Above -80dBm, the likelihood of four layers increases up to 40% as seen below. In [2] it is projected that in those conditions six layers can be achieved. Indeed, this become likely when already in 40% of the observations four layers can be achieved.

Figure 1: PDSCH Number of Layers vs SSB SNR

Figure 2: PDSCH Number of Layers vs SSB RSRP

Observation 1: With a commercial modem equipped with 6Rx antennas in the field scenarios, 4L is seen with a probability between 10% and 20% between SNR = 0 dB and SNR = 10 dB. At SNR > 20 dB, the percentage of 4L even increases to up to 50%.

Observation 2: With a commercial modem equipped with 6Rx antennas in the field scenarios, 4L is seen with a probability between 10% and 20% for RSRP between -100dBm and -80dBm. The likelihood of 4L increases up to 40% above -80dBm.

Observation 3: Given the likelihood of 4L in a commercial device with 6Rx above -80dBm RSRP is up to 40% and up to 50% for SNR > 20 dB in an urban environment, more than 4L can be expected in mid-cell and cell-center scenarios.

3. Assumption on Tx EVM

Tx EVM for feasibility study

Way Forward: FFS on Tx EVM values to be used for feasibility evaluation. RAN4 to further discuss the following:

- Option 1: Use Tx EVM = 1%

- Option 2: Use Tx EVM = 3% for all of modulation order in MCS Table 2 for performance evaluation

- Option 3: Consider TxEVM of 6% for 64QAM and TxEVM 3% for 256QAM

- Option 4: use the existing BS Tx EVM core requirement

- Option 5: The representative BS Tx EVM values provided by the BS vendor.

As part of the simulation assumptions for the feasibility study, RAN4 discussed the assumption on the Tx EVM. The related way forward for this issue is captured below [1].

During the online discussions several network vendors and operators pointed out that the Tx EVM in practical BS equipment can be much lower than what RAN4 assumes in performance requirements and 1% can be assumed for the feasibility study.

Observation 4: The Tx EVM in practical BS equipment can be much lower than what RAN4 assumes in performance requirements.

Given the input from the network vendors and operators, we are of the opinion that 1% Tx EVM is a reasonable assumption for the 6-layer feasibility study.

Proposal 1: Support option 1 (Use 1% Tx EVM for the feasibility evaluation)

4. Assumption on BS Correlation

As part of the simulation assumptions for the feasibility study, RAN4 also discussed the assumption of BS correlation. The related way forward for this issue is captured below [1].

Tx EVM for feasibility study

Way Forward:

- Option 1: 3GPP MIMO correlation matrix with Alpha = 0.1

- Option 2: 3GPP MIMO correlation matrix with Alpha = 0.0

- Other options are not precluded.

Matrices requested from interested BS vendors for RAN4#113

We note that in a practical deployment, the Tx Ports can be configured in a way that produce near zero correlation at the BS. Besides, for the feasibility study, we don’t think it is necessary to consider BS correlation. Therefore, Alpha can be assumed to be zero in 3GPP MIMO Tx correlation matrix.

Proposal 2: Support option 2 (3GPP MIMO correlation matrix with Alpha = 0.0)

5. 6-layer Support

6-layer Support

Way Forward: RAN4 to further discuss the following options (options are not necessarily mutually exclusive).

-	Option 1: 6-layer support is feasible for handheld and FWA based on some company evaluations considering realistic antenna correlation assumptions and deployment scenarios.

-	Option 2: 6-layer support is feasible for FWA devices.

-	Option 3: Companies to further discuss and align on the feasibility criteria and simulation assumptions by RAN4#113 to evaluate 6-Layer feasibility for handhelds using the following as a starting point.

-	Feasibility criteria

-	Consider 4-layer vs 6-layer throughput performance.

-	Simulation assumptions

-	Only consider link-level simulation for performance evaluations.

-	Updated simulation assumptions are provided in Annex A for discussions on alignment at RAN4#113 and can be used by companies to provide simulation results at RAN4#113 for discussion. Companies to clarify and justify (e.g., where the number comes from) the settings used in the simulation assumptions for BS correlation matrix and Tx EVM.

-	Option 4: 6-layer support is not feasible for handheld UEs.

-	Option 5: Companies to further discuss the following aspect of 6-layer support.

-	If the handheld feasibility is confirmed, then the handheld and FWA devices will share the same 6-layer requirements.

-    If the handheld feasibility is not confirmed, 6-layer requirements will be specified for FWA device only.

During the last meeting, RAN4 discussed the feasibility of introducing 6-layer performance requirements for 6Rx devices. To this end, several updated simulation assumptions were agreed for feasibility evaluations. The related way forward is captured below [1]. In this section, we present our simulation results for 6Rx handheld device showing the feasibility of 6-layer support.

5.1 Simulation Assumptions

We consider the assumptions provided in Table 1 for our simulations. We consider a 4L configuration with 6 receive antennas and 8 transmit antenna ports as a reference scenario. Instead of considering 16 transmit antenna ports as in the simulation assumptions [1], we consider 32 transmit antenna ports as the field data indicates 32 transmit antenna ports configuration is widely configured for TDD scenarios. Also, we used fixed PMI for 8Tx simulations and SVD based precoding (to evaluate beamforming gain) for 32Tx. We note that fixed PMI is as mismatched to the channel as random PMI after one period of coherence time of the channel. Since we update at the very beginning and then exclude several slots, the results are expected to be very similar if one is to assume random PMI.

Table 1: Simulation Assumptions

| Parameter |  | Value |
| --- | --- | --- |
| Duplex mode |  | TDD |
| Bandwidth |  | 40MHz |
| SCS |  | 30 |
| Antenna configuration |  | {8Tx, 6Rx} {32Tx, 6Rx} |
| Propagation channel |  | TDLA30-10 |
| Rank |  | Fixed rank {4,5,6} |
| MCS |  | Adaptive MCS (target BLER 10%) MCS Table 1 and Table 2 |
| PDSCH configuration | Mapping type | Type A |
|  | Starting symbol | 2 |
|  | Length | 12 |
|  | PRB bundling size | 2 |
|  | VRB-to-PRB mapping type | Non-interleaved |
|  | Precoding | Fixed PMI (Rel-15 Type 1 codebook) for 8Tx SVD based precoding for 32Tx |
| PDSCH DMRS configuration | DMRS Type | Type 1 [Rel-15] and eType-1 [Rel-18] |
|  | DMRS Configurations | [Rel-18 1+1(6L), Rel-15 2+0(4L)] [Rel-18 1+1(6L), Rel-18 1+1(4L)] [Rel-15 2+2(6L), Rel-15 2+2(4L)] |
|  | Precoding | Fixed PMI (Rel-15 Type 1 codebook) for 8Tx SVD based precoding for 32Tx |
| UE Correlation matrix |  | Option 1: Option 2: 1.000.100.101.000.300.10 0.300.150.100.141.000.12 0.130.120.300.200.200.120.150.140.130.300.120.20 0.121.000.200.110.120.30 0.110.301.000.100.101.00 Option 3: 1.000.220.221.000.300.580.600.150.150.100.050.150.300.600.580.151.000.150.151.000.150.150.150.100.150.050.100.150.150.150.150.101.000.300.301.00 Option 4: 1.000.100.101.000.250.350.500.100.100.050.050.100.250.500.350.101.000.100.101.000.100.150.100.050.100.050.050.100.100.100.150.051.000.150.151.00 |
| BS correlation matrix |  | 3GPP MIMO correlation matrix with Alpha = 0.0 |
| Tx EVM |  | 1% |

5.2 Simulation Results

5.2.1 Impact of Demodulation Reference Symbols

We note that if more than four MIMO layers shall be supported, six or more DMRS antenna ports need to be configured. This can be achieved by multiple DMRS configurations, such as Rel-15 and Rel-18 eType 1 DMRS configurations.

- Rel-15 Type1 DMRS with two frontloaded DMRS symbols: Type1 DMRS supports only four orthogonal DMRS ports with one frontloaded DMRS symbol.

- Rel-18 eType1 DMRS with one frontloaded and one additional DMRS symbols: eType1 DMRS allows to configure up to eight orthogonal DMRS ports with one frontloaded DMRS symbol [3]. The larger number of DMRS ports per symbol comes at the expense of a reduced frequency density of DMRS resource elements. Using only a single FL DMRS is sensitive to higher Doppler spread. Therefore, we consider a configuration with one frontloaded and one additional DMRS symbols.

The simulations apply MCS link adaptation via ACK/NACK feedback targeting 10% first-tx BLER and fixed rank  {4,5,6} for 6Rx antennas. The number of Tx antenna ports is assumed eight and an advanced R-ML receiver is considered for these results. Table 2 shows the relative throughput for five and six layers with respect to a reference scenario for each SNR value with MCS Table 1. The receiver correlation matrix is option 1 from Table 1 and we assume zero Tx antenna correlation. The reference scenario applies four layers and 6Rx antennas with 8Tx antenna ports and configured with Rel-18 1+1 DMRS configuration. A positive relative throughput means that five and/or six layers provide a gain over four layers for 6Rx antennas with the same number of transmit antenna ports.

Table 2: Throughput performance relative to 6Rx/4L Rel-15 1+1 DMRS with MCS Table 1 and correlation matrix from option 1

| RX/Layer | DMRS Configuration | SNR (dB) | Relative Throughput Gains (%) |
| --- | --- | --- | --- |
| 6RX/5L | Rel-15 2+0 | 26 | 9.4 |
|  |  | 30 | 25.2 |
|  |  | 34 | 25.2 |
|  |  | 38 | 25.2 |
|  | Rel-18 1+1 | 26 | 7.2 |
|  |  | 30 | 24.5 |
|  |  | 34 | 25.2 |
|  |  | 38 | 25.2 |
| 6RX/6L | Rel-15 2+0 | 28 | 11.5 |
|  |  | 30 | 26.8 |
|  |  | 34 | 48.9 |
|  |  | 38 | 50.4 |
|  | Rel-18 1+1 | 28 | 8.8 |
|  |  | 30 | 23.1 |
|  |  | 34 | 49.6 |
|  |  | 38 | 50.4 |

From Table 2, it can be observed that Rel-18 1+1 DMRS configurations offers comparable performance with that of Rel-15 2+0 configuration. Having one additional DMRS symbol, it is expected that Rel-18 1+1 DMRS combination will outperform that of Rel-15 2+0 configuration at higher Doppler scenarios. Furthermore, comparing between 5L and 6L performances with Rel-18 1+1 DRMS configurations, it is noted that 6L starts to outperform 5L at around 30dB, showing the feasibility of 6L performance.

Observation 5: Rel-18 1+1 DMRS configuration provides comparable performance with that of Rel-15 2+0 configuration.

Observation 6: It is expected that Rel-18 1+1 DMRS will perform better at higher Doppler compared to that of Rel-15 2+0 configuration due to additional DMRS symbol.

Observation 7: With Rel-18 1+1 DMRS configuration and MCS Table 1, 6L starts to outperform 5L at around 30dB and 5L starts to outperform 4L at around 24dB.

Table 3: Throughput performance relative to 6Rx/4L Rel-15 1+1 DMRS with MCS Table 2 and correlation matrix from option 1

| RX/Layer | DMRS Configuration | SNR (dB) | Relative Throughput Gains (%) |
| --- | --- | --- | --- |
| 6RX/5L | Rel-15 2+0 | 30 | 3.9 |
|  |  | 32 | 4.4 |
|  |  | 38 | 23.9 |
|  |  | 42 | 24.7 |
|  | Rel-18 1+1 | 30 | 1.2 |
|  |  | 32 | 3.9 |
|  |  | 38 | 24.7 |
|  |  | 42 | 24.7 |
| 6RX/6L | Rel-15 2+0 | 30 | 3.5 |
|  |  | 36 | 6.0 |
|  |  | 38 | 13.1 |
|  |  | 42 | 21.3 |
|  | Rel-18 1+1 | 30 | 2 |
|  |  | 32 | 4.3 |
|  |  | 36 | 10.4 |
|  |  | 42 | 32.8 |

With the same simulation assumptions for the results presented in Table 2, now we analyze the performance with MCS Table 2. With MCS table 2, a higher absolute peak throughput is achieved. Besides, it is seen from Table 3 that Rel-18 1+1 DRMS is already outperforming that of Rel-15 2+0 configuration starting 32dB and the performance gap is further widening at higher SNR.

Observation 8: Rel-18 1+1 DMRS configuration shows noticeable performance gain compared to that of Rel-15 2+0 configuration starting from 32dB SNR.

Observation 9: With Rel-18 1+1 DMRS configuration and MCS Table 2, 6L starts to outperform 5L at around 38dB and 5L starts to outperform 4L at around 22dB.

Table 4: Throughput performance relative to 6Rx/4L Rel-18 1+1 DMRS with MCS Table 1 across correlation matrices from options 1 to 4

| RX/Layer | Rel-18 DMRS Configuration/Correlation Matrix | SNR (dB) | Relative Throughput Gains (%) |
| --- | --- | --- | --- |
| 6RX/5L | 1+1 (Option 1) | 28 | 20 |
|  |  | 30 | 24.5 |
|  |  | 34 | 25.2 |
|  |  | 38 | 25.2 |
|  | 1+1 (Option 2) | 28 | 17.7 |
|  |  | 30 | 24.4 |
|  |  | 34 | 25.2 |
|  |  | 38 | 25.2 |
|  | 1+1 (Option 3) | 28 | 13.6 |
|  |  | 30 | 24.3 |
|  |  | 32 | 25.2 |
|  |  | 34 | 25.2 |
|  | 1+1 (Option 4) | 26 | 6.1 |
|  |  | 28 | 18.6 |
|  |  | 30 | 25.2 |
|  |  | 32 | 25.2 |
| 6RX/6L | 1+1 (Option 1) | 28 | 8.8 |
|  |  | 30 | 23.1 |
|  |  | 34 | 49.6 |
|  |  | 38 | 50.4 |
|  | 1+1 (Option 2) | 28 | 6.1 |
|  |  | 30 | 20.2 |
|  |  | 34 | 48.9 |
|  |  | 38 | 50.4 |
|  | 1+1 (Option 3) | 30 | 15.4 |
|  |  | 32 | 33.7 |
|  |  | 34 | 47.4 |
|  |  | 38 | 50.4 |
|  | 1+1 (Option 4) | 30 | 19.5 |
|  |  | 32 | 38.9 |
|  |  | 34 | 48.9 |
|  |  | 38 | 50.4 |

Table 5: Throughput performance relative to 6Rx/4L Rel-18 1+1 DMRS with MCS Table 2 across correlation matrices from options 1 to 4

| RX/Layer | Rel-18 DMRS Configuration/Correlation Matrix | SNR (dB) | Relative Throughput Gains (%) |
| --- | --- | --- | --- |
| 6RX/5L | 1+1 (Option 1) | 32 | 3.9 |
|  |  | 34 | 14.8 |
|  |  | 36 | 23.7 |
|  |  | 38 | 24.7 |
|  | 1+1 (Option 2) | 34 | 3.1 |
|  |  | 36 | 22.5 |
|  |  | 38 | 24.7 |
|  |  | 40 | 24.7 |
|  | 1+1 (Option 3) | 34 | 9.7 |
|  |  | 36 | 20.2 |
|  |  | 38 | 24.7 |
|  |  | 40 | 24.7 |
|  | 1+1 (Option 4) | 32 | 4.1 |
|  |  | 34 | 13.2 |
|  |  | 36 | 23.4 |
|  |  | 38 | 24.7 |
| 6RX/6L | 1+1 (Option 1) | 34 | 2.0 |
|  |  | 36 | 10.4 |
|  |  | 38 | 21.3 |
|  |  | 40 | 27.9 |
|  | 1+1 (Option 2) | 34 | 0.8 |
|  |  | 36 | 8.6 |
|  |  | 38 | 18.0 |
|  |  | 42 | 26.8 |
|  | 1+1 (Option 3) | 36 | 4.4 |
|  |  | 38 | 14.2 |
|  |  | 40 | 21.9 |
|  |  | 42 | 27.9 |
|  | 1+1 (Option 4) | 34 | 0.3 |
|  |  | 36 | 8.0 |
|  |  | 38 | 17.5 |
|  |  | 40 | 25.1 |

Now we evaluate performance with correlation matrices in options 2-4 from Table 1 and compare with performance observed with correlation matrix from option 1 for both MCS Tables 1 and 2. We note that correlation matrix in options 2-4 show higher correlation on average when compared to correlation matrix in option 1. Therefore, it is of interest to see how much performance degradation is observed due to higher correlation across receive antennas. Compared to performance presented in Table 2, we see from Table 4 that 5L performance is similar across options 1 to 4 with MCS Table 1. On the other hand, the degradation for 6L with correlation matrix from option 2 is minimal, i.e., less than 3%. Since correlation matrices in options 3 and 4 show higher correlation on average, so the degradation is slightly higher when compared to degradation seen with option 2, e.g., around 7% and 4% for options 3 and 4, respectively for MCS Table 1.

Observation 10: Compared to performance with correlation matrix from option 1, negligible performance loss (<3%) is observed when compared to performance with correlation matrix from option 2 and MCS Table 1 for 6L.

Observation 11: Compared to performance with correlation matrix from option 1, minimal losses in performance gains are observed, e.g., 8% and 4% performance losses for options 3 and 4, respectively, with MCS Table 1 for 6L. The SNR loss is around 1.5dB for option 3 and around 1dB for option 4.

Looking at the results with MCS Table 2 from Table 5, we notice similar trends for both 5L and 6L performances as in performance with MCS Table 1. In particular, 5L shows similar performance across options 1 to 4 whereas losses in 6L is no greater than 8% when compared to correlation matrix from option 1.

Observation 12: With MCS Table 2, both 5L and 6L shows similar trends as in performance with MCS Table 1 for correlation matrices across options 1 to 4.

Observation 13: Considering MCS Tables 1 and 2, the simulation results with the proposed ECC matrices from four companies show that both 5L and 6L provide gains over 4L.

We now compare performance results with Rel-15 2+2 DMRS configurations. For these results, we assume the reference scenario is 4L/Rel-15 2+2 DMRS configuration with ideal antenna efficiency of 100%. The objective is to compare the 5L and 6L performance against this reference scenario. We note that the DMRS overhead is significantly increased with this configuration, which can be observed with lower peak throughput, e.g., compared with 2+0 configuration. We consider the correlation matrix in option 1 for these simulations with MCS Table 1 and MCS Table 2.

Table 6: Throughput performance relative to 6Rx/4L Rel-15 2+2 DMRS with MCS Table 1

| RX/Layer | DMRS Configuration | SNR (dB) | Relative Throughput Gains (%) |
| --- | --- | --- | --- |
| 6RX/5L | Rel-15 2+2 | 24 | 5.6 |
|  |  | 26 | 18.8 |
|  |  | 28 | 26.2 |
|  |  | 30 | 25.2 |
| 6RX/6L | Rel-15 2+2 | 26 | 3.9 |
|  |  | 28 | 22.8 |
|  |  | 30 | 36.9 |
|  |  | 32 | 49.5 |

Table 7: Throughput performance relative to 6Rx/4L Rel-15 2+2 DMRS with MCS Table 2

| RX/Layer | DMRS Configuration | SNR (dB) | Relative Throughput Gains (%) |
| --- | --- | --- | --- |
| 6RX/5L | Rel-15 2+2 | 30 | 11.2 |
|  |  | 34 | 23.8 |
|  |  | 36 | 24.9 |
|  |  | 38 | 25.7 |
| 6RX/6L | Rel-15 2+2 | 32 | 0.4 |
|  |  | 34 | 8.7 |
|  |  | 36 | 20.9 |
|  |  | 38 | 31.4 |

From Table 6, we observe that 6L starts to outperform both 4L and 5L around 26dB and 28dB, respectively, with Rel-15 2+2 configuration and MCS Table 1. With MCS Table 2, the relative performance is captured in Table 7.  From these results, we observe that 6L starts to outperform both 4L and 5L around 32dB and 37dB, respectively for Rel-15 2+2 configuration.

Observation 14: For Rel-15 2+2 DMRS configuration with MCS Table 1, 6L starts to outperform both 4L and 5L around 26dB and 28dB, respectively.

Observation 15: For Rel-15 2+2 DMRS configuration with MCS Table 2, 6L starts to outperform both 4L and 5L around 32dB and 37dB, respectively.

Observation 16: With Rel-15 2+2 DMRS configuration, significant throughput loss is observed due to increased DMRS overhead. For example, 21% peak throughput loss is observed for both 5L and 6L with MCS Table 1.

5.2.2 Impact of Number of Transmit Antenna Ports

In this section, we provide our results with larger number of transmit antenna ports. To this end, we consider 32Tx/4L Rel-18 1+1 DMRS as a baseline and compared with that of 32Tx antenna ports for both 5L and 6L. It is assumed that SRS antenna switching is available for TDD scenarios and SVD based beamforming at the transmitter. We assume an update periodicity of 40 slots (i.e., 20ms for TDD scenario), which results in a reasonable trade-off between SRS overhead and tracking the time-selective channel in a realistic deployment. As presented earlier, we observed small performance losses with correlation matrices from options 3 and 4. Therefore, for this study we limit our simulations to correlation matrices from options 1 and 2 to ease the simulation burden.

Table 8: Throughput performance relative to 32Tx/4L Rel-18 1+1 DMRS with MCS Table 1 and 32 transmit antenna ports

| RX/Layer | Rel-18 DMRS Configuration/Correlation Matrix | SNR (dB) | Relative Throughput Gains (%) |
| --- | --- | --- | --- |
| 6RX/5L | 1+1 (Option 1) | 24 | 20.3 |
|  |  | 26 | 25.2 |
|  |  | 28 | 25.2 |
|  |  | 30 | 25.2 |
|  | 1+1 (Option 2) | 24 | 20.0 |
|  |  | 26 | 24.9 |
|  |  | 28 | 25.2 |
|  |  | 30 | 25.2 |
| 6RX/6L | 1+1 (Option 1) | 24 | 14.5 |
|  |  | 26 | 26.9 |
|  |  | 28 | 40.3 |
|  |  | 30 | 47.4 |
|  | 1+1 (Option 2) | 24 | 13.0 |
|  |  | 26 | 24.9 |
|  |  | 28 | 36.9 |
|  |  | 30 | 44.5 |

Table 9: Throughput performance relative to 32Tx/4L Rel-18 1+1 DMRS with MCS Table 2 and 32 transmit antenna ports

| RX/Layer | Rel-18 DMRS Configuration/Correlation Matrix | SNR (dB) | Relative Throughput Gains (%) |
| --- | --- | --- | --- |
| 6RX/5L | 1+1 (Option 1) | 32 | 20.9 |
|  |  | 34 | 24.8 |
|  |  | 36 | 24.8 |
|  |  | 38 | 24.8 |
|  | 1+1 (Option 2) | 32 | 19.6 |
|  |  | 34 | 24.2 |
|  |  | 36 | 23.6 |
|  |  | 38 | 24.2 |
| 6RX/6L | 1+1 (Option 1) | 32 | 18.0 |
|  |  | 34 | 29.5 |
|  |  | 36 | 36.1 |
|  |  | 38 | 42.1 |
|  | 1+1 (Option 2) | 32 | 14.8 |
|  |  | 34 | 26.2 |
|  |  | 36 | 34.4 |
|  |  | 38 | 39.9 |

From Table 8, where the performance results are given with MCS Table 1, we observe that 32 Tx antenna ports offer noticeable performance gain with respect to the reference 32Tx scenario even before reaching peak throughput for both correlation options 1 and 2. Similar gains are expected for correlation options 3 and 4 as well. We see a 4dB improvement in 5L to 6L switchover point with 32Tx antenna ports when compared to baseline scenario for both correlation matrices. Even a 4L configuration sees a 20% peak throughput gain with 32 Tx antenna ports at SNR=20dB when compared to that of 8Tx antenna ports. Furthermore, with 32 Tx antenna ports, 6L configuration starts to outperform that of 4L at around 22dB and 5L configuration starts to outperform that of 4L below 20dB for both correlation matrices. With MCS Table 2, as presented in Table 9, we observe that 6L configuration consistently starts to outperform that of 4L at around 28dB and 5L configuration starts to outperform that of 4L below 20dB for both correlation matrices. Furthermore, we see a 5dB improvement in 5L to 6L switchover point with 32Tx antenna ports when compared to baseline scenario for both correlation matrices with MCS Table 2.

Observation 17: Considering MCS Table 1, for the same DMRS configuration, i.e., Rel-18 1+1 with SRS-AS and SVD based BF for TDD, 5L to 6L switchover point with 32Tx antenna ports is improved by 4dB for both correlation matrices.

Observation 18: Considering MCS Table 2, for the same DMRS configuration, i.e., Rel-18 1+1 with SRS-AS and SVD based BF for TDD, 5L to 6L switchover point with 32Tx antenna ports is improved by 5dB for both correlation matrices.

Observation 19: Considering MCS Table 1, for the same DMRS configuration, i.e., Rel-18 1+1 with SRS-AS and SVD based BF for TDD, 6L configuration starts to outperform that of 4L at around 22dB and 5L configuration starts to outperform that of 4L below 20dB for both correlation matrices.

Observation 20: Considering MCS Table 2, for the same DMRS configuration, i.e., Rel-18 1+1 with SRS-AS and SVD based BF for TDD, 6L configuration starts to outperform that of 4L at around 28dB and 5L configuration starts to outperform that of 4L below 20dB for both correlation matrices.

5.2.3 Impact of Non-ideal Antenna Efficiency

During the last meeting, it was discussed that RAN4 should consider antenna efficiency into the envelope correlation coefficients (ECC) for link level simulations. Keeping this discussion in mind, now we consider non-ideal efficiency across 6Rx antennas, where we assume that antenna efficiency is no better than -3.5dB and no worse than -7.5dB across 6Rx antennas. We consider 32Tx antennas and correlation matrix in option 1 for these simulations. The goal is to show that degradation due to non-ideal antenna efficiency can be recouped via BF gain. The throughput performance relative to bassline scenario (32Tx/4L with non-ideal efficiency) are provided in Table 11.

Table 10: Throughput performance relative to 32Tx/4L Rel-18 1+1 DMRS with MCS Table 2 in the presence of non-ideal antenna efficiency

| RX/Layer | Rel-18 DMRS Configuration/Correlation Matrix | SNR (dB) | Relative Throughput Gains (%) |
| --- | --- | --- | --- |
| 6RX/5L | 1+1 (Option 1) | 36 | 22.0 |
|  |  | 38 | 26.2 |
|  |  | 40 | 26.6 |
|  |  | 42 | 26.6 |
| 6RX/6L | 1+1 (Option 1) | 36 | 12.7 |
|  |  | 38 | 24.0 |
|  |  | 40 | 33.3 |
|  |  | 42 | 41.3 |

From Table 10, we see that even in the presence of non-ideal antenna efficiency, 32Tx configuration offer significant performance gain for both 5L and 6L over 4L, with a 5L to 6L switch over SNR of 38dB.

Observation 21: In the presence of non-ideal antenna efficiency, 32Tx configuration still offer significant performance gain for both 5L and 6L over 4L and roughly in line when compared to that of ideal antenna efficiency. The switch over SNR from 5L to 6L is around 38.5dB.

5.4 Summary of Simulation Results

Based on the updated agreed correlation matrices, in this section we have provided simulation results with different DMRS overhead (both Rel-15 and Rel-18), MCS Tables and increased number of transmit antenna ports. It is observed that Rel-18 1 1+1 DMRS configuration provides comparable performance compared to Rel-15 2+0 configurations. We also observed 6Rx/6L configuration outperforms that of 6Rx/4L at around SNR = 27 dB and 6Rx/5L at around SNR = 30dB for MCS Table 1. When MCS table 2 is considered, the switch over points to 6L from 4L and 5L shifts to 33dB and 39dB, respectively. Although higher correlation values are seen in options 3 and 4, we see a small performance loss with these correlation matrices. More importantly, with the ECC matrices from four companies, both 5L and 6L provide gains over 4L at practical SNR values. It is also observed that considering non-ideal antenna efficiency into the ECC values causes a degradation in performance.

Although Rel-15 2+2 DMRS configurations offer same DMRS overhead across 4L/5L/6L, increased DMRS overhead causes 21% peak throughput loss, therefore may not be attractive for practical deployments. Additionally, field data provided in this paper suggests that 4L occurs with up to 50%, and more than 4L can be expected in mid-cell and cell-center scenarios, in which case network may not configure Rel-15 2+2 DMRS configurations for 6L scheduling.

Additionally, we have examined performance with 32 transmit antenna ports and compared with the baseline configuration of 8 transmit antenna ports. It has been noticed that with MCS Table 1, a configuration of 32 transmit antennas can offer significant gain, up to 3dB and 4dB for 5L and 6L, respectively and the switch over point from 5L to 6L is improved by 4dB. With MCS Table 2, a configuration of 32 transmit antennas can offer up to 2dB and 5dB for 5L and 6L, respectively, while the switch over point from 5L to 6L is improved by 5dB. This is assuming all other simulation assumptions remain unchanged. These observations suggest that 32Tx configuration that is commonly observed in the field, can significantly bring down the SNR crossing points. Finally, we show that having large number of transmit antennas can also help to recoup the performance losses observed due to non-ideal antenna efficiency, therefore antenna efficiency may not be a limiting factor in practical deployment scenarios.

Based on the observations from the simulation results with different DMRS types and configurations, as well as number of transmit antenna ports and practical UE and BS correlation assumptions, we think that it is feasible to define 6-layer performance requirements for 6Rx devices.

Proposal 3: RAN4 to introduce 6-layer performance requirements for handheld and FWA devices.

6. Conclusions

In this paper, we share observations from field data and provide link-level simulation results for the 6Rx device. The following have been proposed.

Observation 1: With a commercial modem equipped with 6Rx antennas in the field scenarios, 4L is seen with a probability between 10% and 20% between SNR = 0 dB and SNR = 10dB. At SNR > 20 dB, the percentage of 4L even increases to up to 50%.

Observation 2: With a commercial modem equipped with 6Rx antennas in the field scenarios, 4L is seen with a probability between 10% and 20% for RSRP between -100dBm and -80dBm. The likelihood of 4L increases up to 40% above -80dBm.

Observation 3: Given the likelihood of 4L in a commercial device with 6Rx above -80dBm RSRP is up to 40% and up to 50% for SNR > 20 dB in an urban environment, more than 4L can be expected in mid-cell and cell-center scenarios.

Observation 4: The Tx EVM in practical BS equipment can be much lower than what RAN4 assumes in performance requirements.

Proposal 1: Support option 1 (Use 1% Tx EVM for the feasibility evaluation.

Proposal 2: Support option 2 (3GPP MIMO correlation matrix with Alpha = 0.0)

Observation 5: Rel-18 1+1 DMRS configuration provides comparable performance with that of Rel-15 2+0 configuration.

Observation 6: It is expected that Rel-18 1+1 DMRS will perform better at higher Doppler compared to that of Rel-15 2+0 configuration due to additional DMRS symbol.

Observation 7: With Rel-18 1+1 DMRS configuration and MCS Table 1, 6L starts to outperform 5L at around 30dB and 5L starts to outperform 4L at around 24dB.

Observation 8: Rel-18 1+1 DMRS configuration shows noticeable performance gain compared to that of Rel-15 2+0 configuration starting from 32dB SNR.

Observation 9: With Rel-18 1+1 DMRS configuration and MCS Table 2, 6L starts to outperform 5L at around 38dB and 5L starts to outperform 4L at around 22dB.

Observation 10: Compared to performance with correlation matrix from option 1, negligible performance loss (<3%) is observed when compared to performance with correlation matrix from option 2 and MCS Table 1 for 6L.

Observation 11: Compared to performance with correlation matrix from option 1, minimal losses in performance gains are observed, e.g., 8% and 4% performance losses for options 3 and 4, respectively, with MCS Table 1 for 6L. The SNR loss is around 1.5dB for option 3 and around 1dB for option 4.

Observation 12: With MCS Table 2, both 5L and 6L shows similar trends as in performance with MCS Table 1 for correlation matrices across options 1 to 4.

Observation 13: Considering MCS Tables 1 and 2, the simulation results with the proposed ECC matrices from four companies show that both 5L and 6L provide gains over 4L.

Observation 14: For Rel-15 2+2 DMRS configuration with MCS Table 1, 6L starts to outperform both 4L and 5L around 26dB and 28dB, respectively.

Observation 15: For Rel-15 2+2 DMRS configuration with MCS Table 2, 6L starts to outperform both 4L and 5L around 32dB and 37dB, respectively.

Observation 16: With Rel-15 2+2 DMRS configuration, significant throughput loss is observed due to increased DMRS overhead. For example, 21% peak throughput loss is observed for both 5L and 6L with MCS Table 1.

Observation 17: Considering MCS Table 1, for the same DMRS configuration, i.e., Rel-18 1+1 with SRS-AS and SVD based BF for TDD, 5L to 6L switchover point with 32Tx antenna ports is improved by 4dB for both correlation matrices.

Observation 18: Considering MCS Table 2, for the same DMRS configuration, i.e., Rel-18 1+1 with SRS-AS and SVD based BF for TDD, 5L to 6L switchover point with 32Tx antenna ports is improved by 5dB for both correlation matrices.

Observation 19: Considering MCS Table 1, for the same DMRS configuration, i.e., Rel-18 1+1 with SRS-AS and SVD based BF for TDD, 6L configuration starts to outperform that of 4L at around 22dB and 5L configuration starts to outperform that of 4L below 20dB for both correlation matrices.

Observation 20: Considering MCS Table 2, for the same DMRS configuration, i.e., Rel-18 1+1 with SRS-AS and SVD based BF for TDD, 6L configuration starts to outperform that of 4L at around 28dB and 5L configuration starts to outperform that of 4L below 20dB for both correlation matrices.

Observation 21: In the presence of non-ideal antenna efficiency, 32Tx configuration still offer significant performance gain for both 5L and 6L over 4L and roughly in line when compared to that of ideal antenna efficiency. The switch over SNR from 5L to 6L is around 38.5dB.

Proposal 3: RAN4 to introduce 6-layer performance requirements for handheld and FWA devices.

7. References

[1] R4-2417209, “WF on requirements for 6Rx,” 3GPP TSG RAN WG Meeting #112-bis, Hefei, CN, Oct 14 – 18, 2024.

[2] R4-2411680, “Discussion on the demodulation performance requirements for 6Rx devices,” 3GPP TSG RAN WG Meeting #112, Maastricht, NL, Aug 19 – 23, 2024.

[3] TS 38.211, ‘Physical channels and modulation (Release 18)’, v18.2.0, March 2024