# 3GPP TSG-RAN WG4 Meeting #113                                R4-2418448

# Orlando, US, 18th – 22nd November, 2024

Agenda Item:	7.20.3.1

Source: 	CMCC

Title: 			 Discussion on RRM requirements for UE-to-UE CLI handling

Document for:	Discussion

- Introduction

In RAN #104 meeting, the revised WID on SBFD was approved [1]. According to the WID, the objectives having RAN4 impact are duplicated as following.

- Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4] - Information exchange among gNBs, including [RAN3] - Semi-static cell-specific SBFD time and frequency location configuration - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS - Strongest DL beam information - CLI-mitigation request - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4] - Indication/determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain - PUSCH resource mapping, i.e., rate-matching around the muted REs - UCI resource determination in symbols with muted REs - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource, and configuration/determination of ‘typeD’ QCL assumptions for the CLI measurement resource - At least aperiodic reporting - New report quantities, e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices - UCI bits generation - Priority rules for multiple CSI reporting - CLI measurement accuracy requirement - Note: Without dedicated optimization for dynamic/flexible TDD. - Specify BS RF requirements for SBFD operation at gNB [RAN4] - Specify applicable RRM core requirements for CLI handling mechanisms [RAN4] - Specify other RRM core requirements for SBFD operation, if identified [RAN4]

In last meeting, there was initial discussion on SBFD, and  a WF was agreed [2]. This contribution provides discussion on this topic.

- Discussion

RAN1 #118 Agreements For L1 UE-to-UE CLI measurement and reporting, CLI measurements is performed within the active DL BWP and the following are supported - Method#1: UE measures RSSI within DL subband - Method#2: UE measures RSRP of aggressor UE within UL subband - FFS: Method#3: UE measures RSSI within UL subband For frequency resource allocation of a CLI-RSSI measurement resource, the following are supported - Measurement resource type#1: One CLI-RSSI measurement resource is configured within a DL subband - Measurement resource type#2: One CLI-RSSI measurement resource is configured across two DL subbands - FFS: Number of resources that can be configured - FFS: UE behavior for measurement For L1 UE-to-UE CLI measurement and reporting, the following are supported - Wideband CLI-RSRP reporting - Wideband CLI-RSSI reporting - FFS: Subband CLI-RSSI reporting RAN1 #118bis Agreements For Method#1 (RSSI measurement within DL subband), the frequency resources of CLI-RSSI measurement resource type#2 (One CLI-RSSI measurement resource across two DL subbands) is derived by excluding the frequency resources outside DL usable PRBs. - A single wideband RSSI measurement report is supported - FFS: two per-DL-subband CLI-RSSI measurements reports with wideband report in each DL-subband. - For L1 CLI-RSSI measurement, DL timing is used for Method #1 (UE measures RSSI within DL subband).

As agreed in RAN1, method #1 and method #2 are supported. UE measurement behaviour may be different for different method. In RAN1#118bis meeting, it was agreed that for L1 CLI-RSSI measurement, DL timing is used for Method #1. If different DL timing, e.g. a constant offset relative to the downlink timing, is adopted for method #2, the RRM requirements, e.g. side conditions, schduling restriction, will be different for different method. It is proposed to consider both method #1 and method #2. As for other method, it is pending on  RAN1 further discussion.

Proposal 1: it is proposed that RAN4 define requirements at least for following methods:

- Method#1: UE measures RSSI within DL subband

- Method#2: UE measures RSRP of aggressor UE within UL subband

Agreement in RAN1#118 meeting For frequency resource allocation of a CLI-RSSI measurement resource, the following are supported - Measurement resource type#1: One CLI-RSSI measurement resource is configured within a DL subband - Measurement resource type#2: One CLI-RSSI measurement resource is configured across two DL subbands - FFS: Number of resources that can be configured - FFS: UE behavior for measurement For L1 UE-to-UE CLI measurement and reporting, the following are supported - Wideband CLI-RSRP reporting - Wideband CLI-RSSI reporting - FFS: Subband CLI-RSSI reporting - Agreement in RAN1#118bis meeting For Method#1 (RSSI measurement within DL subband), the frequency resources of CLI-RSSI measurement resource type#2 (One CLI-RSSI measurement resource across two DL subbands) is derived by excluding the frequency resources outside DL usable PRBs. - A single wideband RSSI measurement report is supported - FFS: two per-DL-subband CLI-RSSI measurements reports with wideband report in each DL-subband.

According to RAN1 agreements, there are two measurement resource type: CLI-RSSI measurement resource is configured within a DL subband or the CLI-RSSI measurement resource is configured across two DL subbands. As for reporting, RAN1 agreed that wideband reporting is supported, and FFS for sub-band RSSI reporting. From RAN4 requirements point of view, we can firstly focus on wideband. Whether and how to consider sub-band reporting can be further discussed pending on RAN1 progress.

Proposal 2: it is proposed that RAN4 firstly define requirements based on wideband reporting, and further discuss sub-band reporting pending on RAN1 progress.

Issue 1-1-7: Measurement period - Proposals for L1-SRS-RSRP - Proposal 1 (vivo): - Requirements based on 1 sample measurement assumption should be used. - Proposal 1a (QC, Ericson): - RAN4 to define the measurement delay for aperiodic reporting of L1-SRS-RSRP at least based on a single sample. - Proposal 2 (CATT): - Re-use the framework of R16 CLI measurement period except for the sample number. - Proposal 3 (Nokia): - Follow L1-RSRP measurement approach, and both 1 and 3 samples are considered based on network configuration - Proposal 3a (Samsung): - RAN4 can evaluate L1 SRS-RSRP measurement performance considering both single shot and multiple shots measurements if different side condition identified

For Rel-16 L3 based SRS-RSRP measurements, measurement period are based on 3 samples. For Rel-19 L1-SRS-RSRP, less samples are expected since the major benifit is shorter measurement delay. According to the agreed simulation assumption [4], 1, 2, 3 are considered for the number of samples. The measurement period for L1-SRS-RSRP can be further decided based on the simulation results. As for  CLI-RSSI, one shot is used for L3-CLI-RSSI, which can be reused for L1-CLI-RSSI.

Proposal 3: for L1-CLI-RSSI, the measurement period is defined based on one shot.

Issue 1-1-8: Measurement reporting - Proposals - Proposal 1 (vivo): - Define requirements for periodic, semi-persistent reporting. - Proposal 2 (Samsung, CTC, HW, MTK): - Focus on aperiodic CSI reporting mode for Rel-19 CLI measurement, for other reporting modes hold on the discussion pending on further progress in RAN1 - Proposal 3 (CATT, Nokia, HW, E///): - L1-RSRP measurement reporting requirement for aperiodic reporting can be reused, and wording can be adapted when reporting scheme design is stable. - Proposal 4 (MTK): - FFS whether L1-RSRP measurement reporting requirement can be re-used pending on RAN1 agreement.

As duplicated as following, RAN1 discussion mainly focus on aperiodic reporting. RAN4 discussion can focus on aperiodic reporting, and other reporting modes can be further discussed pending on RAN1 propgress.

Agreements in RAN1#118bis Extend CSI-ResourceConfig to include two CLI measurement resource set lists for SRS-RSRP and CLI-RSSI measurements based on Rel-16 SRS-ResourceConfigCLI and rssi-ResourceConfigCLI defined in MeasObjectCLI for L3 based SRS-RSRP and CLI-RSSI measurement. - The resourceType for the two new CLI measurement resource set lists can be set to periodic, semi-persistent or aperiodic - Note: No new usage needs to be defined for SRS resource sets For aperiodic SRS-RSRP reporting using aperiodic SRS-RSRP measurement resource set, the slot offset between the slot containing the DCI that triggers a set of aperiodic SRS-RSRP resources and the slot in which the SRS-RSRP resource set is measured is configured by higher layer parameter. - FFS: Whether to reuse the legacy SRS resource set - FFS: Whether available slot offset list is also reused For aperiodic CLI-RSSP reporting using aperiodic CLI-RSSI measurement resource set, the slot offset between the slot containing the DCI that triggers a set of aperiodic CLI-RSSI resources and the slot in which the CLI-RSSI resource set is measured is configured by higher layer parameter.

Proposal 4: for L1 CLI measurement, it is proposed that RAN4 firstly focus on aperiodic reporting, other reporting modes can be further discussed pending on RAN1 propgress.

- Conclusion

This contribution provides discussion on RRM requirements for SBFD. The proposals are:

Proposal 1: it is proposed that RAN4 define requirements at least for following methods:

- Method#1: UE measures RSSI within DL subband

- Method#2: UE measures RSRP of aggressor UE within UL subband

Proposal 2: it is proposed that RAN4 firstly define requirements based on wideband reporting, and further discuss sub-band reporting pending on RAN1 progress.

Proposal 3: for L1-CLI-RSSI, the measurement period is defined based on one shot.

Proposal 4: for L1 CLI measurement, it is proposed that RAN4 firstly focus on aperiodic reporting, other reporting modes can be further discussed pending on RAN1 propgress.

- Reference

- RP-241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)

- R4-2416858, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon

- R4-2413900, WF on SBFD RRM, Huawei, HiSilicon

- R4-2416910, Simulation assumption for evaluate L1- SRS-RSRP measurement requirements, Ericsson