# 3GPP TSG-RAN WG4 Meeting #113                                                       R4-2418484

# Orlando, USA, 18th – 22rd November, 2024

Source:	ZTE Corporation, Sanechips

Title:	Discussion on IoT NTN TDD mode

Agenda item:	7.28.2

Document for:	Approval

Introduction

At RAN 105# meeting, the revised WID on IoT NTN TDD mode [1] has been approved, and several objectives in the WID have RAN4 RRM involvement. In this meeting, we will provide the initial analysis on the RAN4 impact on periodic pattern.

Discussion

The work item description can be seen as below:

This work item includes the following objectives: - Study the impact due to the periodic pattern, at least on UE downlink synchronization and other aspects (if identified) [RAN1, RAN4] - Checkpoint in RAN#106 for the completion of the study phase. RAN1 start from Oct’24. RAN4 start from Nov’24 - Specify a new NB-IoT TDD NTN mode based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures, based on the outcome of the study, including: - Definition, configuration (if needed) and signaling (if needed) of the periodic pattern including confirming the value of N, and associated UE procedures [RAN1, RAN2] - Other necessary impacts on higher layers [RAN2] - RRM and RF core requirements [RAN4] - Specify a new NB-IoT TDD operating NTN band for the MSS allocation spanning 1616-1626.5 MHz for DL and UL, based on the outcome of the study, to be used as example band for this WI [RAN4]. - Specify band numbering - Specify SAN and UE RF characteristics - Specify DL and UL channelization. - Specify channel bandwidth as 200 kHz - Note1: No NTN-NTN coexistence study needed. - Note2: Leverage existing work as much as possible for TN-NTN coexistence of adjacent bands Note3: Reuse the existing RAN4 requirements as much as possible (no intention to have any relaxation of the RF emissions requirements)

- In this meeting, we will discuss the scenarios and express our views on potential impact to RAN4 requirements towards the above objective based on RAN1’s discussion.

  - 2.1 RRM impact

- In RAN1, the concrete periodic pattern is still under discussion and the following figure shows us the specific periodic pattern:

- Figure 1 Periodic pattern

- We find that there are three parts which contains:

- Downlink subframe: The number of consecutive DL subframes at the behinning of each DL-UL pattern.

- Uplink subframe: The number of consecutive UL subframesof each DL-UL pattern.

- Guard period: Indicates the downlink to uplink guard period for TDD operation.

- RAN1 is discussing the number of downlink/uplink/guard period without any conclusion. In the current WID, it says that the periodic pattern should consist of non-overlapping set of usable contiguous UL subframes and set of usable contiguous DL subframes, and guard periods, which is periodic every N radio frames, with N=9 as baseline. Thus, the periodicity is 9 and the frame structure will be impacted. First of we can see the FDD mode frame structure as below [2]:

- Figure 2 FDD mode frame structure

- Based on the N=9 periodic pattern, the TDD mode frame structure can be seen as below [3]:

- Figure 3 TDD mode frame structure

- We find that in legacy FDD mode, the NPSS is sent in 6th subframe of every radio frame, while NSSS is sent in the 10th subframe every other radio frames. However, we can see figure 3 that the periodicity of the NPSS and NSS will also increase, at least extend 9 times than legacy FDD mode. Then the UE can only search the NPSS/NSSS with a larger period than legacy FDD, the following table can be seen:

- Table 1 The detection time for NPSS/NSSS/NPBCH (at least N=9)

| - Channel | - Legacy NB-IoT NTN | - TDD NB-IoT NTN |
| --- | --- | --- |
| - NPSS | - Once every 10ms | - Once every 90ms |
| - NSSS | - Once every 20ms | - Once every 180ms |
| - NPBCH | - 64 transmissions in 640ms TTI | - 7 or 8 transmissions in 640ms TTI |

- Actually, other working groups are still discussing the concrete frame structure for IoT NTN TDD mode and based on the latest discussion from RAN1, some RRM measurement requirements will be impacted by extended NPSS and NSSS detection time. To my understanding, the extension of periodicity for NPSS/NSS will increase the latency for performing RRM measurement. The impacted RRM requirements are following:

- Cell re-selection (Tdetect, Tevaluation,Tmeasure);

- Handover (Tinterruption);

- RRC re-establishment (Tsearch, TSI, TPRACH);

- UE measurements procedure in RRC Connected state;

- etc.

- Observation 1: The periodicity of periodic pattern at least equals 9, and the periodicity of NPSS/NSS/NPBCH are also extended.

- Proposal 1: RAN4 shall firstly study the requirements which may be impacted as below:

- Cell re-selection (Tdetect, Tevaluation,Tmeasure);

- Handover (Tinterruption);

- RRC re-establishment (Tsearch, TSI, TPRACH);

- UE measurements procedure in RRC Connected state;

- etc.

- Proposal 2: RAN4 to wait for more progress from other WGs on periodic pattern to discuss the RRM impacts.

- To my understanding, the next impact will be timing requirements. We know that guard period aim to prevent interference between UL and DL, thus we should consider the large propagation delay in NTN scenario. We can see the following from TR 38.811:

When considering TDD mode, a guard time is necessary to prevent UE to simultaneously Transmit and reception. This guard time directly depends on the propagation delay between UE and gNB. This guard time will directly impact the useful throughput and hence the spectral efficiency. When considering Non-Terrestrial networks, this guard time should commensurate the round trip delay.

- In the worst case, the guard period can be as large as 12.89ms for LWO-600km and 20.89ms for LEO-1200km to prevent the overlapping between UL and DL considering the potential propagation delay on the service link. Thus, the large TA exists in IoT NTN TDD mode.  We shall analyze whether the timing requirements will be impacted especially Tp and Tq.  We can see the following contents in current spec:

7.20A UE transmit timing for NB-IoT for Satellite Access 1) The maximum amount of the magnitude of the timing change, apart from a change of NTA, UE−specific due to satellite position update and NTA,common between the previous transmission and the current transmission, in one adjustment shall be 58.33*TS seconds. 2) The minimum aggregate adjustment rate, apart from a change of NTA, UE−specific due to satellite position update and NTA,common between the previous transmission and the current transmission, shall be 7*TS per 1second. 3) The maximum aggregate adjustment rate, apart from a change of NTA, UE−specific due to satellite position update and NTA,common between the previous transmission and the current transmission, shall be 58.33*TS per 200ms. 7.24A UE transmit timing for Category M1 for Satellite Access 1) The maximum amount of the magnitude of the timing change, apart from a change of NTA,adjUE due to satellite position update and NTA,adjcommon between the previous transmission and the current transmission, in one adjustment shall be Tq_NTN_M1 seconds. 2) The minimum aggregate adjustment rate, apart from a change of NTA,adjUE due to satellite position update and NTA,adjcommon between the previous transmission and the current transmission, shall be 7*TS per second. 3) The maximum aggregate adjustment rate, apart from a change of NTA,adjUE due to satellite position update and NTA,adjcommon between the previous transmission and the current transmission, shall be Tq_NTN_M1 per 200ms. Where the maximum autonomous time adjustment step Tq_NTN_M1 is specified in Table 7.24A.2-2. Table 7.24A.2-2: Tq_NTN_M1 Maximum Autonomous Time Adjustment Step CE Mode; Tq_NTN_M1_. A; 17.5*TS. B; 17.5*Ts. Note 1: TS is the basic timing unit defined in TS 36.211. Note 2: This requirement applies regardless of the downlink carrier bandwidth..

- We find that when define the Tp and Tq requirements, the satellite position update has already been precluded. Also, the UE speed in NTN scenario is not similar to HST or ATG which is 3km/h. Thus, the impact of UE speed can be precluded, such as time drift and frequency error etc. However, we find that the legacy Tp and Tq of IoT NTN are reused the value of  legacy IoT TN. To my understanding, the legacy Tp and Tq in IoT NTN can be reused in IoT NTN TDD mode.

- Observation 2: The legacy Tp and Tq of IoT NTN are reused the value of  legacy IoT TN.

- Proposal 3: The legacy Tp and Tq in IoT NTN can be reused in IoT NTN TDD mode.

  - 2.2 UE RF impact

The impact on UE RF requirements due to periodic pattern is listed in the following table.

Table 2: Impact on UE RF requirements due to periodic pattern

| IoT-NTN UE Tx/Rx requirement | Whether there is the impact due to periodic pattern |
| --- | --- |
| - 6.2A.1 UE maximum output power for category M1 | No. |
| - 6.2B.1 UE maximum output power for category NB1 and NB2 |  |
| - 6.2A.2 UE maximum output power reduction for category M1 | No. |
| - 6.2B.2 UE maximum output power reduction for category NB1 and NB2 |  |
| - 6.2A.3 A-MPR for category M1 UE | No. |
| - 6.2B.3 A-MPR for category NB1 and NB2 UE |  |
| - 6.2A.4 Configured transmitted Power for category M1 | No. |
| - 6.2B.4 Configured transmitted Power for category NB1 and NB2 |  |
| 6.3A Output power dynamics for category M1 | No. |
| 6.3B Output power dynamics for category NB1 and NB2 |  |
| 6.4A Transmit signal quality for category M1 | No. |
| 6.4B Transmit signal quality for category NB1 and NB2 |  |
| 6.4.2 Transmit modulation quality |  |
| 6.5A.2 Occupied bandwidth for category M1 | No. |
| 6.5B.2 Occupied bandwidth for category NB1 and NB2 |  |
| 6.5A.3.2 Spectrum emission mask | No. |
| 6.5B.3.2 Spectrum emission mask |  |
| 6.5A.3.3 Additional Spectrum Emission Mask for category M1 | No. |
| 6.5B.3.3 Additional Spectrum Emission Mask for category NB1 and NB2 |  |
| 6.5A.3.4 Adjacent Channel Leakage Ratio for category M1 | No. |
| 6.5B.3.4 Adjacent Channel Leakage Ratio for category NB1 and NB2 |  |
| 6.5A.4.2 General spurious emissions | No. |
| 6.5B.4.2 General spurious emissions |  |
| 6.5A.4.3 Spurious emission band UE co-existence | No. |
| 6.5B.4.3 Spurious emission band UE co-existence |  |
| 6.5A.4.4 Additional spurious emissions | No. |
| 6.5B.4.4 Additional spurious emissions |  |
| 6.6A Transmit intermodulation for category M1 | No. |
| 6.6B Transmit intermodulation for category NB1 and NB2 |  |
| 7.3A Reference sensitivity power level for UE category M1 | No. |
| 7.3B Reference sensitivity power level for UE category NB1 and NB2 |  |
| 7.4A Maximum input level for category M1 | No. |
| 7.4B Maximum input level for category NB1 and NB2 |  |
| 7.5A Adjacent Channel Selectivity for category M1 | No. |
| 7.5B Adjacent Channel Selectivity for category NB1 and NB2 |  |
| 7.6A.2 In-band blocking requirements for category M1 | No. |
| 7.6B.2 In-band blocking requirements for category NB1 and NB2 |  |
| 7.6A.3 Out-of-band blocking requirements for category M1 | No. |
| 7.6B.3 Out-of-band blocking requirements for category NB1 and NB2 |  |
| 7.6A.4 Narrow band blocking for category M1 | No. |
| 7.6B.4 Narrow band blocking for category NB1 and NB2 |  |
| 7.7A Spurious response for category M1 | No. |
| 7.7B Spurious response for category NB1 and NB2 |  |
| 7.8A Intermodulation characteristics for category M1 | No. |
| 7.8B Intermodulation characteristics for category NB1 and NB2 |  |
| 7.9 Spurious emissions | No. |
| A.1 DL reference measurement channels | Yes. NPDSCH Reference Channel for TDD should be introduced. Fixed Reference Channel for TDD should be introduced. |
| A.2 OFDMA Channel Noise Generator (OCNG) | Yes. OCNG Patterns for TDD should be introduced. |
| A.3 Testing related to Satellite Access | No. |
| A.4 UL reference measurement channels | Yes. Reference measurement channels for TDD should be introduced. |

Observation 3: There is impact on UE RF requirements including DL reference measurement channels, OCND Patterns and UL reference measurement channels due to periodic pattern.

  - 2.3 SAN RF requirements

The impact on SAN RF requirements due to periodic pattern is listed in the following table.

Table 3: Impact to SAN RF requirements due to periodic pattern

| IoT-NTN SAN RF Tx/Rx requirements | Whether there is the impact due to periodic pattern |
| --- | --- |
| 6.2 Satellite Access Node output power | No. |
| 6.3.2 RE power control dynamic range | No. |
| 6.3.3 Total power dynamic range | No. |
| 6.4 Transmit ON/OFF power | No. |
| 6.5 Transmitted signal quality | No. |
| 6.6.2 Occupied bandwidth | No. |
| 6.6.3 Adjacent Channel Leakage Power Ratio | No. |
| 6.6.4 Out-of-band emissions | No. |
| 6.6.5 Transmitter spurious emissions | No. |
| 6.7 Transmitter intermodulation | No. |
| 7.2 Reference sensitivity level | No. |
| 7.3 Dynamic range | No. |
| 7.4.1 Adjacent Channel Selectivity (ACS) | No. |
| 7.4.2 In-band blocking | No. |
| 7.5 Out-of-band blocking | No. |
| 7.6 Receiver spurious emissions | No. |
| 7.7 Receiver intermodulation | No. |
| 7.8 In-channel selectivity | No. |
| Annex A (normative): Reference measurement channels | No. |

Observation 4: There is no impact on SAN RF requirements due to periodic pattern.

Conclusion

In this paper we provided our views on RRM requirements on R19 IoT NTN TDD mode enhancement.

- Observation 1: The periodicity of periodic pattern at least equals 9, and the periodicity of NPSS/NSS/NPBCH are also extended.

- Proposal 1: RAN4 shall firstly study the requirements which may be impacted as below:

- Cell re-selection (Tdetect, Tevaluation,Tmeasure);

- Handover (Tinterruption);

- RRC re-establishment (Tsearch, TSI, TPRACH);

- UE measurements procedure in RRC Connected state;

- etc.

- Proposal 2: RAN4 to wait for more progress from other WGs on periodic pattern to discuss the RRM impacts.

- Observation 2: The legacy Tp and Tq of IoT NTN are reused the value of  legacy IoT TN.

- Proposal 3: The legacy Tp and Tq in IoT NTN can be reused in IoT NTN TDD mode.

Observation 3: There is impact on UE RF requirements including DL reference measurement channels, OCND Patterns and UL reference measurement channels due to periodic pattern.

Observation 4: There is no impact on SAN RF requirements due to periodic pattern.

- 4 References

- RP-242415, New WID on introduction of IoT-NTN TDD mode, Iridium Satellite LLC

- TS36.133

- R1-2408871, Qualcomm Incorporated

- R1-2407772, Thales

- R1-2407937, ZTE Corporation, Sanechips