3GPP TSG-RAN4 Meeting  #113	R4-2418531

Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.6.2

Source:	Apple

Title:	On methods for reducing the number of UE Rx chains for fragmented carriers

Document for:	Discussion

# 1	Introduction

The objective of this study is as follows:

-	Identify methods for reducing the number of UE Rx chains (e.g. from separate RF chains per CC to shared RF chains ) needed for single DL band with frequency span ≤ 100 MHz, containing two non-contiguous CCs within a CA combination for the inter-operator co-located scenario, considering:

-	Which RF requirements could be adjusted for the inter-operator co-located BS scenario, e.g. existing UE RF requirements such as ΔRIBNC, ACS and in-band blocking [RAN4];

- Other requirements are not precluded, if identified

-	The ability to semi-statically switch hardware resources (i.e. Rx chains) [RAN4, RAN2 – See note 2];

-	Up to 6 dB DL received power spectral density imbalance between the two non-contiguous CCs [RAN4];

-	Determine a reasonable level for the power spectral density difference between carriers of co-located adjacent channel operators for study [RAN4]

-	Impacts on DL performance [RAN4] ;

-	Means for a UE to inform the network of appropriate CA configuration it can support with adjusted RF requirements [RAN4, RAN2 – See note 2].

At RAN#104, a R19 SI on fragmented carriers was approved [1], with the following objective:

At the last RAN4 meeting, a WF was agreed that captures the agreements and open issues [2]. In this contribution, we share our views on the open issues.

# 2	Discussion

Prerequisite conditions

To allow the UE to support the two non-contiguous CCs with common/shared RF chains instead of separate RF chains, as analyzed in our companion contribution [3], the blocker power level in the gap needs to be manageable to the UE. In other words, the blocker power level needs to be at an appropriate level that the UE can handle with common RF chain. It can either be a specified value in the spec or based on UE capability reporting. The Rx chain configuration/reconfiguration may be based on some assistant information by UE measurement of the blocker.

Proposal 1: To allow the UE to support the two non-contiguous CCs with common/shared RF chains instead of separate RF chains, the blocker power level in the gap needs to be at an appropriate level that the UE can handle with common RF chain.

Signaling aspects

Besides the UE ability to switch, there needs to be signaling support for UE to inform the network of appropriate CA configuration it can support after switching to common RF chains allowed by adjusted RF requirements. Usually, the CA configurations supported by a UE are reported to the network when the UE camps on a cell and receives capability inquiry from the network. In the case of RF chain switching, however, the UE may need to update its supported CA configuration after it is configured with some CA combination by the network through RRC. An alternative way is that the UE reports to network that it is capable of Rx chain sharing for the supported band combination, either by additional signaling on top of existing band combination reporting or by defining a dedicated band combination reporting for UE supporting Rx chain sharing.

On the proposal that gNB triggers UE to measure blocker power in the gap in between two fragments, we agree this is an important aspect if the blocker power is not properly restricted. In this case, UE may or may not need measurement gap to measure the blocker, depending on if the network configures such measurement before or after configuring the CA for the two non-contiguous fragments (CCs). And if such a measurement of blocker is considered, there could be a third alternative on the signaling consideration. For example, the UE can first report high UE capabilities, i.e., support of CCs, assuming non-continuous CCs can be handled by common RF chain. If the blocker power is too high, the UE can inform the network that only fewer CCs can be supported. It is important to note that the need of blocker measurement is only needed if the blocker power cannot be properly restricted by deployment.

In addition, there is a proposal from [4] to consider a new syntax/notation for indicating support of fragmented carrier operation in a CA configuration. We agree this can be further explored.

Proposal 2: In terms of signaling, several alternatives can be considered for band combination reporting for UE capable of Rx chain sharing

- Alt. 1: Legacy band combination reporting scheme + Rx sharing capability indication

- Alt. 2: Band combination capability update with the supported number of CCs

- Alt. 3: New band combination definition e.g. CA_F_n25(2A) + new set of signaling

# 3	Conclusions

In this contribution, we make the following proposals.

Proposal 1: To allow the UE to support the two non-contiguous CCs with common/shared RF chains instead of separate RF chains, the blocker power level in the gap needs to be at an appropriate level that the UE can handle with common RF chain.

Proposal 2: In terms of signaling, several alternatives can be considered for band combination reporting for UE capable of Rx chain sharing

- Alt. 1: Legacy band combination reporting scheme + Rx sharing capability indication

- Alt. 2: Band combination capability update with the supported number of CCs

- Alt. 3: New band combination definition e.g. CA_F_n25(2A) + new set of signaling

# 4	References

RP‑241669, "New SID: Study on NR FR1 DL Fragmented Carriers," MediaTek

R4-2417201, "WF on fragmented DL carriers study," MediaTek

R4-2418532, "On UE RF requirements for fragmented carriers," Apple

R4-2413340, "Discussion on impacts on UE RF requirements and DL performance for Fragmented Carriers," Nokia