# 3GPP TSG-RAN WG4 Meeting #113	R4-2418603

# Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.20.2.2

Source: 	Qualcomm Incorporated

Title: 	Views on existing Tx BS RF requirements for SBFD

Document for:	Approval

# Introduction

In RAN#102, the work item on evolution of NR duplex operation was approved [1]. From RAN4 RF perspective, the objective of the work item is as follows:

The objectives are as follows: - Specify BS RF requirements for SBFD operation at gNB [RAN4]

In RAN4#112bis, a WF was agreed in [3] based on some of the topics listed in the moderator summary, that is summarized in [2]. In this contribution, we share our views on existing BS RF requirements for Rel-19 SBFD.

# Discussion

## ACLR and OBUE

In [2], the following was agreed:

- FFS on the followings: - Whether to define additional ACLR and/or OBUE requirement to ensure feasible coexistence with SBFD-capable gNB firstly? - Option 1: Yes, since legacy ACLR and OBUE requirement might not be sufficient to ensure feasible coexistence. - Option 2: No, since specific co-existence requirements can be covered by regional requirements or customer specific proprietary requirement. - How to define the additional co-existence requirements: - Option 1: based on CL assumption for BS2BS CLI interference and BS DESENS level. - [1dB DESENS in the victim SBFD receiver] - [typical CL values based on typical SBFD-capable gNB implementation] - [frequency offset for the additional OBUE requirement is declared] - Option 2: by improving ACLR1 and ACLR2 requirements - Option 3: others

In Section 11 in TR 38.858, RAN4 has concluded that outcome of the SBFD and legacy TDD adjacent channel coexistence. The SBFD-to-SBFD coexistence case was not investigated since there are no SBFD deployments prior to Rel-19. However, from coexistence point of view, the impact of the inter-BS CLI on the UL subband at the SBFD capable BS that coexists with another SBFD BS will be similar to the impact of the legacy TDD DL on UL subband of an SBFD-capable BS. To address the inter-BS CLI, RAN4 has studied only ACIR enhancements as a solution to ensure feasible coexistence.

On the other hand, for the coexistence case with BS-capable SBFD BS in the adjacent channel, RAN4 has identified coexistence challenges due to inter-BS CLI on the UL subbands. Within the SI phase, RAN4 has considered only ACIR enhancements as a solution to ensure feasible coexistence.

Observation 1: When coexisting with SBFD-capable BS, legacy ACLR requirement might not be sufficient to ensure feasible coexistence. Additionally, RAN4 has only considered ACIR enhancement as a possible solution during the SI phase.

As a way forward, it is viable to consider urban macro deployment since it is the most challenging deployment scenario. RAN4 then should consider a case where additional new ACLR requirement is considered to allow feasible coexistence with new SBFD-capable BS while assuming 1 dB desensitization.

Proposal 1: RAN4 to consider additional ACLR requirement assuming 1 dB desensitization to ensure feasible coexistence with SBFD-capable gNB. As a starting point, urban macro deployments should be considered as that represent the challenging scenario.

Regarding option 2, which relies on proprietary solutions to address the SBFD-to-SBFD coexistence, we can accept this is a viable solution with the agreement that new RF requirements for inter-subband selectivity or blocking will not be defined in RAN4 and will be based on proprietary solution as well.

Proposal 2: If RAN4 agrees to not define additional ACLR/OBUE requirements for SBFD-SBFD coexistence owing to proprietary solutions, then it is viable to not define new in-channel inter-subband selectivity requirements for SBFD capable gNB

Regarding the CL for inter-BS CLI, it is a viable approach that RAN4 would consider typical CL values based on typical implementation, leveraging the work carried out in the study item phase, specifically, the feasibility study and the adjacent channel coexistence work. Due to the different views on the implementation capabilities and coexistence results, RAN4 should try to agree on typical values that serve as a baseline to derive the new ACLR/OBUE requirements when coexisting with new SBFD-capable gNBs.

Proposal 3: Regarding CL for inter-BS CLI, RAN4 to adopt typical CL values based on typical SBFD-capable BS implementation.

# Conclusion

In this contribution, we provided our views on BS RF requirements for SBFD with the following observations and proposals:

Observation 1: When coexisting with SBFD-capable BS, legacy ACLR requirement might not be sufficient to ensure feasible coexistence. Additionally, RAN4 has only considered ACIR enhancement as a possible solution during the SI phase.

Proposal 1: RAN4 to consider additional ACLR requirement assuming 1 dB desensitization to ensure feasible coexistence with SBFD-capable gNB. As a starting point, urban macro deployments should be considered as that represent the challenging scenario.

Proposal 2: If RAN4 agrees to not define additional ACLR/OBUE requirements for SBFD-SBFD coexistence owing to proprietary solutions, then it is viable to not define new in-channel inter-subband selectivity requirements for SBFD capable gNB

Proposal 3: Regarding CL for inter-BS CLI, RAN4 to adopt typical CL values based on typical SBFD-capable BS implementation.

# References

- RP-240789 “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, HiSilicon, 3GPP TSG RAN#103.

- R4-2416585, “Way Forward for [112][308] NR_duplex_evo_General”, RAN4#112bis, Huawei, HiSilicon.