3GPP TSG-RAN WG4 Meeting #113	R4-2418620

Orlando, US, 18th – 22nd November, 2024

Source:	Samsung

Title:	Discussion on spatial channel model for demodulation performance requirements

Agenda item:	7.14.2

Document for:	Discussion

- Introduction

In the RAN#104 meeting, the SID [1] for “New SID: Study on spatial channel model for demodulation performance requirements” has been approved, which starts from RAN4#112 in Aug 2024 and targeting to complete by RAN#108 in June 2025. The objectives are as below:

- Study practical spatial channel modelling methodology for both SU- and MU-MIMO demodulation requirements and CSI reporting requirements: - Identify the limitations of the current (i.e. up to and including Release 18) channel models and corresponding scenarios and how they relate to UE MIMO performance - Consider both Clustered Delay Line (CDL)-based and TDL-based channel modelling approaches - For CDL-based channel modelling, use the tuned repeatable spatial channel model of TR38.827 as the starting point and identify any necessary changes. - Study and verify test methodology feasibility including test complexity and achievable results uncertainty. The test complexity shall not be significantly increased. - The methodology shall include both FR1 (conducted) and FR2 (wireless cable), with first priority for FR1.

In last RAN4#112bis meeting, High level TR skeleton for 38.753 has been agreed [2].  Furthermore, some progress on the SU-MIMO test cases is achieved and the basic antenna array definition for spatial channel modelling methodology are clarified [3]. However, there are still many issues need to be further clarified and discussed for the CDL and TDL modelling.

In this contribution, we share our views and analysis on the open issues for CDL-based model and TDL-based model.

- Discussion

Generally, the channel model with spatial property has been discussed at the beginning of NR, and RAN1 are using geometric based SCM(E) defined in 38.901 for system level simulation since this channel model is more realistic, 38.901 CDL channel model are derived from SCM(E) with some parameters simplified/discarded, and 38.901 TDL are further derived from 38.901 CDL without spatial property [4] as Figure 2-1. RAN4 decided to use simplified TDL model for NR FR1 demodulation test in May 2018 [7] and simplified TDL model for NR FR2 demodulation test in Aug 2018 [8] since majority companies think TDL is enough for demodulation performance requirements and it is hare to align simulation results using 38.901 CDL at that stage. A New SID [9] on the radiated metrics and test methodology for the verification of multi-antenna reception performance of NR UEs was approved in June 2018, which is for OTA test and targeting for 38.827 approval in Dec 2019 and actually approved in June 2020.

Figure 2-1. Channel Models in 3GPP

  - Common for all Methodologies

Channel model is a mathematical model of characteristics which reflect the impacts to the transmit signals from the wireless communication environement including the delay, power and spatial properties. Theoretically once the wireless environement is specified, the channel model characteristics are decided, no matther how many UEs are deployed in this environement. The normal process of MU-MIMO modeling is gNB select candidate UEs to be paired according the correlation between them, in which the correlation is highly related with the locations of candidate UEs. For the MU-MIMO modeling in RAN4 demodulation, we usually assume two UEs are already paired, which means the correlation between them are limited, hence the angle desired for paired UEs should be selected carefully. However, the alignment of channel modeling with spatial property for RAN4 is still on the starting stage, it seems too hasty to deploy MU-MIMO simulations since many undetermined factors for the channel modeling with spatial property.

Proposal 1: For CDL evaluation, prioritize the simulation assumption/results alignment of SU-MIMO scenario alignment and defer MU-MIMO scenario.

  - TDL Based Methodologies

TDL parameters for performance comparsion

Considering the legacy TDL models will be used for CDL performance comparision, and CDL model introduces the correlation cofficient for polarizations between BS and UE side as expjΦmθθκn,m-1expjΦmθϕκn,m-1expjΦmϕθexpjΦmϕϕ, it seems TDLC with ULA Low, which neither include the correlations between polarizations nor the correlation coefficients for different layers (identity matrixs), is an unreasonable and unfair comparison object. Therefore, we prefer to use TDLC X-pol medium correlation as the comparison object for CDL performance comparision.

Observation 1: CDL model introduces the correlation cofficient for polarizations between BS and UE side as expjΦmθθκn,m-1expjΦmθϕκn,m-1expjΦmϕθexpjΦmϕϕ, while TDLC with ULA Low, which neither include the correlations between polarizations nor the correlation coefficients for different layers (identity matrixs), is an unreasonable and unfair comparison object.

Proposal 2: Prefer to use TDLC X-pol medium correlation as the comparison object for CDL performance comparision.

TDL Model Extension

The motivation of the SID [1] is current TDL model does not have spatial property for RAN4 demodulation/CSI reporting requirements, which are needed for some scenarios e.g. High rank, PMI reporting cases. This target could also be achieved by extending current TDL model through introducing spatial clusters as mentioned in [10] multi-cluster TX-RX beam steering with TDL model.

Observation 2: Multi-cluster TX-RX beam steering with TDL model method could also achieve the target for introducing spatial property, which is easier than CDL model since no antenna patterns and antenna combining weight needed.

  - CDL Based Methodologies

In RAN4#112 meeting, 38.827 Uma-CDL-C has been agreed as the first priority to be used for initial comparison and alignment, although several open issues for 38.827 CDL model need to be further discussed after last RAN4#112bis meeting [3].

BS antenna configuration

For the convenience of simulation alignments, the antenna configuration of BS side should be defined.

About the BS antenna polarization, since the agreement is achieved for UE side in last RAN4 meeting as below

UE Antenna Configuration Agreement: UE Antenna Configuration, for DL, for 38.827: Cross Pol (1, n/2, 2), i.e. [ + + + +]. Where ‘n’ is number of Rx with half lambda spacing, with omni antenna element radiation pattern.

The antenna configuration of BS side could select cross Pol with +/- 450 polarization in order to promise both polarizations could receive signals successfully at UE side.

Proposal 3: For BS antenna configuration, select cross Pol with +/- 450 polarization in order to promise both polarizations could receive signals successfully at UE side.

For the radiation power pattern for singla antenna element at BS side, two options were proposed in last RAN4#112bis meeting, one is omni pattern as below Figure 2-2, the other is directional pattern defined in 38.901 Table 7.3-1 as below Figure 2-3.

Figure 2-2. Omni radiation power pattern

Figure 2-3. Directional radiation power pattern defined in 38.901 Table 7.3-1

Considering the typical deployment for BS is 120o coverage per cell, hence if the channel model is based on antenna element level, directional radiation power pattern should be used.

Proposal 4: For BS antenna configuration, the directional radiation power pattern per antenna element defined in 38.901 Table 7.3-1 should be used.

Correlation between polarizations

38.827 assume fixed and pre-defined set of initial phases Φmθθ,Φmθϕ,Φmϕθ,Φmϕϕ and a scalar random initial phase  Φn,m~Uni(-π,π) for each ray m of each cluster n. Hence, for different clusters, actually only one set of initial phases which indicating the correlation introduced by polarization are applied as below Figure 2-4.

Figure 2-4. Fixed and pre-defined set of initial phases Φmθθ,Φmθϕ,Φmϕθ,Φmϕϕ for each cluster

Observation 3: Only one set of initial phases Φmθθ,Φmθϕ,Φmϕθ,Φmϕϕ  which indicating the correlation introduced by polarization are applied for each cluster in 38.827.

The generation of departure and arrival angles

According current description in 38.827, the departure and arrival angles are generated using below equation

Where n,AOA+ cASAm denotes the arrival angles with ray offsets, and  is the mean angle of the original channel model table. Therefore, the technical effects of above equation are only scaling the ray offsets in addition to the mean angle of the original channel model table, the final scaled angel ϕn,m,AOA,scaled are still fixed due to fluctuating around the mean angle, e.g., Uma CDL-C scaled departure/arrival angles in Figure 2-5. Then the drawbacks mentioned in 38.901 section 7.7.5.1 are still exist, i.e. “The PMI statistics can become biased, and a fixed precoder may perform better than open-loop and on par with closed-loop or reciprocity beamforming. Furthermore, a CDL only represents a single channel realization”. Hence, we prefer to replace the  in the last position by the desired mean angle μϕ,desired.

Figure 2-5. Scaled angles with ray offsets defined for Uma CDL-C in 38.827

Observation 4: The technical effects of current departure and arrival angles generation equation in 38.827 is only scaling the ray offsets in addition to the mean angle of the original channel model table, the final scaled angle ϕn,m,AOA,scaled are still fixed due to fluctuating around the mean angle instead of desired angles.

Proposal 5: Replace the mean angle  in the last position of departure and arrival angles generated equation by the desired angle μϕ,desired as ϕn,m,AOA,scaled=ASdesiredASmodeϕn,AOA+cASAαm-μϕ,model+μϕ,desired

Time Varying Beam Direction

Currently, the beam direction is introduced by αs,me,ne in 38.827 as below formula, which is fixed and not changing with time. Per our understanding, the time varying modelling for beam direction is usually used in system level simulation, in which the beam direction may change together with the UE moving trajectory. In link level simulation, fixed angles and beam direction without time varying beam direction should be assumed.

Hu,s,nNLOSt=PnMm=1Mne=1Neme=1MeFrx,u,θθn,m,ZOA,φn,m,AOAFrx,u,φθn,m,ZOA,φn,m,AOATexpjΦmθθκn,m-1expjΦmθϕκn,m-1expjΦmϕθexpjΦmϕϕ

Ftx,s,me,ne,θθn,m,ZOD,φn,m,AODFtx,s,me,ne,φθn,m,ZOD,φn,m,AODαs,me,neexpjΦn,mexpj2πrrx,n,mT.drx,uλ0expj2πrtx,n,mT.dtx,s,me,neλ0expj2πrrx,n,mT.vλ0t

Proposal 6: CDL based link level simulation should focus on predefined and fixed beam direction, i.e., no need to introduce time varying beam direction modelling for CDL based link level simulation.

Antenna Coordinates, Angles, and other parameters for gNB (for UMa-CDL-C)

The Table 7.2.1-8 with angles defined in 38.827 are calculated based on 3.5GHz, and BS height set as 25m for UMa scenario, then the set of angles , ,   which are used for the transformation from a LCS to a GCS (The angle  is called the bearing angle,  is called the down tilt angle and  is called the slant angle) defined as (0, 0, 0). If BS antenna down tilt value is introduce, the value of  should be changed, for example, if 10-degree mechanical down tilt is introduced, the BS antenna orientation should be (alpha = 0 degrees, beta = 10 degrees, gamma = 0 degrees).

Proposal 7: For BS side, frequency Fc=3.5GHz, BS height=25m, BS antenna orientation (alpha = 0 degrees, beta = 0 degrees, gamma = 0 degrees) should be assumed if no mechanical down tilt introduced. BS antenna orientation (alpha = 0 degrees, beta = 10 degrees, gamma = 0 degrees) should be assumed if 10-degree mechanical down tilt introduced.

Antenna Coordinates, Angles, and other parameters for UE (for UMa-CDL-C)

Different with gNB antennas, there are various directions/positions for UE antennas. However, we need to select a typical one in order to proceeding the performance evaluation of CDL modelling. One typical deployment for UE antennas is the azimuth uniform deployment, in which UE antennas uniformly distributed on the horizontal plane (x-y plane), hence the UE antenna orientation could be (alpha = 0 degrees, beta = 90 degrees, gamma = 0 degrees). The second typical deployment for UE antennas is the distributed on the vertical plane (y-z plane), in this case UE antenna orientation could be described as (alpha = 0 degrees, beta = 0 degrees, gamma = 0 degrees). The third typical deployment for UE antennas is spherical uniform deployment, in which UE antennas uniformly distributed on the sphere, this is the most complex one. Considering user habits nowadays, people stare at the UE screen most of the time, which means UE antennas are distributed on the horizontal plane (x-y plane). Hence we think select UE antenna orientation as (alpha = 0 degrees, beta = 90 degrees, gamma = 0 degrees) is a reasonable choice.

Observation 5: Three typical UE antenna distributions: The first one is Azimuth uniform deployment, in which UE antenna orientation could be (alpha = 0 degrees, beta = 90 degrees, gamma = 0 degrees). The second one is the distributed on the vertical plane (y-z plane), UE antenna orientation could be described as (alpha = 0 degrees, beta = 0 degrees, gamma = 0 degrees). The third one is spherical uniform deployment, in which UE antennas uniformly distributed on the sphere, this is the most complex one.

Observation 6: Considering user habits nowadays, people stare at the UE screen most of the time, which means UE antennas are distributed on the horizontal plane (x-y plane).

Proposal 8: Select UE antenna orientation as (alpha = 0 degrees, beta = 90 degrees, gamma = 0 degrees) is a reasonable choice.

BS Antenna Virtualiser choice

Since the number of BS transmit antenna elements is usually large, such as, for FR1 typical number is 128 or 192 AEs, hence the BS antenna virtualization method should be considered if RAN4 would like to define CDL model from antenna elements level. Several options as below are proposed in last RAN4#112bis meeting.

- Option 1: Down Tilt Virtualiser : -10 degrees

- Option 2: Broadside

- Option 3: Max Receive Power

- Option 4: DFT Virtualiser according to TR 36.897 with application of panning and tilting angles (as per Table 7.8-1 of 38.901)

Option 1 mentioned minus 10 degrees virtualization should be considered. Per our understanding, the mechanical down tilt and the antenna elements virtualization are totally two different things. There are two forms of down tilt in general, one is mechanical down tilt and the other is electrical down tilt, which are important means of effectively controlling interference and coverage of the antennas. While, the antenna virtualization is applying suitable combining weights to virtualize multiple antenna elements to less CSIRS/Tx ports. Option 2 are proposed to use to combining weights targeting for broadside direction. Option 3 are targeting for the max receive power, however, which direction is the max receive power direction need further discussion. Option 4 prefer to use the DFT combining weight according to TR 36.897 with application of panning and tilting angles. We think it’s reasonable to use DFT combing weight, but the wording “panning” is ambiguous and need further clarification.

Proposal 9: For BS antenna virtualization, DFT combing weight is reasonable, but RAN4 need to clarify how to implement “panning” in link level simulation and if it’s needed.

BS Antenna down tilt value

As mentioned above, down tilt is usually used in real network. Different down tilt values have different impacts on the BS coverage. Therefore, we need to clarify the down tilt value used in our simulation assumption.

Proposal 10: RAN4 should clarify if BS antenna down tilts is applied in CDL modeling and define the values of BS antenna down tilts if any.

  - Comparison of methodologies

For NR, the antenna array virtualization (AAV) process exists in real network actually, in which AAV means the mapping from multiple antenna elements to one CSIRS/Tx port. However, if the modeling of this process in RAN4 channel modeling are really needed and how to model it need to be discussed.

38.901 CDL model consider the channel response between Tx antenna elements and Rx antenna elements, but leave the AAV process open to the simulator or simulation alignment. Meanwhile, 38.827 CDL model introduce the beamforming weights and combining process for AAV since 38.827 is targeting for OTA test which should include RF. The difference between 38.901 and 38.827 could be described in Figure 2-6.

Figure 2-6. Modeling difference between 38.827 CDL and 38.901 CDL

The AAV rule between BS antenna elements and CSIRS/Tx ports are actually transparent to UEs, subject to BS algorithm design, and different AAV rules may correspond to different throughput performance. However, 38.827 CDL model apply the fully connected mode AAV as below Figure 2-7.

Figure 2-7. Fully connected mode AAV defined in 38.827

There is no problem for 2T2R scenario using current fully connected AAV algorithm defined in 38.827, as one polarization need to support one beam direction. But for 4T4R, one polarization needs to support two spatial multiplex layers in higher rank scheme, this would lead the two layers in one polarization share two different beams in multiple CSIRS/Tx ports and the orthogonality between these two layers would be hard to promise. The higher the layers, the worse the performance.

Observation 7: The AAV rule between BS antenna elements and CSIRS/Tx ports are transparent to UEs, subject to BS algorithm design, and different AAV rules may correspond to different throughput performance. Current 38.827 defines the co-polarized antenna elements of the array are combined to a single RF port.

Observation 8: Current AAV rule for the mapping between antenna elements and CSIRS/Tx ports defined in 38.827 is not suitable for 4T4R and 8T8R with high rank modes.

SNR definition

Currently, For Mode 1 conditions conducted UE demodulation and CSI requirements the SNR is defined as:

SNR=j=1NRXEs(j)j=1NRXNoc(j)

Where

-	NRX denotes the number of receiver antenna connectors and the superscript receiver antenna connector j.

-	The above SNR definition assumes that the REs are not precoded, and does not account for any gain which can be associated to the precoding operation.

TDL and CDL Methodologies

Considering different AAV rules apply virtualization weights for different number of antenna elements, either horizontal domain or vertical domain, which may result in different virtualization gains and different SNR working point requirements for demodulation. Hence, in general, the SNR definition should keep similar principle as before, without precoding and virtualization gain.

Proposal 11: For RAN4 demodulation and CSI reporting simulation, option C (CDL without AAV with the configuration: (M,N,P,Ms,Ns) = (1,4,2,1,1) for 8Tx CSI-RS Ports and (M,N,P,Ms,Ns) = (1,2,2,1,1) for 4Tx CSI-RS Ports) is a good choice to avoid the AAV process which seems useless for RAN4 requirements definition.

Proposal 12: If AAV process is introduced in RAN4 CDL model, update SNR definition accordingly which should not account the virtualization gain.

- Conclusion

In this contribution, we provide analysis and views on SCM topic.

The observations and proposals could be summarized as:

Proposal 1: For CDL evaluation, prioritize the simulation assumption/results alignment of SU-MIMO scenario alignment and defer MU-MIMO scenario.

Observation 1: CDL model introduces the correlation cofficient for polarizations between BS and UE side as expjΦmθθκn,m-1expjΦmθϕκn,m-1expjΦmϕθexpjΦmϕϕ, while TDLC with ULA Low, which neither include the correlations between polarizations nor the correlation coefficients for different layers (identity matrixs), is an unreasonable and unfair comparison object.

Proposal 2: Prefer to use TDLC X-pol medium correlation as the comparison object for CDL performance comparision.

Observation 2: Multi-cluster TX-RX beam steering with TDL model method could also achieve the target for introducing spatial property, which is easier than CDL model since no antenna patterns and antenna combining weight needed.

Proposal 3: For BS antenna configuration, select cross Pol with +/- 450 polarization in order to promise both polarizations could receive signals successfully at UE side.

Proposal 4: For BS antenna configuration, the directional radiation power pattern per antenna element defined in 38.901 Table 7.3-1 should be used.

Observation 3: Only one set of initial phases Φmθθ,Φmθϕ,Φmϕθ,Φmϕϕ  which indicating the correlation introduced by polarization are applied for each cluster in 38.827.

Observation 4: The technical effects of current departure and arrival angles generation equation in 38.827 is only scaling the ray offsets in addition to the mean angle of the original channel model table, the final scaled angle ϕn,m,AOA,scaled are still fixed due to fluctuating around the mean angle instead of desired angles.

Proposal 5: Replace the mean angle  in the last position of departure and arrival angles generated equation by the desired angle μϕ,desired as ϕn,m,AOA,scaled=ASdesiredASmodeϕn,AOA+cASAαm-μϕ,model+μϕ,desired

Proposal 6: CDL based link level simulation should focus on predefined and fixed beam direction, i.e., no need to introduce time varying beam direction modelling for CDL based link level simulation.

Proposal 7: For BS side, frequency Fc=3.5GHz, BS height=25m, BS antenna orientation (alpha = 0 degrees, beta = 0 degrees, gamma = 0 degrees) should be assumed if no mechanical down tilt introduced. BS antenna orientation (alpha = 0 degrees, beta = 10 degrees, gamma = 0 degrees) should be assumed if 10-degree mechanical down tilt introduced.

Observation 5: Three typical UE antenna distributions: The first one is Azimuth uniform deployment, in which UE antenna orientation could be (alpha = 0 degrees, beta = 90 degrees, gamma = 0 degrees). The second one is the distributed on the vertical plane (y-z plane), UE antenna orientation could be described as (alpha = 0 degrees, beta = 0 degrees, gamma = 0 degrees). The third one is spherical uniform deployment, in which UE antennas uniformly distributed on the sphere, this is the most complex one.

Observation 6: Considering user habits nowadays, people stare at the UE screen most of the time, which means UE antennas are distributed on the horizontal plane (x-y plane).

Proposal 8: Select UE antenna orientation as (alpha = 0 degrees, beta = 90 degrees, gamma = 0 degrees) is a reasonable choice.

Proposal 9: For BS antenna virtualization, DFT combing weight is reasonable, but RAN4 need to clarify how to implement “panning” in link level simulation and if it’s needed.

Proposal 10: RAN4 should clarify if BS antenna down tilts is applied in CDL modeling and define the values of BS antenna down tilts if any.

Observation 7: The AAV rule between BS antenna elements and CSIRS/Tx ports are transparent to UEs, subject to BS algorithm design, and different AAV rules may correspond to different throughput performance. Current 38.827 defines the co-polarized antenna elements of the array are combined to a single RF port.

Observation 8: Current AAV rule for the mapping between antenna elements and CSIRS/Tx ports defined in 38.827 is not suitable for 4T4R and 8T8R with high rank modes.

Proposal 11: For RAN4 demodulation and CSI reporting simulation, option C (CDL without AAV with the configuration: (M,N,P,Ms,Ns) = (1,4,2,1,1) for 8Tx CSI-RS Ports and (M,N,P,Ms,Ns) = (1,2,2,1,1) for 4Tx CSI-RS Ports) is a good choice to avoid the AAV process which seems useless for RAN4 requirements definition.

Proposal 12: If AAV process is introduced in RAN4 CDL model, update SNR definition accordingly which should not account the virtualization gain.

- Reference

[1] RP-241610, New SID: Study on spatial channel model for demodulation performance requirements, RAN#104 meeting, Nokia, BT Plc, AT&T, Bell Mobility, Bouygues Telecom, China Telecom, CMCC, Deutsche Telekom, Ericsson, Intel Corporation, KDDI, Keysight, KT Corp., MediaTek, NTT Docomo, Orange, Qualcomm, Rohde & Schwarz, Samsung, SK Telecom, Spark NZ, Telecom Italia, Telefonica, Telenor, Telia Company, Telstra, T-Mobile USA, Verizon, Vodafone, ZTE Corporation

[2] R4-2416549, TR 38.753 Skeleton v0.0.1, RAN4#112bis meeting, Nokia, Ericsson, Qualcomm

[3] R4-2416591, Way Forward Minutes for [112bis][318]NR_SCM, RAN4#112bis meeting, Nokia

[4] 	3GPP TR 38.901 V18.0.0 (2024-03), Study on channel model for frequencies from 0.5 to 100 GHz

[5] 	3GPP TR 38.827 V16.8.0 (2022-09), Study on radiated metrics and test methodology for the verification of multi-antenna reception performance of NR User Equipment (UE)

[6] 	3GPP TS 38.101-4 V18.4.0 (2024-06), User Equipment (UE) radio transmission and reception; Part 4: Performance requirements

[7] R4-1808400, NR Test Methods SI ad-hoc meeting notes, RAN4 #87, Intel Corporation

[8] R4-1812001, RAN4#88 Meeting report, RAN4#88

[9] RP-181325, New SID on Study on radiated metrics and test methodology for the verification of multi-antenna reception performance of NR UEs, RAN#80, CATR, OPPO, Samsung

[10] R4-2412328, Discussion on Spatial Channel Modelling, RAN4#112 meeting, MediaTek Inc.