3GPP TSG-RAN WG4 Meeting #113	R4-2418678

Orlando, Florida, USA, 18 – 22 November, 2024

Title: 	Discussion on remaining issues in R18 NTN Rx beam Gain

Source: 	Huawei, HiSilicon

Agenda item:	5.21.3

Document for:	Discussion

Introduction

RRM performance requirements for R18 NTN Rx beam are discussed in RAN4#112, and WF recorded the process and conclusion [1]. In our view, the following issues need to be further discussed in maintenance phase.

- Rx beam gain max

- Rx beam gain min

In this paper, we will provide our views on remaining issues in R18 NTN performance requirements.

Discussion

Rx beam gain

The following are the conclusions from the last meeting:

Agreement (ad-hoc): - Value of Gmin: - Further discuss the detailed parameter assumptions to be aligned with core specification TS 38.101-5 and TR 38.863. - Separate values will be introduced for NTN VSAT Type 3 and other VSAT types. Agreement (ad-hoc): - Gmax leave to be declaration basis. - Note: FFS whether RAN4 needs to specify the possible range for Gmax

In terms of receiver sensitivity principle, Gmin is able to be derived by the EIS or REFSENS level [2]. The following parameters are used in the deriving process:

BW: channel bandwidth is 50MHz

IM: implementation margin is 2dB

SNR: required for demodulation is -1 dB

NF: noise figure is 6dB NF for VSAT type 3 and 2.5NF for other VSAT types [3]

T: noise temperature is 293.15K (Ambient temperature)

K: Boltzmann constant is 1.38*10-23 J/K

Observation 1: There will be a little error when using a different method to calculate the Gmin.

Proposal 1: The following parameters is able to be used in the Gmin calculation.

BW: channel bandwidth is 50MHz

IM: implementation margin is 2dB

SNR: required for demodulation is -1 dB

NF: noise figure is 6dB NF for VSAT type 3 and 2.5NF for other VSAT types [3]

T: noise temperature is 293.15K (Ambient temperature)

K: Boltzmann constant is 1.38*10-23 J/K

It is noted that G/T also includes the impact of NF. Considering 6dB NF for VSAT type 3 and 2.5dB NF for other VSAT types, and assuming the T value for 20 degrees Celsius, we can get the Gmin for VSAT type 3 as 25dB and other VSAT types as 33 dB.

Proposal 2: Gmin = 25dB for VSAT type 3, and Gmin = 33dB for other VSAT types

For Gmax, a specific range is needed. Although the value of Gmax is not limited by the receiver sensitivity, it still determines the success of RSRP tests.

Proposal 3: RAN4 will define Gmax within a specified range for all VSAT types. The range is from 45 to 55 dB.

Conclusions

Observation 1: There will be a little error when using a different method to calculate the Gmin.

Proposal 1: The following parameters is able to be used in the Gmin calculation.

BW: channel bandwidth is 50MHz

IM: implementation margin is 2dB

SNR: required for demodulation is -1 dB

NF: noise figure is 6dB NF for VSAT type 3 and 2.5NF for other VSAT types [3]

T: noise temperature is 293.15K (Ambient temperature)

K: Boltzmann constant is 1.38*10-23 J/K

Proposal 2: Gmin = 25dB for VSAT type 3, and Gmin = 33dB for other VSAT types

Proposal 3: RAN4 will define Gmax within a specified range for all VSAT types. The range is from 45 to 55 dB.

Reference

- R4-2414100 WF on RRM requirements for NR_NTN_enh

- 3GPP TR 38.863 V18.2.0

- R4-2314934 for UE RF requirements for NR NTN_r3