3GPP TSG-RAN WG4 Meeting #113				                        R4-2418727

Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.21.3.2

Source: 	CMCC

Title: 	Discussion on A-IoT device RF requirements

Document for:	Discussion

# 1. Introduction

In RAN plenary #102 meeting, a new SID[1] of A-IoT is approved with following RAN4 lead RF part objectives:

- Coexistence study of Ambient IoT and NR/LTE.

- RF requirements study for Ambient IoT:

  - Ambient IoT BS transmission and reception

  - Ambient IoT Device, as per the General Scope, transmission and reception

  - Intermediate node (UE), as per the General Scope, transmission and reception

This is the first meeting of RF requirements discussion, in this contribution we list our analysis for A-IoT devices.

- Discussion

Agreement:

- Whether requirements potentially needed or not is summarized in the following table:

| A-IoT device TX (D2R) RF Requirement |  | Potentially needed or not |  |  |
| --- | --- | --- | --- | --- |
|  |  | Device 1 | Device 2a | Device 2b |
| Transmit output power | Maximum output power /output power | NO for maximum output power YES for backscattering power or power backscattering loss | NO for maximum output power YES for backscattering power or power backscattering loss | YES |
| Output power dynamic | Transmit OFF power | No | NO | YES, as RAN4#112 agreed |
|  | Transmit ON/OFF time mask | NO, as RAN4#112 agreed | NO, as RAN4#112 agreed | NO, as RAN4#112 agreed |
|  | Minimum output power | N/A | N/A | YES |
|  | Power control | NO, as RAN4#112 agreed | NO, as RAN4#112 agreed | YES, as RAN4#112 agreed |
| Transmit signal quality | Frequency error | NO, as RAN4#112 agreed | TBD | YES, as RAN4#112 agreed |
|  | Modulation quality requirements | TBD | TBD | TBD |
|  | In band emissions | NO, as RAN4#112 agreed | TBD | TBD. |
|  | Carrier leakage | NO, as RAN4#112 agreed | TBD | YES, as RAN4#112 agreed |
| Output RF spectrum emissions | Occupied bandwidth | YES | YES | YES |
|  | SEM | TBD | TBD | TBD |
|  | ACLR | TBD | TBD | TBD |
|  | Spurious emissions | TBD | TBD | TBD |
| Unwanted emissions |  | TBD | TBD | TBD |
| Transmit intermodulation |  | NO | NO | TBD |

Agreement:

- Whether requirements potentially needed or not is summarized in the following table:

| A-IoT device- RX Requirement | Potentially needed or not |  |  |
| --- | --- | --- | --- |
|  | Device 1 | Device 2a | Device 2b |
| Reference sensitivity | YES, as RAN4#112 agreed | YES, as RAN4#112 agreed | YES, as RAN4#112 agreed |
| Maximum input power | TBD | TBD | TBD |
| ACS | NO | NO | Depend on co-existence study |
| ACSC | NO | NO | NO |
| In-band blocking | NO | NO | TBD |
| Out-of-band blocking | NO | NO | TBD |
| Receiver intermodulation | NO | NO | TBD |
| Rx spurious emission | NO | NO | TBD |
| Spurious response | NO, as RAN4#112 agreed | NO, as RAN4#112 agreed | NO, as RAN4#112 agreed |
| Interference rejection | TBD | TBD | TBD |

2.1 Device 1

For device 1, considering its low cost, it’s better to use RFID spec as baseline which only has output power (backscattering power or power backscattering loss) and unwanted emission requirements. Besides, for Rx part, REFSENSE requirements is needed.

Proposal 1: for device 1, taking RFID RF requirements as reference which only define output power and unwanted emission requirements and REFSENSE requirement.

2.2 Device 2a

Following table lists our initial analysis for Tx requirements of device 2a.

| requirements | Applicable or not |
| --- | --- |
| Transmitter power | All three kinds of device have different output power level. RAN4 further discuss how to define power class based on different device type and architecture. |
| Output power dynamic | Not applicable |
| Power control | Not applicable |
| Transmit signal quality | Frequency error: wait for modulation scheme conclusion EVM: wait for modulation scheme conclusion Carrier leakage: may be needed based on the design of small frequency shift in baseband In-band emission: needed. |
| RF spectrum emissions | Out of band emission: at least for standalone, it’s needed. For in-band and guard-band spectrum deployment mode, FFS how to define such requirements. Spurious emission requirement: needed to meet regulatory requirement. if large frequency shifter is considered for device 2a, RAN4 should further check the spurious emission at specific frequency where harmonic occurs. |

Proposal 2: at least for transmit signal quality related requirement, RAN4 should wait for RAN1 conclusion of How to achieve small frequency shift in baseband and/or FDM(A) among devices.

Proposal 3: if RAN4 only simulate in-band spectrum mode, RAN4 further discuss whether/how to define out of band emission requirement.

Following table list our initial analysis for Tx requirements.

| requirements | Applicable or not |
| --- | --- |
| REFSENSE | Necessary and needs to be separately defined for different devices types. |
| Max input level | Necessary and may needs to be separately defined for different devices types. |
| ACS | For standalone, FFS and details based on architecture. |
| blocking | In-band blocking: necessary Out of band blocking: Further discuss the out of band blocking performance based on RF architecture discussion |
| Spurious response | The same analysis as out of band blocking |
| Spurious emission | Spurious emission: to meet regulatory requirement |

Based on above analysis, we have following proposals.

Proposal 4: REFSENSE and max input level needs to be separately defined for different devices types.

Proposal 5: RAN4 needs to further discuss the out of band blocking performance based on RF architecture discussion.

2.3 Device 2b

For device 2b, it’s suggested to use UE RF framework as baseline and further discuss whether certain relaxation is needed or not.

Proposal 6: For device 2b, it’s suggested to use UE RF framework as baseline and further discuss whether certain relaxation is needed or not.

# 3. Conclusions

In this contribution, RF requirements for devices are discussed with following observations and proposals.

Proposal 1: for device 1, taking RFID RF requirements as reference which only define output power and unwanted emission requirements and REFSENSE requirement.

Proposal 2: at least for transmit signal quality related requirement, RAN4 should wait for RAN1 conclusion of How to achieve small frequency shift in baseband and/or FDM(A) among devices.

Proposal 3: if RAN4 only simulate in-band spectrum mode, RAN4 further discuss whether/how to define out of band emission requirement.

Proposal 4: REFSENSE and max input level needs to be separately defined for different devices types.

Proposal 5: RAN4 needs to further discuss the out of band blocking performance based on RF architecture discussion.

Proposal 6: For device 2b, it’s suggested to use UE RF framework as baseline and further discuss whether certain relaxation is needed or not.

# 4. Reference

- R4-2417088, WF on RF requirement impact of A-IoT, Huawei.

- R4-2414305, WF on RF requirements for A-IoT, Huawei