3GPP TSG-RAN WG4 Meeting # 113                                    R4-2418741

Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.20.2.1

Source: 	CMCC

Title: 	Discussion on new RF requirement for SBFD

Document for:	Discussion

# 1. Introduction

In TR 38.858, the impacts on SBFD requirements have been analyzed. RAN4 has identified which legacy requirements are still applicable, which legacy requirements are not applicable and which new requirements are needed.

In last meeting, WF on SBFD BS requirement is approved with several agreements. [2]

In this contribution, we continue the RF requirements discussion based on the output of study item phase.

- Discussion

2.1 in-channel adjacent sub-band leakage ratio

Following capture the agreements from TR 38.858 for in-channel adjacent subband leakage ratio requirements:

For the potential new requirement of in-channel adjacent subband leakage ratio, it is concluded that further study is needed on the necessity of this requirement in normative phase.

When sub-band Tx leakage fall into Rx sub-band within the same carrier, this interference may contribute to REFSENSE level. Previous agreements are that further study if OTA sensitivity should be defined considering all of the scenarios including self-interference, inter-site interference and inter-sector interference. So this sub-band leakage requirements falling into the same carrier should be discussed together with OTA sensitivity requirements.

When sub-band Tx leakage fall into adjacent carrier, this is something like ACLR. The similar definition methodology could be reused with assumed equal Tx bandwidth and Rx bandwidth. As stated in section 2.4, this general sub-band Tx leakage not applies for co-located case.

Besides, we should also keep in mind that interference may still occur even with infinitely increased SBFD performance. For example when co-exist with legacy NR network, interference may still occur even with infinitely increased SBFD performance. But when co-exist with SBFD of different configuration, interference could be avoided by increased SBFD performance. So before defining sub-band Tx leakage when falling into adjacent carrier, we should identify the assumption of adjacent network performance, i.e. with legacy 3GPP requirement or with enhanced requirements.

Proposal 1: In-channel adjacent sub-band leakage ratio should be defined or reflect by other requirements.

2.6 in-channel adjacent sub-band selectivity or blocking

Last meeting agreement is listed as below:

For the potential new requirements of in-channel adjacent subband blocking and selectivity, it is concluded that further study is needed on the necessity of this requirement in normative phase.

For legacy ACS and blocking requirements, the main difference are the interference signal level and interference signal position. For ACS, the interference signal is at first adjacent channel and interference signal power is calculated based on sensitivity and ACS value. For in-band blocking, the interference signal is at second adjacent channel and interference signal power is 9dB higher derived as 95% point on CDF of all gNB’s total input power level. So the ACS requirement and blocking requirement can’t replace each other.

The same story of ACS and blocking also applies for sub-band case. We need both selectivity and blocking requirements. One is based on co-existence conclusion to meet 5% throughput loss and the other is with higher interference level with another sub-band frequency offset.

Regarding how to conclude sub-band selectivity and blocking requirements, the story is much similar as stated in section for sub-band Tx leakage.

When Tx interference signal is within the same carrier, this sub-band selectivity may contribute to REFSENSE degradation. Previous agreements are that further study if OTA sensitivity should be defined considering all of the scenarios including self-interference, inter-site interference and inter-sector interference. Sub-band selectivity requirements when Tx interference is within the same carrier should be discussed together with OTA sensitivity requirements. If OTA sensitivity will consider encompass these sub-band interference, sub-band selectivity with Tx interference signal within carrier is not needed.

Proposal 2: Both sub-band selectivity and blocking requirements should be defined.

# 3. Conclusions

In this contribution, SBFD RF requirements are discussed with following observations and proposals.

Proposal 1: In-channel adjacent sub-band leakage ratio should be defined or reflect by other requirements.

Proposal 2: Both sub-band selectivity and blocking requirements should be defined.

# 4. Reference

[1] TR 38.858

[1] R4-2416585, Way Forward for [112bis][307] NR_duplex_evo_BSRF