3GPP TSG RAN WG4 Meeting#113                                    R4- 2418748

Orlando, USA, November 18th – 22nd, 2024

Agenda Item:	7.17.2.1

Source:	vivo

Title:	Discussion on testability and interoperability issues for CSI compression

Document for:	Discussion

- Introduction

In R18 SI, we have studied on general aspects, specific issues related to use cases, and interoperability and testability aspects. The agreements have been collected in TR [1]. In Rel-19, the study on interoperability and testability for CSI compression continues in the WI [2].

In last meeting, we have the following agreements for CSI compression.

Agreement: Next step: Interested companies to share decoder and encoder ML models and dataset in next meeting (RAN4#113) - Model and dataset used to generate the data submitted to RAN4#112-bis Additional data to be submitted: - CDF of SGCS per subband - CDF of the largest eigen value of the channel matrix of training data for each subbands (preferably to be submitted in an xls format) Interested companies are invited to bring further proposals for the next steps for the feasibility study of Option 4 (for any of the sub-options of Option 4) - Analysis of commonality with option 3 (whether any steps or data generated for Option 3 can be reused) would be useful

In this contribution, we further provide our views on testability aspects, especially from general test framework perspective.

- Discussion

  - Initial field test result of reference model for CSI compression

Initial field test of reference model for CSI compression is conducted as following. The data is collected from actual 5G network and the whole collecting area including Cell 1 and Cell 2 is about 250m * 150m. The detailed parameters are provided in the below table.

Table 2.1-1. Parameters of field test of CSI compression.

| Parameters | Value |
| --- | --- |
| Scenario | Actual 5G network |
| Carrier frequency | 3.45GHz |
| Subcarrier spacing | 30KHz |
| Frequency resources | 52 RBs, 13 subbands for 4RBs per subband |
| Rank | Max rank 1 |
| gNB antenna | 8 antenna ports |
| UE antenna | 4 antenna ports |
| CSI payload | 55 bits payload |

The performance of different physical cells is analysed in the following. We have tested the coverage of different cells in the industrial park, according to the measured RSRP, RSRQ and SINR of mobile phones. The channel samples are collected by our prototype. The coverage areas of two typical cells in the industrial park are shown in the below figure. Note that in the coverage areas of each cell, the signal of target cell is acceptable in most places but would be not always the strongest among all cells. This means that the map of actual cell section would be different from the following figure.

Figure 2.1-1. The map of data collecting cells.

For Cell 1, we have collected about 144k outdoor samples. For Cell 2, we have collected about 65k outdoor samples. We choose 10% of them as validation samples.

We use the almost the same simulation assumptions of UMa and CDL in Table 2.1-1, except changing gNB 32Tx to 8Tx. Also, it is important to point out that there would be plenty of parameter mismatches between simulation data and field data.

The CNN encoder and the Transformer encoder trained by UMa and CDL are tested on Cell 1 and Cell 2 in field. Transformer decoder is used for both CNN encoder and Transformer encoder. The SGCS results are provided in the below table. It is seen that compared with eType II codebook, UMa transformer has 2.2% SGCS loss in Cell 1, but 2.7% SGCS gain in Cell 2. However, considerable SGCS loss is obtained by using AI/ML model trained by CDL. For UMa training data, Transformer would provide about 4.2%~5.1% SGCS gain compared with CNN. Note that the CNN/Transformer encoder structures used in this section are referred to the proposed model structures in our contributions [3] in the last meeting.

Table 2.1-2. The SGCS results of multiple AI/ML models trained by simulation data and tested on field data.

| 55 feedback bits | SGCS on Cell 1 | SGCS on Cell 2 |
| --- | --- | --- |
| eTypeII | 0.677 | 0.839 |
| Field Transformer encoder (Cell 1 + Cell 2) | 0.779 | 0.915 |
| Field CNN encoder (Cell 1 + Cell 2) | 0.765 | 0.910 |
| UMa CNN encoder | 0.630 | 0.827 |
| UMa Transformer encoder | 0.662 | 0.862 |
| CDL CNN encoder | 0.202 | 0.512 |
| CDL Transformer encoder | 0.309 | 0.557 |

Observation 1: From initial results for field test, the generalization performance of AI/ML model trained by UMa simulation data on field data seems acceptable, which has similar performance as eType II codebook. The generalization performance of AI/ML model trained by CDL simulation data on field data is worse than AI/ML model trained by UMa simulation data.

Based on the results of above table, we further study the case that NW-side and/or UE-side may separately train new decoder and/or encoder compatible to the reference AI/ML model.

The simulation results of different simulation cases are provided in the below table. Since transformer has better performance than CNN, transformer is used in the below table. Note that different from the above table, the results of AI/ML model trained by field data (Case 1) are obtained by using the field data of current cell, that is, the results of cell 1 is obtained by using Cell 1 data.

Case 2A-1 and Case 2A-2 are the cases that only one side finetunes their part based on reference encoder and decoder pair. Here the reference encoder and decoder pair is obtained by training with UMa data, which is generated by RAN4 aligned simulation assumptions. In Case 2A-1, the encoder is finetuned from reference encoder using field data of current cell, while the decoder is fixed to be reference decoder. In Case 2A-2, the decoder is finetuned from reference decoder using field data of current cell, while the encoder is fixed to be reference encoder. Case 2A-3 and Case 2A-4 is the case that both two sides finetune their part based on reference encoder and decoder pair, separately. Since UE and NW would not use the same training data in practical, mismatch on training data between UE and NW is considered in Case 2A-3 and Case 2A-4. In both Case 2A-3 and Case 2A-4, decoder is finetuning from reference decoder using field data of current cell. In Case 2A-3, small mismatch is considered, where encoder is finetuned from reference encoder using field data of Cell 1 and Cell 2. In Case 2A-4, large mismatch is considered, where encoder is finetuned from reference encoder using field data of the other cell.

Compared to directly use reference model (Case 2), it is seen that only finetuning encoder (Case 2A-1) has up to 8.6% SGCS gain and only finetuning decoder (Case 2A-2) has up to 11.0% SGCS gain. Compared to fully trained by field data (Case1), it is seen that Case 2A-1 has up to 9.7% SGCS loss and Case 2A-2 has up to 7.7% SGCS loss.

Compared to Case 2A-2, considering mismatch on training data between UE and NW, encoder and decoder separate training (Case 2A-3, Case 2A-4) has performance loss, and sometimes the performance is terrible (Case 2A-4 on Cell 1). Then, it seems that it is benefit for NW to finetune the decoder from reference decoder using field data, while UE just implements the reference encoder without finetuning.

Table 2.1-3. The SGCS results of AI/ML models trained by simulation data, and then finetuned on field data.

| Index | Simulation case | Performance on Cell 1 |  | Performance on Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SGCS | Gain to eType II | SGCS | Gain to eType II |
| Case 0 | eType II | 0.677 | 0.0% | 0.839 | 0.0% |
| Case 1 | upper bound | 0.796 | 17.6% | 0.929 | 10.7% |
| Case 2 | Reference model (Trained by UMa) | 0.662 | -2.2% | 0.862 | 2.7% |
| Case 2A-1 | Encoder trained by field data, while using reference decoder | 0.719 | 6.2% | 0.866 | 3.2% |
| Case 2A-2 | Decoder trained by field data, while using reference encoder | 0.735 | 8.6% | 0.886 | 5.6% |
| Case 2A-3 | Encoder and decoder separate training considering small mismatch on training data between UE and NW | 0.734 | 8.4% | 0.885 | 5.5% |
| Case 2A-4 | Encoder and decoder separate training considering large mismatch on training data between UE and NW | 0.439 | -35.2% | 0.850 | 1.3% |

Observation 2: From initial results of field test, it is observed that

- Directly use reference model in field (Case 2): similar performance on field data as eType II for one cell and performance loss compared to eType II is observed for another cell.

- Finetuned encoder or decoder using field data against reference decoder/encoder (Case 2A-1, Case 2A-2): performance improved compared to directly use reference model (Case 2), but still has performance loss compared to fully trained by field data (Case1). Finetuned decoder (Case 2A-2) is better than finetuned encoder (Case 2A-1).

- Pairing of finetuned encoder and finetuned decoder (Case 2A-3, Case 2A-4): performance loss in some cases considering mismatch on training data between UE and NW.

Then from the comparison between Case 2A-1, Case 2A-2, Case 2A-3 and Case 2A-4, it is seen that only finetuning decoder is better than only finetuning encoder. Encoder and decoder separate training may be the worst or better than only finetuning encoder, depending of the level of training data mismatch. Thus, reference encoder seems necessary in reference model based inter-vendor training collaboration, and then the reference model is at least the encoder.

Proposal 1: To achieve better field performance, the reference model should at least include the encoder part.

  - Considerations on Option 3 and Option 4 of test decoder

In this subsection, details of Option 3 and Option 4 of test decoder will be discussed. For 2-sided AI/ML model tests, it was agreed that test decoder/encoder is to be used in UE conformance tests and gNB conformance tests, respectively.

Consideration on Reference encoder

Requirement should be considered before the alignment work in Option 3 and Option 4. The test decoder(s) in detailed test case(s), would be used to verify that DUT will meet the defined requirement when communicating with TE. So, we would need reference encoder and decoder, to define requirement.

For both Option 3 and Option 4, since TE vendors will need to implement the test decoder based on the encoder, decoder or dataset in the spec, a verification procedure would be needed to verify that the decoder is correctly implemented and has the right level of performance. A verification encoder will be needed for the verification vendor to verify TE test decoder. TE may also need a verification encoder to test the performance of its test decoder. The simplest and most efficient way is to specify the reference encoder, which is used as verification encoder. The vendor, who is responsible for the TE test decoder verification, could just use the specified reference encoder to test the TE test decoder.

- No extra training procedure to train the verification encoder will be needed. If there is no specified reference encoder, the verification vendor will need considerable training time of hours or days and considerable computation resources, to train the verification encoder.

- No performance degradation from imperfectly matched verification encoder will be caused using different training method and/or train dataset. Currently, there is no aligned training method. RAN4 has aligned training data generation method, but does not have aligned dataset. If there is no specified reference encoder, the verification vendor will need to train the verification encoder using its own training method and/or train dataset. Then, different verification vendor would have different verification encoder, which may imperfectly match with TE test decoder. When the inference performance of test decoder and verification encoder is not so good, it may not be the problem caused by TE vendor.

Also, as discussed in Subsection 2.1, using reference encoder could achieve better field performance, since only finetuning decoder is better than only finetuning encoder.

Currently, RAN1 and RAN4 both work on reference model. RAN4 reference model is designed to derive test decoder, and achieve not worse performance than eType II codebook at least on pre-defined test condition. RAN1 reference model is used to generate encoder and decoder in field, and achieve acceptable performance gain than eType II in field. Both reference encoder and reference decoder are considered in RAN1 now. Then, reference encoder is needed to facilitate Rel-19 AI/ML study for reference model in RAN1 and RAN4.

Then we have the following proposal for reference encoder.

Proposal 2: Reference encoder needs to be specified, due to the following aspects:

- To be used as verification encoder in the TE test decoder verification.

  - No extra training procedure to train the verification encoder will be needed.

  - No performance degradation from imperfectly matched verification encoder will be caused.

- To achieve better field performance.

- To facilitate Rel-19 AI/ML study for reference model in RAN1 and RAN4.

Consideration on sub-options of Option 4

In last meeting, the following agreement of sub-options of Option 4 has been achieved.

Issue 4-4: Options for Option 4 Option 4 sub-options: - 4a: Dataset based: specify at least dataset based on which the test decoder can be implemented by TE vendors FFS whether anything else needs to be specified (e.g. model structure for decoder or a reference encoder) - 4b: Reference encoder based: encoder is documented in the specifications, used to derive a decoder to be implemented by TE vendors FFS whether training dataset (channel information) needs to be specified - 4c: do not specify either reference encoder or dataset. specify enough constraints on the decoder output ordering Companies are invited to bring further analysis on how to perform a feasibility study for these options.

For Option 4a, there would be two sub-options to generate the dataset.

- Option 4a-1: The specified dataset is generated by the aligned reference encoder and reference decoder. TE would train the test decoder based on the specified dataset.

  - Model structure needs to be specified. As seen in Appendix A, model structure (back-bone, parameters, e.g., number of layers, etc) also has significant performance impact even if complexity of model (in terms of FLOPS) are similar. So, if the model structure is not specified, different TE vendor would obtain the test decoder with different performance, which has large impact on the test. Then, it is necessary to specify model structure.

  - Dataset of raw channel or encoder input needs to be aligned. With the aligned reference encoder and reference decoder, to obtain one specified dataset with encoder input and encoder output, dataset of raw channel or encoder input needs to be aligned. There would be two feasible method to generate the raw channel or encoder input. The first method is to aggregate the dataset of raw channel or encoder input from all companies. The second method is to choose one dataset of raw channel or encoder input from all companies.

- Option 4a-2: The specified dataset is aggregated by the datasets provided by all companies. This sub-option is not feasible since TE vendor can not train the test decoder with acceptable performance. The simulation has been done in R18 and will be shown in the following. It is seen that at least the encoder from aggregated dataset could not work, and then different companies could not train the proper encoder to ensure the test decoder from aggregated dataset. Company#1 trains its model using Transformer with configuration#1 for encoder and Transformer with configuration#3 for decoder, and provides dataset#1. Company #2 trains its model using Transformer with configuration#2 for encoder and Transformer with configuration#4 for decoder, and provides dataset#2. Company#3 trains its local model using CNN with configuration#1 for encoder and CNN with configuration#2 for decoder, and provide dataset#3. Each dataset from one company consists of 100K samples, so there are 300K samples in the aggregated dataset. It is seen that

Table 2.2-1. Performance of the dataset aggregated by the datasets provided by companies.

| Test model pair in case 3 for type3 training | SGCS for 64-bit payload |
| --- | --- |
| Encoder from the aggregated dataset, while Decoder from Dataset#1 | 0.204 (no longer workable) |
| Encoder from the aggregated dataset, while Decoder from Dataset#2 | <0.1 (no longer workable) |
| Encoder from the aggregated dataset, while Decoder from Dataset#3 | <0.1 (no longer workable) |

Proposal 3: In Option 4a-1 (the specified dataset is generated by the aligned reference encoder and reference decoder), the model structure needs to be specified, to guarantee the performance of test decoder.

Proposal 4: In Option 4a-1 (the specified dataset is generated by the aligned reference encoder and reference decoder), dataset of raw channel or encoder input needs to be aligned, by aggregating the dataset of raw channel or encoder input from all companies, or choosing one dataset of raw channel or encoder input from all companies.

Proposal 5: The feasibility of Option 4a-2 (the specified dataset is aggregated by the datasets provided by all companies) could not be concluded, since at least the encoder from aggregated dataset could not work.

In Option 4b, channel generation method would also need to be in the spec, so that TE can derive the Training samples including only channel information could be obtained from specified channel generation method. Using the reference encoder and generated channel data, both TE vendor and DUT vendor could derive the decoder, which is paired with the reference encoder. DUT could directly use the reference encoder, or derive a new encoder based on the derived test decoder and generated channel data. Since TE vendors may derive test decoders with different performance, which would impact UE to pass the test. As seen in Appendix, it is suggested to capture the reference decoder model structure as well in the spec.

Proposal 6: In Option 4b (reference encoder based), channel generation method and reference decoder structure need to be in the spec.

Consideration on Reference model (Option 3, Option 4b) and Reference dataset (Option 4a-1)

It is seen from above discussion that both reference model and reference dataset would need the alignment of reference encoder and reference decoder. One of the main differences would be that what will be put in the spec. Reference decoder is at least captured in spec in Option 3, and the reference decoder is at least captured in spec in Option 4b. In Option 4a-1, the dataset, which contains both target channel (encoder input or decoder output) and latent information (encoder output or decoder input), generated from the reference encoder and reference decoder is at least captured in spec.

The main differences between reference model and reference dataset would be discussed in the following.

- Reference dataset would cause some performance loss compared to reference model. For reference dataset, separate training is used to obtain the test decoder, which would have some performance loss. The performance loss would depend on at least the following aspects:

  - The quality of dataset, e.g., the number of samples and the ergodicity of dataset. If the number of samples is not enough or the ergodicity of dataset is not good, the performance loss may be not acceptable.

  - The training method. Different training method would cause different performance. If proper training method is not in the spec or aligned, the performance loss may be also not acceptable.

  - The model structure. As seen from the results in Appendix A, model structure (back-bone, parameters, e.g., number of layers, etc) also has significant performance impact even if complexity of model (in terms of FLOPS) are similar. So, if the model structure is not specified, different TE vendor would obtain the test decoder with different performance, which has large impact on the test.

- The spec impact of writing model and dataset in the spec would be similar. Both model and dataset could be uploaded in cloud or FTP, and the link could be captured in the spec. RAN4 already has aligned method to describe the model structure, and open format file (e. g., ONNX) could provide the details of model. So, the description of model is not an issue.

- Reference dataset needs extra step compared to reference model, which is dataset generating from the aligned reference encoder and reference decoder. After the alignment of reference encoder and reference decoder, the reference encoder or reference decoder can be capture in the spec and work is finished. However, for reference dataset, the dataset would need to be generated by the aligned reference encoder and reference decoder, where dataset of raw channel or encoder input needs to be aligned.

- The storage size of reference dataset is larger than reference model. From current discussion of logistical issues, it is aligned that the dataset would be 1GB~10GB and model would be 7MB~100MB. Note that the model size may be reduced in future RAN4 study.

- Reference model is more flexible than referend dataset for future release. In the future release, new channel model or scenario may be defined, and reference model may be also workable for these channel model or scenario.

From the analysis, it is seen that there is no essential difference between reference model and reference dataset. Reference dataset may have some disadvantages compared to reference model, and the benefit of reference dataset over reference model is unclear.

Proposal 7: There is no essential difference between reference model and reference dataset. Reference dataset may have some disadvantages compared to reference model, and the benefit of reference dataset over reference model is unclear.

- Reference dataset would cause some performance loss compared to reference model.

- The spec impact of writing model and dataset in the spec would be similar.

- Reference dataset needs extra step compared to reference model, which is dataset generating from the aligned reference encoder and reference decoder.

- The storage size of reference dataset is larger than reference model.

- Reference model is more flexible than referend dataset for future release.

Reference encoder and decoder are needed for requirement derivation. Since there is no essential difference between reference model and reference dataset, RAN4 can work on the alignment of reference encoder and reference decoder first, and later to discuss what will be put in the spec.

Proposal 8: Reference encoder and reference decoder are needed for requirement derivation, and could largely be used for test decoder derivation for both Option 3 and Option 4 (Option 4a-1 and Option 4b). RAN4 to work on reference encoder and reference decoder firstly. Later to discuss what will be put in the spec.

For feasibility studies of AI/ML model for test options, we believe similar conclusion can be drawn for both Option 3 and Option 4 (Option 4a-1 and Option 4b). The difference between Option 3 and Option 4 is what AI/ML model(s) for test decoder implementation is captured in the spec. The evaluation should be same in terms of performance. Currently, the discussion was focused on Option 3 due to limited meeting time, but it is important to discuss Option 4 as well since both options have so many things in common. Otherwise, the entire progress of testability for test options will be delayed.

Proposal 9: Similar feasibility conclusion through the simulation campaign of test options can be draw for both Option 3 and Option 4 (Option 4a-1 and Option 4b).

Relationship between reference/test model and requirement/test case

Now we consider the relationship between reference/test model and requirement/test case.

To make the test case and requirement to be simple and effective, one test decoder could be used for one test case or multiple test case (if the generalization performance is well for multiple test case). There is no need to define multiple test decoder in one test case.

Proposal 10: One test decoder could be used for one test case or multiple test case.

Considering model Performance, model complexity and UE implementation flexibility, it is suggested to define different reference encoder for different requirement or test case. Some complex reference encoder but with good performance may be needed for some high requirement or complex test case, while Some simple reference encoder but with lower performance may be needed for some low requirement or simple test case.

Proposal 11: Different reference encoder may be defined for different requirement or test case.

  - Next steps of Option 3 and Option 4

In RAN4#112 meetings, Option 3 next steps have been agreed.

Issue 4-1: Option 3 next steps - Check on performance alignment -> see simulation results from contributing companies - repeat simulations until good alignment is achieved - move to next step after alignment - Share models (encoder or decoder or both)/datasets (training/testing/inference) - framework to share data among companies(dataset/model) to be discussed - Select one or more decoder for further analysis - selection criteria to be discussed - Each company brings results for training of “own encoder” with selected decoder(s) - performance alignment to be checked/discussed - using “own” data or data shared by other companies - Conclude on overall feasibility of Option 3 - consider the conditions under which Option 3 is feasible if found feasible - FFS on what the feasibility criteria Note: some steps could be reused for Option 4, FFS on what can be reused

In last meeting, next steps of Option 3 and Option 4 have been agreed as following.

Next step: Interested companies to share decoder and encoder ML models and dataset in next meeting (RAN4#113) - Model and dataset used to generate the data submitted to RAN4#112-bis Additional data to be submitted: - CDF of SGCS per subband - CDF of the largest eigen value of the channel matrix of training data for each subbands (preferably to be submitted in an xls format) Interested companies are invited to bring further proposals for the next steps for the feasibility study of Option 4 (for any of the sub-options of Option 4) - Analysis of commonality with option 3(whether any steps or data generated for Option 3 can be reused) would be useful

In the following, next steps of Option 3 and Option 4 for RAN4#113 will be discussed.

Consider Option 3 first. Since companies will share encoder and decoder in RAN4#113, the next step of Option 3 would be the decoder selection. In this Step, selection criteria need to be discussed. Since the datasets would be shared along with models, the aggregated dataset with only encoder input could be obtained. Then companies could provide their performance on the aggregated dataset, and then RAN4 can choose the decoder from the encoder and decoder pair, which has the best SGCS performance on the aggregated dataset with only encoder input. If there are multiple encoder and decoder pair has similar SGCS performance, one pair could be chosen randomly.

In final step of Option 3, feasibility criteria will be needed to find whether Option 3 is feasible. In Option 3, at least the test decoder is put in the spec and TE will implement the specified test decoder. Based on the specified test decoder, DUT vendor would train the encoder considering the test scenario and real wireless channel, and it is expected that the trained encoder could work well with the test decoder and pass the RAN4 test. So, the feasibility criteria could be that nearly all companies can obtain acceptable performance using their “own encoder” paired with selected decoder(s).

Proposal 12: Option 3 next steps for RAN4#113:

- Select one or more decoder for further analysis

  - selection criteria: select the decoder from the encoder and decoder pair, which has the best SGCS performance on the aggregated dataset with only encoder input.

    - If there are multiple encoder and decoder pair has similar SGCS performance, one pair could be chosen randomly.

- Company brings results for training of “own encoder” with selected decoder(s)

  - performance alignment to be checked/discussed

  - using “own” data or data shared by other companies

- Conclude on overall feasibility of Option 3

  - consider the conditions under which Option 3 is feasible if found feasible

  - feasibility criteria: nearly all companies can obtain acceptable performance using their “own encoder” paired with selected decoder(s).

The main difference between Option 3 and Option 4a-1 is that reference decoder is specified in Option 3 but dataset is specified in Option 4a-1. So that the new round of performance alignment and models/datasets sharing is not needed, which would take a lot of efforts but provide little benefits. Note that a pair of encoder and decoder is chosen in Option 3 in fact and the pairs of encoder and decoder will be shared by companies in RAN4#113. Companies can put the channel inputs into the encoder and then obtain the corresponding latent information (PMI), so that the slected dataset with encoder input and encoder output is obtained. Option 4a-1 next steps can be slightly modified from Option 3 next steps.

Proposal 13: Option 4a-1 next steps for RAN4#113:

- Select one or more dataset(s) for further analysis

  - selection criteria: select the dataset generated from the encoder and decoder pair, which has the best SGCS performance on the aggregated dataset with only encoder input.

    - If there are multiple encoder and decoder pair has similar SGCS performance, one pair could be chosen randomly.

- Company brings results for training of “own encoder and decoder” with selected dataset(s)

  - performance alignment to be checked/discussed

  - using “own” training method or training method shared by other companies

- Conclude on overall feasibility of Option 4a-1

  - consider the conditions under which Option 4a-1 is feasible if found feasible

  - feasibility criteria: nearly all companies can obtain acceptable performance using their “own encoder and decoder” paired with selected dataset(s).

The main difference between Option 3 and Option 4b is that reference decoder is specified in Option 3 but encoder is specified in Option 4b. Option 4b next steps can be slightly modified from Option 3 next steps.

Proposal 14: Option 4b next steps for RAN4#113:

- Select one or more encoder for further analysis

  - selection criteria: select the encoder from the encoder and decoder pair, which has the best SGCS performance on the aggregated dataset with only encoder input.

    - If there are multiple encoder and decoder pair has similar SGCS performance, one pair could be chosen randomly.

- Company brings results for training of “own decoder” with selected encoder(s)

  - performance alignment to be checked/discussed

  - using “own” data or data shared by other companies

- Conclude on overall feasibility of Option 4b

  - consider the conditions under which Option 4b is feasible if found feasible

  - feasibility criteria: nearly all companies can obtain acceptable performance using their “own decoder” paired with selected encoder(s).

The summary of main steps of Option 3, Option 4a-1 and Option 4b is shown in the below table. It is seen that these options have the similar steps and could work in parallel.

Table 2.3-1. The summary of main steps of Option 3, Option 4a-1 and Option 4b

|  | Option 3 | Option 4a-1 | Option 4b |
| --- | --- | --- | --- |
| Step 1 (finished) | Check on performance alignment -> see simulation results from contributing companies |  |  |
| Step 2 (to be finished in RAN4#113) | Share models (encoder or decoder or both)/datasets (training/testing/inference) |  |  |
| Step 3 | Select one or more encoder and decoder pair(s) |  |  |
|  | Decoder(s) are from the encoder and decoder pair(s) | Dataset(s) are generated from the selected encoder and decoder pair(s) | Encoder(s) are from the encoder and decoder pair(s) |
| Step 4 | Company brings results for training of “own encoder” with selected decoder(s) | Company brings results for training of “own encoder and decoder” with selected dataset(s) | Company brings results for training of “own decoder” with selected encoder(s) |
|  | Performance alignment to be checked/discussed |  |  |
| Step 5 | Conclude on overall feasibility for each option. |  |  |
| Step 6 | Determine which option(s) to be used for test. FFS whether the decision is made in the SI phase. |  |  |

Proposal 15: The feasibility study of Option 3, Option 4a-1 and Option 4b could work in parallel.

  - Simulation results for aligned reference encoder and reference decoder

In previous RAN4 meeting, the simulation assumptions have been agreed for feasibility study of aligning model among companies. In the e-mail discussion after RAN4#111 meeting, a CNN based model structure pair for both encoder and decoder has been aligned for feasibility study. In last meeting, more details of data generation and training method have also been aligned to further reduce the mismatch between companies. The detailed agreements are listed in the Appendix B.

Based on the above aligned model structure and simulation assumptions, we have trained the new encoder and decoder pair. The SGCS of AI/ML model for 64 bits payload is 0.732, AI/ML model for 64 float value is 0.784 and the SGCS of eType II is 0.720. Note that we use the baseline simulation assumptions in the SLS simulation assumption table agreed in the last meeting to perform the simulations. However, the BS Tx power equals 41dBm for 10 MHz BW, which is the baseline assumption, is missed in the table.

The CDF curve of SGCS for aligned model structure and simulation assumptions is shown in the below figure. It is seen that AI/ML model has better mean performance but worse variance performance, compared to eType II. For example, the 5% SGCS of eType II is about 0.5, while the 5% SGCS of AI/ML model is about 0.2. The reason would be that the loss function of AI/ML model is the mean SGCS. If modifying the loss function, the variance performance of AI/ML model could be largely improved.

Besides, we also provide the CDF of SGCS per sub-band for further performance alignment, which is shown in figure 2.3-2. It can be observed that the performance of each sub-band is different and the biggest variance between sub-band could reach to almost 0.2.

Observation 3: Based on the aligned model structure and simulation assumptions, the SGCS of AI/ML model for 64 bits payload is 0.732, AI/ML model for 64 float value is 0.784, and the SGCS of eType II is 0.720. The CDF curve of SGCS is shown in Figure 2.4-1. The CDF of SGCS per sub-band is shown in Figure 2.4-2.

|  | eType II | AI/ML model for 64 bits | AI/ML model for 64 float value |
| --- | --- | --- | --- |
| SGCS | 0.720 | 0.732 | 0.784 |

Figure 2.4-1. CDF curve of SGCS for aligned model structure and simulation assumptions

Figure 2.4-2. CDF curves of SGCS per sub-band for aligned model structure and simulation assumptions

Observation 4: It is seen that AI/ML model has better mean performance but worse variance performance, compared to eType II. The loss function for training may be further studied in the future.

Note that in above FLOPs, one FLOP is one multiplication and one addition. If one FLOP is one multiplication or one addition, the FLOPs of encoder is 94M and the FLOPs of decoder is 1480M.

Currently, the FLOPs of encoder and decoder are much higher than the FLOPs of eType II. In Future actual reference encoder and reference decoder design, RAN4 may consider smaller model, for the convenience of implementation. Transformer, MLP, MLP-mixer may be also considered in future.

Proposal 16: In future actual reference encoder and reference decoder design, RAN4 may consider smaller model, for the convenience of implementation.

  - Suggested simulation assumptions for reference model for both encoder and decoder

In order to consider model performance with more concrete details, companies are encouraged to bring parameters/values proposals, considering RAN4 existing test configuration or RAN1 baseline scenario captured in TR 38.843 in Table 6.2.1-2 for example.

Below table presents the baseline link level simulation assumptions of CDL channel for reference model derivation, where some unsuitable information is removed.

Table 2.5-1. Baseline link level simulation assumptions of CDL channel for reference model derivation.

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | FDD OFDM |
| Carrier frequency | 2GHz |
| Bandwidth | 20MHz |
| Subcarrier spacing | 15kHz |
| Nt | 32: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Nr | 4: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ |
| Channel model | CDL-C |
| UE speed | 3kmhr |
| Delay spread | 30ns |
| Channel estimation | Realistic channel estimation algorithms (e.g., LS or MMSE) as a baseline. Ideal DL channel estimation is optionally taken into the baseline of evaluation methodology for the purpose of calibration and/or comparing intermediate results (e.g., accuracy of AI/ML output CSI, etc.). Up to companies to report whether/how ideal channel is used in the dataset construction and performance evaluation/inference. |
| Rank per UE | Rank 1-4. Companies are encouraged to report the Rank number, and whether/how rank adaptation is applied |
| Note: the baseline EVM is used to compare the performance with the benchmark release, while the AI/ML related parameters (e.g., dataset construction, generalization verification, and AI/ML related metrics) can be of additional/different assumptions. The conclusions for the use cases in the SI should be drawn based on generalization verification over potentially multiple scenarios/configurations. |  |

Based on our proposed LLS assumptions for CDL-C channel model and CNN model structures proposed in our contributions [3] in the previous meeting, below table shows the SGCS results for different model structures under the different tests and training datasets. Note that the UMa dataset is generated based on the SLS assumption from RAN1 and related parameters are same as our proposed LLS assumptions for CDL-C. The mixed dataset contains the partial CDL-C dataset and UMa dataset, which are mixed together in one dataset randomly for model training.

Table 2.5-2. SGCS under different channel model for the proposed CNN and Transformer.

| Model structure |  | CNN |  | Transformer |  |
| --- | --- | --- | --- | --- | --- |
| CSI feedback bits | Training dataset | Test dataset |  | Test dataset |  |
|  |  | CDL-C | UMa | CDL-C | UMa |
| 116 bits | CDL-C | 0.994 | 0.336 | 0.996 | 0.253 |
|  | UMa | 0.791 | 0.787 | 0.831 | 0.821 |
|  | Mixed | 0.982 | 0.773 | 0.988 | 0.801 |
| CNN encoder: FLOPS 126M, size 0.1M. CNN decoder: FLOPS 1974M, size 2.4M. Transformer encoder: FLOPS139M, size 5.6M. Transformer decoder: FLOPS 139M, size 5.6M Note: Mixed training dataset includes CDL-C and UMa channel model. |  |  |  |  |  |

It can be observed that, the models trained under CDL-C dataset could provide a very high SGCS results under the CDL-C test datasets, while the performance degrades severely under the UMa test datasets. However, the models trained under UMa could provide a medium good performance of SGCS for UMa test dataset, and provide a higher SGCS under the CDL-C test dataset compared with the model training under CDL-C but tested in UMa. For the model trained under mixed dataset, it could provide good SGCS results under CDL-C and UMa test datasets and only degrade a little compared with the results of <CDL-C trained, CDL-C tested> and <UMa trained, UMa tested>. Thus, the best option is to use the mixed dataset for training and one of types of the dataset for testing. Since the TDL channel is a simpler channel compare with CDL and UMa, it can also be used for mixing and RAN4 test case. Similar to the previous test case, RAN4 tests could only use TDL channel. Based on the following analysis, we could have the following proposal.

Proposal 17: Using the mixed dataset for model training, including the mixing of TDL, CDL and UMa, while using the TDL dataset for RAN4 tests. Other mixing rules are not precluded.

- Summary

Based on above analysis, following proposals and observations are present.

Observation 1: From initial results for field test, the generalization performance of AI/ML model trained by UMa simulation data on field data seems acceptable, which has similar performance as eType II codebook. The generalization performance of AI/ML model trained by CDL simulation data on field data is worse than AI/ML model trained by UMa simulation data.

Observation 2: From initial results of field test, it is observed that

- Directly use reference model in field (Case 2): similar performance on field data as eType II for one cell and performance loss compared to eType II is observed for another cell.

- Finetuned encoder or decoder using field data against reference decoder/encoder (Case 2A-1, Case 2A-2): performance improved compared to directly use reference model (Case 2), but still has performance loss compared to fully trained by field data (Case1). Finetuned decoder (Case 2A-2) is better than finetuned encoder (Case 2A-1).

- Pairing of finetuned encoder and finetuned decoder (Case 2A-3, Case 2A-4): performance loss in some cases considering mismatch on training data between UE and NW.

Proposal 1: To achieve better field performance, the reference model should at least include the encoder part.

Proposal 2: Reference encoder needs to be specified, due to the following aspects:

- To be used as verification encoder in the TE test decoder verification.

  - No extra training procedure to train the verification encoder will be needed.

  - No performance degradation from imperfectly matched verification encoder will be caused.

- To achieve better field performance.

- To facilitate Rel-19 AI/ML study for reference model in RAN1 and RAN4.

Proposal 3: In Option 4a-1 (the specified dataset is generated by the aligned reference encoder and reference decoder), the model structure needs to be specified, to guarantee the performance of test decoder.

Proposal 4: In Option 4a-1 (the specified dataset is generated by the aligned reference encoder and reference decoder), dataset of raw channel or encoder input needs to be aligned, by aggregating the dataset of raw channel or encoder input from all companies, or choosing one dataset of raw channel or encoder input from all companies.

Proposal 5: The feasibility of Option 4a-2 (the specified dataset is aggregated by the datasets provided by all companies) could not be concluded, since at least the encoder from aggregated dataset could not work.

Proposal 6: In Option 4b (reference encoder based), channel generation method and reference decoder structure need to be in the spec.

Proposal 7: There is no essential difference between reference model and reference dataset. Reference dataset may have some disadvantages compared to reference model, and the benefit of reference dataset over reference model is unclear.

- Reference dataset would cause some performance loss compared to reference model.

- The spec impact of writing model and dataset in the spec would be similar.

- Reference dataset needs extra step compared to reference model, which is dataset generating from the aligned reference encoder and reference decoder.

- The storage size of reference dataset is larger than reference model.

- Reference model is more flexible than referend dataset for future release.

Proposal 8: Reference encoder and reference decoder are needed for requirement derivation, and could largely be used for test decoder derivation for both Option 3 and Option 4 (Option 4a-1 and Option 4b). RAN4 to work on reference encoder and reference decoder firstly. Later to discuss what will be put in the spec.

Proposal 9: Similar feasibility conclusion through the simulation campaign of test options can be draw for both Option 3 and Option 4 (Option 4a-1 and Option 4b).

Proposal 10: One test decoder could be used for one test case or multiple test case.

Proposal 11: Different reference encoder may be defined for different requirement or test case.

Proposal 12: Option 3 next steps for RAN4#113:

- Select one or more decoder for further analysis

  - selection criteria: select the decoder from the encoder and decoder pair, which has the best SGCS performance on the aggregated dataset with only encoder input.

    - If there are multiple encoder and decoder pair has similar SGCS performance, one pair could be chosen randomly.

- Company brings results for training of “own encoder” with selected decoder(s)

  - performance alignment to be checked/discussed

  - using “own” data or data shared by other companies

- Conclude on overall feasibility of Option 3

  - consider the conditions under which Option 3 is feasible if found feasible

  - feasibility criteria: nearly all companies can obtain acceptable performance using their “own encoder” paired with selected decoder(s).

Proposal 13: Option 4a-1 next steps for RAN4#113:

- Select one or more dataset(s) for further analysis

  - selection criteria: select the dataset generated from the encoder and decoder pair, which has the best SGCS performance on the aggregated dataset with only encoder input.

    - If there are multiple encoder and decoder pair has similar SGCS performance, one pair could be chosen randomly.

- Company brings results for training of “own encoder and decoder” with selected dataset(s)

  - performance alignment to be checked/discussed

  - using “own” training method or training method shared by other companies

- Conclude on overall feasibility of Option 4a-1

  - consider the conditions under which Option 4a-1 is feasible if found feasible

  - feasibility criteria: nearly all companies can obtain acceptable performance using their “own encoder and decoder” paired with selected dataset(s).

Proposal 14: Option 4b next steps for RAN4#113:

- Select one or more encoder for further analysis

  - selection criteria: select the encoder from the encoder and decoder pair, which has the best SGCS performance on the aggregated dataset with only encoder input.

    - If there are multiple encoder and decoder pair has similar SGCS performance, one pair could be chosen randomly.

- Company brings results for training of “own decoder” with selected encoder(s)

  - performance alignment to be checked/discussed

  - using “own” data or data shared by other companies

- Conclude on overall feasibility of Option 4b

  - consider the conditions under which Option 4b is feasible if found feasible

  - feasibility criteria: nearly all companies can obtain acceptable performance using their “own decoder” paired with selected encoder(s).

Proposal 15: The feasibility study of Option 3, Option 4a-1 and Option 4b could work in parallel.

Observation 3: Based on the aligned model structure and simulation assumptions, the SGCS of AI/ML model for 64 bits payload is 0.732, AI/ML model for 64 float value is 0.784, and the SGCS of eType II is 0.720. The CDF curve of SGCS is shown in Figure 2.4-1. The CDF of SGCS per sub-band is shown in Figure 2.4-2.

|  | eType II | AI/ML model for 64 bits | AI/ML model for 64 float value |
| --- | --- | --- | --- |
| SGCS | 0.720 | 0.732 | 0.784 |

Figure 2.4-1. CDF curve of SGCS for aligned model structure and simulation assumptions

Figure 2.4-2. CDF curves of SGCS per sub-band for aligned model structure and simulation assumptions

Observation 4: It is seen that AI/ML model has better mean performance but worse variance performance, compared to eType II. The loss function for training may be further studied in the future.

Proposal 16: In future actual reference encoder and reference decoder design, RAN4 may consider smaller model, for the convenience of implementation.

Proposal 17: Using the mixed dataset for model training, including the mixing of TDL, CDL and UMa, while using the TDL dataset for RAN4 tests. Other mixing rules are not precluded.

- References

- 3GPP TR38.843, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18)”, V18.0.0, 2023-12.

- RP-242399, “Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, RAN#105, Sep 9-12, 2024.

- R4-2408294, vivo, “Discussion on testability and interoperability issues for CSI compression and CSI prediction”, RAN4#111, May 20 – 24, 2024.

- Appendix A: Comparison of AI model performance for CSI compression

In order to better understand the performance difference for different AI models, initial evaluation was conducted under the CDL-A channel model with 30ns delay. Fig. A-1 shows the simulation procedure of the CSI compression.

Fig. A-1. Illustration of the simulation procedure of CSI compression

The simulation results are provided in Table A-1 with transformer encoder and Table A-2 with CNN encoder, respectively.

Table A-1. Performance comparison for different decoder structure and complexity (Transformer encoder)

| Encoder | Decoder |  |  |  | SGCS (R16 CB: 0.8375) |
| --- | --- | --- | --- | --- | --- |
|  | Back-bone | Model parameter | Number of model parameter | FLOPS |  |
| Back-bone: Transformer Number of model parameter: 4.1M FLOPS: 5×107 Depth: 10 transformer blocks Width: embedding=140 | Transformer | Depth: 18 transformer blocks Width: embedding=148 | 8.0M | 2×108 | 0.80 |
|  |  | Depth: 10 transformer blocks Width: embedding=198 | 8.0M | 2×108 | 0.92 |
|  |  | Depth: 20 transformer blocks Width: embedding=140 | 7.9M | 2×108 | 0.62 |
|  | CNN | Depth: conv layers Width: 165 feature maps | 0.2M | 2×108 | 0.88 |
|  |  | Depth: 17 conv layers Width: 41 feature maps | 0.2M | 2×108 | 0.91 |
|  |  | Depth: 150 conv layers Width: 20 feature maps | 0.2M | 2×108 | 0.92 |
|  | MLP | Depth: 1 FC layers Width: 9500 neurons | 98.7M | 2×108 | 0.91 |
|  |  | Depth: 10 FC layers Width: 3200 neurons | 109.6M | 2×108 | 0.89 |
|  |  | Depth: 15 FC layers Width: 2600 neurons | 106.6M | 2×108 | 0.83 |

In Table A-1, transformer encoder is used in the evaluation for verifying performance of different decoders. It can be seen that there could be large performance variance if model structure, including back-bone, parameters of decoder are different even if the complexity (FLOPs) are similar. For same type of decoder, if parameters are different, especially for transformer and MLP type of decoders, performance variance can be observed. The performance of CNN decoder seems not very sensitive to some parameters as listed in the table.

Table A-2. Performance comparison for different decoder structure and complexity (CNN encoder)

| Encoder | Decoder |  |  |  | SGCS (R16 CB: 0.8375) |
| --- | --- | --- | --- | --- | --- |
|  | Back-bone | Model parameter | Number of model parameter | FLOPS |  |
| Back-bone: CNN Number of model parameter: 4.1M FLOPS: 5×107 Depth: 17 conv layers Width: 32 feature maps | CNN | Depth: 17 conv layers Width: 16 feature maps | 0.08M | 4×107 | 0.88 |
|  | Transformer | Depth: 4 transformer blocks Width: embedding =110 | 1.4M | 4×107 | 0.84 |
|  | MLP | Depth: 3 FC layers Width: 2200 neurons | 16.5M | 4×107 | 0.89 |

In table A-2, CNN encoder is used in the evaluation for verifying performance of different decoders. Similar observations can be made.

- Appendix B: Agreement of assumptions for aligned reference encoder and reference decoder

In previous RAN4 meeting, the simulation assumptions have been agreed for feasibility study of aligning model among companies.

System-level simulation parameters (from TR): Parameter; Value. Duplex, Waveform; FDD, OFDM. Multiple access; OFDMA. Scenario; Dense Urban (Macro only). Frequency Range; FR1 only, [2GHz, 4GHz]. Inter-BS distance; 200m. Channel model; According to TR 38.901. Antenna setup and port layouts at gNB; Companies need to report which option(s) are used between - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ. Antenna setup and port layouts at UE; 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for (rank 1-4). BS Tx power; 44dBm for 20MHz. BS antenna height; 25m. UE antenna height & gain; Follow TR36.873. UE receiver noise figure; 9dB. Numerology; Slot/non-slot; 14 OFDM symbol slot. SCS; Baseline: 15kHz for 2GHz; Optional: 30kHz for 4GHz. Simulation bandwidth; Baseline: 10 MHz for 15kHz Optional: 20 MHz for 30kHz. Frame structure; Slot Format 0 (all downlink) for all slots. MIMO scheme; SU-MIMO. MIMO layers; Baseline: 1 Optional: 2. CSI feedback; Feedback assumption at least for baseline scheme - CSI feedback periodicity (full CSI feedback): 5 ms (baseline) - Scheduling delay (from CSI feedback to time to apply in scheduling): 4 ms. Traffic load (Resource utilization); Baseline: 50% Optional: 20/70%. UE distribution; CSI compression: 80% indoor (3 km/h), 20% outdoor (30 km/h). UE receiver; MMSE-IRC as the baseline receiver. Feedback assumption; Realistic. Channel estimation; Realistic or ideal channel estimation.

In the e-mail discussion after RAN4#111 meeting, a CNN based model structure pair for both encoder and decoder has been aligned for feasibility study.

Based on the two rounds of discussions, the following parameters are proposed: Fig 1. Detailed model diagrams of encoder. Fig 2. Detailed model diagrams of decoder. Table 1. CNN model parameters for encoder. Model architecture parameters for encoder (convolutional); Model type; CNN (Resnet like design). FLOPs; 47M (vs. 1.6M for eTypeII). Model size; 448Kbyte. Num of Resnet blocks; 6. Layer after resnet blocks; 2 layers (one FC layer, one Conv layer). Layer before resnet blocks; 1 layer (one conv layer). Layer within a resnet block; 2 Conv layer and direct combination. CNN kernel parameters; - dilation =1 (which is the default parameter) - padding type = zero padding (which is the default parameter) - The value of groups =1 (which is default parameter). Kernel/Channel size within a resnet block; (3,3,32). Size of message (feature map) passed across layers; (13,32). Table 2. CNN model parameters for decoder. Model architecture parameters for decoder (convolutional); Model type; CNN (Resnet like design). FLOPs; 740M. Model size; 7.2Mbyte. Num of Resnet blocks; 6. Layer before resnet blocks; 2 layers (one FC layer, one Conv layer). Layer after resnet blocks; 1 layer (one conv layer). Layer within a resnet block; 2 Conv layer and direct combination. CNN kernel parameters; - dilation =1 (which is the default parameter) - padding type = zero padding (which is the default parameter) - The value of groups =1 (which is default parameter). Kernel/Channel size within a resnet block; (3,3,128). Size of message (feature map) passed across layers; (13,32). Other model related parameters: Activation function; ReLU. Latent message size; 32 float values. Payload size, bits; 64 per layer.. Quantization; 2 bits. System parameters: Frequency range; 2GHz (4GHz optional). MIMO Layers; 1 layer (2 layers optional, companies to. Sub-bands; 13. Channel estimation; ideal. Metrics; SGCS of AI CSI feedback and eTypeII CSI feedback, both relative to ideal CSI.. Companies are also invited to provide the training parameters used.

In last meeting, more details of data generation and training method have also been aligned to further reduce the mismatch between companies

Parameter/Configurations; Options/Values; Company comments. Channel/data-set related. UE antenna element assumptions; - Option 1: Omni-directional antenna elements (Ericsson) - Option 2: Directional antenna elements - Option 3: Other; [Apple] We provided 2 simulation results: Directional (both horizontal/vertical) Omni (both horizontal/vertical) [QC] Option 1 (both horizontal and vertical) [Intel] Option 1 [OPPO]:Option1 [CMCC] option 1. Configuration for alignment: - Ideal omni-directional antenna elements following RAN1 assumptions. Sub-band channel averaging; - [Nokia, QC, Apple, vivo, Ericsson,OPPO] Average of covariance channel matrix per RBs in the sub-band before SVD - [ZTE]: Per sub-band - [CMCC]: Average of raw channel matrix per RBs in the sub-band before SVD; CATT: Prefer not to do the averaging before SVD.. Configuration for alignment: - Average of covariance channel matrices per PRBs in the sub-band - 1 sample per PRB is selected. Any other pre-processing; - To be listed if used - [QC, Ericsson] Normalize the phase of all entries of the eigenvector based on the phase of the 1st entry of the eigenvector - [OPPO] Normalize the phase of all entries of the eigenvector based on the phase of the last entry of the eigenvector - [Apple]: Normalize the phase of all entries of the eigenvector based on the phase of that entry that has the maximum amplitude averaged over the sub-bands; [Intel] prefer no other pre-processing. Configuration for alignment: - Apply phase normalization: - Normalize the phase of all entries of the eigenvector based on the phase of the 1st entry of the eigenvector - i.e. the phase of the first value of the eigenvector is 0. - L2 norm of the eigenvector is equal to 1.. Data collection assumptions; - [Nokia, CATT, ZTE, Ericsson, CMCC] Each channel realization is collected from independent UE drop - Multiple channel realization can be collected from the same UE - Other - [QC] Multiple channels realizations are collected from multiple UEs and multiple drops of same UE - [Apple, Intel, OPPO]: Multiple drops. Each drop collect data from multiple UEs. Additionally collect samples from channel realization from each UE spaced at de-correlated interval.. Configuration for alignment: - Strive to ensure that the samples are decorrelated, especially when collected from the same UEs.. Training dataset size; - [Nokia]: 630K - [QC]: 450K - [Samsung]: 25K - [R&S]: 614,440 - [MTK]: 700K - [Ericsson]: 800K - [Intel]: 200K - [CATT]: 102,600 - [ZTE]: 600k - [Apple]: 600K - [vivo]: 243000 - [OPPO]：600k - [CMCC]: 100K. Configuration for alignment: - Aim to use the dataset around 600K. - At least 100K sample to be used.. Dataset quantization; - Float32. Other data-set characteristics/statistics; - Companies to propose. - Companies are encouraged to report some statistics about the dataset. - Statistics are FFS - Companies are encouraged to report the CDF of the SGCS across sub-bands and input samples. - If possible, also report CDF for eTypeII.. Model-related. Quantization of encoder output; - [Nokia]: Non-trainable quantization (bypassed during training) - [QC, CMCC] Quantization aware training - [Apple]: Quantization in forward and approximation in backward to enable learning. (avoid zero gradient of round function) - [vivo] VQ with quantization aware training - [Intel] Quantization aware training. Scalar quantization with non-trainable quantization grid. - [Ericsson] 2 bit scalar quantization, quantizer aware training; CATT: Consider to align the activation function and quantization method.. - Report results without quantization of the FC layer outputs for validation purposes only, i.e., float32 is sent in CSI report - Companies still need to report quantization results: - Companies to report whether/how quantization aware training is implemented (e.g., how quantization is treated at back propagation) - Companies can try quantization aware training with bypassing of the quantization in back propagation - Quantization options, companies to provide the details: - Fixed unform quantization with equal ranges - Trainable quantization. Normalization before quantization in the encoder; - [Nokia, Ericsson]: Normalization with tanh function after FC layer - [CATT, ZTE, vivo, CMCC]: Sigmoid function used after FC layer - [QC] Normalization not done due to the use of quantization aware training - [Apple]: Normalization with tanh function after FC layer. We don’t apply inverse of that in de-quantization - [Intel] Sigmoid after FC layer. Inverse sigmoid after de-quantization stage in decoder.. Configuration for alignment: - For the results with quantization, to use a sigmoid function in the encoder for normalization - Sigmoid in the decoder is optional.. Use and configuration of drop-out; - [Nokia]: Use of on single drop-out layer after FC layer in the encoder and decoder, Drop-out rate = 0.4. - [CATT, QC, ZTE, Apple, vivo, Intel, Ericsson, CMCC]: No drop-out used.. Configuration for alignment: - Not to use drop-out layers. Training loss function, use of regularization; - [Nokia]: - SGCS with L2 regularization with weight decay, λ = 10-2 - [QC]: SGCS + additional terms for quantization aware training - [Apple, CATT, CMCC]: 1- SGCS - [R&S, MTK, Intel]: SGCS - [Ericsson]: NMSE - [ZTE]: MSE - [vivo]: 1-SGCS plus VQ loss for encoder output (MSE between original and quantized encoder output) - [Intel]: SGCS, no L1/L2 regularization. Configuration for alignment: - Use (1-SGCS) as loss function as reference. Optimizer; - [Samsung, QCM, Apple, MTK, Intel, CATT, Ericsson, CMCC]: Adam - [Nokia]: RMSProp (Root Mean Square Propagation) - [ZTE, vivo]: AdamW. Configuration for alignment: - Use Adam. Learning rate; - [Nokia]: - [QC, R&S, CATT, Ericsson]: 10^-4 - [R&S]: Learning rate adaptation: Reduce on plateau by factor 0.5 with patience of 50 epochs. - [Apple]: 0.0025 - [Samsung, Intel, MTK, CMCC]: 0.001 - [ZTE]:1.5e-3 - [vivo]: Warm-up to initial lr 1e-3 and then linearly decay to final lr 8e-4, update after each batch. Configuration for alignment: - Fixed learning rate of 10^-4. Batch size; - [Nokia]:1024 - [QC, MTK, CMCC]: 128 - [Apple]: 256 - [Samsung]: 200 - [R&S, CATT, vivo, Ericsson]: 512 - [Ericsson]: 32 - [Intel]: 10 - [ZTE]:400. Configuration for alignment: - 256. (Max) Number of epochs; - [QC]: 150 - [Apple, vivo, Intel]: 100 - [R&S, Nokia, CATT]: 200 - [MTK, ZTE]: 300 - [CMCC]: 500. Configuration for alignment: - 150 epochs. Stopping criteria; - [Nokia, Apple, CATT, QC, ZTE, Intel, CMCC]: No change in metric over a given number of epochs - Validation patience: [Intel, Nokia] 10, [QC] 50 - + Ericsson with constraint on max Epochs - [vivo] Stop when reaching the configured number of epochs. Configuration for alignment: - No improvement over 25 epochs.. Weight initialization; - [Apple, CATT, QC, vivo, Intel, CMCC]: Default - Ericsson: Variance scaling. Configuration for alignment: - Default. Kernel Parameters: - Dilation - Padding Type The value of groups (VG); - [CMCC, Samsung, Vivo, CATT]: Dilation:1, Padding type=0, VG= 1 - [QC, ZTE, Apple, Intel]: Default. Configuration for alignment: - Default.