3GPP TSG-RAN WG4 Meeting #113            	R4-2418935

Orlando, USA, 18th-22nd, November, 2024

Agenda item:	7.16.3

Source:	Samsung

Title:	View and initial simulation results for BS demodulation requirement with MMSE-IRC receiver

Document for:	Discussion

- Introduction

In the last RAN4 meeting, the performance part of Rel-19 WI on demodulation performance requirements enhancement was under discussed. The general assumption for MMSE-IRC and interference profile was discussed, the related agreements are captured in the WF [1] as following

- General - Test scenarios - On HomNet and HetNet scenarios: - Cover HomNet and HetNet scenarios. - FFS whether to introduce additional applicability rule - On scenario with unaligned TDD pattern - For NR_demod_Ph5 WI, only consider interference model with aligned TDD pattern between cells. - Interference modeling - Interference modeling criteria - Use INR-based modelling and transfer the DIP value into INR value - Interference profile - On interference modelling: - Reuse the same interference modelling as captured in B.6.2 in TS36.104, to transmit random 16QAM symbol for the neighbour PUSCH - On how to account for HPUE in existing LTE profiles: - Reuse the LTE interference power level as a start point, and check performance gain of MMSE-IRC over baseline MMSE. - It is not precluded for companies to bring system level simulation on other profiles. - Proposals on how to account for HPUE in existing LTE profiles. - Proposal 1: Perform new system-level simulations with the inclusion of HPUEs - Proposal 2: Consider artificially increasing DIP/INR with additional 3 or 7 dB - Number of interferes - For 4Rx and 8Rx, consider 2 interferers - Check the performance gain of 1 or 2 interferes for 2Rx and make decision based on simulation results - Test Parameters and simulation assumptions - Test scope - 15kHz SCS FDD and TDD with pattern 3D1S1U, S=10D:2G:2U - 30 kHz SCS FDD and TDD with pattern 7D1S2U, S=6D:4G:4U - same requirements are applicable to both FDD and TDD with different UL-DL pattern. - with adding note in the spec that ‘same requirements are applicable to both FDD and TDD with different UL-DL pattern’. - Channel bandwidth - For CP-OFDM: - For performance evaluation purpose, Minimum channel bandwidth for each SCS, i.e., 5MHz for 15kHz SCS and 10MHz for 30kHz. - Further discuss the CHBW for requirement definition - Antenna configuration - Cover 2/4/8 Rx with 1Tx for simulation evaluation purpose - Propagation condition - For serving cell: - Option 1: TDLA30-10 and TDLC300-100 for HetNet and HomNet respectively - Option 2: TDLA30-10 and TDLB100-100 for HetNet and HomNet respectively - Option 3: Cover TDLA30-10, TDLB100-400, TDLC300-100 - Option 4: Use TDLB100-400 - For interference cell - Option 1: Introduce new propagation channel for testing with desired delay spread 1000ns for the interference PUSCH, i.e., TDLC1000 - Option 2: Same propagation condition for serving and interference PUSCH - Option 3: Use TDLC300-100 - MCS - Option 1: Cover QPSK/16QAM/64QAM - Option 1A: Use MCS 5(QPSK, R=379/1024), MCS 13(16QAM, R=490/1024) and MCS 24(64QAM, R=772/1024) - Option 1B: MCS index 2, 12, 20 in MCS table 1 - Option 1C: Cover MCS 2, 16, 20 - Option 2: Choose only one MCS configuration - Option 3: Consider MCS 5(QPSK, R=379/1024) and MCS 13(16QAM, R=490/1024) and FFS 64QAM (ZTE) - Waveform - Cover CP-OFDM and FFS DFT-s-OFDM - DMRS configuration - Option 1: Cover DMRS configuration type 1 and type 2 - Option 2: DMRS configuration type 1 - DMRS sequence generation - Option 1: - NID0=0, nSCID =0 for CP-OFDM - NID0=0, group hopping and sequence hopping are disabled for DFT-s-OFDM - Option 2: - nSCID =0 for all cells - Different NID0=0 for serving and interference cells - PUSCH mapping type - Cover both PUSCH mapping Type A and Type B - PUSCH time domain allocation S=0, L=14 - Test metric - Use 70% of maximum throughput as metric for NR BS MMSE-IRC demodulation - Test requirement definition - Option 1: Using SINR based requirement definition - Option 2: SNR based requirement definition - SINR definition - For BS type 1-C and BS type 1-H The SINR used in this clause is specified based on a single carrier and defined as: SINR = S / N’ Where: S is the total signal energy of one intra-cell UE in the slot on a single antenna connector (for BS type 1-C) or on a single TAB connector (for BS type 1-H). N’ is the summation of the received energy of the strongest inter-cell interfering signals explicitly defined in a test procedure plus the white noise energy N in a bandwidth corresponding to the transmission bandwidth over the same duration where signal energy exists on a single antenna connector (for BS type 1-C) or on a single TAB connector (for BS type 1-H). The respective energy of each inter-cell interfering signal relative to N is defined by its associated INR value.. - For BS type 1-O The SINR used in this clause is specified based on a single carrier and defined as: SINR = S / N’ Where: S is the total signal energy of one intra-cell UE in a slot on a RIB. N’ is the summation of the received energy of the strongest inter-cell interfering signals explicitly defined in a test procedure plus the white noise energy N in a bandwidth corresponding to the transmission bandwidth over the same duration where signal energy exists on a RIB. The respective energy of each inter-cell interfering signal relative to N is defined by its associated INR value..

In this contribution, the view on the remaining issue of test setup for BS demodulation with MMSE-IRC are provides, meanwhile, the initial simulation results are provided for initial analysis and alignment.

# 2	General

## 2.1	Test scenario

For test scenario, both HomNet and HetNet scenarios were agreed to cover the performance requirement of MMSE-IRC receiver. Regarding the applicability for requirement in different scenarios, it will be further discussed.

In LTE, for requirement in HomNet scenario was not appliable for Local Area BS and Home BS, since the HomNet is mainly targeting for macro cell only as following table

Table 1: Enhanced performance requirement type A for PUSCH, 1.4 MHz Channel Bandwidth

| Number of TX antennas (Note 1) | Number of RX antennas (Note 1) | Propagation conditions and correlation matrix (Annex B) (Note 2) |  |  | DIP set | FRC (Annex A) | Fraction of maximum throughput | SINR [dB] (Note 3) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Tested signal | Interferer 1 | Interferer 2 |  |  |  |  |
| 1 | 2 | EPA 5 Low | ETU 5 Low | N/A | Set 2 | A12-1 | 70% | -4.8 |
|  |  | EVA 70 Low | ETU 70 Low | N/A | Set 1* | A12-1 | 70% | -2.0 |
|  | 4 | EPA 5 Low | ETU 5 Low | ETU 5 Low | Set 2 | A13-1 | 70% | -4.1 |
|  |  | EVA 70 Low | ETU 70 Low | ETU 70 Low | Set 1* | A13-1 | 70% | -0.1 |
|  | 8 | EPA 5 Low | ETU 5 Low | ETU 5 Low | Set 2 | A4-3 | 70% | -4.7 |
|  |  | EVA 70 Low | ETU 70 Low | ETU 70 Low | Set 1* | A4-3 | 70% | 0.0 |
| Note*: Not applicable for Local Area BS and Home BS. |  |  |  |  |  |  |  |  |

For NR, only three categories of BS class were defined as wide area base station, medium Range base station and local area range base station in current specification. Therefore, we think the similar approach can be applied for NR to differentiate the different BS classes.  The requirement for DIP set 2 with a note that not applicable for local area BS can be considered. Regarding the Home BS scenario, since it is not introduced in current specification, we do not need to mention it in current release. If home BS was introduced into future release, the applicable of requirement can be updated.

Meanwhile, from the baseband processing perspective of MMSE-IRC, there is no difference foreseen. In our view there is no necessary to test both if BS declared to support MMSE-IRC receiver for these two scenarios, pending on the deployment, for example, in Marco cell or Micro cell or Pico cell.

To reduce the test effort, we propose to introduce the test applicability rule for these two requirements (DOP) testing. The following test applicability rule can be considered as

  - Unless other stated, if a BS declared to support [wide area] or [medium range] and pass the tests for DIP1, can also consider the test for DIP2 as passed

Observation 1:  The requirement in HomNet (DIP set 1) is not applicable for local Area BS and Home BS in LTE system.

Observation 2:  Only three categories of BS class were defined in NR system as wide area base station, medium Range base station and local area range base station

Proposal 1:  Apply the similar approach for NR with a note to indicate the requirement in HomNet ( DIP set 1)  is not appliable for local Area BS.

Proposal 2: Introduce the test applicability rule for these two requirements (DIP set 1) testing. The following test applicability rule can be considered as

  - Unless other stated, if a BS declared to support [wide area] or [medium range] and pass the tests for DIP1, can also consider the test for DIP2 as passed

## 2.2	Interference modeling

Interference profile

Regarding the interference Profile, it was agreed in the last meeting that to reuse the LTE interference power level as a start point, and to check performance gain of MMSE-IRC over baseline MMSE receiver.  As shown in our simulation results, the significant gain over MMSE receiver can be achieved based on LTE interference profile. Therefore, it is feasible to reuse LTE interference power for specifying PUSCH requirement with MMSE-IRC receiver.

Different with LTE, HPUEs including PC1(7dB higher than PC3 UE) or PC2 (3dB higher than PC3 UE) HPUE are widely deployed for NR system, therefore, the expected interference power will be increased. In that case, how to account for HPUE in existing LTE profiles should be further discussion. In order to account the HPUE impact, in general, it is more accurate to derive the interference profile based on system level simulation under the given assumption of HPUE distribution. While considering the system level simulation effort and alignment work, as well as the schedule of this WI, we prefer to add an artificial interference value into the existing interference profiles. Regarding the value to be added, it cannot guarantee the full power will be transmitted at the UE side, although 3 or 7dB higher than PC3 UE. While considering RAN4 is to define the minimum requirement and worse case, 7dB can be considered to add into the existing interference profile. And it can be also used to check the performance with MMSE-IRC receiver under high interference condition and try to match the real deployment with avoiding the system level simulation effort. Therefore, we slightly prefer to add 7dB to check the worse scenario.

Regarding how to add the value to reflect the impact of HPUE, the DIP is defined as the ratio of the power of a given dominant interferer over the total power of all interferers along with the white noise. It is not feasible to add the value into the DIP directly. In the last meeting, the INR-based modeling was agreed, where INR value is transferred from DIP. Therefore, to account the impact of HPUE, 7 dB can be added into the INR value directly as following table

Table 2: Interference profile with and without impact on HPUE

| Scenario | (DIP1, DIP2) | (INR1, INR2) | (INR1, INR2) with considering HPUE |
| --- | --- | --- | --- |
| HetNet with 1 interferer | (-0.43, N/A) | (9.82, N/A) | (16.82, N/A) |
| HetNet with 2 interferers | (-0.43, -13.78) | (12.38, -0.97) | (19.38, 6.03) |
| HomNet with 1 interferer | (-1.11, N/A) | (5.35, N/A) | (12.35, N/A) |
| HomNet with 2 interferers | (-1.11, -10.91) | (7.28, -2.52) | (14.28, 4.48) |

Proposal 3:  Consider artificially increasing INR with additional 7dB to account the impact of HPUE, the following INR value can be considered

| Scenario | (INR1, INR2) with considering HPUE |
| --- | --- |
| HetNet with 1 interferer | (16.82, N/A) |
| HetNet with 2 interferers | (19.38, 6.03) |
| HomNet with 1 interferer | (12.35, N/A) |
| HomNet with 2 interferers | (14.28, 4.48) |

Number of Interferers

Regarding number of interferer UE modeling, it was agreed to check the performance with 1 interferer and 2 interferers for 2Rx. Meanwhile, we also check the performance for 1 interferer and 2 interferers condition for HPUE scenario.

Based on initial simulation, there is also a significant gain achieved by MMSE-IRC compared with MMSE receiver for both HomNet and HetNet scenario,

Also, for the interference profile with HPUE, the simulation results show the gain of MMSE-IRC compared with MMSE receiver.

Considering the impact of HPUE, the gain of MMSE-IRC under 2 interferers is slightly smaller than that under 1 interferer scenario.

Observation 3: The gain can be achieved with MMSE-IRC receiver compared with MMSE receiver for the 2 interferers modeling, under the interference profile with and without the impact of HPUE.

Observation 4: Considering the impact of HPUE, the gain of MMSE-IRC under 2 interferers is slightly smaller than that under 1 interferer scenario.

Proposal 4:  2 interferers can be considered for 2Rx to specify the PUSCH requirement with MMSE-IRC receiver

# 3	Test parameters and simulation assumption

In this section, the remaining simulation assumptions are provided for evaluation.

Channel Bandwidth

Regarding the SCS and channel bandwidth, minimum CBW for each SCS has been considered for performance evaluation purpose. As for requirement definition, in our view, the typical CBW with 10MHz for 15KHz, and 40MHz for 30kHz can be chosen, considering they are widely applied in current field. Therefore, for CP-OFDM and DFT-s-OFDM waveform, the following channel bandwidth can be considered for requirement definition.

- For CP-OFDM

  - 15kHz SCS, 5MHz, 10MHz

  - 30KHz SCS, 10MHz, 40MHz

- For DFT-s-OFDM

  - 15kHz SCS, 5MHz

  - 30KHz SCS, 10MHz

Proposal 5:  The following channel bandwidth can be considered to specify PUSCH requirement with MMSE-IRC receiver

- For CP-OFDM

  - 15kHz SCS, 5MHz, 10MHz

  - 30KHz SCS, 10MHz, 40MHz

- For DFT-s-OFDM

  - 15kHz SCS, 5MHz

  - 30KHz SCS, 10MHz

Propagation condition

To check the per-PRB interference covariance estimation, it is better to choose the channel with high delay spread for the interferers. In Rel-15, several channel models are specified to cover different delay spread and doppler spread, including TDLA30-10, TDLB400-100 and TDLC300-100, where TDLC300-100 can be considered as a starting point. In LTE BS performance with MMSE-IRC receiver, different channel models were applied for target UE and interfered UE. For NR PDSCH with MMSE-IRC, the same channel was applied for both target UE and interfered UE.  Therefore, whether to differentiate the channel model used for target UE and interfered UE should be further discussed and evaluated.  To align the similar principle, we prefer to apply different channel model used for target UE and interfered UE for performance specification with MMSE-IRC receiver.

Proposal 6: Reusing the existing channel models to specify the PUSCH requirement with MMSE-IRC receiver

The following candidate propagation channel can be considered as initial evaluation

  - Targeting cell:  TDLA30-10, TDLC300-100 and TDLB100-400

  - Interference cell: TDLC300-100

MCS

Regarding the MCS, 16QAM for interfered UE can be considered for initial evaluation, which is more typical scenario, similar as LTE MMSE-IRC receiver evaluation.

For target cell, MCS 2, MCS 16 and MCS 20 were considered in existing NR PUSCH requirement, they can be considered as candidate for initial evaluation. There is no necessary to consider other new MCS value for evaluation and requirement, considering to minimize the effort of FRC specification. For requirement definition, FFS to down select with only one MCS for requirement definition.

Proposal 7: Consider MCS 16 for interfered UE, MCS 2, MCS 16 and MCS 20 as candidate for initial performance evaluation with MMSE-IRC receiver.

  - Targeting cell:  MCS 2, MCS 16 and MCS 20

  - Interference cell: MCS 16

For requirement definition, FFS to down select one MCS for requirement.

Waveform

Considering only 1Tx UE scenario, both CP-OFDM and DFT-s-OFDM are feasible. Therefore, for requirement definition, both CP-OFDM and DFT-s-OFDM waveforms can be considered to guarantee the test coverage.

Proposal 8: For requirement definition, both CP-OFDM and DFT-s-OFDM waveforms can be considered to specify PUSCH requirement with MMSR-IRC receiver to guarantee the test coverage

DMRS configuration

Regarding the DMRS configuration, both type 1 and type 2 were considered as one of option. Type 2 is mainly targeting for multi user scenarios. In legacy NR PUSCH requirement, RAN4 only specified the requirement with type 1. We do not think it is necessary to specify the requirement with DMRS type 2.

Proposal 9: Only specify the PUSCH requirement with MMSR-IRC for DMRS type 1.

DMRS sequence generation

For DMRS generation with different cell, for simplicity, the cell ID for targeting cell, and interference cells can be set as 0, 1, 2, respectively. And the scrambling ID can be set as cell ID, nSCIDλ=0  and quantity nSCID =0. And the group hopping and sequence hopping are disabled for both CP-OFDM and DFT-s-OFDM waveform

|  | Targeting cell | Interferer 1 | Interferer 2 |
| --- | --- | --- | --- |
| DM-RS port | 0 | 0 | 0 |
| Cell ID | 0 | 1 | 2 |
| DM-RS sequence generation | NIDnSCIDλ=NIDcell nSCIDλ=0 λ=0 Group hopping and sequence hopping are disabled |  |  |

Proposal 10: Considering the following for DMRS configuration and cell ID to specify PUSCH requirement with MMSE-IRC receiver

|  | Targeting cell | Interferer 1 | Interferer 2 |
| --- | --- | --- | --- |
| DM-RS port | 0 | 0 | 0 |
| Cell ID | 0 | 1 | 2 |
| DM-RS sequence generation | NIDnSCIDλ=NIDcell nSCIDλ=0 λ=0 Group hopping and sequence hopping are disabled |  |  |

Test requirement definition

In the last meeting, 70% of maximum throughput as metric to derive the requirement for PUSCH with MMSE-IRC receiver. Regarding the requirement definition, both SNR based and SINR based method can be considered. In LTE MMSE-IRC, the SINR is applied to derive the related requirement.

| Number of TX antennas (Note 1) | Number of RX antennas (Note 1) | Propagation conditions and correlation matrix (Annex B) (Note 2) |  |  | DIP set | FRC (Annex A) | Fraction of maximum throughput | SINR [dB] (Note 3) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Tested signal | Interferer 1 | Interferer 2 |  |  |  |  |
| 1 | 2 | EPA 5 Low | ETU 5 Low | N/A | Set 2 | A12-1 | 70% | -4.8 |
|  |  | EVA 70 Low | ETU 70 Low | N/A | Set 1* | A12-1 | 70% | -2.0 |
|  | 4 | EPA 5 Low | ETU 5 Low | ETU 5 Low | Set 2 | A13-1 | 70% | -4.1 |
|  |  | EVA 70 Low | ETU 70 Low | ETU 70 Low | Set 1* | A13-1 | 70% | -0.1 |
|  | 8 | EPA 5 Low | ETU 5 Low | ETU 5 Low | Set 2 | A4-3 | 70% | -4.7 |
|  |  | EVA 70 Low | ETU 70 Low | ETU 70 Low | Set 1* | A4-3 | 70% | 0.0 |
| Note*: Not applicable for Local Area BS and Home BS. |  |  |  |  |  |  |  |  |
| Note 1: Antenna configuration applies for each of the tested signal, interferer 1 and interferer 2. Note 2: The propagation conditions for the tested signal, interferer 1 and interferer 2 are statistically independent. Note 3: SINR corresponds to of the tested signal as defined in clause 8.1. |  |  |  |  |  |  |  |  |

For NR PDSCH requirement with MMSE-IRC, SNR was applied to derive the related requirement

Table 5.2.2.1.15-3: Minimum performance for PDSCH with rank 1 and with inter-cell interference

| Test num | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-2.1 FDD | 10 / 15 | 16QAM, 0.48 | TDLC300-100 | 2x2, ULA Low | 70 | 15.4 |
| 1-2 | R.PDSCH.1-2.1 FDD | 10 / 15 | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 12.5 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: Bandwidth/ Sub carrier spacing, Propagation Condition, Correlation matrix and antenna configuration parameters apply for each of Cell 1, Cell 2 and Cell 3. |  |  |  |  |  |  |  |

Observation 5: The SINR was applied for test requirement definition for PUSCH requirement with MMSE-IRC receiver in LTE

Observation 6: The SNR was applied for test requirement definition for PDSCH requirement with MMSE-IRC receiver in NR

Based on our initial simulation results, the performance gain over MMSE receiver is similar for both SNR and SINR-based requirement definition.  In general, either approach is fine, since the interference power is fixed. To reflect the performance with certain interference condition, SINR based requirement definition is more proper. Therefore, similar as LTE MMSE-IRC, we can still apply the SINR based requirement definition to derive the targeting value based on 70% of TP. While we are also open to further discuss whether SNR based metric is more proper from conference test perspective, since TE vendor can transmit the PUSCH with target power. Without additional calculation to derive the signal power based on SINR and INR.

Proposal 11: Slightly prefer to use the 70% of maximum throughput as metric for NR BS MMSE-IRC receiver to derive the SINR value. Open to discuss the SNR based metric.

SINR definition

In general, the SINR definition in the WF [1] is fine. Currently, there is no agreement the requirement is derived based on SINR or SNR metric, we can further discuss whether the SINR based definition is needed and captured into the specification.

Proposal 12: FFS on SINR based definition is needed and captured into the specification

# 4	Initial Simulation Results

In this section, the initial simulation results are provided to check the performance gain of MMSE-IRC over MMSE receiver.

As starting point, only 15KHz SCS, 5MHz with CP-OFDM waveform for both HomNet and HetNet are under evaluation with and without the impact on HPUE scenario.

Table 1.  PUSCH performance with HomNet scenario, channel model for target cell is TDLA30-10

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SINR@70% (MMSE) | SINR@70% (MMSE-IRC) | Gain (SINR) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 2 | TDLA30-10 | TDLC300-100 | (5.35, N/A) | 1 | 1+1 (2,11) | MCS 2 | MCS16 | -4.13 | -7.22 | 3.09 | 2.34 | -0.75 | 3.09 |
| 2 | 1 | 2 | TDLA30-10 | TDLC300-100 | (5.35, N/A) | 1 | 1+1 (2,11) | MCS 16 | MCS16 | 7.34 | 4.43 | 2.91 | 13.81 | 10.9 | 2.91 |
| 3 | 1 | 2 | TDLA30-10 | TDLC300-100 | (5.35, N/A) | 1 | 1+1 (2,11) | MCS 20 | MCS16 | 10.54 | 7.66 | 2.88 | 17.01 | 14.13 | 2.88 |
| 4 | 1 | 2 | TDLA30-10 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 2 | MCS16 | -4.16 | -7.43 | 3.27 | 4.25 | 0.97 | 3.28 |
| 5 | 1 | 2 | TDLA30-10 | TDLC300-100 | 7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 7.34 | 4.33 | 3.01 | 15.74 | 12.73 | 3.01 |
| 6 | 1 | 2 | TDLA30-10 | TDLC300-100 | 7.28, -2.52) | 2 | 1+1 (2,11) | MCS 20 | MCS16 | 10.55 | 7.53 | 3.02 | 18.95 | 15.93 | 3.02 |

Table 2.  PUSCH performance with HomNet scenario, channel model for target cell is TDLC300-100

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SINR@70% (MMSE) | SINR@70% (MMSE-IRC) | Gain (SINR) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 2 | TDLC300-100 | TDLC300-100 | (5.35, N/A) | 1 | 1+1 (2,11) | MCS 2 | MCS16 | -4.19 | -7.32 | 3.13 | 2.28 | -0.85 | 3.13 |
| 2 | 1 | 2 | TDLC300-100 | TDLC300-100 | (5.35, N/A) | 1 | 1+1 (2,11) | MCS 16 | MCS16 | 8.18 | 5.42 | 2.76 | 14.65 | 11.89 | 2.76 |
| 3 | 1 | 2 | TDLC300-100 | TDLC300-100 | (5.35, N/A) | 1 | 1+1 (2,11) | MCS 20 | MCS16 | 11.5 | 8.93 | 2.57 | 17.97 | 15.4 | 2.57 |
| 4 | 1 | 2 | TDLC300-100 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 2 | MCS16 | -4.15 | -7.5 | 3.35 | 4.25 | 0.9 | 3.35 |
| 5 | 1 | 2 | TDLC300-100 | TDLC300-100 | 7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 8.21 | 5.32 | 2.89 | 16.61 | 13.72 | 2.89 |
| 6 | 1 | 2 | TDLC300-100 | TDLC300-100 | 7.28, -2.52) | 2 | 1+1 (2,11) | MCS 20 | MCS16 | 11.56 | 8.77 | 2.79 | 19.97 | 17.06 | 2.91 |

Table 3.  PUSCH performance with HomNet scenario, channel model for target cell is TDLB100-400

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SINR@70% (MMSE) | SINR@70% (MMSE-IRC) | Gain (SINR) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 2 | TDLB100-400 | TDLC300-100 | (5.35, N/A) | 1 | 1+1 (2,11) | MCS 2 | MCS16 | -3.89 | -7.08 | 3.19 | 2.58 | -0.61 | 3.19 |
| 2 | 1 | 2 | TDLB100-400 | TDLC300-100 | (5.35, N/A) | 1 | 1+1 (2,11) | MCS 16 | MCS16 | 9.73 | 7.26 | 2.47 | 16.2 | 13.72 | 2.48 |
| 3 | 1 | 2 | TDLB100-400 | TDLC300-100 | (5.35, N/A) | 1 | 1+1 (2,11) | MCS 20 | MCS16 | 15.06 | 13.21 | 1.85 | 21.53 | 19.68 | 1.85 |
| 4 | 1 | 2 | TDLB100-400 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 2 | MCS16 | -3.89 | -7.27 | 3.38 | 4.51 | 1.13 | 3.38 |
| 5 | 1 | 2 | TDLB100-400 | TDLC300-100 | 7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 9.77 | 7.18 | 2.59 | 18.17 | 15.59 | 2.58 |
| 6 | 1 | 2 | TDLB100-400 | TDLC300-100 | 7.28, -2.52) | 2 | 1+1 (2,11) | MCS 20 | MCS16 | 15.36 | 13.51 | 1.85 | 23.76 | 21.92 | 1.84 |

Table 4.  PUSCH performance with HetNet scenario, channel model for target cell is TDLA30-10

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SINR@70% (MMSE) | SINR@70% (MMSE-IRC) | Gain (SINR) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 2 | TDLA30-10 | TDLC300-100 | (9.82, N/A) | 1 | 1+1 (2,11) | MCS 2 | MCS16 | -4.3 | -9.73 | 5.43 | 5.96 | 0.53 | 5.43 |
| 2 | 1 | 2 | TDLA30-10 | TDLC300-100 | (9.82, N/A) | 1 | 1+1 (2,11) | MCS 16 | MCS16 | 7.3 | 2.12 | 5.18 | 17.56 | 12.38 | 5.18 |
| 3 | 1 | 2 | TDLA30-10 | TDLC300-100 | (9.82, N/A) | 1 | 1+1 (2,11) | MCS 20 | MCS16 | 10.47 | 5.27 | 5.2 | 20.72 | 15.53 | 5.19 |
| 4 | 1 | 2 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 2 | MCS16 | -4.29 | -9.96 | 5.67 | 8.52 | 2.84 | 5.68 |
| 5 | 1 | 2 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 7.28 | 2.01 | 5.27 | 20.09 | 14.81 | 5.28 |
| 6 | 1 | 2 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 20 | MCS16 | 10.48 | 5.13 | 5.35 | 23.29 | 17.94 | 5.35 |

Table 5.  PUSCH performance with HetNet scenario, channel model for target cell is TDLC300-100

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SINR@70% (MMSE) | SINR@70% (MMSE-IRC) | Gain (SINR) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 2 | TDLC300-100 | TDLC300-100 | (9.82, N/A) | 1 | 1+1 (2,11) | MCS 2 | MCS16 | -4.26 | -9.79 | 5.53 | 5.99 | 0.47 | 5.52 |
| 2 | 1 | 2 | TDLC300-100 | TDLC300-100 | (9.82, N/A) | 1 | 1+1 (2,11) | MCS 16 | MCS16 | 8.14 | 3.25 | 4.89 | 18.4 | 13.51 | 4.89 |
| 3 | 1 | 2 | TDLC300-100 | TDLC300-100 | (9.82, N/A) | 1 | 1+1 (2,11) | MCS 20 | MCS16 | 11.48 | 6.33 | 5.15 | 21.74 | 16.59 | 5.15 |
| 4 | 1 | 2 | TDLC300-100 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 2 | MCS16 | -4.23 | -9.99 | 5.76 | 8.58 | 2.82 | 5.76 |
| 5 | 1 | 2 | TDLC300-100 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 8.17 | 3.21 | 4.96 | 20.98 | 16.02 | 4.96 |
| 6 | 1 | 2 | TDLC300-100 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 20 | MCS16 | 11.54 | 6.27 | 5.27 | 24.35 | 19.08 | 5.27 |

Table 6.  PUSCH performance with HetNet scenario, channel model for target cell is TDLB100-400

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SINR@70% (MMSE) | SINR@70% (MMSE-IRC) | Gain (SINR) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 2 | TDLB100-400 | TDLC300-100 | (9.82, N/A) | 1 | 1+1 (2,11) | MCS 2 | MCS16 | -3.97 | -9.58 | 5.61 | 6.29 | 0.67 | 5.62 |
| 2 | 1 | 2 | TDLB100-400 | TDLC300-100 | (9.82, N/A) | 1 | 1+1 (2,11) | MCS 16 | MCS16 | 9.69 | 5.15 | 4.54 | 19.95 | 15.41 | 4.54 |
| 3 | 1 | 2 | TDLB100-400 | TDLC300-100 | (9.82, N/A) | 1 | 1+1 (2,11) | MCS 20 | MCS16 | 15.59 | 11.64 | 3.95 | 25.85 | 21.89 | 3.96 |
| 4 | 1 | 2 | TDLB100-400 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 2 | MCS16 | -3.97 | -9.76 | 5.79 | 8.84 | 3.05 | 5.79 |
| 5 | 1 | 2 | TDLB100-400 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 9.77 | 5.14 | 4.63 | 22.57 | 17.95 | 4.62 |
| 6 | 1 | 2 | TDLB100-400 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 20 | MCS16 | 15.99 | 12.48 | 3.51 | 28.8 | 25.29 | 3.51 |

Table 7.  PUSCH performance with HomNet scenario, channel model for target cell is TDLA30-10 with HPUE

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SINR@70% (MMSE) | SINR@70% (MMSE-IRC) | Gain (SINR) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 2 | TDLA30-10 | TDLC300-100 | (12.35, N/A) | 1 | 1+1 (2,11) | MCS 2 | MCS16 | -4.35 | -11.16 | 6.81 | 8.25 | 1.43 | 6.82 |
| 2 | 1 | 2 | TDLA30-10 | TDLC300-100 | (12.35, N/A) | 1 | 1+1 (2,11) | MCS 16 | MCS16 | 7.28 | 0.77 | 6.51 | 19.87 | 13.36 | 6.51 |
| 3 | 1 | 2 | TDLA30-10 | TDLC300-100 | (12.35, N/A) | 1 | 1+1 (2,11) | MCS 20 | MCS16 | 10.44 | 3.88 | 6.56 | 23.03 | 16.48 | 6.55 |
| 4 | 1 | 2 | TDLA30-10 | TDLC300-100 | (14.28, 4.48) | 2 | 1+1 (2,11) | MCS 2 | MCS16 | -4.24 | -9.65 | 5.41 | 10.62 | 5.21 | 5.41 |
| 5 | 1 | 2 | TDLA30-10 | TDLC300-100 | (14.28, 4.48) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 7.34 | 4.33 | 3.01 | 15.74 | 12.73 | 3.01 |
| 6 | 1 | 2 | TDLA30-10 | TDLC300-100 | (14.28, 4.48) | 2 | 1+1 (2,11) | MCS 20 | MCS16 | 10.55 | 7.53 | 3.02 | 18.95 | 15.93 | 3.02 |

Table 8.  PUSCH performance with HomNet scenario, channel model for target cell is TDLC300-100 with HPUE

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SINR@70% (MMSE) | SINR@70% (MMSE-IRC) | Gain (SINR) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 2 | TDLC300-100 | TDLC300-100 | (12.35, N/A) | 1 | 1+1 (2,11) | MCS 2 | MCS16 | -4.28 | -11.17 | 6.89 | 8.31 | 1.43 | 6.88 |
| 2 | 1 | 2 | TDLC300-100 | TDLC300-100 | (12.35, N/A) | 1 | 1+1 (2,11) | MCS 16 | MCS16 | 8.12 | 2.14 | 5.98 | 20.72 | 14.74 | 5.98 |
| 3 | 1 | 2 | TDLC300-100 | TDLC300-100 | (12.35, N/A) | 1 | 1+1 (2,11) | MCS 20 | MCS16 | 11.48 | 5.01 | 6.47 | 24.07 | 17.6 | 6.47 |
| 4 | 1 | 2 | TDLC300-100 | TDLC300-100 | (14.28, 4.48) | 2 | 1+1 (2,11) | MCS 2 | MCS16 | -4.2 | -9.63 | 5.43 | 10.66 | 5.23 | 5.43 |
| 5 | 1 | 2 | TDLC300-100 | TDLC300-100 | (14.28, 4.48) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 8.21 | 3.6 | 4.61 | 23.06 | 18.46 | 4.6 |
| 6 | 1 | 2 | TDLC300-100 | TDLC300-100 | (14.28, 4.48) | 2 | 1+1 (2,11) | MCS 20 | MCS16 | 11.56 | 6.72 | 4.84 | 26.42 | 21.58 | 4.84 |

Table 9.  PUSCH performance with HomNet scenario, channel model for target cell is TDLB100-400 with HPUE

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SINR@70% (MMSE) | SINR@70% (MMSE-IRC) | Gain (SINR) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 2 | TDLB100-400 | TDLC300-100 | (12.35, N/A) | 1 | 1+1 (2,11) | MCS 2 | MCS16 | -4 | -10.98 | 6.98 | 8.6 | 1.62 | 6.98 |
| 2 | 1 | 2 | TDLB100-400 | TDLC300-100 | (12.35, N/A) | 1 | 1+1 (2,11) | MCS 16 | MCS16 | 9.74 | 4.02 | 5.72 | 22.33 | 16.62 | 5.71 |
| 3 | 1 | 2 | TDLB100-400 | TDLC300-100 | (12.35, N/A) | 1 | 1+1 (2,11) | MCS 20 | MCS16 | 16 | 11.02 | 4.98 | 28.6 | 23.61 | 4.99 |
| 4 | 1 | 2 | TDLB100-400 | TDLC300-100 | (14.28, 4.48) | 2 | 1+1 (2,11) | MCS 2 | MCS16 | -3.94 | -9.44 | 5.5 | 10.92 | 5.42 | 5.5 |
| 5 | 1 | 2 | TDLB100-400 | TDLC300-100 | (14.28, 4.48) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 9.78 | 5.61 | 4.17 | 24.64 | 20.47 | 4.17 |
| 6 | 1 | 2 | TDLB100-400 | TDLC300-100 | (14.28, 4.48) | 2 | 1+1 (2,11) | MCS 20 | MCS16 | 16.17 | 13.64 | 2.53 | 31.02 | 28.5 | 2.52 |

Table 10.  PUSCH performance with HetNet scenario, channel model for target cell is TDLA30-10 with HPUE

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SINR@70% (MMSE) | SINR@70% (MMSE-IRC) | Gain (SINR) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 2 | TDLA30-10 | TDLC300-100 | (16.82, N/A) | 1 | 1+1 (2,11) | MCS 2 | MCS16 | -4.4 | -13.31 | 8.91 | 12.51 | 3.6 | 8.91 |
| 2 | 1 | 2 | TDLA30-10 | TDLC300-100 | (16.82, N/A) | 1 | 1+1 (2,11) | MCS 16 | MCS16 | 7.24 | -1.28 | 8.52 | 24.15 | 15.63 | 8.52 |
| 3 | 1 | 2 | TDLA30-10 | TDLC300-100 | (16.82, N/A) | 1 | 1+1 (2,11) | MCS 20 | MCS16 | 10.43 | 1.82 | 8.61 | 27.34 | 18.73 | 8.61 |
| 4 | 1 | 2 | TDLA30-10 | TDLC300-100 | (19.38, -6.03) | 2 | 1+1 (2,11) | MCS 2 | MCS16 | -4.34 | -11.74 | 7.4 | 15.29 | 7.89 | 7.4 |
| 5 | 1 | 2 | TDLA30-10 | TDLC300-100 | (19.38, -6.03) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 7.27 | 0.45 | 6.82 | 26.9 | 20.08 | 6.82 |
| 6 | 1 | 2 | TDLA30-10 | TDLC300-100 | (19.38, -6.03) | 2 | 1+1 (2,11) | MCS 20 | MCS16 | 10.44 | 3.49 | 6.95 | 30.07 | 23.12 | 6.95 |

Table 11.  PUSCH performance with HetNet scenario, channel model for target cell is TDLC300-100 with HPUE

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SINR@70% (MMSE) | SINR@70% (MMSE-IRC) | Gain (SINR) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 2 | TDLC300-100 | TDLC300-100 | (16.82, N/A) | 1 | 1+1 (2,11) | MCS 2 | MCS16 | -4.3 | -13.23 | 8.93 | 12.61 | 3.68 | 8.93 |
| 2 | 1 | 2 | TDLC300-100 | TDLC300-100 | (16.82, N/A) | 1 | 1+1 (2,11) | MCS 16 | MCS16 | 8.1 | 0.56 | 7.54 | 25.01 | 17.47 | 7.54 |
| 3 | 1 | 2 | TDLC300-100 | TDLC300-100 | (16.82, N/A) | 1 | 1+1 (2,11) | MCS 20 | MCS16 | 11.51 | 3.35 | 8.16 | 28.42 | 20.26 | 8.16 |
| 4 | 1 | 2 | TDLC300-100 | TDLC300-100 | (19.38, -6.03) | 2 | 1+1 (2,11) | MCS 2 | MCS16 | -4.27 | -11.64 | 7.37 | 15.36 | 7.99 | 7.37 |
| 5 | 1 | 2 | TDLC300-100 | TDLC300-100 | (19.38, -6.03) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 8.16 | 2.01 | 6.15 | 27.79 | 21.65 | 6.14 |
| 6 | 1 | 2 | TDLC300-100 | TDLC300-100 | (19.38, -6.03) | 2 | 1+1 (2,11) | MCS 20 | MCS16 | 11.55 | 4.79 | 6.76 | 31.18 | 24.43 | 6.75 |

Table 12.  PUSCH performance with HetNet scenario, channel model for target cell is TDLB100-400 with HPUE

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SINR@70% (MMSE) | SINR@70% (MMSE-IRC) | Gain (SINR) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 2 | TDLB100-400 | TDLC300-100 | (16.82, N/A) | 1 | 1+1 (2,11) | MCS 2 | MCS16 | -4.3 | -13.04 | 8.74 | 12.88 | 3.87 | 9.01 |
| 2 | 1 | 2 | TDLB100-400 | TDLC300-100 | (16.82, N/A) | 1 | 1+1 (2,11) | MCS 16 | MCS16 | 9.73 | 2.47 | 7.26 | 26.64 | 19.38 | 7.26 |
| 3 | 1 | 2 | TDLB100-400 | TDLC300-100 | (16.82, N/A) | 1 | 1+1 (2,11) | MCS 20 | MCS16 | 16.37 | 10.57 | 5.8 | 33.28 | 27.48 | 5.8 |
| 4 | 1 | 2 | TDLB100-400 | TDLC300-100 | (19.38, -6.03) | 2 | 1+1 (2,11) | MCS 2 | MCS16 | -4 | -11.44 | 7.44 | 15.63 | 8.19 | 7.44 |
| 5 | 1 | 2 | TDLB100-400 | TDLC300-100 | (19.38, -6.03) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 9.76 | 3.99 | 5.77 | 29.39 | 23.62 | 5.77 |
| 6 | 1 | 2 | TDLB100-400 | TDLC300-100 | (19.38, -6.03) | 2 | 1+1 (2,11) | MCS 20 | MCS16 | 16.6 | 14.08 | 2.52 | 36.23 | 33.72 | 2.51 |

Based on above initial simulation results, the following observations can be achieved

Observation 7: As number of interferers increasing from 1 to 2, the achieved SNR@70% Throughput is increasing, while the achieved SINR @ 70%Throughput is decreasing

Observation 8:  For the high interference scenario in HetNet DIP set 2, the performance gain over MMSE is more compared with HomNet DIP set 1 scenario considering with and without the impact of HPUE.

Observation 9: The performance gain over MMSE receiver is similar in terms of both SNR-based requirement definition and SINR-based requirement definition

Observation 10:  For targeting cell with MCS 20 and TDLB100-400 channel condition, the achieved SNR is higher than 20dB, especially for high interference scenario HetNet DIP set 2 with 2Rx.

Observation 11:  For targeting cell with MCS 16, MCS 20 underTDLA30-10, TDLC300-100, TDLB100-400 channel condition, the achieved SNR is higher than 20dB, for both scenarios HomNet and HetNet and with 2Rx under the impact of HPUE

Based on our simulation results and above observations, to reduce the test effort, the following MCS and channel combination can be considered for specifying PUSCH requirement with MMSE-IRC receiver.

  - MCS 2 and TDLB400-100 for both HomNet and HetNet

  - MCS 16 and TDLC300-100 for both HomNet and HetNet

  - MCS 20 and TDLA30-10 for HomNet and HetNet

Proposal 13:  The following MCS and channel combination can be considered for specifying PUSCH requirement with MMSE-IRC receiver

  - MCS 2 and TDLB400-100 for both HomNet and HetNet

  - MCS 16 and TDLC300-100 for both HomNet and HetNet

  - MCS 20 and TDLA30-10 for HomNet and HetNet

# 5	Conclusion

In this contribution, the view on the BS demodulation requirement with MMSE-IRC receiver. And the initial simulation results are provided to check the performance gain over MMSE based on LTE interference profile.

Observation 1:  The requirement in HomNet (DIP set 1) is not applicable for local Area BS and Home BS in LTE system.

Observation 2:  Only three categories of BS class were defined in NR system as wide area base station, medium Range base station and local area range base station

Proposal 1:  Apply the similar approach for NR with a note to indicate the requirement in HomNet ( DIP set 1)  is not appliable for local Area BS.

Proposal 2: Introduce the test applicability rule for these two requirements (DIP set 1) testing. The following test applicability rule can be considered as

  - Unless other stated, if a BS declared to support [wide area] or [medium range] and pass the tests for DIP1, can also consider the test for DIP2 as passed

Proposal 3:  Consider artificially increasing INR with additional 7dB to account the impact of HPUE, the following INR value can be considered

| Scenario | (INR1, INR2) with considering HPUE |
| --- | --- |
| HetNet with 1 interferer | (16.82, N/A) |
| HetNet with 2 interferers | (19.38, 6.03) |
| HomNet with 1 interferer | (12.35, N/A) |
| HomNet with 2 interferers | (14.28, 4.48) |

Observation 3: The gain can be achieved with MMSE-IRC receiver compared with MMSE receiver for the 2 interferers modeling, under the interference profile with and without the impact of HPUE.

Observation 4: Considering the impact of HPUE, the gain of MMSE-IRC under 2 interferers is slightly smaller than that under 1 interferer scenario.

Proposal 4:  2 interferers can be considered for 2Rx to specify the PUSCH requirement with MMSE-IRC receiver

Proposal 5:  The following channel bandwidth can be considered to specify PUSCH requirement with MMSE-IRC receiver

- For CP-OFDM

  - 15kHz SCS, 5MHz, 10MHz

  - 30KHz SCS, 10MHz, 40MHz

- For DFT-s-OFDM

  - 15kHz SCS, 5MHz

  - 30KHz SCS, 10MHz

Proposal 6: Reusing the existing channel models to specify the PUSCH requirement with MMSE-IRC receiver

The following candidate propagation channel can be considered as initial evaluation

  - Targeting cell:  TDLA30-10, TDLC300-100 and TDLB100-400

  - Interference cell: TDLC300-100

Proposal 7: Consider MCS 16 for interfered UE, MCS 2, MCS 16 and MCS 20 as candidate for initial performance evaluation with MMSE-IRC receiver.

  - Targeting cell:  MCS 2, MCS 16 and MCS 20

  - Interference cell: MCS 16

For requirement definition, FFS to down select one MCS for requirement.

Proposal 8: For requirement definition, both CP-OFDM and DFT-s-OFDM waveforms can be considered to specify PUSCH requirement with MMSR-IRC receiver to guarantee the test coverage

Proposal 9: Only specify the PUSCH requirement with MMSR-IRC for DMRS type 1.

Proposal 10: Considering the following for DMRS configuration and cell ID to specify PUSCH requirement with MMSE-IRC receiver

|  | Targeting cell | Interferer 1 | Interferer 2 |
| --- | --- | --- | --- |
| DM-RS port | 0 | 0 | 0 |
| Cell ID | 0 | 1 | 2 |
| DM-RS sequence generation | NIDnSCIDλ=NIDcell nSCIDλ=0 λ=0 Group hopping and sequence hopping are disabled |  |  |

Observation 5: The SINR was applied for test requirement definition for PUSCH requirement with MMSE-IRC receiver in LTE

Observation 6: The SNR was applied for test requirement definition for PDSCH requirement with MMSE-IRC receiver in NR

Proposal 11: Slightly prefer to use the 70% of maximum throughput as metric for NR BS MMSE-IRC receiver to derive the SINR value. Open to discuss the SNR based metric.

Proposal 12: FFS on SINR based definition is needed and captured into the specification

Observation 7: As number of interferers increasing from 1 to 2, the achieved SNR@70% Throughput is increasing, while the achieved SINR @ 70%Throughput is decreasing

Observation 8:  For the high interference scenario in HetNet DIP set 2, the performance gain over MMSE is more compared with HomNet DIP set 1 scenario considering with and without the impact of HPUE.

Observation 9: The performance gain over MMSE receiver is similar in terms of both SNR-based requirement definition and SINR-based requirement definition

Observation 10:  For targeting cell with MCS 20 and TDLB100-400 channel condition, the achieved SNR is higher than 20dB, especially for high interference scenario HetNet DIP set 2 with 2Rx.

Observation 11:  For targeting cell with MCS 16, MCS 20 underTDLA30-10, TDLC300-100, TDLB100-400 channel condition, the achieved SNR is higher than 20dB, for both scenarios HomNet and HetNet and with 2Rx under the impact of HPUE

Proposal 13:  The following MCS and channel combination can be considered for specifying PUSCH requirement with MMSE-IRC receiver

  - MCS 2 and TDLB400-100 for both HomNet and HetNet

  - MCS 16 and TDLC300-100 for both HomNet and HetNet

  - MCS 20 and TDLA30-10 for HomNet and HetNet

# References

R4-2416548, WF on [112bis][319] NR_demod_Ph5_Part1_General_BS