3GPP TSG-RAN WG4 Meeting # 113	R4-2419093

# Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.17.2.1

Source: 	National Taiwan University

Title: 	On AI/ML Interoperability and Standardization of Testing and Verification for CSI Use Case – A PCA based approach

Document for:	Approval

Introduction

We present our view on partially specified decoder and reference decoder derivation procedure in this contribution.

Discussion

Discussion and Proposals for Option 4

Three sub-options for option 4 are listed in the previous meeting WF:

Option 4 sub-options: - 4a: Dataset based: specify at least dataset based on which the test decoder can be implemented by TE vendors FFS whether anything else needs to be specified (e.g. model structure for decoder or a reference encoder) - 4b: Reference encoder based: encoder is documented in the specifications, used to derive a decoder to be implemented by TE vendors FFS whether training dataset (channel information) needs to be specified - 4c: do not specify either reference encoder or dataset. specify enough constraints on the decoder output ordering

However, none of these options are complete enough for TE vendors to derive the test decoders that meet the test repeatability requirements. For example, in 4a, there are different ways to train the test decoders and the achieved loss functions from the specified dataset can vary a lot, and we still don’t have any test repeatability guarantee across different test decoders. Similarly, the joint training with reference encoder and yield different test decoders without guaranteed test repeatability. Therefore, additional requirements/specifications are needed for dataset based and reference encoder-based approach. We provide the suggestions for further specifications for each option in the following:

Dataset based approach 4a: we can specify a loss/similarity function of the test decoder output and the specified dataset for a given input as follows

z∈Zfgref(z),gtest(z)<δ,

where Z is the set of decoder inputs (latent messages) in the specified dataset, and f is the chosen loss/similarity function to evaluate the difference between the decoder output from the dataset corresponding to the latent message z, gref(z), and the test decoder output from z, gtest(z).

The drawback of the approach is that both the test decoder (and implicitly the DUT encoder) are heavily restricted by the dataset due to the specified constraint, and we don’t expect that the DUT encoder can work in practice unless in an environment that is very closely resembled with the specified dataset. In fact, the purpose of the test is, as agreed, “verify whether the minimum performance of AI/ML functionality/feature can be achieved”. But we don’t expect such dataset shared between the UE vendors and infra vendors is available for all the application scenarios, and therefore the performance achieved here is definitely not the minimum performance we can expect in the field. Alternatively, if we consider the approach proposed in the previous meeting (and included also in section 2.2), we can derive a more flexible requirement that can enable both TE vendors and UE vendors to develop decoders and encoders that can be trained by different datasets than the specified one. Such a flexibility can get us closer to the agreed test goal, since we allow the encoders and decoders to be trained by other dataset as long as the specified requirement in the form of latent message/decoder output ordering is fulfilled. To be more specified, the alternative requirement is:

gref(z)-gtest(z)=miny∈Ytestgref(z)-y for all z∈Z

where Ytest is the test decoder output space.

Similarly, we can add the requirement for option 4b as

x∈Xfgref(z(x)),gtest(z(x))<δ,

where z(x) is the reference encoder output (latent message) of encoder input x, and X is the sampled encoder input during test decoder verification procedures. Therefore, we also need to specified the generation procedures of X.

Alternatively, based on the proposal in section 2.2, we can specify the requirements of the test decoder as

gref(z)-gtest(z)=miny∈Ytestgref(z)-y for all z∈Z

where Ytest is the test decoder output space and Z is the reference encoder output space.

In summary, we have the following proposal:

Proposal 1: Next steps for option 4a and 4b feasibility study:

- Reuse the decoder/encoder pair derived in option 3 as the reference pair

- Use the reference pair to generate dataset, or reuse any agreed dataset in option 3 discussion for option 4a, and use the encoder in the reference pair as the reference encoder for option 4b

- Define the decoder verification criterion from one of the following options (for 4a and 4b, respectively):

(Option 4a: specify dataset)

- 4a-1: RAN4 derive the threshold δ via simulation study (setup following option 3 discussion), and check whether it’s feasible to derive a test decoder satisfying

z∈Zfgref(z),gtest(z)<δ,

where Z is the set of decoder inputs (latent messages) in the specified dataset (from step 2), and f is the chosen loss/similarity function gref(z) is the test decoder output and gtest(z) is from the dataset.

- 4a-2: RAN4 studies whether it is feasible to derive a test decoder satisfying

gref(z)-gtest(z)=miny∈Ytestgref(z)-y for all z∈Z

where the notations are the same as 4a-1, and Ytest is the test decoder output space.

(Option 4b: specify reference encoder)

- 4b-1: RAN4 derive the threshold δ and the encoder input X generation procedure via simulation study (setup following option 3 discussion), and check whether it’s feasible to derive a test decoder satisfying

x∈Xfgref(z(x)),gtest(z(x))<δ,

where z(x) is the reference encoder output (latent message) of encoder input x, and X is the sampled encoder input in the test decoder verification procedures. Note that RAN4 needs to specified the generation procedures of X and specifying

- 4b-2: RAN4 studies whether it is feasible to derive a test decoder satisfying

gref(z)-gtest(z)=miny∈Ytestgref(z)-y for all z∈Z

where Ytest is the test decoder output space and Z is the reference encoder output space.

- RAN4 check if the decoders derived from the above procedure can satisfy test repeatability requirement, i.e., the loss function delta between two different decoders when connecting to the same encoder, is within a RAN4 agreed margin. If yes, we can conclude that option 4 (at least one sub-option) is feasible.

Observation 1: Option 4a-1 and 4b-1 require both TE vendors and UE vendors to train the test decoder and DUT encoder with the dataset or reference encoder, while option 4a-2 and 4b-2 allow TE vendors and UE vendors to train the test decoder and DUT encoder with arbitrary dataset or encoders/decoders as long as the specified ordering requirements are satisfied.

New Proposal for Partially Specified Decoder

Based on the summary provided in Table 3 in [1], the options proposed to derive/specify partially specified decoder rely on a common dataset and/or a reference encoder/decoder pair. Those approaches can possibly achieve test repeatability, which is very important for standardized test and can provide good guidance for interoperability, but they also impose many limitations on encoder/decoder implementation since the training/design of the test decoder or DUT encoder is based on the reference encoder/decoder pair (or the data generated by the pair).

Such limitation may reduce the potential gain of using AI/ML models, or implicitly leads to a specialized DUT encoder for RAN4 test that deviates a lot from the encoders used in the field. Therefore, we want to carefully examine the mathematical structure of the CSI encoding/decoding function, and propose a new approach for partially specified decoder with much lower limitation on test decoder and DUT encoder limitation.

When we consider the complete CSI information, e.g, the precoder from SVD, as the input to the encoder, the encoder is a mapping between the precoder (from a real-value vector space) and the corresponding latent message (from a finite set of numbers); and the decoder is a mapping between the latent message (from a finite set of numbers) to the recovered precoder (from a real-value vector space but there are finite number of recovered precoders, limited by the size of the latent message set). When the encoder and decoder are jointly trained or trained by a common dataset, at least within the train dataset, we expect a recovered precoder is very close to the expectation of the subspace of the precoder that is mapped to the latent message corresponding to the recovered precoder. However, when the decoder and encoder are trained by different entities, i.e., we get the encoder from a jointly trained pair (pair A), and get the decoder from another pair (pair B), these pairs may not have aligned precoder subspaces to generate the latent message by the encoder. Therefore, even each pair have the optimal recovered precoder for each of their precoder subspace, the recovered precoder from the decoder in pair B may not be the optimal recovered precoder when the latent message is encoded by the encoder in pair A since the subspaces in pair A and B are different. The even worse scenario is that the decoder may map the latent message to a recovered precoder very far away from the subspace of precoders mapped to the latent message by the encoder if the same latent message are mapped to two very far apart subspaces by the two encoders (from pair A and B).

We illustrate the two types of errors in the following figure. When the pairs are jointly trained, like ENC-A and DEC-A, ENC-B and DEC-B, ENC-C and DEC-C, we expect the recovered precoder is very close to the (probability weighted) center of the subspace of the precoder linked by the corresponding latent message (sub-figure 1, 2, 3). However, when we connect the encoders and decoders trained separately, as explained above, the recovered messages are not ideal. The first type of error due to subspace boundary misalignment from different models are illustrated in sub-figure 4, when we connect ENC-A to DEC-B. We can see that the recovered precoder (e.g., the dark red dot) is no longer at the probability center of the precoder subspace linked by the corresponding latent message (e.g., the dark green circle), but is still very close to it and might be within the precoder subspace. However, the second type of error due to mapping order difference to the latent message, as illustrated in sub-figure 5 (connecting ENC-A to DEC-C), is very harmful. We can observe that the dark green precoder subspace is mapped to the light yellow recovered precoder through the dark blue latent message instead of the dark yellow recovered precoder, which is much closer to the dark green precoder subspace. Now the recovered precoder is very far away from the precoder space it is representing, and therefore we expect to have a very bad performance. If we can ensure that DEC-A and DEC-C are (to some extent) consistent: any latent message is mapped to close by recovered precoders by DEC-A and DEC-C, we can prevent the second type of error.

ENC-A

DEC-D

5

ENC-A

DEC-B

4

ENC-C

DEC-C

3

ENC-B

DEC-B

2

ENC-A

DEC-A

1

In our proposal, we want to first resolve the second issue illustrated in the previous paragraph: even if different pairs trained separately may have subspaces that are not completely aligned, we want to propose a set of decoder specifications that can guarantee nearby subspaces from different pairs mapping to the same latent message. Therefore, we can eliminate the mismatch due to far apart subspaces mapped to the same latent message, and the residual mismatch are only from subspace mismatches between different pairs.

We first formulate the mathematical problem corresponding to the above requirement in the following. First, we assume the encoder input space X is common across all the encoders, and use the following mapping to describe two encoder and decoder pairs:

Pair 0:X→Z:Z=z0,…,zk, X0=x0,0,…,x0,k

Pair 1:X→Z:Zz0,…,zk,X1=x1,0,…,x1,k

Note that zm→x0,m in pair 0 and zm→x1,m in pair 1.

We want to have any two pairs of encoder and decoder satisfy the following property: for any z∈Z, denote decoder output of pair 0: z→x0 and pair 1: z→x1, the decoders has to satisfy: x0-x1=minx∈X1x0-x. To fit in the test decoder standardization framework, we can consider pair 0 as the reference encoder/decoder pair, and we require the pair 1 (with the test decoder) satisfying the constraint listed above.

We provide a novel approach below to guide the derivation of the test decoder satisfying the above constraints. We can start with a reference decoder, or any mapping between latent messages and the recovered precoders, e.g., e-Type II precoding can be an option. Denote this mapping by

Z→Xref, Z:z0,…,zk, Xref:xref,0,…,xref,k

Any test decoder, Z: z0,…,zk→X: x0,…,xk, has to follow the constraints below

x0=argminXxref,0-x

x1=argminX\x0xref,1-x…

xk-1=argminX\x0,…,xk-2xref,k-1-x

xk-1=X\x0,…,xk-2,xk-1

The above approach can be considered as a greedy algorithm to sort a achieve the proposed test decoder requirements: x0 is guaranteed to satisfy the proposed requirement, but if x0=minXxref,1-x, we can’t guarantee that x1 satisfies the proposed requirements. Based on the above observation, we can conclude that the greedy algorithm can satisfy the requirements under an additional constraint:

argminx∈X1xn-x≠argminx∈X1xm-x

For any n≠m.

If an encoder input dataset is given, we can derive the latent message ordering in the following steps.

Step 1: Perform principle component analysis (PCA) on encoder input dataset, i.e., derive the SVD of the covariance matrix of the encoder input dataset.

Step 2: Select the eigenvectors corresponding to the M largest eigenvalues as dominant eigenvectors.

Step 3: Derive the coefficients of each of the decoder output on the (incomplete) basis composed of the selected M eigenvectors, i.e., projecting each decoder output on M eigenvectors to derive the coefficients

Step 4: Suppose the latent message length is N. Distribute the N bits to represent the coefficients of the M eigenvalues proportional to the magnitude of the corresponding eigenvalues, e.g, if eigenvalue 1 is 15, eigenvalue 2 is 5, when eigenvalue 1 gets 2 bits, eigenvalue 2 gets 1 bit.

Step 5: Quantize the coefficients to form the latent message corresponding to each decoder output. The mapping from the new latent message from these steps to the original latent message before reordering is added before decoder input. The mapping from the original encoder output to the new latent message from these latent messages is added to the encoder output.

The encoder/decoder pair with the additional mappings added in step 5 will be compatible with other encoder/decoder pairs (derived following the same procedures).

Proposal 2: Next steps for feasibility study of option 4c

        - Use the encoder input part of the dataset from option 3 feasibility study, derive the sample covariance matrix, and use principle component analysis to derive eigenvectors and eigenvalues.

        - RAN4 decides how many dominant eigenvectors, and the number of quantization bits for the coefficients of eigenvectors (sum to the number of bits in the latent message) to include in indexing procedure in proposal 3.

        - Companies propose any encoder-decoder pair, with the latent message reordering based on the procedures in proposal 3 and the dominant eigenvectors decided in step 1

        - RAN4 check if the decoders derived from the above procedure can satisfy test repeatability requirement, i.e., the loss function delta between two different decoders when connecting to the same encoder (also following the latent message reordering procedure), is within a RAN4 agreed margin. If yes, we can conclude that option 4c is feasible.

Proposal 3: The latent message reordering procedure is in the following:

Step 1: Perform principle component analysis (PCA) on encoder input dataset, i.e., derive the SVD of the covariance matrix of the encoder input dataset.

Step 2: Select the eigenvectors corresponding to the M largest eigenvalues as dominant eigenvectors.

Step 3: Perform Gram-Schmidt process on each of the decoder output with the dominant eigenvectors as basis.

Step 4: Suppose the latent message length is N. Distribute the N bits to represent the coefficients of the M eigenvalues proportional to the magnitude of the corresponding eigenvalues.

Step 5: Quantize the coefficients to form the latent message corresponding to each decoder output. The mapping from the new latent message from these steps to the original latent message before reordering is added before decoder input. The mapping from the original encoder output to the new latent message from these latent messages is added to the encoder output.

The above approach, if proven to be feasible, has the following advantages:

Observation 2: The advantages of partial mapping alignment approach for partial specified test decoder option are:

- A standardized training data free approach: 3GPP standard doesn’t have to capture the training data.

- If the reference decoder can be described analytically with a set of equations, e.g., e-Type II CSI, RAN4 doesn’t have to agree a reference encoder/decoder pair to derive the test decoders.

- Best forward compatible: either dataset or reference encoder/decoder pair approaches require UE vendors to train the decoder based on the encoder or dataset from a specific model designed based on the currently available AI/ML model structures. When new and superior models are developed by AI research communities, the dataset or reference encoder may limit the implementation flexibility of the UE vendors to adopt the new models.

Since we don’t require the test decoder output matching exactly to the reference decoder output, we need take the mismatches between the test decoder, the reference decoder, and the decoder UE vendors training their encoders into consideration. However, the constraint in proposal 1 significantly reduces the mismatches between the decoders by ensuring that the decoder output mapped from the same latent message are the closest within the decoder output space.

Proposal 4: Additional margin to account for the mismatch xref(z)-xtest(z)=minx∈Xtestxref(z)-x need to be considered when determining requirement.

Test Decoder Selection Criterion

We summarize the potential criterions for test decoder selection below:

- Achievable performance metrics: SGCS or NMSE

- Complexity: including flops or model storage size, can consider to set an upper bound

- Robustness (based on performance metrics): whether the decoder can achieve satisfactory performance when connecting to encoder with different structures with the decoder, but trained with the decoder’s input and output dataset

Given the agreements on decoder structure, complexity is mostly bounded, and therefore we may not need to consider it. Most of discussions so far were focusing on the achievable performance as criterion. However, in the agreed procedure, after selected a decoder, the next step is:

Each company brings results for training of “own encoder” with selected decoder(s)

  - performance alignment to be checked/discussed

  - using “own” data or data shared by other companies

in which we need to check the performance alignment across the encoders trained by a common dataset (as an option), similar to the robustness mentioned above. In order to achieve performance alignment, it is preferred to select a decoder that achieves similar performance when connecting to different encoders with different structures, e.g., the structure options discussed previously for the test decoder, but trained with the same dataset produced by the decoder, i.e., the decoder input and output dataset. Otherwise, even if a decoder can achieve a better performance when connected to the jointly trained encoder, that decoder might fail to achieve performance alignment in the agreed next step, and RAN4 has to reselect the decoder. Based on the above analysis, we have the following proposal:

Proposal 4: Consider the test decoder selection criterion based on the agreed step, i.e., check whether performance alignment across contributing companies’ “own encoder”, which may have different structures, can be achieved. 
Therefore, companies can bring the test decoder proposal together with performance metrics achieved when connecting to different encoders with different structures, e.g., the structure options for test decoder discussion, but trained with the decoder input and output dataset from the proposed decoder.

Conclusion

Proposal 1: Next steps for option 4a and 4b feasibility study:

        - Reuse the decoder/encoder pair derived in option 3 as the reference pair

        - Use the reference pair to generate dataset, or reuse any agreed dataset in option 3 discussion for option 4a, and use the encoder in the reference pair as the reference encoder for option 4b

        - Define the decoder verification criterion from one of the following options (for 4a and 4b, respectively):

(Option 4a: specify dataset)

- 4a-1: RAN4 derive the threshold δ via simulation study (setup following option 3 discussion), and check whether it’s feasible to derive a test decoder satisfying

z∈Zfgref(z),gtest(z)<δ,

where Z is the set of decoder inputs (latent messages) in the specified dataset (from step 2), and f is the chosen loss/similarity function gref(z) is the test decoder output and gtest(z) is from the dataset.

- 4a-2: RAN4 studies whether it is feasible to derive a test decoder satisfying

gref(z)-gtest(z)=miny∈Ytestgref(z)-y for all z∈Z

where the notations are the same as 4a-1, and Ytest is the test decoder output space.

(Option 4b: specify reference encoder)

- 4b-1: RAN4 derive the threshold δ and the encoder input X generation procedure via simulation study (setup following option 3 discussion), and check whether it’s feasible to derive a test decoder satisfying

x∈Xfgref(z(x)),gtest(z(x))<δ,

where z(x) is the reference encoder output (latent message) of encoder input x, and X is the sampled encoder input in the test decoder verification procedures. Note that RAN4 needs to specified the generation procedures of X and specifying

- 4b-2: RAN4 studies whether it is feasible to derive a test decoder satisfying

gref(z)-gtest(z)=miny∈Ytestgref(z)-y for all z∈Z

where Ytest is the test decoder output space and Z is the reference encoder output space.

        - RAN4 check if the decoders derived from the above procedure can satisfy test repeatability requirement, i.e., the loss function delta between two different decoders when connecting to the same encoder, is within a RAN4 agreed margin. If yes, we can conclude that option 4 (at least one sub-option) is feasible.

Proposal 2: Next steps for feasibility study of option 4c

      - Use the encoder input part of the dataset from option 3 feasibility study, derive the sample covariance matrix, and use principle component analysis to derive eigenvectors and eigenvalues.

      - RAN4 decides how many dominant eigenvectors, and the number of quantization bits for the coefficients of eigenvectors (sum to the number of bits in the latent message) to include in indexing procedure in proposal 3.

      - Companies propose any encoder-decoder pair, with the latent message reordering based on the procedures in proposal 3 and the dominant eigenvectors decided in step 1

      - RAN4 check if the decoders derived from the above procedure can satisfy test repeatability requirement, i.e., the loss function delta between two different decoders when connecting to the same encoder (also following the latent message reordering procedure), is within a RAN4 agreed margin. If yes, we can conclude that option 4c is feasible.

Proposal 3: The latent message reordering procedure is in the following:

Step 1: Perform principle component analysis (PCA) on encoder input dataset, i.e., derive the SVD of the covariance matrix of the encoder input dataset.

Step 2: Select the eigenvectors corresponding to the M largest eigenvalues as dominant eigenvectors.

Step 3: Derive the coefficients of each of the decoder output on the (incomplete) basis composed of the selected M eigenvectors.

Step 4: Suppose the latent message length is N. Distribute the N bits to represent the coefficients of the M eigenvalues proportional to the magnitude of the corresponding eigenvalues.

Step 5: Quantize the coefficients to form the latent message corresponding to each decoder output. The mapping from the new latent message from these steps to the original latent message before reordering is added before decoder input. The mapping from the original encoder output to the new latent message from these latent messages is added to the encoder output.

Proposal 4: Additional margin to account for the mismatch xref(z)-xtest(z)=minx∈Xtestxref(z)-x need to be considered when determining requirement.

Proposal 5: Consider the test decoder selection criterion based on the agreed step, i.e., check whether performance alignment across contributing companies’ “own encoder”, which may have different structures, can be achieved. 
Therefore, companies can bring the test decoder proposal together with performance metrics achieved when connecting to different encoders with different structures, e.g., the structure options for test decoder discussion, but trained with the decoder input and output dataset from the proposed decoder.

Reference

[1] R4-2407334